3GPP TSG-RAN WG2 Meeting #131-bis                                                             R2-2506942

Prague, Czech Republic Oct 13th – 17th, 2025

Source:	CATT

Title:	Discussion on remaining issues for ambient IoT

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

Based on the progress of RAN2#131 [1] and the guidance of open issue list [2], this contribution further discusses the remaining open issues suggested by the rapporteur. In details, the following aspects are covered by our contribution,

- Potential impact from SA3 LS

- Potential impact from SA2 LS

- No response of upper layer command

Section 2 gives the detailed discussion, and followed by the Section 3 that summarizes the conclusion of this paper.

2. Discussion

## 2.1 Potential impact from SA3 LS (Issue 1-7)

Regarding security parameters in paging message, there is the following open issue captured in RAN2#131 post email discussion [2]:

| Subgroup: Paging message content |  |  |
| --- | --- | --- |
| Issue 1-7: Security parameter | How to include the security parameters in Paging message. - Based on SA3 LS in S3-252392, it’s understood that security is a mandatory feature to be supported by the Rel-19 devices. But companies also raised question during online discussion whether the security parameters can be optional, for instance the feature can be activated/controlled by network. And companies also mentioned that the signaling overhead of the current paging message design is an important aspect from RAN2 point of view. Then how to capture the security parameter has been postponed and expected more discussion in Oct meeting. In this case, this issue is classified as “to be discussed by contribution”, and the rapporteur would like to suggest companies also to consider more from RAN2 perspective when designing signaling in their contributions, in terms of e.g., signaling compatibility, flexibility, signaling overhead, and scalability. - SA3 provided more information about the security design in LS S3-252933 in Aug meeting, which should be take into account. - Relevant agreements: - RAN2 thinks it is feasible from a signalling perspective to add the 128 bits. However, from RAN2 perspective the less overhead the better, so SA3 should avoid adding additional parameters if possible. - Indicate to SA3 that RAN2 tries to minimize number of bits required. Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG. - RAN2 will wait for SA3 conclusions in October on whether the “128bit random number in the paging request message” is always required or not. - Reply to SA3 (The LS is approved in R2-2506465) - RAN2 thinks it is feasible from a signalling perspective to add the 128 bits. However, from RAN2 perspective the less overhead the better, so SA3 should avoid adding additional parameters if possible. - Indicate to SA3 that RAN2 tries to minimize number of bits required. Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG. - Indicate space pressure from all the WG Status in running CR: not captured yet | To be discussed by company contributions |

According to SA3 TS 33.369 [3],

SA3 TS 33.369-030 5.2.2 Authentication procedure The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2]. Figure 5.2.1-1: Authentication procedure 0. Step 1-6 of clause 6.2.2 Procedure for Inventory or clause 6.2.3 Procedure for command in TS 23.369 [2] is performed. 1. ADM shall generate RANDAIOT_n. AIOTF shall retrieve RANDAIOT_n from ADM. 2. AIOTF shall send inventory request message including RANDAIOT_n to NG-RAN. 3. NG-RAN shall include RANDAIOT_n in the paging request message to the AIoT device in addition to other device identification information. … 4. Upon receiving the paging request message, if the device determines it needs to respond based on the device identification information, AIoT device shall generate RANDAIOT_d, calculate RESAIOT using KAIoT_root and RANDAIOT_n (see Annex A.2) for network authenticating AIoT Device. …

In any case of the inventory or command procedure, the device will use the RANDAIOT_n to calculate RESAIOT using KAIoT_root and RANDAIOT_n for network authenticating AIoT Device. Besides, CT1 TS 24.368 [4] also captured the following descriptions on security parameter in the A-IoT Paging message:

CT1 TS 24.368-100 4.2.2 Authentication Authentication of the AIoT device is done in the inventory procedure. A network generated RANDAIOT_n is provided by the AIOTF and received by the AIoT device in the paging request. The AIoT device which is target of the paging request (see clause 5.2.3), uses the RANDAIOT_n and its long-term KAIOT to derive a RESAIOT and includes the RESAIOT and a RANDAIOT_d in the INVENTORY REPORT message to the AIOTF. The network verifies the RESAIOT to authenticate the AIoT device.

So the 128-bit security parameter RANDAIOT_n should be mandatory present in the A-IoT paging message.

Proposal 1:  (Issue 1-7) 128-bit security parameter is mandatory in the A-IoT Paging message.

Reagrding the temporary ID in the A-IoT Paging message in case of single device inventory, SA3 TS 33.369 captured the following description:

SA3 TS 33.369-030 ### 5.4.3 Procedure for AIoT Device identifier protection with Temp ID update during Individual inventory For the protection of AIoT device permanent identifier during the inventory procedure with AIoT device identifier described in clause 5.2.2, the following changes shall apply: … - In step 2 and 3 the AIOTF includes indication of type of T-ID handling. T-ID can be either concealed type or stored type. The concealed type can be based on either the stored T-ID or the permanent identifier. If needed the handling also indicates whether the stored T-ID type shall be updated with or without a command. NG-RAN includes the T-ID handling in the paging message. - In step 4, the AIoT device, based on the T-ID handling indication in the paging message, generates the T-ID in the same way as the ADM did in step 1. The AIoT device determines it needs to reply to the NG-RAN if the generated T-ID matches with the received T-ID. In case the stored T-ID update shall be done without a command, the AIoT Device generates a new Temp_ID_n+1 as specified in Annex B.1 and stores the new Temp ID_n+1.

We can see for the single device inventory case, an indication needs to be included in the A-IoT Paging message to enable the device to generate a temporary ID, and this temporary ID is used to match the received T-ID.

Proposal 2a: (Issue 1-7) For the case of single device inventory, 1-bit T-ID handling indication should be introduced in the paging message.

Then another issue is how to capture this 1-bit indication in the current paging format. Since this 1-bit T-ID indication is only applied for single device inventory, it is straightfoward to make it optional in the paging message, i.e., additional 1-bit to indicate the presence of the T-ID handling indication field.

That means the reader determines the single device inventory request from NG message, then the reader further set the T-ID handling indication field to 0 or 1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Device Identification for Paging | M |  |  |  |
| >Single Device |  |  |  |  |
| >>Single Device Identity | M |  | BIT STRING (SIZE(1..1024, ...)) | Includes the AIoT device Identifier defined in TS 23.369 [z]. |
| >Group Devices |  |  |  |  |
| >>Group Device Identity | M |  | BIT STRING (SIZE(1..1024, ...)) | Includes the Filtering Information defined in TS 23.369 [z]. |
| >All Devices |  |  | NULL |  |

From the device perspective, benefit from this optional feature, the device can determine the device ID information is for single device case if the T-ID handling indication field is present and then determine whether to generate the temporary ID to match this T-ID in the paging.

Based on above analysis, it is further proposed that,

Proposal 2b: (Issue 1-7) The 1-bit T-ID handling indication field is optionally present in the paging message. Introduce additional 1-bit to indicate the presence of this T-ID handling indication field.

## 2.2 Potential impact from SA2 LS (Issue 1-3)

In RAN2#131 post email discussion, there is the following open issue on paging ID length field:

| Subgroup: Paging message content |  |  |
| --- | --- | --- |
| Issue 1-3: Paging ID length field | The field to indicate the paging ID length, e.g., how many bits, taking into account of CT4 and SA2 inputs. - Relevant agreements: - 8-bit length field (in unit of bit) is assumed to indicate the paging ID length, based on current SA2/CT4 conclusion. - However, in Aug meeting, SA2 sent a new LS to RAN2 in S2-2507793 on AIoT Device Permanent ID Length, indicating SA2 identified requirements for longer device ID, and the rapporteur understand this will impact the length of paging ID as well, e.g., 600 bits are needed instead of 256 bits. Therefore, in Oct meeting, RAN2 needs to discuss how to reply the SA2 LS, and how to handle the impact to paging ID length field. Status in running CR: the field name is captured in 6.2.1.1 with the detailed format. | To be discussed by company contributions |

SA2 sent LS to RAN2 [5] about AIoT device permanent ID length, and SA2 discussed extending the length of the AIoT device permanent ID (i.e., the identification information part) to 256-bit and 496-bit. SA2 also roughly estimated that the filtering information will be around 600-bit in case the identification information is 496-bit, and around 360-bit in case the identification information is 256-bit. The first impact to RAN2 is we need to change the paging ID length field from 8-bit to 10-bit to cover this case of around 600-bit filtering information as the paging ID.

Observation 1: If the device identification formation is extended to maximum 496-bit, one potential impact on RAN2 is to change the paging ID length field from 8-bit to 10-bit to cover this case of around 600-bit filtering information as the paging ID.

Then we can take the filtering information for example as it is the largest bit-consuming (up to 596-bit for single device ID case) to check whether it is feasible to do this extension. According to the paging format for CBRA,

| Field | Remarks | Field | Remarks |
| --- | --- | --- | --- |
| R2D msg type | 3-bit | Paging ID | Maximum around 600-bit for filtering information |
| TBS | 7-bit | Number of access occasions | 4-bit |
| CBRA indication | 1-bit | D2R scheduling information | 28-bit |
| Transaction ID | 6-bit | K value | 1-bit |
| Paging ID Present Indication | 1-bit | Security parameters (Our proposal 1) | 128-bit |
| Paging ID length | Change 8-bit to 10-bit | Present indication of T-ID handling indication | 1-bit |
| Fill bits | X | T-ID handling indication | 1-bit |

As summarized in above table, the maximum paging content may be around 800-bit, considering the filtering information for the device ID. For now the maximum R2D TBS is 125-byte, so it can ensure based on the reader implementation to not perform segmentation for the R2D paging message. Therefore, it is suggested relaying LS to SA2 that it is feasible from RAN2 perspective to extend the AIoT device permanent ID to 256-bit and 496-bit.

Proposal 3a: (Issue 1-3) Reply LS to SA2 that it is feasible to extend the AIoT device permanent ID to 256-bit and 496-bit.

Proposal 3b: (Issue 1-3) Change the paging ID length field from 8-bit to 10-bit to cover this case of around 600-bit filtering information as the paging ID.

## 2.3 No response of upper layer command (Issue 3-7)

There is also an open issue about whether there is new case for “no upper layer data respnse” other than long writing operation.

| Subgroup: D2R message content for data transmission |  |  |
| --- | --- | --- |
| Issue 3-7: more data indication handling | How to set “more data indication” value in case of no NAS response available (i.e., zero SDU) - Relevant agreements: - The reader determines no data available case by SDU length 0. As more data indication is mandatory, the device sets this bit to "0". - The reader, in response to 0 SDU in the device’s MAC response, may send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device. - The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command. - RAN2 would like to check if there is a case where NAS doesn’t provide a response at all. If this case exists, RAN2 will discuss this issue. - Regarding the above check point, it can be up to companies’ contribution. Status in running CR: captured. | To be discussed by company contributions |

According to the progress on CT1, for read/write/permanent disable Command, there is always the D2R response from the device. Therefore, we proposed that,

Proposal 4: (Issue 3-7) RAN2 to confirm no other use case will lead to a no response or delayed response than the write command. No further MAC spec impact.

3. Conclusion

Based on the analysis in the Section 2, the conclusions of this paper are summarized as follows:

Observation 1: If the device identification formation is extended to maximum 496-bit, one potential impact on RAN2 is to change the paging ID length field from 8-bit to 10-bit to cover this case of around 600-bit filtering information as the paging ID.

Potential impact from SA3 LS (Issue 1-7)

Proposal 1:  (Issue 1-7) 128-bit security parameter is mandatory in the A-IoT Paging message.

Proposal 2a: (Issue 1-7) For the case of single device inventory, 1-bit T-ID handling indication should be introduced in the paging message.

Proposal 2b: (Issue 1-7) The 1-bit T-ID handling indication field is optionally present in the paging message. Introduce additional 1-bit to indicate the presence of this T-ID handling indication field.

Potential impact from SA2 LS (Issue 1-3)

Proposal 3a: (Issue 1-3) Reply LS to SA2 that it is feasible to extend the AIoT device permanent ID to 256-bit and 496-bit.

Proposal 3b: (Issue 1-3) Change the paging ID length field from 8-bit to 10-bit to cover this case of around 600-bit filtering information as the paging ID.

No response of upper layer command (Issue 3-7)

Proposal 4: (Issue 3-7) RAN2 to confirm no other use case will lead to a no response or delayed response than the write command. No further MAC spec impact.

4. References

[1]	RAN2#131 Chair’s notes.

[2]	Summary of A-IoT MAC open issues

[3]	TS 33.369-030   Security aspects of Ambient IoT service for isolated private networks SA3

[4]  TS 24.368-100   Ambient IoT Non-Access-Stratum (AIoT NAS) protocol for 5G System (5GS) CT1

[5]	S2-2507793   LS on AIoT Device Permanent ID Length