3GPP TSG-RAN WG2 Meeting #131bis	R2-2506949

# Prague, Czech Republic, October 13 – 17, 2025

Agenda item:	10.2

Source:	Samsung, Verizon

Title:	General aspects for 6G Radio protocol

Document for:	Discussion

1	Introduction

This paper is to discuss general aspects for designing 6G Radio protocols based on the objectives in RAN SID on 6G Radio [1]. It includes high level design principles, deployment scenarios and RAN architectures, radio interface protocol architecture and procedures, 5G to 6G migration, and UE capability signalling framework.

2	Discussion

2.1		Design principle for 6G Radio Protocol

The study should aim to develop a single technology framework for 6G Radio (6GR) to support both existing and new services, fulfilling the deployment scenarios and requirements identified in the RAN study on 6G scenarios and requirements [2]. This framework should be built on a stand-alone architecture that ensures an acceptable performance and complexity trade-off.

The key principles for 6GR protocol can be outlined as follows. These principles aim to establish a foundation for 6G networks that deliver user-centric innovative services while meeting environmental, technological, and security demands.

- Enhancing user experience and improving performance require minimizing connection setup latency and handover interruption delay, which are particularly critical for interactive and immersive services.

  - Handover framework enhancement:

    - While 5G has introduced numerous advanced mobility features—such as conditional handover, dual active protocol stack (DAPS), and L1/L2-triggered mobility (LTM)—only a subset of these features have been widely deployed in commercial networks. RAN2 study on 6G handover framework could start from existing 5G NR handover mechanisms to evaluate and identify essential and practical features. For 6G system, the handover framework can be advanced to minimize mobility interruption time to support a seamless application performance, ensure robustness, and reduce configuration complexity.

  - Optimization of RRC_INACTIVE mobility:

    - In 5G NR, RRC_INACTIVE mobility (i.e., RAN paging) and RRC_IDLE mobility (i.e., CN paging) are specified, with these two mobility functions are almost duplicated. The latency associated with UE context setup at the base station for RRC_INACTIVE UE is typically shorter compared to that for RRC_IDLE UE, enabling faster RRC resumption relative to RRC_IDLE. Additionally RRC_INACTIVE state allows UE to save more power than the power saving mechanisms available in RRC_CONNECTED. For 6G system, RRC_INACTIVE mobility can be specified as a default function without involving CN-based paging. By leveraging fast UE context retrieval at RAN level, RRC_INACTIVE mobility can be further optimized compared to that in 5G NR to significantly reduce connection setup latency.

  - AI/ML based mobility enhancements:

    - AI/ML framework can be integrated to enhance mobility management. 5G NR works on AL/ML mobility and measurement functionalities can be starting point for AI/ML based 6G mobility optimization.

Observation 1: Advancements in handover framework, RRC_INACTIVE mobility optimization, and AI/ML integration can contribute to achieving 6G’s latency and reliability requirements for interactive and immersive applications.

- Radio interface designed for environmental sustainability involves implementing energy saving mechanisms to maximize energy conservation for both the UE and the network.

  - Coordinated UE and NW energy saving:

    - Network energy saving functionalities were introduced as part of 5G-Advanced technology. However the NES functionalities must not impact legacy UEs, which limits the ability of 5G NR NES to maximize energy saving. In 6G system, UE power saving mechanisms such as DRX and network energy saving mechanisms should be defined in coordinated manner from the beginning of 6G to fully exploit opportunities for cell DTX/DRX.

  - Energy-oriented initial access and dynamic carrier(cell) on/off:

    - 6G system should support an energy efficient initial access mechanism. To conserve network energy, it is unnecessary for every carrier to remain continuously active or to transmit always-on signals with the same short periodicity. The 6G system can operate with differentiated carrier types based on usage scenarios, such that anchor carrier transmits always-on signals to facilitate UE system access and non-anchor carrier can be only activated as needed for high data rate services and deactivated when not in use. Additionally, even for the anchor carrier, always-on signals (e.g., synchronization signals) could be transmitted with a longer periodicity than in 5G NR, provided that UE performance, such as access latency, is not compromised.

  - Energy efficient system information message transmission/reception:

    - In the design of 6G systems, the periodic broadcast of system information should be avoided, and the on-demand system information acquisition functionality introduced in 5G NR should be adopted. Further energy saving benefits can be achieved in 6GR through advanced mechanisms such as spatial system information message transmission instead of broadcasting on-demand system information across all beam directions, and enhanced system information update procedures, including finer granularity of SI change notifications and avoiding the transmission and reception of unchanged or common information in multiple cells, and SIB-based SI request mechanism.

Observation 2: 6G systems should integrate coordinated UE and network energy-saving mechanisms, differentiate carrier operations, and optimize system information transmission to maximize energy efficiency.

- Implementation-friendly and simplified User Plane protocol design requires addressing the challenges posed by the misalignment between the pre-processing capacity leveraged by fast hardware processing power and some of existing 5G NR user plane protocol. This is particularly critical when targeting the ultra-high data rates expected in 6G.

  - Study on 6G User Plane protocols and architecture:

    - To maximize the potential of fast hardware processing power and support immersive services in 6G, enhanced packetization mechanisms such as concatenation or segmentation can be leveraged. RAN2 can study mechanisms for pre-concatenation to reduce overhead associated with AS layer header and AS layer security processing. RAN2 can study on optimized segmentation mechanism (e.g., RLC segmentation enhancement), which is essential to ensure consistent memory allocation, enabling faster and more efficient packet processing.

Observation 3: The design of 6G UP protocol can be explored through approaches aimed at achieving greater alignment with hardware capabilities. Such optimization can enhance performance for ultra-high data rates and immersive applications.

- Enhanced Access Stratum (AS) security improves system stability by mitigating threats by fake base stations and rogue users.

  - Study on AS layer security enhancement:

    - The growing utilization of MAC Control Elements (MAC CEs) and broadcast system information (e.g., SIBs) introduces new vulnerabilities that can be exploited to disrupt network operations or compromise user privacy. As 6G networks would evolve toward more dynamism and distribution, ensuring the security of the AS —particularly Layer 2 signalling—becomes increasingly critical. 6G study on AS security aspects requires alignment with security requirements from SA3 and coordination with SA3.

Observation 4: 6G AS security must address MAC CEs/SIBs vulnerabilities, aligning with SA3 requirements.

- A scalable and forward compatible design should target diverse device types by leveraging a common 6G Radio.

  - Common 6G Radio design to support a wide range of device types:

    - Starting from 5G, a wide variety of device types have emerged, and 5G system has defined and implemented functionalities specifically tailored to support the diversity for device types. With the evolution to 6G, an even broader range of device types is anticipated. While 6GR can prioritize eMBB devices targeting higher capability compared to those available in 5G today, it is essential to define 6G Radio protocols in a future-proof approach from the early stages of 6G system design. This enables to support different devices types with different device capabilities such as smartphone device type, FWA device type, and IoT device type, without requiring separate solutions for each use case.

Observation 5: A unified and adaptable 6G Radio protocol is critical to accommodate the diverse device types.

Based on the above observations, we propose the design principles for 6G Radio protocol in RAN2 study:

Proposal 1: The design principles for 6G Radio protocol should encompass the following:

(1) Optimized mobility framework to achieve the stringent latency and reliability requirements for emerging interactive and immersive applications; (2) Optimized power and energy saving technologies to maximize energy efficiency for both UE and NW; (3) Implementation-friendly and simplified UP Protocol to support high-speed and immersive traffic efficiently; (4) Enhanced AS security to improve system stability by mitigating potential security threats; (5) Scalable and forward-compatible protocols to accommodate diverse device types

2.2		Deployment scenarios and architectures

The study on 6G Radio protocol should consider the deployment scenarios for 6G Radio as outlined in TR 38.914 section 4 [2]. Unlike 5G, where only a few options are commercially deployed despite many RAN architecture options being standardized, the study on 6G RAN architecture should focus on a standalone and single RAT from the beginning of 6G study. It is assumed that 6G RAN logical architecture and interfaces, including CU-DU split and CP-UP separation, similar to the NG-RAN architecture, will be studied in RAN3 WG.

Key considerations associated with RAN2 work:

- Overall RAN architecture

  - This requires careful consideration to support emerging technologies such as AI-native operations and integrated terrestrial-satellite networks. Close collaboration between RAN2 and RAN3 is essential to align air interface protocols with architectural aspects.

- RAN internal functional split, interfaces, protocol and procedures

  - RAN functional partitioning and interfaces must accommodate both CU-DU split and CP-UP separation to enable network optimizations while maintaining well-defined functional boundaries.

  - The RAN-CN interface must ensure that latency levels are maintained at the same standard as those of previous-generation communication systems. This is critical for ensuring seamless connectivity and a consistent user experience across different network generations.

Observation 6: Deployment scenarios, RAN architecture, and RAN internal interface aspects will primarily be studied by RAN and RAN3.

From the perspective of 6G Radio protocol, no immediate RAN2 work is deemed necessary for deployment scenarios and RAN architecture. Instead, RAN2 can monitor the studies by TSG RAN on deployment scenarios and by RAN3 on RAN architecture, internal functional split, interfaces, protocol and procedures to identify decisions that may impact on 6G Radio protocol aspects.

Based on the above observation, we propose:

Proposal 2: RAN2 to await progress on 6G deployment scenarios and 6G RAN architecture from TSG RAN and RAN3 WG to identify associated 6G Radio protocol issues.

2.3	Radio interface protocol architecture and procedure

In the development of 6G communication systems, the protocol specifications must comprehensively support all the essential functionalities and features of earlier generations to support existing services. At the same time, they must introduce significant enhancements to address the growing demands of next-generation applications as well as the lessons learned from 5G. The study on radio interface protocol architecture and procedures should include at the least the following.

- User plane protocol architecture and protocols for 6G radio

  - RAN2 can study the user protocol stack and protocols for 6GR, using the user plane protocol stack and protocols for 5G NR as a starting point. While there are no critical issues with reusing the 5G NR user plane protocol stack and architecture—where SDAP, PDCP, RLC, and MAC sublayers terminated in gNB on the network side—RAN2 may explore potential improvements to user plane protocol stack alongside functions enhancements.

  - For implementation-friendly and simplified user plane protocol enhancements, RAN2 can study the optimized packetization mechanisms, such as concatenation and segmentation, Additionally, RAN2 can investigate further enhancements to support delay-critical services and applications, leveraging the work for XR services in 5G NR as a starting point. Cross-layer optimization, which involves sharing information between protocol layers to improve decision-making, e.g., proactive ARQ retransmission by utilizing HARQ feedback, can yield significant performance gains. 6G UP protocol should be designed with a homogeneous and integrated approach to accommodate a diversity of mechanisms such as delay-aware Layer 2 protocols and a new BSR table for higher data rates introduced during 5G-Advanced.

Observation 7: 5G NR user plane protocol stack can be leveraged as a baseline while exploring enhancements for 6G, focusing on simplified designs, cross-layer optimization, and mechanisms to support immersive services.

- Control plane protocol architecture and protocols for 6G radio

  - The control plane protocol stack architecture in 5G NR can serve as the baseline for 6GR. The 6GR control plane protocol stack includes 6G RRC, terminated in 6G sNB on the network side, and 6G NAS control protocol, terminated in the control function of core network on the network side. The sublayer(s) below RRC can be defined based on the study for 6GR user plane protocol stack, with these sublayer(s) also terminating in 6G sNB on the network side.

  - RAN2 should design the 6GR control plane protocol with a focus on reducing configuration and signalling complexity and avoiding redundant options. Services and functions, as outlined in Table 1, can be defined for RRC in 6G Radio, adopting the services and functions of RRC sublayer in 5G NR as a baseline.

Table 1 Services and functions of RRC in 6G Radio

The services and functions of RRC sublayer in 6G Radio include: - Broadcast of system Information related to AS and NAS - Paging – FFS to support both 6G-RAN initiated paging and 6G core network initiated paging - Establishment, maintenance and release of an RRC connection between the UE and 6G-RAN - Security functions including key management - Establishment, configuration, maintenance and release of radio bearers – FFS on radio bearer types supported in 6G Radio - Mobility functions including: - Handover and context transfer - UE cell selection and reselection and control of cell selection and reselection - Inter-RAT mobility - UE measurement reporting and control of the reporting - Detection of and recovery from radio link failure; - NAS message transfer to/from NAS from/to UE. - QoS management functions

  - In 6G RAN, three RRC states—RRC_CONNECTED, RRC_INACTIVE, and RRC_IDLE—can be defined for UE. As stated in Section 2.1, RAN2 discussions aim to improve UE mobility management, particularly simplifying handover procedure and paging procedure.

Observation 8: RAN2 should design the 6GR control plane protocol focusing on minimizing complexity, avoiding redundancy, and defining services and functions based on the 5G RRC sublayer as a baseline.

Based on the above observations, we propose:

Proposal 3: RAN2 to study 6G UP protocol stack with leveraging 5G NR UP protocol stack as a baseline. For 6G CP protocol, RAN2 can discuss the services and functions outlined in Table 1 for 6G RRC.

- New plane architecture and protocol

  - For emerging operator services such as AI/ML and sensing, RAN2 needs to study how to integrate with 6GR. This study is closely related with the work tasks outlined in SA2 6G SID (refer to the highlighted texts in Table 2). From the perspective of 6G Radio protocol, RAN2 can study new plane architecture, protocols, and radio bearer aspects if a dedicated new plane—separate from the control plane and user plane radio bearers—is deemed necessary. Developing a new plane architecture and protocol will require collaboration with other relevant WGs such as SA2 and RAN3.

Table 2 Related work tasks in SA2 6G SID [3]

<SA2 6G SID in SP-250806[3]> WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network: NOTE 1: The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split. 1.1.Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following: a. Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities. b. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities. c. Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services. (omitted) WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework. NOTE 6: The work split with SA3, SA5 and RAN WGs will require TSG coordination

Observation 9: A new plane will be investigated to support signalling and data transmission for emerging services such as AI/ML and sensing.

Based on the observation, we propose:

Proposal 4: RAN2 to study new plane architecture, protocols, and radio bearer aspects if a dedicated new plane is deemed necessary to accommodate new services such as AI/ML and sensing.

2.4 5G-to-6G Migration

To ensure the successful introduction of 6G networks while maintaining end-user satisfaction with 5G networks, a smooth transition from 5G to 6G is required. According to 6G Radio SID [1], RAN2 is involved in studying all three migration objectives—labelled a), b), and c)—from 5G NR to 6GR, as outlined in Table 3:

Table 3 Related objective in 6GR SID [1]

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Regarding 5G-6G MRSS under objective a), similar to LTE and 5G NR DSS, RAN2 may study signalling aspects to support MRSS between 5G NR and 6G Radio once RAN1 makes progress on this topic.

For Inter-RAT handover with 5G under objective c), like all inter-RAT handovers for the previous generations, the handover procedure can be managed and performed through the core network. So there is no urgency for RAN2 to initiate a study on inter-RAT handover with 5G at this stage.

Observation 10: RAN2 can begin studying 5G NR-6GR MRSS only after RAN1 makes progress on this aspect.

Observation 11: Inter-RAT handover with 5G should be handled through the core network. No immediate study by RAN2 is necessary.

Based on the above observations, we propose:

Proposal 5: RAN2 to hold off until RAN, RAN1 and SA make substantial progress on 5G-6G migration and mobility.

2.5	UE capability signalling framework

The 5G NR UE capability signalling framework defines functionalities such as NW-filtered UE capability reporting, feature set, and fallback band combination, which can be utilized in 6G Radio. However, enhancements and improvements are needed to address issues identified in 5G NR, particularly excessive UE capability signalling, as outlined below:

- With EN-DC deployment preceding standalone NR, reporting for multiple RAT types (nr, eutra, eutra-nr) is necessary.

- The number of component carriers in NR band combinations has increased. While NR CA supports up to 16 CCs, RAN4 specifies up to 5 CC CA cases.

- Numerous lower-level UE capabilities, such as supported bandwidth per band per CC and MIMO parameters per band per CC, are considered.

- Additionally, the complexity of signalling increases due to the optional nature of most features and feature sets.

Since Release 16 in 5G NR, enhanced mechanisms have been introduced to reduce UE capability signalling and manage excessive UE capability information, including:

- RACS (Radio UE Capability Signaling Optimization): To reduce signalling overhead, an ID (e.g., PLMN-assigned ID) representing UE radio capabilities can be provided in NAS signalling.

- UL RRC segmentation: A UE Capability Information message can be segmented if the size of the encoded RRC message PDU exceeds the maximum PDCP SDU size.

In 5G NR, there is no standardized prioritization rule for feature sets or their combinations within RRC messages when RRC limitations exist. Even with UL RRC segmentation, some UE capability information may be unreported due to insufficient UL grants or segment limitations. The transmission of multiple segmented UE capability reports may also introduce latency in UE Capability transfer procedures during RRC connection establishment.

Considering these limitations in the current 5G NR framework, RAN2 can study a standardized framework to prioritize basic and essential UE capabilities over additional UE capabilities. Basic and essential UE capabilities represent mandatory capabilities for a feature group. To avoid latency in the RRC connection establishment procedure and prioritize the transfer of basic and essential UE capabilities, RAN2 can further investigate a step-wise UE capability reporting mechanism. Under this approach, the UE would report basic and essential UE capabilities first during the RRC connection establishment procedure and report additional UE capabilities later upon request from the 6G sNB after the RRC connection establishment procedure is completed. Additionally, RAN2 can study improved NW filters and ID-based UE capability reporting to mitigate excessive UE capability reporting. The improved NW filter can be used to identify basic and essential UE capabilities for 6G specification releases or versions.

Observation 12: Prioritized UE capability reporting with basic and essential UE capabilities, ID-based UE capability reporting, and improved NW filters can simplify the UE capability reporting framework and reduce excessive UE capability signalling.

RAN2 may also explore the feasibility of a UE category-based UE capability framework, similar to the approach taken in LTE systems. In LTE, UE categories are defined to specify maximum downlink and uplink data rates. To accommodate additional downlink and uplink data rates, further UE categories were introduced in LTE. Since LTE Release 12, specific categories for IoT devices, such as LTE Cat 0, LTE-M, and LTE Cat 1bis, have been defined. By LTE Release 18, a total of 26 downlink categories and 26 uplink categories have been specified. Building on lessons learned from LTE, 5G NR adopts a distinct approach by using an equation based on the number of aggregated carriers within a UE’s supported bands or band combinations to calculate maximum downlink and uplink data rates, rather than defining specific UE categories. Additionally, 5G NR introduced a feature set framework to represent a collection of features associated with a particular band. However, the concept of features and feature sets in 5G NR is insufficient to specify discrete IoT devices.

If a UE category-like concept is necessary for clear device type classification in the 6G system, it is essential to establish a product-oriented device classification by anchoring device requirements to real-world product groups rather than abstract use cases. This approach ensures that physical layer, signalling, and architectural designs remain practical and scalable. Table 4 presents an example of the classification of 6G device types. As shown in Table 4, multiple 6G device types can be defined with distinctive key parameters to identify each device type’s requirements or capabilities. Note that the device type in Table 4 is illustrative, and the concrete classification for 6G device types must be specified through collaboration with RAN, RAN1, and RAN4.

Table 4 6G device types

| Parameter list | Type IV (FWA) | Type III (Smartphone) | Type II (Wearable) | Type I (IoT) | NTN (if applicable) |
| --- | --- | --- | --- | --- | --- |
| TX antenna | 1-4Tx (depending on FRs) (Note: 2TX can be baseline) | 1Tx baseline (for all FRs), up to 2Tx (optional) | 1Tx | 1Tx | 1Tx to 4 Tx (depending on PC). |
| RX antenna | 4, 6 or 8 RX (optional) | Below 1 GHz: 2RX, up to 4 RX (optional) Around 3.5 GHz: 4 RX, Around 7 GHz: 4RX, up to 6 RX (optional) Around 24 GHz : 2 RX, up to 4 RX (optional) | 1Rx | 1Rx and up to 2RX (optional) | Depending on device type |
| MIMO layer | Up to 4, 6 or 8 Layer | Up to 2 or 4 layer (depending on FR) | 1 layer | 1 or 2 layer (optional) | 1 layer |
| RF BW (max) | 400 MHz | 200 or [400] MHz | 20 MHz | 20 MHz | Depending on device type |
| Maximum Power class | 29 dBm [or 31 dBm] | FDD 23 dBm, TDD 26 dBm | FDD 23 dBm, TDD 26 dBm | FDD 23 dBm, TDD 26 dBm | [Depending on outcome of 5G work] |
| DL max modulation order | 1024QAM or [4096QAM] | 1024QAM or [4096QAM] | 64 QAM | 256 QAM | Depending on device type |
| UL max modulation order | 256QAM or [1024QAM] | 256QAM or [1024QAM] | 64 QAM | 64 QAM | Depending on device type |
| - Type I: This type spans from cost-sensitive industrial modules to ultra-low-power modules for IoT services. These devices will be deployed in massive scale and must operate under extremely tight cost and energy budgets. 6GR should provide simplified PHY/MAC designs, scalable addressing and identification mechanisms, and highly efficient paging/wake-up signalling. - Type II: Type II is to support wearables (e.g., smartwatches). Wearable devices represent a distinct category with unique constraints in terms of battery size, smallest form factor, and connectivity requirements. Unlike smartphones, wearables typically operate with reduced complexity (e.g., limited bandwidth, fewer antennas) while requiring robust coverage and efficient signalling to support continuous operation under low power. Wearable devices such as smartwatches face unique RF challenges due to their small form factor, including limited number of antenna, constrained PA/filter design, and strict power/thermal budgets. These factors typically result in higher error vector magnitude (EVM) compared to smartphones or IoT devices. Consequently, while generic IoT devices can rely on similar requirements as normal UE, wearable devices require a separate classification in 6G with mid-level modulation capability (e.g., up to 64 QAM) but with more relaxed EVM requirements than smartphones. This distinction justifies treating wearables as a device type separate from IoT in 6GR. - Type III: Smartphones will continue to be the most widely adopted 6G device type, serving as the baseline for feature-rich capabilities. Smartphones must support high spectrum flexibility, multi-antenna operation, and advanced AI-assisted resource management while maintaining strict energy efficiency and form-factor constraints. However, in practical smartphone form factors, achieving flexible multi-band frequency support is fundamentally constrained since adding more antennas and transmit chains is extremely difficult. The compact device architecture must already accommodate displays, batteries, cameras, and processors, leaving very limited space for RF front-end components. Each additional antenna requires its own matching network, filters, and often dedicated power amplifiers or switches, which not only increases board area but also raises power consumption and thermal challenges. As a result, the physical and electrical limitations of smartphone design make it unrealistic to scale antenna counts and RF chains in the same way as larger form-factor devices such as laptops or FWA. - Type IV: This is to support high capable UE for fixed Wireless Access (FWA) service. FWA devices will remain an essential category in 6G, providing broadband connectivity to households and enterprises. Compared to smartphones, FWA UEs typically allow less stringent size and power constraints, enabling support for high-gain antennas, multi-band reception, and advanced beam management. For FWA UEs, 6G standardization must ensure efficient support for wideband carrier aggregation, optimized mobility handling for semi-static installations, and seamless integration with wired backhaul networks. |  |  |  |  |  |

Along with device type classification, RAN2 can study a UE capability signalling framework that utilizes basic and essential capabilities associated with each device type to reduce signalling overhead for UE capability reporting. For instance, if RAN2 defines basic and essential capabilities for each device type, the device type information alone can be reported in UE capability signalling instead of reporting individual parameters for basic and essential capabilities. This approach would streamline the signalling process and provide a more structured framework for defining diverse device capabilities in 6G.

Based on the above observations, we propose:

Proposal 6: RAN2 to study improvements to UE capability signalling mechanisms to avoid excessive capability report signalling while leveraging NR UE capability signalling framework. This includes exploring NW-filtered capability reporting, the feature set concept, fallback band combinations, and segmented UL signalling.

3	Conclusion

Based on the discussion, we made the following observations:

Observation 1: Advancements in handover framework, RRC_INACTIVE mobility optimization, and AI/ML integration can contribute to achieving 6G’s latency and reliability requirements for interactive and immersive applications.

Observation 2: 6G systems should integrate coordinated UE and network energy-saving mechanisms, differentiate carrier operations, and optimize system information transmission to maximize energy efficiency.

Observation 3: The design of 6G UP protocol can be explored through approaches aimed at achieving greater alignment with hardware capabilities. Such optimization can enhance performance for ultra-high data rates and immersive applications.

Observation 4: 6G AS security must address MAC CEs/SIBs vulnerabilities, aligning with SA3 requirements.

Observation 5: A unified and adaptable 6G Radio protocol is critical to accommodate the diverse device types.

Observation 6: Deployment scenarios, RAN architecture, and RAN internal interface aspects will primarily be studied by RAN and RAN3.

Observation 7: 5G NR user plane protocol stack can be leveraged as a baseline while exploring enhancements for 6G, focusing on simplified designs, cross-layer optimization, and mechanisms to support immersive services.

Observation 8: RAN2 should design the 6GR control plane protocol focusing on minimizing complexity, avoiding redundancy, and defining services and functions based on the 5G RRC sublayer as a baseline.

Observation 9: A new plane will be investigated to support signalling and data transmission for emerging services such as AI/ML and sensing.

Observation 10: RAN2 can begin studying 5G NR-6GR MRSS only after RAN1 makes progress on this aspect.

Observation 11: Inter-RAT handover with 5G should be handled through the core network. No immediate study by RAN2 is necessary.

Observation 12: Prioritized UE capability reporting with basic and essential UE capabilities, ID-based UE capability reporting, and improved NW filters can simplify the UE capability reporting framework and reduce excessive UE capability signalling.

Base on the observations, RAN2 is requested to discuss and agree the following proposals:

Proposal 1: The design principles for 6G Radio protocol should encompass the following:

(1) Optimized mobility framework to achieve the stringent latency and reliability needs for interactive and immersive applications; (2) Optimized power and energy saving technologies to maximize energy efficiency for both UE and NW; (3) Implementation-friendly and simplified UP Protocol to support high-speed and immersive traffic efficiently; (4) Enhanced AS security to improve system stability by mitigating potential security threats; (5) Scalable and forward-compatible protocols to accommodate diverse device types

Proposal 2: RAN2 to await progress on 6G deployment scenarios and 6G RAN architecture from TSG RAN and RAN3 WG to identify associated 6G Radio protocol issues.

Proposal 3: RAN2 to study 6G UP protocol stack with leveraging 5G NR UP protocol stack as a baseline. For 6G CP protocol, RAN2 can discuss the services and functions outlined in Table 1 for 6G RRC.

Proposal 4: RAN2 to study new plane architecture, protocols, and radio bearer aspects if a dedicated new plane is deemed necessary to accommodate new services such as AI/ML and sensing.

Proposal 5: RAN2 to hold off until RAN, RAN1 and SA make substantial progress on 5G-6G migration and mobility.

Proposal 6: RAN2 to study improvements to UE capability signalling mechanisms to avoid excessive capability report signalling while leveraging NR UE capability signalling framework. This includes exploring NW-filtered capability reporting, the feature set concept, fallback band combinations, and segmented UL signalling.

4	References

[1] RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone (Moderator)

[2] TR 38.914 V0.2.0, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on 6G Scenarios and Requirements; (Release 20)

[3] SP-250806, Study on Architecture for 6G System, SA WG2