3GPP TSG-RAN WG2 Meeting#131bis	R2-2506963

Prague, Czech Republic, 13th – 17th Oct, 2025

Source:	vivo

Title:	Discussion on Topology 2

Agenda Item:	9.2.2

Document for:	Discussion and Decision

- Introduction

The WID [1] has been approved to standardize a second phase of RAN aspects for Ambient IoT, by identifying the following Topology 2 related RAN2 objectives:

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify a single architecture by down-selection between RRC-based architecture and UP-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. The work is subject to the following constraints: RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling.

In this contribution, we will discuss the support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader.

- Discussion

For A-IoT topology 2, RRC-based solution in Uu interface is the only solution since UP-based solution and NAS-based solution have been ruled out by SA2. One example of the signaling procedure is described in [2] as:

Figure 1: Message flow for A-IoT Inventory in Topology 2 - RRC-based solution

## UE Reader selection & Uu Behavior

### UE Reader selection

During study phase for Topology2 aspect, RAN2 has agreeed that[2]:

From TR 38769-j00 For the UE reader selection, it is up to RAN3 and SA2 on whether BS or CN selects the UE reader(s) and what information is shared between BS and CN. From RAN2 perspective, it is assumed, at least for RRC based solution, there may be radio related reasons for the BS to be involved in the UE reader selection.

In RAN architecture aspects, it is specified that “In RRC based solution, Reader selection may need coordination between A-IoT-enabled gNB and A-IoT CN”. As RRC based solution has been selected, further discussion is needed on how to perform reader selection. In Rel-19, the A-IoT CN node initiates the inventory procedure by sending the Inventory Request message to the gNB, the gNB may select readers based to the Requested Service Area Information (list of A-IoT Area IDs and/or reader list) included in the the Inventory Request message. For Topology2, the UE reader may be selected in the same manner, i.e., based on A-IoT Area IDs and/or reader list, or entirely by gNB.

Although UE Reader can perform AIoT operations only in RRC_CONNECTED state, the UE may be in a non-Connected state (e.g., RRC_IDLE or RRC_INACTIVE) when the network selects it as a UE reader. In case that the UE is in non-Connected state, the NW shall page the UE to enter Connected state firstly before performing A-IoT Inventory procedure.Therefore, we propose the following:

- A UE capable of acting as an A-IoT reader in all RRC states may be selected act as a UE Reader.

In addition to the A-IoT Area and/or reader list information, the NW may select the UE reader based on additional criteria, such as:

- The UE is an AIoT-enabled UE, which relates to its capability.

- The receiving signal strength is sufficient, e.g., the UE is allowed to act as a UE reader only if the serving cell RSRP is above a certain RSRP threshold . The RSRP threshold is used to control interference from the AIoT interface to the Uu interface.

In the RRC_CONNECTED state, the network can easily apply the above criteria through implementation. When the UE is in non-Connected state, the NW shall page the UE to perform the inventory procedure. If the Uu RSRP is too low, potentially causing the UE to fail to complete the A-IoT procedure properly, the UE may not accept the paging to become a UE reader. In other word, whether the UE to respond to paging for A-IoT purpose depends on whether the Uu RSRP threshold is met. Therefore, we propose the following:

- The UE capable of acting as an A-IoT reader responds to a Uu Paging message for A-IoT purpose only if selection criteria is met, e.g., Uu RSRP threshold is fulfilled.

### UE Reader Camping and Access

Before providing AIoT services, a UE reader must find a cell that supports UE reader access and establish a radio connection in that cell. This cell is expected to support the following:

- RRC based A-IoT message transmission on the Uu interface.

- AIoT radio resource allocation for the UE reader.

Before accessing the cell, the UE reader relies on the system message to determine whether a cell supports UE reader access. To facilitate this, a UE-reader-support indicator can be provided in SIB1, enabling the UE reader to identify whether a cell supports UE reader access during the cell searching procedure. For a cell shared by multiple PLMNs, a separate UE-reader-support indicator can be provided for each PLMN.

- Introduce an indicator in SIB1 to indicate tha a cell supports the UE reader.

A UE reader operates as an access node for AIoT devices and it must be served by an AMF that supports AIoT services. After establishing a radio connection, the serving gNB should initiate the UE context setup procedure by sending an INITIAL UE MESSAGE to a selected AMF capable of providing AIoT service. To enable this, the UE reader must indicate to its serving gNB that it is accessing as a UE reader before the serving gNB selects AMF and sends the INITIAL UE MESSAGE. To minimize the delay for UE context setup on the NG interface, it is preferable for the UE reader to send a UE reader indication to the serving gNB during random access procedure. An example flowchart is provided below.

UE reader

gNB

AMF

Random Access (+ reader indication)

INITIAL UE MESSAGE

INITIAL UE CONTEXT SETUP REQUEST

SecurityModeProcedure

UECapbilityProcedure

INITIAL UE CONTEXT SETUP RESPONSE

Figure 2: Flow chart example of UE reader indication in access procedure

- The UE acting as an A-IoT reader should indicate to gNB that it is accessing as a UE reader during the Uu access procedure.

## Inventory and command procedure

### New SRB

For A-IoT Topology 2, as RRC-based solution is the only option, RRC message should be used to transmit A-IoT Inventory and Command PDUs, e.g. in the form of containers, along with related control information such as A-IoT session assistance information and resource allocation for the A-IoT interface. Legacy RRC messages are transmitted via SRB1, which has the highest priority, infinite Prioritized Bit Rate (PBR), high reliability requirement, no service continuity requirement, etc. These transmission attributes of SRB1 seems unsuitable for A-IoT data, as A-IoT data may have lower priority, different PBR, and distinct reliability requirements compared to SRB1. Furthermore, A-IoT data is transmitted between device and CN, which may require service continuity during handover scenarios.

- RRC messages containing A-IoT data may have different transmission requirements from those of SRB1, e.g. lower priority and distinct PBR or reliability requirements.

Introducing a new SRB, e.g. SRBx, may be a feasible and reasonable solution for more efficient forwarding A-IoT data over the Uu interface. The new SRBx would be used to carry RRC messages containing A-IoT data or related information and could be configured on-demand with appropriate priority, PBR, reliability parameters, etc.

- Introduce a new SRB, e.g. SRBx, to transfer RRC messages containing A-IoT data or related information between an A-IoT UE reader and its serving gNB.

The new SRBx should be managed (i.e., addition/modification/release) by the serving gNB of the A-IoT UE reader, e.g. via RRC Reconfiguration procedure. When the serving gNB determines that a UE will act as a A-IoT UE reader to transmit A-IoT data or information over the Uu interface, it configures SRBx for that UE. Consistent with legacy rules, SRBx must be configured after AS security activation.

- The New SRBx can be configured to an A-IoT UE reader only after AS security activation.

Furthermore, normal UE without any DRB/MRB is not permitted to stay in RRC_CONNECTED mode for an extended period, which is unsuitable for an A-IoT UE reader. When an A-IoT UE reader has no user services of its own but only A-IoT transmission requirements, it should be allowed to stay in RRC_CONNECTED state until A-IoT transmission is complete. In other words, a configuration with the new SRBx for A-IoT data without any DRB/MRB is supported for a A-IoT UE reader.

- For an A-IoT UE reader, a configuration with new SRBx for A-IoT data, without any DRB/MRB, is supported.

### Inventory procedure

In the Uu interface, the RRC inventory procedure is triggered by Inventory Request of NG interface. According to recent RAN3 progress[3], Inventory Request from the NG interface may include the following fields:

- Message type,

- AIOTF Identifier,

- A-IoT Correlation Identifier,

- A-IoT Device Identification Requested,

- Inventory Assistance Information: Expected D2R Message Size, Approximate Number of Target A-IoT Devices and Time Interval for report,

- Follow-on Command Indication.

The first three fields indicate the type and identification of A-IoT service, which are useful for the Uu interface. However, RRC message can use their own encoding method, e.g. unified ASN.1 method for Message type, node ID, and session ID which can be encoded or included directly as is. Details can be left to Stage 3.

The field of A-IoT Device Identification Requested carries necessary paging ID information in the device’s NAS PDU format, which should be encapsulated in an RRC container. The Inventory Assistance Information and Follow-on Command Indication fields provide critical information for the inventory procedure and are useful for UE reader to allocate the paging parameters and schedule subsequent operations on the A-IoT interface. Therefore, this information should be included in the RRC message.

Furthermore, RRC Inventory Request is typically the first A-IoT related message on the Uu interface. It can also carry resource allocation information for the A-IoT interface, as the serving gNB can provide appropriate resources based on the approximate scale of the inventory and follow-on command indication. This allocation method reduces delay and enables A-IoT interface to start operating as soon as possible. Alternatively, resource allocation can be transmitted via separate procedure, e.g. reusing RRC Reconfiguration procedure, similar to the legacy resource allocation method.

- The RRC Inventory Request may at least include the following information:

- A-IoT Device Identification Requested;

- Inventory Assistance Information: Expected D2R Message Size, Approximate Number of Target A-IoT Devices and Time Interval for report;

- Follow-on Command Indication;

- Optional A-IoT Resource allocation info.

In the second procedure, the RRC Inventory Report is sent from UE reader to its serving gNB. Consistent with the same rule, Inventory Report on the NG interface should include the following:

- Message type;

- AIOTF Identifier;

- A-IoT Correlation Identifier;

- Global gNB ID;

- Reader Report List: Each item includes Reader Index and Device Report List;

- Device Report List: Each item includes A-IoT NAS PDU and optional RAN A-IoT Device NGAP ID (e.g. NGAP ID is present in the case of subsequent Command).

The first three fields can be handled in the same manner as in the RRC Inventory Request and left to stage3. On the NG interface, the A-IoT message operates per-gNB granularity, and there may be multiple cell readers under the same gNB, e.g. Global gNB ID and Reader Index differentiate device report list from each cell reader under a specific gNB. For a UE reader, only one device report list is needed as there is only one A-IoT reader.

The key issue is how to differentiate devices within a UE reader and establish one-to-one device ID mapping between the Uu interface and NG interface for accurate UL and DL addressing and routing. There are 3 options for device IDentification between a UE reader and its gNB:

- Option 1: Device local ID allocated by gNB;

- Option 2: Device local ID allocated by UE reader;

- Option 3: Reusing AS ID in R19 A-IoT interface.

Option 1 aligns most closely with the principle of centralized control by the gNB. However, Option 1 requires the UE reader to first report device information to the gNB and wait for device local ID allocation, which introduces signalling overhead and additional latency. Both option 2 & 3 involves allocating the device ID by the UE reader, which is closer to the source of device ID requirement, reducing signalling overhead and latency. Compared to Option 2, Option 3 is simpler and more reasonable, as the AS ID in Rel-19 A-IoT interface is a unique identifier within the scope of a UE reader. A newly or separately allocated ID, as in Option 2, offers no advantages and only add complexity.

- The AS ID in Rel-19 A-IoT interface is the simplest method to uniquely identify a device within the scope of a UE reader.

From the gNB’s perspective, there may be multiple UE readers and even coexistence with gNB reader(s) under the same gNB. gNB should establish and maintain a one-to-one mapping between the AS ID of a specific UE reader and the RAN A-IoT Device NGAP ID, as shown in the following mapping table (Table 1). When the RRC Inventory Report message first includes a new AS ID from A-IoT interface, the gNB allocates a new RAN A-IoT Device NGAP ID to this device and associates it with the pair of UE reader Index and AS ID, completing the establishment of the ID mapping relationship. Subsequently, A-IoT Command messages from CN are addressed using this RAN A-IoT Device NGAP ID. The gNB receives CN message, identifies the corresponding UE reader Index and device AS ID from the RAN A-IoT Device NGAP ID, and delivers the A-IoT Command message to the appropriate UE reader and, subsequently, to the correct device.

Table 1 Example of Device ID mapping

|  | AS ID | RAN A-IoT Device NGAP ID |
| --- | --- | --- |
| UE reader 1 | AS ID 1 | NGAP ID 1 |
|  | AS ID 2 | NGAP ID 2 |
|  | … | … |
|  | AS ID m | NGAP ID m |
| UE reader 2 | AS ID 1 | NGAP ID m+1 |
|  | … | … |
|  | AS ID n | NGAP ID m+n |
| UE reader 3 | … | … |

- The gNB can generate a device ID for use on the NG interface (i.e. RAN A-IoT Device NGAP ID) based on the pair of UE reader Index and device AS ID of Rel-19 A-IoT interface. the gNB can then use the mapping relationship for subsequent A-IoT unicast command addressing and routing.

Hence, we propose:

- The RRC Inventory Report should include at least a Device Report List: each item includes A-IoT NAS PDU and an optional Device ID within the scope of this UE reader.

- The Device AS ID in the A-IoT interface (i.e., allocated by UE reader, as in Rel-19) can be reused as Device ID in A-IoT RRC messages between the UE reader and its serving gNB.

### Command procedure

After the Inventory procedure, the RRC Command procedure is triggered by a Command Request from the NG interface. According to the newest TS38.413 CR for A-IoT, the Command Request from the NG interface may include the following fields:

- Message type;

- AIOTF Identifier;

- A-IoT Correlation Identifier;

- RAN A-IoT Device NGAP ID;

- A-IoT NAS PDU;

- Command Assistance Information: Estimate of Expected D2R Message Size;

The first three fields are also left to stage 3. The RAN A-IoT Device NGAP ID should be translated into the appropriate pair of UE reader Index and Device AS ID using the Device ID mapping (e.g., Table 1). Thus, the gNB can include the Device AS ID in the RRC Command Request sent to the appropriate UE reader, which then delivers it to the correct device. This ensure proper DL routing of A-IoT data.

- For routing and transmitting an A-IoT Command Request from CN, the gNB can translate the RAN A-IoT Device NGAP ID in the NG message into the appropriate pair of UE reader Index and Device AS ID.

Hence, we propose:

- RRC Command Request should include at least the following information:

- Device AS ID;

- A-IoT NAS PDU;

- Command Assistance Information: Estimate of Expected D2R Message Size.

### Failure case

On the NG interface, nearly all of DL A-IoT Request messages will have a corresponding separate Failure message to report the reason for NG interface failure. Similarly, RAN2 needs to discuss whether failure may also occur on the Uu interface and the reasons for failures should be reported from the UE reader to the gNB. If Failure reporting is deemed necessary, the failure causes for the Uu interface should be designed separately based on specific Uu interface requirements and should not reuse those defined for the NG interface.

- The RRC Inventory/Command Failure case, if necessary, may report Uu interface failure cause(s) to the serving gNB.

## AIoT Radio Resource Allocaiton and Validity

### A-IoT resource allocation request procedure

According to TR 38.769, two alternatives for the radio resources allocation request have been studied, which are describled as below:

From TR 38769-j00 For the radio resources allocation request, following alternatives are studied: - Alternative 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or - Alternative 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN. NOTE 2: The above two alternatives can co-exist. The configuration for radio resource allocation may also include the validity criteria (if applicable, see the time period in previous option 2 of those scenarios)

It is evident that the two alternatives above can coexist, although it has not been decided whether both will be supported (see highlighted yellow text). We understand that both alternatives is beneficial for different use cases. For example, Alt2 can be used for initial resource configuration upon service arrival, while Alt1 can be used for resource reconfiguration e.g., adjusting (adding or releasing) resources based on UE assistance information e.g., status info related to access collision, D2R failure, or service completion. Therefore, it is proposed to confirm that both of the the above two alternatives will be supported.

- RAN2 confirms that, for the radio resources allocation requests, both of the following alternatives will be supported for different purposes e.g., use Alt-2 for initial configuration and use Alt-1 for reconfiguration on-demand:

- Alt-1: BS configures/allocates a set of A-IoT radio resource to the UE reader, in response to a request from the UE reader;

- Alt-2: BS configures/allocates a set of A-IoT radio resource to the UE reader, based on the service request from CN.

### A-IoT resource allocation singnaling option

During study phase, RAN2 agreed that the A-IoT radio resources are not shared among UE readers, and are only configured to the UE readers via dedicated signaling by the network, as highlighted as following:

From TR 38769-j00 The radio resources used by A-IoT radio interface between the A-IoT device(s) and UE reader are controlled by the network. The radio resources, which are dedicated for a UE reader, are only configured to the UE reader via dedicated signalling. The mechanisms for shared resource pool amongst UE readers are not considered in this release. The UE reader in coverage of BS scenario is supported.

During the study phase,, configuration granularities beyond per UE reader were proposed. Examples are as below:

- Per A-IoT message (e.g., paging, MSG1, etc.)

- Per D2R and R2D transmission pair

- Per A-IoT procedure

In our view, thise finer configuration granularities represent over-optimzation, causing singnificant Uu singaling overhead between the UE reader and BS without clear benefits. Therefore, we think all of the above configuration granularities should not be considered. Therefore, it is proposed to confirm that the agreement made during SI phase is directly used for further standardization work.

- RAN2 confirms that the A-IoT radio resource configuration is allocated per UE reader via dedicated RRC signalling. Finer configuration granularities, such as per A-IoT message or per A-IoT procedure, are excluded.

Regarding the candidate singnaling options for defining the available A-IoT resources to a UE reader, three options under consideration:

- Option 1: RRC-based Configured Grant resource (e.g., like SL CG type-1 configuration IE SL-ConfiguredGrantConfig)

- Option 2: A-IoT resource pools (e.g., similar to the SL resource pool configuration IE SL-ResourcePool), with each pool configured exclusively to a UE reader

- Option 3: A-IoT specific Gap configuration (e.g., similar to the Uu Gap configuration needed for certain feature)

For Option 1, the RRC parameters are similar to the SL CG type-1 configuration IE SL-ConfiguredGrantConfig. However, this option introduces specification complexity. For example, RRC must provide a set of parameters for the configured grant (e.g., the periodicity, the time and the frequency resource location etc), and a new MAC CE for Configured Grant Confirmation is required. Additionally,, exsiting RRC parameters, such as HARQ process ID and the PUCCH resource for HARQ feedback, are not applicable to A-IoT radio interface. Consequently, adopting Option 1 would require further work to decouple the HARQ concept from the existing configured grant mechanism.

For Option 2, the singnaling is simair to the SL resource pool configuration IE SL-ResourcePool, However, since the A-IoT radio interface supports only two physical channels types, the parameters for SL resource pool can be significantly simplified. For instance, a single resource pool for PRDCH and PDRCH, defined by a repeating bitmap with periodicity within a SFN cycle, may suffice. However, the stringent timeline between A-IoT R2D transmission and corresponding D2R transmisison, as defined in RAN1 specificaition, may lead to resource fragmention within the A-IoT resource pool defined by this bitmap approach. As a result, Option 2 is less favorable due to potential low resource ulitllization.

For Option 3, the Uu Gap configuration signaling, such as the IE MUSIM-GapConfig can be reused as baseline. By introducing a Gap configuration that provides periodic or aperiodic time window, a balance between BS control flexibility and resource untilization can be achieved to effectively operate on A-IoT radio interface. Based on this analysis, Option 3 to slightly preferred.

- RAN2 to discuss and decide which of the following singnaling options to adopt for defining available A-IoT resources to a UE reader:

        - Option 1: RRC-based Configured Grant resource (e.g., similar to SL CG type-1 resource);

        - Option 2: A-IoT resource pools, with each Pool configured exclusively for a UE reader (e.g., similar to SL resource pool);

        - Option 3: A-IoT specific Gap configuration (e.g., similar to Uu gap configuration needed for certain feature).

### A-IoT resource allocation validity

Based on the latest WID scope[1], only RRC CONNECTED UE reader, including RLF and handover, are considered, without specific enhancement for other temporary out-of-connection scenarios. Meanwhile, candidate solutions for A-IoT resource allocation validity have been agreed upon for further down-selection during the WI phase, see highlighted text in yellow below:

From TR 38769-j00 For the case when the radio resources are allocated semi-statically by the network, the radio resource validities are studied in the following scenarios with candidate options (down-selection can be decided in the WI phase): - Scenario 1: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface: - Option 1: UE reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly. This would be supported by the NR Uu RRC reconfiguration; and/or - Option 2: UE reader receives the resources configuration in NR Uu dedicated RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the resources remain valid for that time period, unless the resource configuration is explicitly released by the network by the NR Uu RRC reconfiguration. - Scenario 2: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases): - Option 1: The UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition. - Option 2: The UE reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations. NOTE 1: The A-IoT radio resource can be (re)configured by UE reader’s RRC reconfiguration (including during handover procedure, after re-establishment of the UE reader, etc.)

We believe that validity time period concept is benificial for the following use cases:

- Preventing an RRC CONNECTED UE reader from permanently occupying periodic resources, thereby improving TDM resource utilization efficiency.

- Addressing the loss of explicit release signaling for the resource configuration by the network during temporary out-of-connection scenarios (e.g., RLF).

- Reducing potential A-IoT service interruption and ensuring A-IoT service continuity for at least a specific period.

Therefore, we propose that RAN2 agree to support the validity time period concept, with a flexible duration to address both Scenario 1 and Scenaro 2 and achieve the benefits listed above .

- RAN2 agrees that the UE reader can continue performing A-IoT operations in RRC CONNECTED state and during temporary out-of-connection scenarios (e.g., RLF and handover), until the UE reader determines that the A-IoT radio resource invalidity condition is met .

With regard to how the UE reader determines the validity of configured or maintained A-IoT radio resources, our illustration is shown as in below Figure 3:

- Firstly, the UE reader in RRC CONNECTED state can receive A-IoT radio resource (re)configuration from its serving cell. The A-IoT radio resource (re)configuration includes the duration of a new validity timer T3xx.

  - Note: The exact timing for A-IoT radio resource (re)configuration depends on the alternatives in A-IoT resource allocation request procedure. The timer duration is determined by the BS to allocate sufficient A-IoT time resources, considering all relevant factors, including the assistance information from CN and/or from UE reader.

- Secondly, the UE reader maintains the (re)configured A-IoT radio resources and (re)starts the validity timer T3xx. And at this point, the UE reader considers the configured or maintained A-IoT radio resource as valid.

- Thirdly, the UE reader can continuously perform A-IoT operations in RRC CONNECTED and during temporary out-of-connection scenarios (e.g., RLF and handover), until the UE reader determines that the A-IoT radio resource invalidity is met.

- Lastly, given that a principle that A-IoT radio resource of the UE reader is only valid within the serving cell as documented in the WID, the invalidity condition(s) are further refined based on the following four cases:

  - Case 1: Handover case. In this case, the target BS may or may not have available A-IoT radio resource for the UE reader in target cell during handover preparation phase, e.g.,   due to some reasons such as radio resource control constraints or lack of support for AIoT operations. Consequently, upon reception of handover command, the UE reader’ serving cell changes. The previously configured or maintained A-IoT radio resource is no longer invalid while if there is no A-IoT radio resource for target cell’s configuration, the UE reader cannot continue to perform A-IoT operations.

  - Case 2: RLF case and any other Uu failure cases that leads to RRC re-establishment. In this case, cell selection procedure is performed firstly during RRC re-establishment procedure. In case that the UE rader’s serving cell changes upon cell selection, the previously configured or maintained A-IoT radio resource is no longer invalid. The difference between handover case and this case is that, only after successfully RRC re-establishment procedure, the UE may receive RRC Reconfigurition containing A-IoT radio resource. In other word, the UE reader has no way to continue to perform A-IoT operations upon cell selection during RRC re-establishment procedure.

  - Case 3: Upon receiving an explicit release indicator from the serving cell.  In this case, the  UE reader receives an explicit release indicator for the A-IoT radio resource configuration from the cell on which the A-IoT radio resource is configured. This explicit release method can be utilized for resource efficienty by re-allocation to other UE readers when this UE reader has completed the A-IoT service procedure earlier that expected by the BS.

  - Case 4: Upon validity timer expiry. In this case, the new validity timer T3xx, corresponding to the A-IoT radio resource (re)configuration expires. Validity timer mechanism is beneficial for the UE reader’s service continouity and is complementory to above cases. E.g., when the serving cell change doesn’t occur to the UE reader, it can continue to perform A-IoT operations until T3xx is expired.

Figure 3. Conditions for deciding A-IoT radio resource valid/invalid

- The UE reader determines the A-IoT radio resource invalidity is met when any of the condition(s) is met:

        - Upon reception of handover command and no  A-IoT radio resource reconfiguration in target cell;

        - Upon cell change due to cell selection during RRC re-establishment;

        - Upon receiving an explicit release indicator from the serving cell;

        - Upon validity timer expiry.

## Collision handling between Uu and AIoT

As an intermediate node between gNB and AIoT device, a UE reader has the following restriction as described in the WID[1]:

- The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band

- The UE reader operates as a UE on Uu interface and as a reader in A-IoT interface, but simultaneous Uu and AIoT transmission/reception in the same band is not supported.

From the UE reader’s perspective, Uu traffic may arrive while A-IoT traffic is ongoing on its A-IoT interface, or A-IoT traffic may arrive while Uu traffic is ongoing on its Uu interface. The Uu traffic includes DL/UL data traffic transception and signaling/functional traffic (e.g. beam failure recovery, measurements, RLF etc). The AIoT traffic may include inventory, and command procedure. As simultaneous operation of Uu and AIoT in the same band is not supported, a mechanism is needed for the UE reader to manage the trade-off between Uu operation and the AIoT operations. For example, if a UE reader is engaged in an ongoing A-IoT service, a voice call may arrive, a beam failure may occur, or a measurement report may be triggered. In such cases, the UE reader must determine whether to prioritize Uu traffic handling or continue A-IoT traffic until completion. Similarly, if the UE reader is engaged in ongoing Uu traffic, A-IoT traffic may arrive. It is unclear whether the gNB implementation should delay A-IoT traffic until Uu traffic is completed or allow the UE to switch between Uu and A-IoT traffic based on specific rules.

- RAN2 to consider the collision handling mechanism for the UE reader to manage conflicts between Uu and AIoT operations, for instance:

        - Mechanism to handle the Uu traffic occurance (e.g. incoming call, measurement report and BFR etc) for UE reader with ongoing AIoT service.

        - Mechanism to handle the incoming AIoT traffic for an UE reader with ongoing Uu traffic.

- Conclusion

On a basis of above analysis, we make the following observations and proposals:

UE Reader selection & Uu Behavior

- A UE capable of acting as an A-IoT reader in all RRC states may be selected act as a UE Reader.

- The UE capable of acting as an A-IoT reader responds to a Uu Paging message for A-IoT purpose only if selection criteria is met, e.g., Uu RSRP threshold is fulfilled.

- Introduce an indicator in SIB1 to indicate tha a cell supports the UE reader.

- The UE acting as an A-IoT reader should indicate to gNB that it is accessing as a UE reader during the Uu access procedure.

Inventory and command procedure

- RRC messages containing A-IoT data may have different transmission requirements from those of SRB1, e.g. lower priority and distinct PBR or reliability requirements.

- The AS ID in Rel-19 A-IoT interface is the simplest method to uniquely identify a device within the scope of a UE reader.

- The gNB can generate a device ID for use on the NG interface (i.e. RAN A-IoT Device NGAP ID) based on the pair of UE reader Index and device AS ID of Rel-19 A-IoT interface. the gNB can then use the mapping relationship for subsequent A-IoT unicast command addressing and routing.

- For routing and transmitting an A-IoT Command Request from CN, the gNB can translate the RAN A-IoT Device NGAP ID in the NG message into the appropriate pair of UE reader Index and Device AS ID.

Inventory and command procedure

- Introduce a new SRB, e.g. SRBx, to transfer RRC messages containing A-IoT data or related information between an A-IoT UE reader and its serving gNB.

- The New SRBx can be configured to an A-IoT UE reader only after AS security activation.

- For an A-IoT UE reader, a configuration with new SRBx for A-IoT data, without any DRB/MRB, is supported.

- The RRC Inventory Request may at least include the following information:

- A-IoT Device Identification Requested;

- Inventory Assistance Information: Expected D2R Message Size, Approximate Number of Target A-IoT Devices and Time Interval for report;

- Follow-on Command Indication;

- Optional A-IoT Resource allocation info.

- The RRC Inventory Report should include at least a Device Report List: each item includes A-IoT NAS PDU and an optional Device ID within the scope of this UE reader.

- The Device AS ID in the A-IoT interface (i.e., allocated by UE reader, as in Rel-19) can be reused as Device ID in A-IoT RRC messages between the UE reader and its serving gNB.

- RRC Command Request should include at least the following information:

- Device AS ID;

- A-IoT NAS PDU;

- Command Assistance Information: Estimate of Expected D2R Message Size.

- The RRC Inventory/Command Failure case, if necessary, may report Uu interface failure cause(s) to the serving gNB.

AIoT Radio Resource Allocation and Validity

- RAN2 confirms that, for the radio resources allocation requests, both of the following alternatives will be supported for different purposes e.g., use Alt-2 for initial configuration and use Alt-1 for reconfiguration on-demand:

- Alt-1: BS configures/allocates a set of A-IoT radio resource to the UE reader, in response to a request from the UE reader;

- Alt-2: BS configures/allocates a set of A-IoT radio resource to the UE reader, based on the service request from CN.

- RAN2 confirms that the A-IoT radio resource configuration is allocated per UE reader via dedicated RRC signalling. Finer configuration granularities, such as per A-IoT message or per A-IoT procedure, are excluded.

- RAN2 to discuss and decide which of the following singnaling options to adopt for defining available A-IoT resources to a UE reader:

        - Option 1: RRC-based Configured Grant resource (e.g., similar to SL CG type-1 resource);

        - Option 2: A-IoT resource pools, with each Pool configured exclusively for a UE reader (e.g., similar to SL resource pool);

        - Option 3: A-IoT specific Gap configuration (e.g., similar to Uu gap configuration needed for certain feature).

- RAN2 agrees that the UE reader can continue performing A-IoT operations in RRC CONNECTED state and during temporary out-of-connection scenarios (e.g., RLF and handover), until the UE reader determines that the A-IoT radio resource invalidity condition is met .

- The UE reader determines the A-IoT radio resource invalidity is met when any of the condition(s) is met:

        - Upon reception of handover command and no  A-IoT radio resource reconfiguration in target cell;

        - Upon cell change due to cell selection during RRC re-establishment;

        - Upon receiving an explicit release indicator from the serving cell;

        - Upon validity timer expiry.

Collision handling between Uu and A-IoT

- The UE reader operates as a UE on Uu interface and as a reader in A-IoT interface, but simultaneous Uu and AIoT transmission/reception in the same band is not supported.

- RAN2 to consider the collision handling mechanism for the UE reader to manage conflicts between Uu and AIoT operations, for instance:

        - Mechanism to handle the Uu traffic occurance (e.g. incoming call, measurement report and BFR etc) for UE reader with ongoing AIoT service.

        - Mechanism to handle the incoming AIoT traffic for an UE reader with ongoing Uu traffic.

- Reference

- RP-252894 New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2

- 38769-j00 Study on solutions for Ambient IoT (Internet of Things) in NR

- 38413_CR1260r7_(Rel-19)_R3-255966 Ambient IoT_NGAP CR_R19_v2.docx