3GPP TSG RAN WG2 #131bis                                R2-2506968

Prague, Czech Republic, October 13th - 17th, 2025

Source:	Sharp

Title:	Discussion on Deployment Scenario 2 with Topology 2 in A-IoT

Agenda Item:	9.2.2

Document for:	Discussion and Decision

Introduction

A revised work item on solutions for Ambient IoT [1] for Rel-20 was approved in RAN#109, with objectives including the following:

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. The work is subject to the following constraints: [...] RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling.

In this document, we share our views on a few aspects of support for Deployment Scenario 2 with Topology 2.

Discussion

A-IoT deployment scenarios and device types

According to [1], for UE Readers, both D2T2-B (see Figure 1) for Device 1 (as already supported in Rel-19) and D2T2-C (see Figure 2) for active devices (i.e. Device 2b/C, to be supported in Rel-20) are studied. On the other hand, as also noted in [1], “coexistence between Device 1 and Device 2b/C is not considered in the same deployment in the same band”. Therefore, for a given UE Reader, A-IoT radio interface operations are either only for Device 1, or only for Device 2b/C, i.e. there is no need to consider scenarios with a mix of Device 1 and Device 2b/C.

Figure 1.  Deployment Scenario 2 with Topology 2 - B (D2T2-B), according to TR 38.769 [2]

Figure 2.  Deployment Scenario 2 with Topology 2 - C (D2T2-C), according to TR 38.769 [2]

- From UE Reader perspective, A-IoT radio interface operations are either only for Device 1 or only for Device 2b/C.

Furthermore, according to [1], “air interface for active device(s) studied for outdoor will be reused for supporting active device(s) in indoor scenarios”. And according to RAN1 agreements for active devices so far (see [3]), a large number of air interface enhancements to Rel-19 A-IoT are expected to be introduced in Rel-20 for Device 2b/C, e.g. new R2D signal for frequency synchronization / acquisition, and new D2R signal for measurement. As a result, for a UE Reader, supporting Device 2b/C implies excessive complexity in terms of R2D transmissions and/or D2R receptions when comparing to supporting only Device 1. For indoor deployments with only Device 1 (i.e. based on Rel-19 A-IoT radio interface), support for Rel-20 A-IoT air interface features means unnecessary complexity (and cost) to the UE Readers, and should thus not be mandated for the UE Readers.

- As a baseline, a UE Reader supports only A-IoT radio interface operations for Device 1 (i.e. as already specified in Rel-19).

A-IoT radio resource configuration

According to [1], for a UE Reader, spectrum relevant for A-IoT is “FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum”. And regarding A-IoT radio resource validity, “A-IoT radio resource of the UE reader is only valid within the serving cell”.

For a serving cell configured with A-IoT radio resources, we think the specification should support not only Case 1 but also Case 2 below, because Case 1 is not feasible for operators with no spectrum allocated in band n8 for 5G eMBB services:

- Case 1: The A-IoT radio resources and the uplink carrier of the serving cell are in the same band, i.e. band n8 which is currently the only band supported by A-IoT. In this case, A-IoT radio resources can be configured as part of the resources in the serving cell.

- Case 2: The A-IoT radio resources and the uplink carrier of the serving cell are not in the same band, i.e. the former in band n8 and the latter in a band other than band n8. In this case, A-IoT radio resources cannot be configured as part of the resources in the serving cell.

- For A-IoT radio resources configured in a serving cell of a UE Reader, support both of the following two cases:

      - Case 1: The A-IoT radio resources and the uplink carrier of the serving cell are in the same band.

      - Case 2: The A-IoT radio resources and the uplink carrier of the serving cell are not in the same band.

In the frequency domain, A-IoT radio resources should be respectively configured for R2D and D2R, e.g. a number of RBs for R2D and another number of RBs for D2R. Note that for D2R, although Device 1 does not care about the actual frequency location of a D2R transmission, the UE Reader needs to be aware of such information for D2R reception purposes.

Furthermore, according to [1], for D2T2-B for Device 1 (where a D2R transmission is back-scattered on an external carrier-wave, i.e. “CW”), there is “no specification support for CW control”. In our understanding, on one hand, it is true that CW is transparent to Device 1 and CW control itself can be considered a network implementation issue (and there is no need for specification support for CW control); on the other hand, a UE Reader has to be aware of the frequency location for CW transmission in order to properly perform D2R receptions (e.g. in terms of determining frequency location corresponding to any given small frequency shift factor value).

- For a UE Reader, in the frequency domain, A-IoT radio resources include the following

      - A number of RBs for R2D.

      - A number of RBs for D2R.

      - The carrier-wave frequency for back-scattering for Device 1.

In the time domain, since the total time for A-IoT radio interface operations for an inventory or command procedure is not deterministic (e.g. due to re-transmissions of R2D and corresponding D2R), the A-IoT radio resources, once allocated to the UE Reader, can be kept valid until completion of the A-IoT Session Release procedure.

- For a UE Reader, in the time domain, A-IoT radio resources can be released upon completion of the A-IoT Session Release procedure.

Message Mapping between NG and Uu

Inventory procedure and Command procedure are two procedures supported for Topology 1 in R19 A-IoT. In R20, deployment scenario 2 with Topology 2 will be discussed. Figure 3 shows a general message flow for A-IoT inventory based on RRC-based solution in topology 2 [2].

Figure 3: Message flow for A-IoT Inventory in Topology 2 - RRC-based solution

Based on the message flow, the mapping between the messages transmitted on NG and on NR Uu should be discussed. In R19, the NG message including Inventory request, Inventory response, Inventory Report, Command request and Command response. With the introduction of intermediate Node, how to construct Uu message for the corresponding NG message should be considered. One alternative is 1-1 mapping between the NG messages and Uu messages that is the corresponding RRC message are defined for each NG message. For example, RRC Inventory request is defined for Inventory request, RRC Inventory response is defined for Inventory response, and so on. Another alternative is n-1 mapping between the NG messages and Uu messages that is multiple NG messages are mapped to a Uu messages. For example, both Inventory request and Command request are mapped to RRCAIoT request message, both Inventory response and Command response are mapped to RRCAIoT response message.

- RAN2 to discuss the mapping between the NG message and Uu message is 1-1 or n-1.

In addition, inventory report aggregation which is supported in R19 should also be supported in R20. With involving of the intermediate Node, RAN2 should consider which Node (i.e., gNB or UE reader) performs the inventory report aggregation when requested by AIOTF. Either gNB or UE reader can perform the inventory report aggregation. On one hand, performing the inventory aggregation in gNB can reduce the complexity of UE reader. On the other hand, performing the inventory aggregation by UE reader can reduce the signalling overhead on Uu. To fulfil the time interval requested by AIOTF, RAN2 should discuss which node performs the Inventory report aggregation.

- RAN2 to discuss which Node (i.e., gNB or UE reader) performs Inventory report aggregation.

Conclusion

In this contribution, we discuss a few aspects of support for Deployment Scenario 2 with Topology 2, and make the following observation and proposals:

- From UE Reader perspective, A-IoT radio interface operations are either only for Device 1 or only for Device 2b/C.

- As a baseline, a UE Reader supports only A-IoT radio interface operations for Device 1 (i.e. as already specified in Rel-19).

- For A-IoT radio resources configured in a serving cell of a UE Reader, support both of the following two cases:

      - Case 1: The A-IoT radio resources and the uplink carrier of the serving cell are in the same band.

      - Case 2: The A-IoT radio resources and the uplink carrier of the serving cell are not in the same band.

- For a UE Reader, in the frequency domain, A-IoT radio resources include the following

      - A number of RBs for R2D.

      - A number of RBs for D2R.

      - The carrier-wave frequency for back-scattering for Device 1.

- For a UE Reader, in the time domain, A-IoT radio resources can be released upon completion of the A-IoT Session Release procedure.

- RAN2 to discuss the mapping between the NG message and Uu message is 1-1 or n-1.

- RAN2 to discuss which Node (i.e., gNB or UE reader) performs inventory report aggregation.

References

- RP-252894, “Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2”, Huawei, RAN#109.

- TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”, v19.0.0.

- Draft Report of 3GPP TSG RAN WG1 #122 v0.1.0, MCC Support.