3GPP TSG-RAN WG2 Meeting #131-bis                                     R2-2506971

Prague, CZ, 13-17 Oct 2025

Source:        ZTE Corporation

Title:           Discussion on MAC-2 and MAC-3 for SBFD

Agenda item:   8.11.2

Document for:  Discussion and Decision

- Introduction

The MAC open issue in Rel-19 maintenance for SBFD is summarized as below [1]:

Issues for further discussion with tdoc contributions in RAN2#131bis: - MAC-2: How to clarify "the preambleReceivedTargetPower used in Msg3 transmission power derivation is that configured for legacy RO, in the case that Msg3 is transmitted in non-SBFD symbols and SBFD RACH config option 2 is configured." - MAC-3: Whether, and (if support) how, to support the compensation for the difference in preamble received target power between SBFD RO and legacy RO (in addition to the compensation for the power ramping difference that is already supported), for ensuring preamble transmit power continuity, during RO type switching.

This contribution proposes our view on the open issue.

- Discussion

MAC-2

The principle is to allow UE to apply preambleReceivedTargetPower configured in legacy RACH resource when UE performs Msg3 transmission on non-SBFD symbol.

According to current RRC configuration structure, the RACH-ConfigCommon for SBFD RACH and the RACH-ConfigCommon for legacy RACH share same parent IE: BWP-UplinkCommon and AdditionalRACH-Config-r17, but have different parent IE sbfd-RACH-DualConfig-r19. If we follow the logic that ‘UE should select preambleReceivedTargetPower configured in xxx’, it should be ‘UE selects preambleReceivedTargetPower configured in RACH-ConfigCommon from BWP-UplinkCommon ①, or configured in RACH-ConfigCommon from AdditionalRACH-Config-r17 ②③’. However, this statement is also unclear since RACH-ConfigCommon (④⑤⑥) containing in sbfd-RACH-DualConfig-r19 is technically also configured ‘in RACH-ConfigCommon from BWP-UplinkCommon, or in RACH-ConfigCommon from AdditionalRACH-Config-r17’. So this statement cannot clearly exclude the case that preambleReceivedTargetPower should be outside sbfd-RACH-DualConfig-r19.

Therefore to reduce any unclear wording (which is also the motivation of raising MAC-2), we suggest to say ‘UE should select preambleReceivedTargetPower not configured in yyy’. The LGE’s TP looks good:

… 5> else (i.e., the received UL grant indicates that the corresponding PUSCH transmission is in non-SBFD symbols as specified in clause 11.1 of TS 38.213 [6]): 6> indicate the preambleReceivedTargetPower (not included in sbfd-RACH-DualConfig) and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE).

Proposal 1: For MAC-2, adopt the TP as LGE proposes, i.e., to clarify that when Msg3 is sent on non-SBFD symbol, the UE adopts the preambleReceivedTargetPower which is not included in sbfd-RACH-DualConfig.

MAC-3

Current preamble transmission power is determined according to the following formula:

1> if the selected PRACH occasion is of the second PRACH occasions (as defined in TS 38.213 [6]) and sbfd-RACH-SingleConfig-preambleReceivedTargetPower is configured for the Random Access Procedure: 2> set PREAMBLE_RECEIVED_TARGET_POWER to sbfd-RACH-SingleConfig-preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE. 1> else if the selected PRACH occasion is of the second PRACH occasions (as defined in TS 38.213 [6]) and sbfd-RACH-DualConfig is configured for the Random Access Procedure: 2> set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower (included in the sbfd-RACH-DualConfig) + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE. 1> else: 21> set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE.;

The power offset of power ramping counter is given as below:

6> set POWER_OFFSET_RO_TYPE to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP).

The field description of preambleReceivedTargetPower is:

preambleReceivedTargetPower The target power level at the network receiver side (see TS 38.213 [13], clause 7.4, TS 38.321 [3], clauses 5.1.2, 5.1.3). Only multiples of 2 dBm may be chosen (e.g. -202, -200, -198, ...). This field is set to the same value for different repetition numbers associated with a specific FeatureCombination.

It can be seen that the calculation of preamble transmitting power only compensates the power ramping step, not the preambleReceivedTargetPower. The reasons can be:

- The difference between power ramping step is an accumulated value (multiplied by N-1), but the difference of preambleReceivedTargetPower is a one-shot value (which only added once in the formula). So the difference of preambleReceivedTargetPower is not that significant compared with the difference of power ramping step;

- Most likely, gNB will configure the preambleReceivedTargetPower to be same in SBFD RACH option 2 and legacy, since preambleReceivedTargetPower is to describe the target power level at the network receiver side. There is no reason for NW to configure this value to be (largely) different for SBFD and non-SBFD RACH resource;

- Usually the preambleReceivedTargetPower is related to the preamble format, when the longer preamble format is configured, NW should provide the smaller preambleReceivedTargetPower. Although SBFD RACH option 2 and legacy RACH config may have different configured preamble format, the formula also contains a DELTA_PREAMBLE which is already there to compensate the power difference caused by different preamble format;

- Note that 2-step RACH config can also configure different preambleReceivedTargetPower that 4-step RACH config (since the 2-step RACH can have a separate MsgA-ConfigCommon-r16). However when 2-step falls back to 4-step, only the power ramping step is compensated, not the preambleReceivedTargetPower. This proves that the difference between preambleReceivedTargetPower is acceptable to UE, assuming NW will provide reasonable value.

Based on the above analysis, there is no need to further compensate the difference of preambleReceivedTargetPower.

Proposal 2: For MAC-3, there is no need to further compensate the difference of preambleReceivedTargetPower.

- Conclusion

In this contribution, we propose the following proposals:

Proposal 1: For MAC-2, adopt the TP as LGE proposes, i.e., to clarify that when Msg3 is sent on non-SBFD symbol, the UE adopts the preambleReceivedTargetPower which is not included in sbfd-RACH-DualConfig.

Proposal 2: For MAC-3, there is no need to further compensate the difference of preambleReceivedTargetPower.

- Reference

- List of Rel-19 SBFD MAC open issues for maintenance, Samsung, 2025-09