3GPP TSG RAN WG2 Meeting #131-bis	 R2-2506973

Prague, Czech Republic, 13-17 Oct 2025

Agenda Item:	10.4 Mobility

Source:	Fraunhofer HHI, Fraunhofer IIS

Title:	On 6G-Mobilty

Document for:	Discussion

# Introduction

Mobility is a fundamental capability of cellular systems. In 6G, mobility must evolve to support new services while addressing challenges that remain in 5G. Experience from 5G highlights the need to reduce latency, improve signalling efficiency, and ensure robust performance under frequent handovers and diverse deployment scenarios.

This contribution considers some enhancements for 6G mobility. Specifically, it discusses the maintenance of HARQ processes during handover, the design of a unified signalling framework for mobility procedures, and the use of uplink reference signal measurements, such as SRS, as a basis for mobility decisions.

# Enhancement for 6G-Mobility

## HARQ Maintenance

Hybrid Automatic Repeat Request (HARQ) has been a key mechanism in 5G to ensure reliable data transmission. By running multiple processes in parallel, each with a dedicated process ID, the system can transmit new packets while waiting for ACK/NACK feedback. This parallelism reduces delay and improves robustness, especially in high throughput scenarios.

In the current design, HARQ entities are reset at handover. The UE flushes all HARQ buffers, ongoing processes are terminated, and the target cell starts new HARQ entities. Packets awaiting retransmission or acknowledgement are lost, resulting in higher latency and lower spectral efficiency. The impact is even more severe at higher frequencies (e.g., mmWave, sub-THz) where link quality fluctuates quickly and handovers occur frequently. In LTM, where latency degradation is the primary concern, restarting HARQ retransmissions from scratch at every event is inefficient and not suitable for 6G.

For example, in a dense urban scenario with mmWave coverage, a UE may perform multiple mobility events within seconds. Resetting HARQ at each handover forces retransmission of partially delivered packets, increasing both latency and signalling overhead. Similarly, in high-speed vehicular mobility, frequent handovers would repeatedly discard useful HARQ states, degrading efficiency.

Observation 1: Current HARQ design forces the UE to start new processes at each handover, causing redundant retransmissions and higher latency.

We propose that RAN2 study HARQ maintenance in 6G mobility procedures. In this manner, the target gNB would be able to resume ongoing retransmissions seamlessly, without forcing the UE to restart from the first transmission attempt. The potential benefits are substantial. Latency can be reduced by avoiding redundant retransmissions, radio resources can be saved, and user experience can be improved in scenarios with frequent handovers.

Proposal 1: RAN2 study HARQ maintenance across handovers to allow seamless continuation of retransmissions and acknowledgements. This would reduce latency, save radio resources, and improve user experience in scenarios with frequent mobility.

## Unified Mobility Signalling

Mobility procedures rely heavily on RRC signalling. In 5G, the UE maintains a full configuration, and gNBs send RRCReconfiguration messages to indicate delta changes. However, in practice, large sets of configuration IEs are often retransmitted even if unchanged. Furthermore, the fullConfig IE can force the UE to discard its configuration entirely, introducing unnecessary latency, signalling overhead, and power consumption.

These inefficiencies are especially visible in frequent or diverse mobility scenarios. In L3 handover, long configuration lists are often repeated unnecessarily. In LTM, forcing a full reconfiguration slows down recovery from radio link failure. In conditional mobility e.g., conditional L3 handover or conditional LTM, unchanged IEs are duplicated across multiple target configurations, further increasing signalling burden.

Observation 2: Current RRC signalling repeats unchanged configuration elements and may trigger full reconfigurations, increasing latency, signalling load, and power usage.

To address this, a unified signalling design for target cell preparation and configuration is needed. The benefits are clear: smaller RRC message sizes, faster mobility execution, and reduced UE processing and power consumption.

Proposal 2: RAN2 is to ensure signalling efficiency for target cell preparation and configuration in different handover scenarios.

Proposal 3: RAN2 is to design a unified signalling framework that supports flexible and incremental configuration updates across mobility events.

## Use of SRS-Measurements for UE Mobility:

UE Mobility in 3GPP is primarily based on utilizing measurements performed on downlink reference signals. Use of uplink reference signals, such as SRS, where configured, could be to determine the measurement in the uplink could benefit from reduced need to perform measurement and reporting.

For UE positioning, the neighbouring cells can be configured to perform measurement on sounding reference signals that are configured by the serving cell for a given UE. This is already done for uplink positioning (UL-TDOA). For mobility, the neighbouring cells can perform measurement on SRS transmitted by the cell and report the measurement to the serving cell, so that the serving cell can initiate handover or send handover configuration to the UE when needed.

Observation 3:  In addition to UE reported measurements and events, the network shall also utilise existing reference signals like the SRS to perform measurement at different RAN nodes, to determine measurements and events.

It is observed that for co-located case, AI/ML models could be utilised to perform inter-frequency domain prediction. By analogy, if the SRS from UE is received at a neighbouring cell, and the neighbouring cell performs measurement, then it could also apply analogously an AI/ML model to predict measurement in an inter-frequency cell, where the SRS is received. Hence, utilising as SRS measurement at a neighbouring cell could enable an inter-frequency prediction at a target cell that is not collocated with the special cell.

Observation 4:  Inter-frequency measurement for co-located cells may be predicted using measurements of uplink reference signals like SRS.

Training such AI model on the network side needs training data and validation. To enable the network to train and perform monitoring, the UE shall indicate which of the measurements are conducted measurement and which of the measurement are predicted measurement.

Hence, we propose that RAN2 studies the use of uplink reference signal to determine the measurement and events.

Proposal 4: RAN2 studies the use of uplink reference signal (like SRS) to determine or predict uplink measurement for UE mobility in addition to current measurement and reporting framework based on downlink reference signals.

# Conclusions

In this contribution, we discussed various aspects related to enhancements for 6G mobility. Based on the discussions, we have made the following observations and proposals:

Observation 1: Current HARQ design forces the UE to start new processes at each handover, causing redundant retransmissions and higher latency.

Observation 2: Current RRC signalling repeats unchanged configuration elements and may trigger full reconfigurations, increasing latency, signalling load, and power usage.

Observation 3:  In addition to UE reported measurements and events, the network shall also utilise existing reference signals like the SRS to perform measurement at different RAN nodes, to determine measurements and events.

Proposal 1: RAN2 study HARQ maintenance across handovers to allow seamless continuation of retransmissions and acknowledgements. This would reduce latency, save radio resources, and improve user experience in scenarios with frequent mobility.

Proposal 2: RAN2 is to ensure signalling efficiency for target cell preparation and configuration in different handover scenarios.

Proposal 3: RAN2 is to design a unified signalling framework that supports flexible and incremental configuration updates across mobility events.

Proposal 4: RAN2 studies the use of uplink reference signal (like SRS) to determine or predict uplink measurement for UE mobility in addition to current measurement and reporting framework based on downlink reference signals.

# References

| [1] | TS 22.870, "Study on 6G Use Cases and Service Requirements," 3GPP, 2025-06. |
| --- | --- |