3GPP TSG RAN WG2 Meeting #131-bis	R2-2506975

Prague, Czech Republic 13-17 Oct 2025

Agenda Item:	10.2 General Aspects

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Enhancement of Public Safety Support

Document for:	Discussion

# Introduction

Public safety services increasingly rely on wireless networks to deliver critical emergency alerts, such as the Earthquake and Tsunami Warning System (ETWS) defined in 3GPP TS 23.041. In a typical current implementation, a UE displays warning message and a warning ring tone. The expectation is that the warning enables the user to take appropriate action.

Disasters like earthquakes and Tsunami have the potential not only to disrupt communication infrastructure but also may lead to collapse of infrastructure. The standard for building is not harmonized in the world. In certain regions, strict building codes and their enforcement, the warning may be sufficient for the user to take protective actions. However, there are several regions in the world where the building codes are not strict or are not enforced. There a disaster (e.g. a strong earthquake) can lead to

- Incapacitated victims – e.g. unable to call emergency services because they lie under a rubble

- Collapsed infrastructure

- Massive network load after disasters.

The problem is exacerbated in roaming scenario. In a roaming scenario, the users are typically unfamiliar with the surroundings, exit path, relief mechanism and possibly language. Furthermore, the information that a victim was at the location and requires assistance may not be known to the local rescue organization without additional information.

Observation 1: 3GPP has mechanism to alert the user of impending disaster (like earthquake and Tsunami) but does not support by default collection of information aiding rescue.

SA2 has identified several use cases TS 22.870 [1], Section 6.31, Section 7.1, Section 8.5, Section  8.11 and Section 8.12), which highlight the need for 6G to support disaster mitigation and rescue. The RAN-level impact, and specially RAN2 level impact needs to be studied.

# Proposed Way Forward

One way to improve rescue operations after disaster is to collect information about the UEs that are within the coverage of a cell. This information may be useful for rescue and search operation in the aftermath of the disaster.

One mechanism could be that the UE, upon receiving emergency alerts, be required to respond with information the UE is configured to respond. This could be handled by enhancing the emergency SMS mechanism or using SDT feature.

The information that a UE could provide, which could help the rescue operation could be:

- UE location (upon receiving alert and after a finite time), this should enable determination whether a user is trapped or not.

- Information from sensing the geographical location, (e.g. identification of locations where the buildings have collapsed).

- Information from UE sensors or wearables associated with the UE (e.g. smartwatch) could provide valuable information to aid rescue (e.g. prioritization).

A user initiated emergency call may not always be possible in critical situations. Therefore, automated means for initiating such operation needs to be specified. This may be done by associating a transmission of emergency SMS to an emergency warning system.

Of course, the information is private information under normal circumstances, but collection of such information during disaster could be a reasonable behaviour and could be subject to user’s consent. It would also be possible that the data collected could be encrypted, such that the data is only accessible by PSAPs (and the content is or some of the contents are transparent to the network operator).

Proposal 1: RAN2 shall study mechanisms to extend emergency alert systems (e.g., ETWS/PWS) to request and collect UE-assisted data (e.g., location, user status) during disasters.

# Conclusions

In this contribution, we highlighted how 6G system may be extended to support rescue services in a disaster location.

Proposal 1: RAN2 shall study mechanisms to extend emergency alert systems (e.g., ETWS/PWS) to request and collect UE-assisted data (e.g., location, user status) during disasters.

# References

| [1] | TS 22.870, "Study on 6G Use Cases and Service Requirements," 3GPP, 2025-06. |
| --- | --- |