3GPP TSG RAN WG2 Meeting #131bis            	                    R2-2506993

Prague, Czech Oct 13th – 17th, 2025

Agenda Item:	8.10.2

Source:	Xiaomi

Title:	Discussion on Rel-19 SONMDT RILs

Document for:	Discussion and Decision

Introduction

In the post RAN2 #131 meeting 38.331 ASN.1 review procedure, some RILs are indicated "Todo", which need to be brought for online decision.

In this contribution, we discuss RILs releated to MRO for NTN.

Discussion

MRO for NTN related RILs

RIL: X551

In the ASN.1 review discussion, the status of RIL: X551 is set to "PropReject". However, following offline discussions with the rapporteur, it appears that not all parties have reached a consensus on how to handle the prior agreement stating "UE suspends logging, if it cannot obtain location information"; this matter requires further discussion in the upcoming online meeting. To help companies understand the background of this issue, the agreement proposed during the [AT131][607][SONMDT] email discussion [1] is attached below. This agreement aims to address the problem where a UE cannot obtain location information when configured to perform MDT logging for NTN scenarios:

UE suspends logging, if it cannot obtain location information.

Based on the aforementioned agreement, we consider that there are two scenarios that will lead to the suspending of measurement logging. For example, logging shall be suspended when an IDC problem occurs, or when location information is unavailable. This needs to be clarified to ensure the procedure is unambiguous. However, some companies hold the view that a modification is not necessary, as the latest updates indicate that the UE only suspends logging due to IDC problems. In cases where location information is unavailable, the UE "skips" but not "suspends" logging for a single interval. This perspective is inconsistent with the current agreement, which explicitly supports logging suspension not only due to IDC problems but also due to location information unavailability. Additionally, some companies argue that the UE's "skipping" of logging for an interval is also a kind of "suspending", which aligns with the current agreement. These differing opinions require discussion and alignment during the meeting. Therefore, it is proposed that RAN2 conduct further deliberations on this matter.

If RAN2 clarifies that the UE's "skipping" of logging for an interval qualifies as "suspending", the existing procedural texts will be deemed ambiguous. For instance, the current CR only states "if measurement logging is suspended, ..." but fails to specify whether the "suspending" results from an IDC problem or the unavailability of location information. Consequently, this sentence needs to be revised for clarity. If RAN2 clarifies that the UE's "skipping" of logging for an interval does not constitute "suspending", the existing procedural texts will be inconsistent with the agreement. In this case, it is advisable to adjust the current implementation and use the term "suspend" instead of "skip". The following TP in the "Annex" clause are attached to address the two aforementioned understandings of the issue.

- If companies agree that the UE’s “skipping” of logging for an interval is a form of “suspending”, the first proposed change shall be adopted.

- If companies agree that the UE’s “skipping” of logging for an interval is not a form of “suspending”, the second proposed change shall be adopted.

To resolve the above two issues outlined above, it's proposed as below:

Proposal 1: (X551) To align with the RAN2 agreement that “the UE suspends logging if it cannot obtain location information”, RAN2 to confirm the correctness of one of the following interpretations of the current procedural text:

Option 1: The UE’s “skipping” of logging for an interval is also a form of “suspending”.

Option 2: The UE’s “skipping” of logging for an interval is not a form of “suspending”.

Proposal 2: (X551) If RAN2 agrees that the UE’s “skipping” of logging for an interval qualifies as “suspending”, RAN2 to adopt the TP (First change) proposed in Tdoc R2-2506993.

RIL: X554

- In the ASN.1 review discussion, the status of RIL: X554 is set to “PropTodo”. This decision is based on the rapporteur’s view that RAN3 has agreed to exclude the (quasi-) Earth-fixed scenario, in accordance with the conclusion from “RAN3#125bis: For the Earth-fixed case, there is no need to report the measured distance from the UE.” Nevertheless, further discussion of this RIL remains valuable, as the agreements reached by RAN2 and RAN3 appear to combine MRO for NTN and MDT enhancement for NTN. The following is an analysis of the matter:

- a) The ReferenceLocation-r17 IE (of the referenceLocation IE type) is exclusively used for (quasi-) Earth-fixed cell scenarios, whereas only the movingReferenceLocation-r18 IE is used for NTN Earth-moving cell scenarios. It is therefore strange to state that we only support Earth-moving cell scenarios while using an IE that is solely defined for (quasi-) Earth-fixed cell coverage.

- b) Although RAN3 has agreed to the conclusion that “For the Earth-fixed case, there is no need to report the measured distance from the UE”, this agreement clearly applies only to MRO optimization for NTN—not to MDT optimization for NTN. For MDT logging and reporting, RAN2 has only discussed the configuration of geographical location, with no deliberations involving the reporting of “measured distance”.

- c) If RAN2 decides to clarify that MDT logging is also restricted to Earth-moving cell scenarios, ambiguity will remain regarding how the network performs optimization when the reference location changes continuously within the elapsed time (epochTime). One potential solution is to define a geographical area using movingReferenceLocation, epochTime, and radius; however, this would increase complexity. Furthermore, defining the scope of a remote logging area would likely contradict the original objective of MDT optimization—namely, to log measurements within a specific, fixed area of a large-coverage NTN cell. Therefore, the reasonable and intuitive approach is to consider MDT optimization for NTN in (quasi-) Earth-fixed cell scenarios. Thus, the definition of the referenceLocation IE in the current specifications should be clarified and restricted to (quasi-) fixed NTN cell scenarios, and RIL: 554 shall be approved.

- In the event that RAN3 identifies other issues related to this matter, RAN2 may send a LS to RAN3 to inform RAN2’s decision. RAN3 may then provide feedback if it identifies any critical issues.

Observation 1: (X554) The legacy referenceLocation IE in the current specifications applies to (quasi-) Earth-fixed NTN cell scenarios.

Proposal 3: (X554) RAN2 to agree that MDT optimization for NTN is restricted to (quasi-) Earth-fixed NTN cell scenarios (not NTN Earth-moving cell scenarios) and adopt the TP (Third change) proposed in Tdoc R2-2506993.

Proposal 4: (X554) RAN2 to send a LS to inform RAN3 of the conclusion regarding the applicable scenarios for MDT optimization for NTN.

Conclusion

In this contribution, we propose:

Observation 1: (X554) The legacy referenceLocation IE in the current specifications applies to (quasi-) Earth-fixed NTN cell scenarios.

Proposal 1: (X551) To align with the RAN2 agreement that “the UE suspends logging if it cannot obtain location information”, RAN2 to confirm the correctness of one of the following interpretations of the current procedural text:

Option 1: The UE’s “skipping” of logging for an interval is also a form of “suspending”.

Option 2: The UE’s “skipping” of logging for an interval is not a form of “suspending”.

Proposal 2: (X551) If RAN2 agrees that the UE’s “skipping” of logging for an interval qualifies as “suspending”, RAN2 to adopt the TP (First change) proposed in Tdoc R2-2506993.

Proposal 3: (X554) RAN2 to agree that MDT optimization for NTN is restricted to (quasi-) Earth-fixed NTN cell scenarios (not NTN Earth-moving cell scenarios) and adopt the TP (Third change) proposed in Tdoc R2-2506993.

Proposal 4: (X554) RAN2 to send a LS to inform RAN3 of the conclusion regarding the applicable scenarios for MDT optimization for NTN.

Reference

[1] [AT131][607][SONMDT] Area config without having location (Ericsson)

Annex TP

First Change

While T330 is running and SDT procedure is not ongoing, the UE shall: 1> if measurement logging is suspended due to IDC problems: 2> if during the last logging interval the IDC problems detected by the UE is resolved, resume measurement logging; 1> if AreaConfigurationNTN-List is included in VarLogMeasConfig: 2> if location information is available, and is outside of all areas indicated by AreaConfigurationNTN-List; or 2> if location information is not available: 3> skip the execution of the remainder of clause 5.5a.3.2 for the current logging interval (i.e. do not perform measurement logging for this interval);

End of first Change

Second Change

While T330 is running and SDT procedure is not ongoing, the UE shall: 1> if measurement logging is suspended due to IDC problems: 2> if during the last logging interval the IDC problems detected by the UE is resolved, resume measurement logging; 1> if AreaConfigurationNTN-List is included in VarLogMeasConfig: 2> if location information is available, and is outside of all areas indicated by AreaConfigurationNTN-List; or 2> if location information is not available: 3> suspend measurement logging; 2> else if location information is available, and is inside of all areas indicated by AreaConfigurationNTN-List during the last logging interval: 3> resume measurement logging;

End of second Change

Third Change

| LoggedMeasurementConfiguration field descriptions |  |
| --- | --- |
| Omitted.... |  |
| referenceLocation Indicates the NTN coverage area reference location and is used in logged measurement for NTN (quasi-)Earth fixed cell deployment. |

End of Third Change