3GPP TSG-RAN WG2 Meeting #131bis	R2-2507003

Prague, Czech Republic, 13th – 17th October, 2025

Agenda Item:	8.11.2

Source:	Huawei, HiSilicon

Title:	Discussion on residual issues for MAC spec

Document for: Discussion and Decision

# 1	Introduction

In the post RAN2#131 meeting discussion on MAC spec, there are two open issues that need to be further discussed. Besides, we identified one more residual issue with the current MAC spec that is also discussed below.

# 2	Discussion

2.1	Issue MAC-2 on restriction for preambleReceivedTargetPower

In the post RAN2#131 discussion on MAC spec, there was an issue raised by LG to clarify the preambleReceivedTargetPower used for MSG3 transmission, and no conclusion about how to indicate it has been reached.

Rapp summary for MAC-2: All the companies consider further changes necessary, where: - five companies prefer negative statement ("not in xx"), considering the tricky point in explicitly indicating a commonly applicable parent IE for different contexts inclusively, and - four companies prefer positive statement ("in yy"), for clearer and simpler UE implementation. Proposed WF for MAC-2: To secure more time for careful review of the relevant IE structure, no correction will be considered for MAC-2 in Rapp CR. Companies are invited to submit tdoc contributions with the preferred TP for MAC-2.

We support the positive statement "preambleReceivedTargetPower included in yy" between the two alternatives, considering the UE implementation would be always based on a "positive indication" not a "negative indication". With "negative indication", the UE would anyway need to figure out the right parameter by excluding the negatively-indicated parameter from all possible parameters. The concern regarding the complexity of specification wording with positive statement should be secondary issue, also the concern seems come from the confusion of RRC IEs and RRC fields.

After further checking the RRC specification for SBFD, we support the restriction to be "preambleReceivedTargetPower included in rach-ConfigCommon" as proposed by LG (we appreciate LG raising this WF). This is because rach-ConfigCommon (with or without suffix) is the only field to be used for non-SBFD based transmission with SBFD RACH configuration Option 2. For SBFD based transmission with SBFD RACH configuration Option 2, the filed sbfd-RACH-DualConfig would be used.

Proposal for MAC-2: For MAC-2 issue, support to add the restriction as "preambleReceivedTargetPower included in rach-ConfigCommon".

2.2	Issue MAC-3 UE transmit power continuity during RO type switching

Proposed WF for MAC-3 (by Rapp): Given that MAC-3 was identified too late for sufficient offline discussion, companies are invited to submit tdoc contributions on: - whether, and (if support) how, to support the compensation for the difference in preamble received target power between SBFD RO and legacy RO (in addition to the compensation for the power ramping difference that is already supported), for ensuring preamble transmit power continuity, during RO type switching, taking into account the issue description and the TP, made by Nokia.

For this issue, we understand the intention of the proposed change is to also count the effect of different preambleReceivedTargetPower values between legacy RO and SBFD RO for SBFD RACH configuration Option 2 into the power ramping compensation. The question would be then, with the current power ramping compensation regarding different power ramping steps, would it be really needed to add further compensation regarding preamble received target power values. Note RAN2 agreeing to support power ramping compensation based on different power ramping steps was mainly by referring to the existing compensation mechanism between 2-step RACH and 4-step RACH. We think this new function/restriction would be better discussed in RAN1 when the justification would be based on performance calculation. Nevertheless on RAN1 side, RAN1 didn't even achieve consensus on the power ramping compensation at all, meaning RAN1 would accept zero compensation. It could be understood as also acceptable to RAN1 not to further optimize the current power ramping compensation mechanism in order to achieve the complete power continuity. If no further change is agreed upon the current power ramping compensation mechanism, and if a larger power ramp-down or power ramp-up than 6dB is caused by RO type switching, we can leave it to UE implementation. Thus, there seems no need to further optimize the current power continuity mechanism.

Proposal for MAC-3: For MAC-3 issue, no further optimization upon the current power ramping compensation mechanism in RAN2.

2.3	Issue MAC-x on Msg1 repetition number fallback procedure

In the current MAC for SBFD, the UE shall determine whether to select another RO type using the variable PREAMBLE_TRANSMISSION_COUNTER, i.e., determine whether PREAMBLE_TRANSMISSION_COUNTER = preambleTransMaxRO-Type + 1 is satisfied. After the RO type switching, the UE shall further determine whether to select another Msg1 repetition number also using the same variable PREAMBLE_TRANSMISSION_COUNTER if the Random Access Preamble is transmitted with repetitions, i.e., determine whether PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1 is satisfied, as following:

2> if the Random Access procedure is not completed: 3> if preambleTransMaxRO-Type is applied, and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure, and PREAMBLE_TRANSMISSION_COUNTER = preambleTransMaxRO-Type + 1: 4> if the RO_TYPE is set to 2nd-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same or higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), than the current set of Random Access resources, is available for the first PRACH occasions as defined in TS 38.213 [6]: 5> set the RO_TYPE to 1st-RO; 5> select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure; 5> if sbfd-RACH-DualConfig is configured for the Random Access procedure (see TS 38.331 [5]): 6> set PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP to PREAMBLE_POWER_RAMPING_STEP; 6> (re-)initialize the parameters specified in clause 5.1.1 for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources; 6> re-initialize PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI as specified in clause 5.1.1a; 6> set POWER_OFFSET_RO_TYPE to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP). 4> else if the RO_TYPE is set to 1st-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same or higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), than the current set of Random Access resources, is available for the second PRACH occasions as defined in TS 38.213 [6]: 5> set the RO_TYPE to 2nd-RO; 5> select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure; 5> if sbfd-RACH-DualConfig is configured for the Random Access procedure (see TS 38.331 [5]): 6> set PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP to PREAMBLE_POWER_RAMPING_STEP; 6> (re-)initialize the parameters specified in clause 5.1.1 for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources; 6> re-initialize PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI as specified in clause 5.1.1a; 6> set POWER_OFFSET_RO_TYPE to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP). 3> if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure: 4> if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or 4> if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1: 5> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available: 6> select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure; 6> initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

Since the preambleTransMaxRO-Type can be configured to values n1, n2, n4, n6, n8, n10, n20, n50, n100, n200, and the preambleTransMax-Msg1-Repetition can be configured to values n1, n2, n4, n6, n8, n10, n20, n50, n100, n200, considering the limited number for those values especially for the larger values, there could be non-negligible probability both conditions are satisfied at the same time. That is, according to the above action sequence in the MAC spec, it is possible that when the RO fallback condition is met, the Msg1 repetition number fallback condition is also satisfied, i.e., the RO type fallback and Msg1 repetition number fallback conditions are met at the same time. In this case, the UE shall first select a RACH resource set with different RO type with a same or next higher Msg1 repetition number, and then immediately (however unnecessarily) select another RACH resource set with further higher Msg1 repetition number.

According to the agreements in RAN2#130 and in RAN2#131, once the conditions for both RO type fallback and Msg1 repetition number fallback are met, UE should perform RO type switch. The Msg1 repetition number for the other RO type would be the same or the next higher Msg1 repetition number if the same Msg1 repetition number is not available. It is understood in this case, the UE shall not increase the Msg1 repetition number immediately after the RO type switch where the same Msg1 repetition number or the next higher Msg1 repetition number has been chosen and the transmission quality could be already improved by switching the RO type.

A straightforward way to solve this issue is to bypass the check for Msg1 repetition number fallback after the UE performs RO type fallback. The TP for MAC is given as below. As only one time RO type fallback is allowed for a RACH procedure, this restriction could be sufficient for this scenario (both conditions are met).

3> if the Random Access Preamble is transmitted with repetitions, and the RO type has not been changed from 1st-RO to 2nd-RO or vice versa, and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure: 4> if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or 4> if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1: 5> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available: 6> select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure; 6> initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

Proposal for MAC-x: Agree the TP of bypassing the immediate Msg1 repetition number fallback when the UE performs RO type fallback.

# 3	Conclusion

Based on the above discussion we have the following proposals for open issues:

Proposal for MAC-2: For MAC-2 issue, support to add the restriction as "preambleReceivedTargetPower included in rach-ConfigCommon".

Proposal for MAC-3: For MAC-3 issue, no further optimization upon the current power ramping compensation mechanism in RAN2.

Proposal for MAC-x: Agree the TP of bypassing the immediate Msg1 repetition number fallback when the UE performs RO type fallback.

# 4	Reference

- RP‑241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)