3GPP TSG-RAN WG2 Meeting #131bis		R2-2507013

Prague, Czech Republic, 13th – 17th Oct., 2025

Source:	vivo

Title:	List of MAC open issues for R19 mobility

Agenda Item:	8.6.1

Document for:	Discussion and Decision

- Introduction

Per instruction from the chairlady, this document is to collect MAC open issues identified for Rel-19 Mobility. More specifically, please provide here open issues that were identified but not addressed during the running CR review phase or we expect to come from other WGs.

Please provide your input no later than Friday September 19 18:00 UTC. After the deadline, I will provide a draft CR for companies to review before submission. In addition, I will also identify issues that are difficult to resolve and should be discussed by contributions, if any, as suggested by the chairlady.

- Contact information

Please provide your contact information in the table below.

| Company | Name | Email |
| --- | --- | --- |
| Vivo | Chenli | Chenli5g@vivo.com |
| ASUSTeK | Xinra | Xinra_Kung@asus.com |
| Samsung | Aby | Aby.abraham@samsung.com |
| Nokia | Endrit | endrit.dosti@nokia.com |
| Huawei | David Lecompte | david.lecompte@huawei.com |
| Qualcomm | Ozcan Ozturk | oozturk@qti.qualcomm.com |
| MediaTek | Xiaonan Zhang | Xiaonan.zhang@mediatek.com |
| Ofinno | Fasil | flatheef@ofinno.com |
| LGE | SungHoon Jung | Sunghoon.jung@lge.com |

- Open issues

  - Inter-CU LTM

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| vivo | RACH based LTM with MIMO 2TA How to address the mismatch issue that the TCI state ID indicated in the LTM Cell Switch Command MAC CE and TA value in the RAR within the RACH-based LTM procedure are associated with different TAG? [Huawei] This is a Rel-18 issue, it was already discussed and change proposals were not pursued. We should not re-open this now. [Rapp] That is true. The conclusion in Rel-18 is leave this to NW implementation, and this is one possible way for Rel-19. But it is better to explicitly discuss it. | Issue Type: Not essential Not important How to address it: based on companies contribution Issue Number: MAC-V01 |
| vivo | Shared CFRA resource in LTM CSC MAC CE FFS whether the CFRA resource provided by LTM candidate cells and assigned by source DU in LTM CSC MAC CE is an shared resource, similar as the RACH resource for early TA acquisition. [Huawei] This is a Rel-18 issue, we should not open this now. [Rapp] That is true, there is some discussion in Rel-18. But it is better to explicitly discuss and conclude it in Rel-19. | Issue Type: Not essential Not important How to address it: based on companies contribution Issue Number: MAC-V02 |

  - Event triggered L1 measurement

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| vivo | Similar as L3 measurement reporting, if an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed from or modified in the current UE configuration, the MAC entity shall remove the measurement reporting entry for this ltm-CSI-ReportConfigId from the MR_LIST, stop the periodical reporting timer, and reset the associated information (e.g. timeToTrigger) for this ltm-CSI-ReportConfigId. [Huawei] Agree but this should be initiated from TS 38.331, as well as the execution of 5.x.3, see below. [Rapp] It is a possible way. Let’s discuss it. | Issue Type: Not essential but important How to address it: based on companies contribution Issue Number: MAC-V03 |
| Samsung | There is a note as below in TS 38.321, Section 6.1.3.x Event Triggered L1 Measurement Report MAC CE NOTE: For the measurement report triggered by LTM2, the RS with Type of 00 is the current beam, which is always included in the last octet, i.e. the current RS of serving cell and the corresponding RS type are not included in the first two octets. The higkighed part is added according to the following agreement to avoid the case of UE adding current beam measurements twice in the event triggered MR. Serving beam measured result is only included at the end of a (truncated) MR MAC CE for LTM2. Howevert the highlighted part in the note is not true when the allowReportAnyBeam is not configured. In this case, the UE needs to send event triggered L1 MR with only the measurements of current beam. i.e. the octet2 to octet 2M+1 of the below MAC CE from Figure 6.1.3.x-1: event triggered L1 measurement report and truncated event triggered L1 measurement report MAC CE will not be included. When LTM2 is triggered, actual event triggered MAC CE send transmitted over air will be as below in case allowReportAnyBeam is not configured. Type can be set to 00 or 01 depending on whether entering condition or leaving condition is fulfilled. So the note needs to be updated as below: NOTE: For the measurement report triggered by LTM2, the RS with Type of 00 or 01 is the current beam, which is always included in the last octet. If the allowReportAnyBeam is not configured, current beam measurements are always included only in the second octet. Otherwise the current RS of serving cell and the corresponding RS type are not included in the first two octets. [Rapp] Thanks. Actually, my original proposal is to remove the last sentence, which was added based on companies’ feedback. My assumption is the first sentence in the Note is enough. Let’s discuss it in the CR. | Issue Type: Not essential but important How to address it: Rapporteur will include the change in the CR and discuss it during CR phase. Issue Number: MAC-S01 |
| Samsung | UE reporting of event triggered L1 MR is specified using the following variables. - BEAM_ENTERING_LIST: includes the reference signaling resource index of LTM candidate cell(s) for each ltm-CSI-ReportConfigId, for which the L1 measurement report entering conditions have been met for TTT for the triggered L1 measurement report; - BEAM_LEAVING_LIST: includes the reference signaling resource index of LTM candidate cell(s) for each ltm-CSI-ReportConfigId, for which has been reported in the (Truncated) L1 measurement report MAC CE, and the L1 measurement report leaving conditions have been met for TTT for the triggered L1 measurement report; Handling of these variables in section 5.x.4 Measurement report while the UE sends truncated MR is not proper, as the RS whose measurements are reported in the truncated L1 MR is not removed from the above lists in the below text. 2> else if the UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the Truncated L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the Truncated L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as defined in clause 6.1.3.x according to the measurement report information in the MR_LIST by selecting the RS(s) based on a decreasing order of the priority for the type of beam: RS(s) in BEAM_ENTERING_LIST, RS(s) in BEAM_LEAVING_LIST, RS(s) in BEAM_REPORTED_LIST, and other RS(s) not in these three lists; We raised the issue during the CR discussions, rapporteur has informed that the below note can handle the case: Note: After sending a Truncated L1 measurement report MAC CE, if the subsequent UL grant is still not big enough to fit all the remaining beams, it is up to UE implementation to select the beam(s) which were not included in the previous MAC CE However, this note seems to be covering only a specific case where the UE does not get enough resources to send the MR after sending a truncated MR, which is a corner case. In case, the UE sends a MR (not truncated MR) after sending the truncated MR, the UE needs to only include the RS(s) which are not reported in the truncated MR. It can be achieved by simply removing the RS(s) whose measurements are reported from the BEAM_ENTERING_LIST and BEAM_LEAVING_LIST, and the UE behaviour will be clear. The current procedural text is also not inline with the below note, as the RS for which truncated MR is send are not removed from BEAM_ENTERING_LIST, when the leaving condition of same RS is met, UE doesn’t send the event triggered MR. NOTE Z: When a measurement report is triggered by entry condition for one or more RS(s), and included in the BEAM_ENTERING_LIST, another measurement report is triggered by leaving condition for the same RS(s), all the corresponding measurement reports are cancelled. When a measurement report is triggered by leaving condition for one or more RS(s), and included in the BEAM_LEAVING_LIST, another measurement report is triggered by entry condition for the same RS(s), all the corresponding measurement reports are cancelled. So we suggest to capture the behaviour as below highlighted without any ambiguities. 2> else if the UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the Truncated L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the Truncated L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as defined in clause 6.1.3.x according to the measurement report information in the MR_LIST by selecting the RS(s) based on a decreasing order of the priority for the type of beam: RS(s) in BEAM_ENTERING_LIST, RS(s) in BEAM_LEAVING_LIST, RS(s) in BEAM_REPORTED_LIST, and other RS(s) not in these three lists; 3>include the RS(s) from BEAM_ENTERING_LIST whose measurements are reported into BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId. 3> remove the RS(s) whose measurements are reported from BEAM_ENTERING_LIST and/or BEAM_LEAVING LIST for this ltm-CSI-ReportConfigId. If there is still a differet understanding, please mark for discussion in the next meeting. [Huawei] Agree with Samsung, but with some refinement of the TP. Then, the note is useless and can be removed. [Ofinno] Agree with Samsung. We indicated this during the CR discussion as well and think the existing note for the subsequent UL grant not being enough does not address the procedure completely. [Rapp] Based on our previous discussion and conclusion, the understanding “In case, the UE sends a MR (not truncated MR) after sending the truncated MR, the UE needs to only include the RS(s) which are not reported in the truncated MR” is not correct. In this case, UE should report the whole MR MAC CE, rather than include the RS which are not reported. That is why we didn’t capture the suggested TP in the normative text. We have discussed that the beam could be removed from BEAM_ENTERING_LIST only when the whole MR MAC CE is sent. Thus current CR could work well, there is no need to introduce any optimization. Regarding the understanding “The current procedural text is also not inline with the below note, as the RS for which truncated MR is send are not removed from BEAM_ENTERING_LIST, when the leaving condition of same RS is met, UE doesn’t send the event triggered MR.” The triggered beam should be evaluated on the leaving condition before the normal MR is sent, and when the leaving condition is met, UE won’t sent the MR as captured in the note, which has been discussed based on Apple’s proposal before, which is reasonable and there is no issue forseen. Considering several companies have same understanding, let’s discuss it to align companies’ understanding. | Issue Type: Not essential but important How to address it: based on companies contribution Issue Number: MAC-S02 |
| Huawei | 5.x.2 does not have a single "The MAC entity shall" or "The UE shall", it is written like stage 2. Either it contains a UE requirement, and then it should be specified with "The MAC entity shall" or "The UE shall" or it should be removed. At first glance, it does not contain any requirement, so it could be removed. [Rapp] In MAC, there are a many places mentioning “The UE shall”. By the way, I didn’t found the information in this section is captured in stage-2. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Proposal from Proponent is welcome. Issue Number: MAC-H01 |
| Huawei | In 5.x.3.1, there is: The UE maintains the following UE variables for event triggered L1 measurement and report procedure: [...] - candidateSpecificOffset: offset for event condition that is applicable for all the reference signals belonging to the candidate cell with the candidate cell ID ltm-CandidateReportConfigId; - candidateSpecificOffsetS: offset for event condition that is applicable for all the reference signals belonging to the serving cell with the candidate cell ID ltm-CandidateReportConfigId. These two things are clearly not UE variables, so they have nothing to do there, they should be moved to 5.x.1. [Rapp] Maybe a better way is to add the below description for this two parameters “The RRC configures the following parameters in the LTM-CSI-ReportConfig for event triggered L1 measurement and corresponding reporting procedure:”. | Issue Type: Not essential not important How to address it: Rapporetur will address it in the updated CR. Issue Number: MAC-H02 |
| Huawei | Generally, the description of variables in 5.x.3 requires the reader to put back words in the correct order. This should be fixed. (can provide a TP). [Rapp] Not sure about your comments. Let’s see the TP. | Issue Type: Not essential not important How to address it: Discuss the possible wording update during the CR phase. Companies are welcome to provide views by then. Issue Number: MAC-H03 |
| Huawei | In 5.x.3.1: The MAC entity shall for LTM event evaluation procedure: 1> for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig: in 5.x.4: For the event triggered L1 measurement reporting, for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig, the MAC entity shall: However: - LTM-CSI-ReportConfig contains a single ltm-CSI-ReportConfigId, so each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig does not make any sense - there are multiple LTM-CSI-ReportConfig, configured in ltm-CSI-ReportConfigToAddModList-r18 in each serving cell, plus one in LTM-Candidate, 5.x.3 should not be executed for every LTM-CSI-ReportConfig (e.g., it should not be executed for the ones that are for reporting on PUSCH/PUCCH and for CSI reporting). The procedure text should say for which LTM-CSI-ReportConfig the UE shall execute 5.x.3. Since this is determined by RRC parameters, procedure text in TS 38.331 should indicate to MAC for which LTM-CSI-ReportConfig to perform measurement and executed 5.x.3, and the MAC procedure text could be 1> for each ltm-CSI-ReportConfigId for which upper layers require to perform L1 measurement and event-triggered report: [Rapp]: Regarding your comments on the first part, i.e. LTM-CSI-ReportConfig contains a single ltm-CSI-ReportConfigId, it is correct. I will update the specification accordingly. Regarding the cross-layer indication, it could be further discussed based on companies’ contributions. | Issue Type: Not essential not important How to address it: Rapporetur will address the first part in the updated CR. Regarding the cross-layer indication, it can be discussed based on companies contribution Issue Number: MAC-H04 |
| Huawei | In 5.x.3, there is The MAC entity shall for LTM event evaluation procedure: 1> for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig: 2> if the corresponding ltm-ReportConfigType is set to eventTriggered and there is ltm-EventTriggeredReportContent configuration: [..] 2> if the entry condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams, i.e. reference signalling associated with SSB-Index or NZP-CSI-RS-ResourceID in the LTM-CSI-ResourceConfig associated with the LTM-CSI-ReportConfig, which is not in the BEAM_ENTERING_LIST and not in the BEAM_REPORTED_LIST, for the measurement from lower layer during TTT defined for this event: [...] The highlighted condition if the first 2> bullet excludes any LTM-CSI-ReportConfig which is not for L1 measurement and event-triggered reporting, but it does not exclude them for what is after the other 2> bullets, while none of what is there should apply to them. Acording to our above suggestion, 5.x.3 should be made applicable only to what is indicated by RRC, and then the bullet 2> can be removed. [Rapp]: This issue was discussed before. The UE behaviour under this highlighted “2>” is for event triggered report, as there is a condition on “there is ltm-EventTriggeredReportContent configuration”, while the other “2>” doesn’t apply the corresponding UE behaviour, we don’t see any issue here. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Proposal from Proponent is welcome. Issue Number: MAC-H05 |
| Huawei | In 5.x.3, there is: 3> if the eventLTM2 is configured in the corresponding ltm-CSI-ReportConfigId: 4> consider only the current beam of serving cell, i.e. the beam which corresponds to the RS configured in the indicated TCI State or the RS QCLed with the RS configured in the indicated TCI State indicated by TCI State in the serving cell as defined in clause 5.1.5 in TS 38.214, with the same RS type as the beam of LTM candidate cell, i.e. the RSs configured in LTM-CSI-ResourceConfig which associated with this ltm-CSI-ReportConfigId, to be applicable; 3> if the eventLTM3, eventLTM4, or eventLTM5 is configured in the corresponding ltm-CSI-ReportConfigId: 4> if ltm-CandidateReportConfigList is configured: 5> consider any beam of LTM candidate cell (except the serving cell) configured in ltm-CandidateReportConfigList, i.e. the RSs configured in LTM-CSI-ResourceConfig which is associated with this ltm-CSI-ReportConfigId, to be applicable; The text is overcomplicated. It has "in the indicated TCI State indicated by TCI State", which does not seem to make any sense. Besides, it is unclear which LTM-CSI-ResourceConfig is meant here, it should be the one indicated by ltm-ResourcesForChannelMeasurement. We can try to come up with a clearer wording. [Rapp]: This part was discussed and decided in RAN1. The candidate cell for LTM2 in the current specification is used to determine the RS type of current serving cell. I don’t see any issue here. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Proposal from Proponent on the TP is welcome. Issue Number: MAC-H06 |
| Huawei | In 5.x.3, there is: 2> if the entry condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams, i.e. reference signalling associated with SSB-Index or NZP-CSI-RS-ResourceID in the LTM-CSI-ResourceConfig associated with the LTM-CSI-ReportConfig, which is not in the BEAM_ENTERING_LIST and not in the BEAM_REPORTED_LIST, for the measurement from lower layer during TTT defined for this event: 3> if the MR_LIST does not include a measurement reporting entry for this ltm-CSI-ReportConfigId (a first RS triggers the event): 4> include a measurement reporting entry in the MR_LIST for this ltm-CSI-ReportConfigId; 3> include the SSBRI or CRI of the concerned beam(s) in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId; However, BEAM_ENTERING_LIST and BEAM_REPORTED_LIST only exists for items in the MR_LIST, so testing BEAM_ENTERING_LIST and BEAM_REPORTED_LIST first, and later MR_LIST is not logical, and the opposite of what is done for RRC measurements. The highlighted conditions should be removed and put before the second 3> bullet. [Rapp]: No issue is forseen. For the beams in BEAM_ENTERING_LIST and not in the BEAM_REPORTED_LIST, there is no need to evaluate its entry condition, so this highlighted condition is absolutely essential. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Proposal from Proponent is welcome. Issue Number: MAC-H07 |
| Huawei | In 5.x.3, there is: 3> if the beam is in BEAM_LEAVING_LIST: 4> remove the concerned beam(s) in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId; 4> include the SSBRI or CRI of the concerned beam(s) in the BEAM_REPORTING_LIST for this ltm-CSI-ReportConfigId; However, it is unclear "the beam" refers to what beam. This should be: 3> if one or more beam that fulfil the entry condition is in BEAM_LEAVING_LIST of the ltm-CSI-ReportConfigId: Same issue there (and same solution): 2> else if the leaving condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams included in the BEAM_ENTERING_LIST or BEAM_REPORTED_LIST for the measurement from lower layer during TTT defined for this event: 3> if the beam is in BEAM_ENTERING_LIST: 4> remove the concerned beam(s) in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId; 3> if the beam is in BEAM_REPORTED_LIST: 4> remove the concerned beam(s) in the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId; 4> include the SSBRI or CRI of the concerned beam(s) in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId; [Rapp] No issue is forseen. We could consider to change “the beam” to “the concerned beam” or some other better wording, but I think it is crystal clear now, we can infer it from the context. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Proposal from Proponent is welcome. Issue Number: MAC-H08 |
| Huawei | In 5.x.3, there is: 3> initiate the measurement reporting procedure, as specified in 5.x.4. 4> initiate the measurement reporting procedure, as specified in 5.x.4; 3> initiate the measurement reporting procedure, as specified in 5.x.4. However, in 5.x.4, the procedure is expected to be a loop when L1 measurement was triggered for an ltm-CSI-ReportConfigId and a corresponding non-truncated MR MAC CE was not sent yet. So this should be changed to: 3> consider L1 measurement reporting to be triggered for the ltm-CSI-ReportConfigId. [Rapp] OK. Let’s update it. | Issue Type: Not essential not important How to address it: Rapporetur will address it in the updated CR. Issue Number: MAC-H09 |
| Huawei | In 5.x.4, there is: For the event triggered L1 measurement reporting, for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig, the MAC entity shall: 1> if at least one L1 measurement report has been triggered as specified in 5.x.3 and not cancelled: Accordingly, it looks like what follows needs to be executed "for each ltm-CSI-ReportConfig" whenever L1 measurement reporting has been triggered on any ltm-CSI-ReportConfig. This is clearly incorrect. It should be: For each ltm-CSI-ReportConfigId, the MAC entity shall: 1> if L1 measurement reporting is has been triggered for the ltm-CSI-ReportConfig as specified in 5.x.3 and not cancelled: [Rapp] Thanks. As mentioned above, let’s change it as: For the event triggered L1 measurement reporting, for each ltm-CSI-ReportConfigId included in the serving cell, the MAC entity shall: 1> if at least one L1 measurement report has been triggered for the ltm-CSI-ReportConfigId as specified in 5.x.3 and not cancelled: | Issue Type: Not essential not important How to address it: Rapporetur will address it in the updated CR. Issue Number: MAC-H10 |
| Huawei | In 5.x.4, there is For the event triggered L1 measurement reporting, for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig, the MAC entity shall: 1> if at least one L1 measurement report has been triggered as specified in 5.x.3 and not cancelled: When the non-truncated MR is sent, there is: 3> cancel the triggered L1 measurement report; So when the periodic timer reporting expires, the "L1 measurement report" has previously been cancelled, then accoding to the above no MAC CE will be sent. To avoid this issue, we could change to: For each ltm-CSI-ReportConfigId, the MAC entity shall: 1> if L1 measurement reporting is has been triggered for the ltm-CSI-ReportConfig as specified in 5.x.3: [...] 3> consider L1 measurement report not to be triggered for the ltm-CSI-ReportConfig; [Rapp]: This was discussed before, while the condition “and not cancelled” is needed due to the below note: NOTE Z: When a measurement report is triggered by entry condition for one or more RS(s), and included in the BEAM_ENTERING_LIST, another measurement report is triggered by leaving condition for the same RS(s), all the corresponding measurement reports are cancelled. When a measurement report is triggered by leaving condition for one or more RS(s), and included in the BEAM_LEAVING_LIST, another measurement report is triggered by entry condition for the same RS(s), all the corresponding measurement reports are cancelled. The other wording suggestion, which is similar as above, will be considered in the updated version. | Issue Type: Not essential not important How to address it: No issue on the condition of “not cancelled” from Rapporteur point of view. And Rapporetur will address the other wording update in the updated CR. Issue Number: MAC-H11 |
| Huawei | In 5.x.4, there is: 4> stop the periodical reporting timer, if running; However, there is no case where this timer can be running when this bullet (2 occurrences) is executed, so it should be removed. [Rapp]: it is clear in the spec that this timer will be running in case the reportAmount is configured, e.g. 5> restart the periodical reporting timer with the value of reportInterval for this ltm-CSI-ReportConfigId as defined within the corresponding LTM-CSI-reportConfig; So no issue is forseen from Rapporteur point of view. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Proposal from Proponent is welcome. Issue Number: MAC-H12 |
| Huawei | In 5.x.4, there is: 4> if at least one L1 measurement report associated with the ltm-CSI-ReportConfigId has been triggered due to the reason other than the expiry of the periodical reporting timer and the L1 measurement report is not cancelled: 5> set the MR_SENT_COUNTER to 0 for this ltm-CSI-ReportConfigId; This text is a little complicated, it could be much easier to read if the MR_SENT_COUNTER would be set to 0 in 5.x.3.1 (i.e., when L1 reporting is triggered for an ltm-CSI-ReportConfig, except for the case of periodic reporting timer expiry). [Rapp]: This has been extensively discussed during CR phase. If we changed it to 5.x.3.1, there will be problem for the case that the L1 MR is triggered at T1, while it is cancelled before UL grant at T2. So let’s keep it as it is. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Issue Number: MAC-H13 |
| OPPO | In 5.4.4, the following proedure is captured: As long as at least one SR is pending, the MAC entity shall for each pending SR: 1> if the MAC entity has no valid PUCCH resource configured for the pending SR; and 1> if there is no ongoing RACH-less LTM cell switch; and 1> if rach-LessHO is not configured: 2> initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR. In 5.x.4, the highlighted part is not needed as legacy SR procedure already specified the UE behavior on RACH triggering. 2> else: 3> if the dedicated SR configuration for L1 measurement report MAC CE transmission is configured: 4> trigger the SR using the dedicated SR configuration for L1 measurement report; 3> else: 4> initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR; 3> stop the periodical reporting timer for this ltm-CSI-ReportConfigId, if running. To avoid the duplication, we can change is as: 2> else: 3> if the dedicated SR configuration for L1 measurement report MAC CE transmission is configured: 34> trigger the SR using the dedicated SR configuration for L1 measurement report; 3> else: 4> initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR; 3> stop the periodical reporting timer for this ltm-CSI-ReportConfigId, if running. [Rapp]: The description in 5.x.4 is for measurement report, while what you mentioned in 5.4.4 is only for the case “rach-LessHO is not configured”. I assume MR is not covered by this case. But further elaboration is welcome. Thanks. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Proposal from Proponent is welcome. Issue Number: MAC-O01 |
| OPPO | In 5.4.4, there are several note captured to clarify the SR/PUCCH resource handling for the cases that PUCCH resources are overlapped among the triggered SRs NOTE 1: Except for the cases specified in NOTE 3 below, the selection of which valid PUCCH resource for SR to signal SR on when the MAC entity has more than one overlapping valid PUCCH resource for the SR transmission occasion is left to UE implementation. NOTE 3: When the MAC entity has pending SR for SCell beam failure recovery and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery of a BFD-RS set) overlapping with PUCCH resource for SCell beam failure recovery for the SR transmission occasion, the MAC entity considers only the PUCCH resource for SCell beam failure recovery as valid. When the MAC entity has pending SR for beam failure recovery of a BFD-RS set of Serving Cell and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery) overlapping with PUCCH resource for beam failure recovery of that BFD-RS set for the SR transmission occasion, the MAC entity considers only the PUCCH resource for beam failure recovery of that BFD-RS set as valid. NOTE 6: When the MAC entity has PUCCH resource for pending SR for SCell beam failure recovery overlapping with PUCCH resource for pending SR for beam failure recovery of a BFD-RS set for the SR transmission occasion, it is up to UE implementation to select PUCCH resource for SCell beam failure recovery or PUCCH resource for beam failure recovery of a BFD-RS set. For pending SR for event triggered L1 MR MAC CE, similer text is needed for charlfication, especially for the case that PUCCH resource for pending SR for event triggered L1 MR MAC CE has overlapping with PUCCH resource for beam failure recovery. [Rapp]: As we have no conclusion on how to handle the case when pending SR for event triggered L1 MR MAC CE overlapping with other PUCCH resource, this issue is wroth to be discussed. | Issue Type: Not essential How to address it: Based on companies’ contribution. Issue Number: MAC-O02 |
| NEC | In 5.x.3.1, when the entry condition is fulfilled for one beam for TTT, the UE includes the the beam in the BEAM_ENTERING_LIST. Additionally, if the beam is in BEAM_LEAVING_LIST, the UE should also include the beam in the BEAM_REPORTING_LIST. This results in that the beam can be include in two beam lists, i.e., the BEAM_ENTERING_LIST and BEAM_REPORTING_LIST, which is not correct because we understand that one beam can only be included in one beam list: 2> if the entry condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams, i.e. reference signalling associated with SSB-Index or NZP-CSI-RS-ResourceID in the LTM-CSI-ResourceConfig associated with the LTM-CSI-ReportConfig, which is not in the BEAM_ENTERING_LIST and not in the BEAM_REPORTED_LIST, for the measurement from lower layer during TTT defined for this event: 3> if the MR_LIST does not include a measurement reporting entry for this ltm-CSI-ReportConfigId (a first RS triggers the event): 4> include a measurement reporting entry in the MR_LIST for this ltm-CSI-ReportConfigId; 3> include the SSBRI or CRI of the concerned beam(s) in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId; 3> if the beam is in BEAM_LEAVING_LIST: 4> remove the concerned beam(s) in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId; 4> include the SSBRI or CRI of the concerned beam(s) in the BEAM_REPORTING_LIST for this ltm-CSI-ReportConfigId; So we suggest to update the text as: 3> if the beam is not in BEAM_LEAVING_LIST: 4> include the SSBRI or CRI of the concerned beam(s) in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId; 3> if the beam is in BEAM_LEAVING_LIST: 4> remove the concerned beam(s) in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId; 4> include the SSBRI or CRI of the concerned beam(s) in the BEAM_REPORTING_LIST for this ltm-CSI-ReportConfigId; [Rapp]: Thanks. That is true. Let’s fix it. | Issue Type: Not essential not important How to address it: Rapporetur will address it in the updated CR. Issue Number: MAC-W01 |
| Qualcomm | Event LTM2 handling Clause 5.x.3.1 and 5.x.4 in the current MAC CR don’t have sufficient description on Event LTM2. - [Clause 5.x.3.1] MR_LIST, BEAM_ENTERING_LIST, BEAM_LEAVING_LIST, and BEAM_REPORTED_LIST only mention RSs from candidate cell(s) that have fulfilled the event condition for TTT. The current beam from serving cell which triggered the event condition for Event LTM2 needs to be captured. - [Clause 5.x.4] Correspondingly, the procedures for regular L1 MR MAC CE do not capture the Event LTM2 correctly, i.e., the following is not applicable to Event LTM2. 3> include the SSBRI or CRI in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId, if any, into the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId; 3> clear the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId; 3> clear the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId; [Rapp]: Thanks. This issue indeed needs a solution. Let’s discuss it. | Issue Type: Not essential but important How to address it: Based on companies’ contribution. Issue Number: MAC-Q01 |
| Qualcomm | Type01/10/11 beam reporting description in regular L1 MR MAC CE The regular L1 MR MAC CE procedure in clause 5.x.4 doesn’t have any description on reporting Type01/10/11 beams at all. [Rapp]: No issue is forseen. The corresponding description has been already captured in 6.1.3.x, while there is no Type 01/10/11 in 5.x.4. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Issue Number: MAC-Q02 |
| Qualcomm | Minimum truncated L1 MR MAC CE size The following statement in Clause 6.1.3.x doesn’t cover neither (i) the Event LTM2 case (the triggered RS is same as the current RS) nor (ii) the minimum size when reportCurrentBeam is not configured. For a truncated event triggered L1 measurement report MAC CE, at least the following fields should be included: report ID field, at least one triggered RS with corresponding measured quantity, and the current RS of serving cell as described in TS 38.215 [24] with corresponding measured quantity, if UE is configured to report the measurement result of current RS of the serving cell by reportCurrentBeam. [Rapp]: For i), there is another Note. As the current RS is the triggered RS, according to the note, it is clear that only report ID + current RS of serving cell are included. For ii) on the minimum size when reportCurrentBeam is not configured, as the current RS is included only when reportCurrentBeam is configured. If this parameter is not configured, it is clear that only report ID field, and at least one triggered RS with corresponding measured quantity will be included according to current specification. With this, no issue is forseen. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Proposal from Proponent is welcome. Issue Number: MAC-Q03 |
| Lenovo | The following is described in 5.18.x. UE keeps SP CSI-RS/CSI-IM resource after cell switch. And UE will change the states from activated to deactivated for SP CSI-RS/IM resource. After discussing with RAN1 colleague, the early CSI acquisition related to mobility is one shot. Therefore, RAN2 needs to further check the maintenance of CSI resource for early CSI acquisition. Besides SP CSI resource, periodic CSI-RS/CSI-IM resource may be configured to UE as well. Therefore, we need to check both SP CSI resource and periodic CSI-RS/CSI-IM resource together which also impact RRC specification. 5.18.x: After reconfiguration with sync that is triggered by LTM, the configured Semi-Persistent CSI-RS/CSI-IM resource sets for all candidate cell(s), except the target cell, are deactivated. After CSI reporting at the target cell after or during cell switch triggered by LTM as specified in clause 5.2.4a in TS 38.214, the configured Semi-Persistent CSI-RS/CSI-IM resource sets for the target cell are deactivated. [Rapp]: After further checking the MAC and RRC specification, SP CSI resource is configured in LTM-CSI-ResourceConfig, which is in LTM configuration. It is common CSI configuration, which cannot be released. That is why only “deactivated” is performed after switching to the target cell. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Proposal from Proponent is welcome. Issue Number: MAC-B01 |
| LGE | In 5.x.3.1, regarding the yellow part below, the entering beam should be added back to BEAM_REPORTED_LIST (BRL) only in case the beam has been removed from the BRL due to fulfilling leaving condition. To see this, let us take an example cases below: - Case a) B1_type10 (i.e., B1 in BRL) B1_type01 if B1 becomes type00, B1 should added back to BRL. - Case b) B1_type00 (i.e. B1 not in BRL) B1_type01if B1 becomes type00, B1 should not be added into BRL In case a) it is correct that the entering beam B1 should be added back to BRL because B1 was originally in BRL but it has beem removed from BRL, but in case b), the entering beam B1 should not be added back to BRL because the B1 was not in BRL from the beginning. But the current yellow text does not distinguish these two cases. As a result, if it is the case b), the resulting BRL incorrectly contains B1, even though B1 has not reported as type00 before. Curret text in clause 5.x.3.1 …. 2> if the entry condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams, i.e. reference signalling associated with SSB-Index or NZP-CSI-RS-ResourceID in the LTM-CSI-ResourceConfig associated with the LTM-CSI-ReportConfig, which is not in the BEAM_ENTERING_LIST and not in the BEAM_REPORTED_LIST, for the measurement from lower layer during TTT defined for this event: 3> if the MR_LIST does not include a measurement reporting entry for this ltm-CSI-ReportConfigId (a first RS triggers the event): 4> include a measurement reporting entry in the MR_LIST for this ltm-CSI-ReportConfigId; 3> include the SSBRI or CRI of the concerned beam(s) in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId; 3> if the beam is in BEAM_LEAVING_LIST: 4> remove the concerned beam(s) in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId; 4> include the SSBRI or CRI of the concerned beam(s) in the BEAM_REPORTING_LIST for this ltm-CSI-ReportConfigId; … Proposed change: … 2> if the entry condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams, i.e. reference signalling associated with SSB-Index or NZP-CSI-RS-ResourceID in the LTM-CSI-ResourceConfig associated with the LTM-CSI-ReportConfig, which is not in the BEAM_ENTERING_LIST and not in the BEAM_REPORTED_LIST, for the measurement from lower layer during TTT defined for this event: 3> if the MR_LIST does not include a measurement reporting entry for this ltm-CSI-ReportConfigId (a first RS triggers the event): 4> include a measurement reporting entry in the MR_LIST for this ltm-CSI-ReportConfigId; 3> include the SSBRI or CRI of the concerned beam(s) in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId; 3> if the beam is in BEAM_LEAVING_LIST: 4> remove the concerned beam(s) in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId; 4>if the SSBRI or CRI of the concerned beam(s) has been removed in BEAM_REPORTING_LIST 5> include the SSBRI or CRI of the concerned beam(s) in the BEAM_REPORTING_LIST for this ltm-CSI-ReportConfigId; [Rapp]: It is true. This condition should be added. | Issue Type: Not essential not important How to address it: Rapporetur will address it in the updated CR. Issue Number: MAC-L01 |

  - Conditional intra-CU LTM

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| vivo | How to handle the CLTM TAT of target cell when receiving an LTM Cell switch Command MAC CE with valid TA? Which is related the previous MAC-14 open issue. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: MAC-V04 |
| vivo | When current serving cell is an LTM candidate cell, whether UE maintain and update its CLTM TAT based on legacy Timing Advance Command MAC CE? [Huawei] The UE could be allowed to use the valid TA value it has for the serving cell, this could be a note. [Rapp]: This has not been concluded. Let’s discuss it. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: MAC-V05 |
| ASUSTeK | If the source cell is an LTM candidate cell, upon Cell switch, does the UE store the valid TA and update its CLTM TAT of the source cell for possible RACH-less CLTM back to the source cell (e.g., if the CLTM TAT for the Source Cell is not running at the time upon Cell switch)? [Rapp]: This issue is same as above issue raised by vivo, i.e. MAC-V05. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: MAC-K01 |
| ASUSTeK | In the last meeting, it was agreed that if the UE receives more TAs beyond its capability, it’s up to UE implementation to decide which one is released. Since the NW does not know which TA is released, whether the UE needs to report currently stored TAs to the NW to avoid unnecessary RA for early UL sync? [Huawei] The conclusion was to do nothing else, it is up to the network to avoid this. [Rapp]: I assume the conclusion didn’t imply it is up to the network to avoid it. While it is indeed a solution, and another solution is to inform network. Let’s discuss it. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: MAC-K02 |
| Nokia | Although the TAs of other candidates are maintained, the TA of the previous serving cell, which may also be a candidate, is discarded as it is maintained by the servingCell TAT and not the candidate TAT. For example, the corresponding candidate TAT may not have been started or it may have not been restarted upon reception of a serving cell TAC MAC CE. To address the issue: FFS whether to store the previous serving cell TA for the corresponding candidate and remaining TAT time in the CLTM TAT of the corresponding candidate upon cell switch or, as discussed above by vivo, maintain and update the cooresponding CLTM TAT based on reception of legacy TAC MAC CE for the serving cell. The former approach allows maintaining only one timer for the cell, the legacy TAT when it is serving and the candidate TAT when not. [Rapp]: This issue is same as above issue raised by vivo and ASUSTek, i.e. MAC-V05, and MAC-K01 | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: MAC-N01 |
| Huawei | In 5.y.3 there is The MAC entity shall: 1> if the event for conditional LTM is satisfied based on L1 measurements as specified in clause 5.y.2: 2> select the SSB(s) or CSI-RS(s) corresponding to the SSB-Index or NZP-CSI-RS-ResourceID in the LTM-CSI-ResourceConfig associated with the LTM-CSI-ReportConfig in which the satisfied event is included; This seems to say the UE selects multiple RSs, if multiple RSs fulfil the event. Perhaps it should be "an SSB". [NEC] we agree with the issue. And we think even we change it to “an”, the sentence still doesn’t reflect the agreement “For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition” , as there may be multiple beams associated with the LTM-CSI-ResourceConfig, and the UE must select one fulfiling the execution condition. We think we can, for example, change the sentence to: 2> select the SSB or CSI-RS satisfying the entry condition for the event associated with ltm-CSI-ReportConfigId for TTT as specified in clause 5.y.2 and corresponding to the SSB-Index or NZP-CSI-RS-ResourceID in the LTM-CSI-ResourceConfig associated with the LTM-CSI-ReportConfig in which the satisfied event is included; [Rapp]: The intention is to select multiple beams here, while the UE will determine one of the selected beam(s) when checking the CG related condition later, e.g. “if the ltm-Candidate-TimeAlignmentTimer associated with the CLTM target cell is running in the first available CG occasion corresponding to one of the selected SSB/CSI-RS for initial uplink transmission according to clause 5.8.2”. If only one beam is selected in 5.y.3, while the condition related to CG resource for RACH-less is not satisfied for this beam, then, RACH-based CLTM should be peformed. Therefore, to increase the chances of using RACH-less CLTM, multiple eligible beams will be selected in advance. This point has been discussed before. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Issue Number: MAC-H14 |
| Huawei | 2> indicate to upper layers that the event for the LTM cell switch procedure is triggered, and the Target Configuration ID, corresponding to ltm-CandidateId minus 1, for which the associated L1 measurement based event is satisfied. The text is unnecessarily complicated. Suggest 2> indicate to upper layers to trigger LTM cell switch to the LTM candidate cell for which the event is satisfied [Rapp]: Actually, this issue has also been discussed. The target Configuration ID is also needed for RRC to apply the corresponding candidate configuration. The suggested wording is to re-white the Rel-18 procedure as in 5.18.35, which doesn’t include the target Configuration ID. So there is no issue forseen. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Issue Number: MAC-H15 |
| OPPO | How to handle CLTM TAT if the CLTM execution condition is released but candidate cofiugration is kept, i.e. NW reconfig CLTM to LTM candidate. [Rapp]: This issue is similar as above issue raised by vivo, i.e. MAC-V04. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: MAC-O03 |
| OPPO | In 5.12, the highlighted condition is not necessary to be captured. The CLTM TAT will be stopped if CLTM candidate is released, which means the CLTM candidate config is always remained if there is running CLTM TAT. Suggest to remove the highlighted condition. 1> if upper layers indicate SCG deactivation and bfd-and-RLM with value true is configured for the deactivated SCG: 2> stop (if running) all timers except beamFailureDetectionTimer associated with PSCell and timeAlignmentTimers. 1> else if upper layers indicate the reset is triggered by conditional LTM, or triggered by LTM while there is CLTM candidate configuration(s), or triggered by handover while there is CLTM candidate configuration(s): 2> stop (if running) all timers, except MBS broadcast DRX timers, ltm-Candidate-TimeAlignmentTimer, and ltm-Candidate-TimeAlignmentTimerTAG2, if configured; 2> consider all timeAlignmentTimers as expired and perform the corresponding actions in clause 5.2; 1> else: 2> stop (if running) all timers, except MBS broadcast DRX timers; 2> consider all timeAlignmentTimers, inactivePosSRS-TimeAlignmentTimer, and cg-SDT-TimeAlignmentTimer, if configured, as expired and perform the corresponding actions in clause 5.2; [Rapp]: Assuming the current specification is correct. If we removed the highlighted condition, it seems UE will peform the behaviour when SCG is deactivated, which is not expected. Not sure whether the clarification addressed you concern. Otherwise, more detailed elaboration is welcome. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Proposal from Proponent is welcome. Issue Number: MAC-O04 |
| MediaTek | There is no clear guidance that UE will perform RACH-based LTM after CLTM recovery in stage-3 spec. In RRC spec, two different conditions (CLTM recovery, fulfilment of CLTM condition) have the same action: 5.3.5.18.6 LTM cell switch execution …… 1> else (LTM cell switch triggered upon cell selection performed while timer T311 was running or upon the fulfilment of LTM cell switch execution conditions (as specified in clause 5.3.5.18.x): 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.5.18.x or 5.3.7.3) according to clause 5.3.5.3;. In MAC spec, either 5.18.35 (Enhanced) LTM Cell Switch Command or 5.y.3 Conditional LTM execution seems not reflect the action for CLTM recovery case. And it is not clear if CLTM fast recovery will go to 5.y.3 Conditional LTM execution, as the condition of CLTM fast recovery is not included 5.y.3 Conditional LTM execution The conditional LTM cell switch procedure is triggered when: - the MAC entity determines that the event for conditional LTM is satisfied based on L1 measurements as specified in clause 5.y.2; or - the event(s) for conditional LTM is satisfied based on L3 measurements indicated by upper layers.. For R18 LTM fast recovery UE does not have other stored TA value so it is natural to fallback to RACH-based LTM. While for R19 CLTM fast recovery, it is better to be specified somewhere to clarify UE’s behavior, as UE may have other stored TA and may be capable to perform RACH-less LTM. We can discuss whether to capture that “UE only could perform RACH-based LTM for CLTM recovery” in MAC spec, or keep it in stage-2 only. [Rapp]: It is true, i.e. we could discuss it. | Issue Type: Not essential not important How to address it: based on companies’ contribution. Issue Number: MAC-M01 |
| Ofinno | In 5.12, list of conditions when LTM candidate TA timers are continued are provided: 1> else if upper layers indicate the reset is triggered by conditional LTM, or triggered by LTM while there is CLTM candidate configuration(s), or triggered by handover while there is CLTM candidate configuration(s): 2> stop (if running) all timers, except MBS broadcast DRX timers, ltm-Candidate-TimeAlignmentTimer, and ltm-Candidate-TimeAlignmentTimerTAG2, if configured; 2> consider all timeAlignmentTimers as expired and perform the corresponding actions in clause 5.2; 1> else: 2> stop (if running) all timers, except MBS broadcast DRX timers; 2> consider all timeAlignmentTimers, inactivePosSRS-TimeAlignmentTimer, and cg-SDT-TimeAlignmentTimer, if configured, as expired and perform the corresponding actions in clause 5.2; The yellow highlighted text includes all mobility types except CHO. Since ltm configuration is not released on executing CHO, we think the same principle should apply for CHO as well. i.e., the candidate TA timers are continued if the MAC reset is triggered by CHO. Further, RRC specification does not capture an indication to MAC about the trigger for MAC reset. Does MAC really know whether the reset is triggered by CLTM of L3 based handover? We think it is better to keep the condition based on reconfiguration with sync and not basedon handover type. Based on 5.2x, LTM candidate timers can be running only when CLTM candidate cells are configured on the UE. When a CLM candidate configuration is released, then LTM candidate timer associated with that CLTM candidate configuration is stopped. MAC specification does not allow LTM candidate TA timer to be running without the UE already having the corresponding CLTM candidate configuration. We think the blue highlighted text is redundant and unnecessary. We suggest to update as: 1> else if upper layers indicate the reset is triggered by reconfiguration with sync: 2> stop (if running) all timers, except MBS broadcast DRX timers, ltm-Candidate-TimeAlignmentTimer, and ltm-Candidate-TimeAlignmentTimerTAG2, if configured; 2> consider all timeAlignmentTimers as expired and perform the corresponding actions in clause 5.2; 1> else: 2> stop (if running) all timers, except MBS broadcast DRX timers; 2> consider all timeAlignmentTimers, inactivePosSRS-TimeAlignmentTimer, and cg-SDT-TimeAlignmentTimer, if configured, as expired and perform the corresponding actions in clause 5.2; [Rapp]: It is reasonable. Let’s change it as suggested. | Issue Type: Not essential not important How to address it: Rapporetur will address it in the updated CR. Issue Number: MAC-F01 |
| Ofinno | In 5.y.3, UE selects the valid CG for L3-based CLTM based on the selected beam above a threshold. In our understanding, RAN2 did not introduce any new threshold for this. However, the selection seems to be based on beam being above cg-LTM-RSRP-ThresholdSSB. Further, this field is not defined in 38.331. 3> if cg-LTM-Configuration is configured for the CLTM target cell (i.e. the SpCell corresponding to the target configuration indicated by Target Configuration ID), if two TAGs are not configured for the CLTM target cell, and if at least one of the SSB(s) of the CLTM target cell with SS-RSRP above cg-LTM-RSRP-ThresholdSSB and amongst the SSBs associated with the CG is available: 4> select the SSB(s) with SS-RSRP above the cg-LTM-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant; 4> if the ltm-Candidate-TimeAlignmentTimer associated with the CLTM target cell is running in the first available CG occasion corresponding to one of the seleted SSB for initial uplink transmission according to clause 5.8.2: 5> process the stored Timing Advance Command associated with the running ltm-Candidate-TimeAlignmentTimer (see clause 5.2); 5> consider the RACH-less CLTM cell switch to be ongoing; We suggest to either re-use cg-RRC-RSRP-ThresholdSSB or define cg-LTM-RSRP-ThresholdSSB in 38.331. In either case, there is change required in 38.331. [Rapp]: This should be defined in RRC, as we discussed and concluded before. Similar threshold as cg-RRC-RSRP-ThresholdSSB should be introduced for LTM. This is an RRC issue. [Rapp2]: After coordination with RRC Rapp/Tony, let’s discuss this issue in MAC. | Issue Type: Not essential not important How to address it: based on companies’ contribution. Issue Number: MAC-F02 |

  - Others, please specify

Companies are invited to describe any other identified open issues not currently included within this document.

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| Nokia | For the deactivation of semi-persistent CSI-RSs associated with the target cell, it has been agreed that after CSI reporting at the target cell after or during a cell switch, the configured semi-Persistent CSI-RS resources for the target cell are deactivated. However, this agreement only applies to CSI-RSs configured for CSI acquisition. For CSI-RSs configured for L1-RSRP measurements, no report is sent to the target cell after or during the cell switch. The L1-RSRP reporting after the cell switch will follow the LTM CSI report configuration provided in the target configuration. Therefore, it remains an open issue when the active semi-persistent CSI-RSs associated with the target cell and configured for L1-RSRP should be deactivated. It would make sense to deactivate them upon the cell switch, but this still needs to be agreed. [Rapp]: We have discussed this issue clearly in RAN2#131 meeting, and concluded that there is no need to differentiate the SP CSI-RS for L1-RSRP and for CSI. With this, it seems there is no need to have separate handling for SP CSI-RS for CSI and for L1-RSRP. More elaboration from proponent is still welcome. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Proposal from Proponent is welcome. Issue Number: MAC-N02 |
| Qualcomm | Deactivation of SP CSI-RS of target cell for L1-RSRP measurement Based on the previous RAN2 agreements, Clause 5.18.x only captures (i) deactivation of SP CSI-RS/CSI-IM resources for all candidate cells other than target AND (ii) deactivation of SP CSI-RS/CSI-IM resources of target cell for early CSI acquisition. And SP CSI-RS for target cell L1-RSRP measurement is left uncovered as CSI reporting of L1-RSRP at target cell cannot happen. For SP CSI-RS for target cell L1-RSRP measurements, it would be logical to deactivate them just like the ones for other candidate cells. [Rapp]: See above response. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Proposal from Proponent is welcome. Issue Number: MAC-Q04 |
| Qualcomm | SP CSI-RS/CSI-IM Resource Set Act/deact for Candidate Cell MAC CE (1) In Clause 6.1.3.12a, the presence of CSI Resource Configuration ID2 is implicitly determined at UE based on the configuration of SP CSI-IM resource set for candidate cell(s) in RRC, but this implicit approach is not clear and would lead to unnecessary inclusion of CSI Resource Configuration ID2. - If CSI Resource Configuration ID1 corresponds to multiple NZP-CSI-RS resource IDs from different candidate cells, which candidate cell(s)’ CSI-IM resource configuration should be used to determine the presence of CSI Resource Configuration ID2? - CSI Resource Configuration ID2 is used only for CSI-IM for early CSI acquisition, not for L1-RSRP measurements. If this MAC CE is intended for SP CSI-RS for L1-RSRP measurements, should CSI Resource Configuration ID2 be still present just because CSI-IM resource is configured in RRC? To avoid any confusion, it would be better to use an explicit indicator such as “IM” in legacy SP CSI-RS/CSI-IM activation/deactivation MAC CE, like below, where CSI Resource Configuration ID2 is present when IM is set to 1, and absent otherwise. If IM is set to 0, TCI State ID0 will be present in Octet 2, next to IM. [Rapp]: This issue was indeed discussed in RAN2#130 meeting. Everyone agreed that UE could easily identify whether this CSI Resource Configuration ID is CSI Resource Configuration ID1 or CSI Resource Configuration ID2 based on RRC configuration due to the the following agreement： - M resource and NZP CSI resource cannot be configured for a CSI-RS resource configuration id. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Issue Number: MAC-Q05 |
| Qualcomm | SP CSI-RS/CSI-IM Resource Set Act/deact for Candidate Cell MAC CE (2) As per RAN2 agreements, in Clause 6.1.3.12a, all NZP-CSI-RS resources within an LTM CSI resource configuration (CSI Resource Configuration ID1) are activated with TCI state IDs assigned or deactivated. This is a different situation from the legacy SP CSI-RS activation procedure. In the legacy, the minimum unit of activation/deactivation was an NZP-CSI-RS resource set, so it was possible for a serving cell to activate a subset of NZP-CSI-RS resource sets within a single CSI resource configuration. But in LTM, the minimum unit is an LTM CSI resource configuration which can have a long list of NZP-CSI-RS resource IDs from different candidate cells. This will cause inefficient SP CSI-RS management at serving cell such as - Some of activated SP CSI-RS resources may not be useful at all as the channel condition could dynamically change from the LTM configuration time. - If serving cell creates multiple LTM CSI resource configurations each of which covers a small set of candidate cells, then those many LTM CSI report configurations should be also configured to fully utilize the LTM CSI resource configurations. To address this issue, RAN2 needs to discuss the method of ‘partial’ activation and/or deactivation of NZP-CSI-RS resources within an LTM CSI resource configuration. [Rapp]: If I understand the comment correctly, this issue was discussed in RAN2#130, and the following agreement is achieved: - Instead of candidate cell id and SP CSI-RS resource set id, LTM-CSI-ResourceConfigId is included into SP CSI-RS activation/deactivation MAC CE. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Issue Number: MAC-Q06 |

- Conclusion

In this contribution, we collect the open issues for mobility enhancements in MAC as below:

Proposal 1: Following open issues are suggested to be discussed based on companies’ contributions:

- Inter-CU LTM: MAC-V01, V02,

- Event triggered L1 measurement: MAC-V03, S02, H03, H04(Part II), O02, Q01,

- Conditional intra-CU LTM: MAC-V04, V05, K01, K02, N01, O03, M01, F02(it is an RRC issue).

Proposal 2: Rapporteur understands there is no problem on the below open issues (For some open issues, proposal from Proponent is also welcome):

- Event triggered L1 measurement: MAC-H01, H05, H06, H07, H08, H11(Part I), H12, H13, O01, Q02, Q03, B01,

- Conditional intra-CU LTM: MAC-H14, H15, O04,

- Others: MAC-N02, Q04, Q05, Q06.

Proposal 3: Following open issues are suggested to be addressed in the CR directly, while Rapporteur will include the corresponding change in the updated CR:

- Event triggered L1 measurement: MAC-S01, H02, H04(Part I), H09, H10, H11(Part II), W01, L01

- Conditional intra-CU LTM: MAC-F01.

- References

- R2-506229, Introduction of NR mobility enhancements Phase 4 in MAC, vivo.