3GPP RAN WG2 Meeting #131-bis                                                                               R2-2507039

Prague, Czech Republic, Oct. 13th – 17th, 2025

Agenda Item:	9.7.2

Source:	HONOR

Title:	Discussion on IoT-NTN to support IMS voice call

Document for:	Discussion and Decision

- Introduction

In Rel-20 Non-Terrestrial Networks (NTN) for Internet of Things (IoT) WID [1], the following objectives were achieved:

The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives: - Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2]

In this contribution, we will discuss some issues needed to be addressed for supporting IMS voice call over GSO.

- Discussion

2.1 CP solution vs UP solution

An IMS-based session requires at least two distinct EPS bearers with different reliability requirements, mapped to specific RLC transmission modes.

- IMS Signalling Bearer (Default EPS Bearer): This bearer carries SIP messages (TCP-based), which require high reliability. The loss of a single SIP message can cause call failure. Therefore, this bearer must be configured with RLC Acknowledged Mode (AM).

- Media Bearer (Dedicated EPS Bearer): This bearer carries real-time voice packets (UDP-based). It prioritizes low latency over perfect reliability and should be configured with RLC Unacknowledged Mode (UM) to minimize transmission delay.

- The Control Plane CIoT EPS Optimisation (CP solution) is designed for infrequent, small, and delay-tolerant data transmission. It encapsulates user data within NAS PDUs, which are transported over Signalling Radio Bearers (SRBs).

- As per 3GPP specifications (TS 36.331), SRBs must operate in RLC Acknowledged Mode (AM) to guarantee the delivery of critical control signalling. This mandatory use of RLC-AM is fundamentally incompatible with the low-latency requirements of real-time voice traffic.

- Attempting to reconfigure an SRB to use RLC Unacknowledged Mode (UM) for voice packets would be a fundamental violation of legacy design principle.

The User Plane CIoT EPS Optimisation (UP solution) establishes standard Data Radio Bearers (DRBs), allowing for:

- Appropriate RLC Mode Selection: A dedicated DRB for voice media can be configured with RLC Unacknowledged Mode (UM), minimizing transmission delay by avoiding retransmissions.

- Distinct QoS Handling: Separate DRBs can be established for IMS signalling (RLC-AM) and media (RLC-UM), enabling the network to apply differentiated scheduling and prioritization.

- Protocol Efficiency: Direct transmission over DRBs avoids the excessive header overhead associated with encapsulating voice packets within NAS PDUs.

- Spec Impact: It has no spec impact expect remove the limitation of 2-DRBs for IOT device.

Proposal 1:  Support UP solution for normal and emergency services.

Proposal 2:  Separate DRBs can be established for IMS signalling (RLC-AM) and media (RLC-UM), enabling the network to apply differentiated scheduling and prioritization.

  - Semi-persistent scheduling

In the previous SA2 discussion, the issue over/under scheduling was identified, which will have a significant impact on RAN2. Due to the significant difference in size between the voice packets transmitted during the talk phase and the silence packets transmitted during the silence phase, how to allocate radio resource will be a problem that has to be considered when applying SPS to IMS voice call. One solution is to reserve the SPS resource based on the largest packet size. However, it will cause over-scheduling, which will result in inefficiency on radio resource utilization. Moreover, in general, the scheduling interval of voice packet is 20ms and the scheduling interval of silence is 160ms. If the SPS resource period is configured to 20ms, this will further exacerbate the impact of radio resource utilization.

Another approach is to allocate two different types of SPS resource for transmitting silence packet in silence phase and voice packet in talk phase respectively. How to dynamically switch between the two SPS configurations should be taken into account in the subsequent discussion. By comparing these two solutions, it is obvious that the latter one makes higher efficiency on radio resource utilization.

Proposal 3: It is suggested to define two SPS configurations for silence packets and voice packets transmission respectively.

In the current spec, for the purpose of coverage enhancement, the repetition transmission mechanism has been introduced for IOT terminals. When considering that SPS configuration is used for the transmission of voice packet, the impact of the repetition transmission on SPS configuration needs to be taken into account. In addition, usually, a voice service will last for a relatively long period of time. During this period, there is a possibility that the coverage level may change, and it might affect the use of SPS configuration.

Proposal 4: RAN2 to study SPS configuration for voice packet transmission with consideration of the impact of coverage enhancement.

  - RRC connection setup

Enhancements related to RRC connection setup are introduced considering the characteristic of NB-IoT. For example, massive NB-IoT UEs could trigger access at the same time which may cause access congestion. Extended Access

Barring mechanism enables the NW control the access for these delay-tolerant NB-IoT UEs. However, based on the performance requirements for satellite access from SA1, the call set up time needs to be less than 30s. The voice service for IoT NTN is not delay-tolerant which is quite different from other IoT services. Considering the RTT for the NTN network (especially for communication using GSO satellite access), the RRC connection setup latency should be reduced for voice service compared with other services for NB-IoT UEs.

It is important to increase the success probability and reduce the latency for the access triggered by voice service (e.g., IMS voice call and emergency call),

Proposal 5: RAN2 to study how to reduce the latency for the access triggered by voice service (e.g., IMS voice call and emergency call).

One possible solution is that the UE's initial access attempt could be recognized and prioritized by the network. It is straight forward to introduce a new value in the establishmentCause for the voice service. A NB-IoT UE initiating a voice service could set the specific establishmentCause in the RRC connection request message (MSG3). Upon receiving an RRC connection request with the specific establishmentCause, the eNB could prioritize this request over other access attempts.

Proposal 6: Add a new value for the voice service (e.g., IMS voice call and emergency call) to the EstablishmentCause enumeration.

  - Emergency call

In terrestrial networks, UEs indicate the need for emergency service establishment during RRC connection and NAS signaling. The network responds by prioritizing the access procedure and establishing EPS bearers with appropriate QoS.

For IoT-NTN, characterized by long delays and potentially limited resources, explicit signaling and prioritization mechanisms are crucial to ensure emergency calls can be established reliably. Without these mechanisms, a IoT device's emergency request may be delayed or blocked.

An IoT-NTN device must be able to quickly determine if the cell it is camped on supports emergency services. Without this indication, the device may waste energy attempting registration or service request procedures on a cell that cannot fulfil its emergency request. We propose introducing an emergencyServiceSupport indicator in the relevant IoT-specific SIB. A NB IoT UE requiring emergency service shall only attempt access on a cell where the relevant SIB indicates emergencyServiceSupport = true.

Proposal 7: Introduce an emergencyServiceSupport indicator in the relevant IoT-specific SIB.

- Conclusions

In conclusion, we discuss the issues related to semi-persistent scheduling, RRC connection setup and emergency call. Based on the discussion, the following proposals are concluded:

Proposal 1:  Support UP solution for normal and emergency services.

Proposal 2:  Separate DRBs can be established for IMS signalling (RLC-AM) and media (RLC-UM), enabling the network to apply differentiated scheduling and prioritization.

Proposal 3: It is suggested to define two SPS configurations for silence packets and voice packets transmission respectively.

Proposal 4: RAN2 to study SPS configuration for voice packet transmission with consideration of the impact of coverage enhancement.

Proposal 5: RAN2 to study how to reduce the latency for the access triggered by voice service (e.g., IMS voice call and emergency call).

Proposal 6: Add a new value for the voice service (e.g., IMS voice call and emergency call) to the EstablishmentCause enumeration.

Proposal 7: Introduce an emergencyServiceSupport indicator in the relevant IoT-specific SIB.

- References

- RP-251867 Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4