3GPP TSG-RAN2#131bis	R2-2507074

Prague, Czech Republic, 2025-10-13 - 2025-10-17

Agenda Item:	10.3.3 - Common User plane and Control plane aspects

Source:	Ericsson

Title:	Common user and control plane aspects for 6G

Document for:	Discussion, Decision

# 1	Introduction

This contribution discusses aspects which impact both user plane and control plane.

# 2	Discussion

## 2.1      Mapping control messages to appropriate channels

Control messages for the Uu interface are defined by various working (sub-) groups in TSG RAN. UE and gNB exchange those messages via different “channels”. The gNB sends an RRCReconfiguration message through an SRB, i.e., as PDCP SDU. MAC Activation/Deactivation control elements and Buffer Status Reports are injected directly into the payload of the PxSCH transport blocks. CSI reports and HARQ feedback are encoded separately and mapped into physical resources at the edge of the carrier (PUCCH) or multiplexed into resource elements that would otherwise have been used for PUSCH.

The different channels offer different functionality (error detection (CRC), retransmissions, segmentation, concatenation, ciphering/integrity, …) and hence it would be appropriate to map different control messages onto different channels according to their properties and requirements. However, it appears that the messages’ requirements have less impact on the chosen channel than the working group that introduces the control message. Messages introduced by RAN2’s control plane group are by default sent via an SRB. When the RAN2 user plane experts introduce a new control message they map it typically to a MAC CE. And RAN1 chooses DCI/UCI by default. Taking BSR and CQI reports as examples, one can see that despite similar size and similar demands in terms of delay and robustness they were mapped to very different channels.

Working groups tend to map control elements to their channels rather than choosing the channel offering the required properties in terms of robustness, message size and security.

In recent years there has been a trend to map more downlink control messages to MAC PDUs or DCIs rather than sending them through SRBs. The reasoning is typically that DCI and MAC PDUs have more predictable delivery times. However, control messages that are mapped to SRBs experience long latency only in case of transmission errors. Depending on the error case it may take time until the error is detected, and a retransmission triggered. If a control message that is carried in a MAC PDU or DCI is subject to the same transmission error, it is the responsibility of the “application protocol” to catch and recover from it. Unfortunately, those error cases and race conditions are often overlooked and hence the specification does not describe how UE and network shall discover and solve the problem. Especially rare error cases tend to occur only after the functionality has been introduced in the field. For example, NR UEs falsely detected DCIs that (seemingly) task the UE to switch to a different BWP. Since the NW has never sent those DCIs, it expects the UE to be listening on the original BWP. UEs were not required to acknowledge a BWP switch and even if they were, the NW would not have received a confirmation that the UE would have sent after the switch. This example shows that especially downlink control messages that trigger a persistent change in the UE’s configuration should be mapped to a channel that makes false detection very unlikely and that offers means to detect and correct transmission errors.

The seemingly higher/unpredictable latency of SRBs compared to MAC CEs (or MAC CEs compared to DCIs) is the price to pay for the robustness that the former offers. Choosing the latter requires additional specification and implementation efforts and tends to result in the same effective latency.

One area that we would like to pay attention to in 6G is the uplink control signalling, i.e., the diverse types of reports in which the UE conveys radio measurements, (H)ARQ feedback, buffer state information, assistance information and many more. On one hand, we consider it important to review and possibly adjust how NR’s UL MAC-CEs and UL RRC reports (if needed in 6G at all) are specified, configured and transmitted. For example, the list of MAC CEs defined in 38.321 clause 6.1.3 seems to grow out of control. The description of message formats and their mapping into MAC Sub-PDUs appears unnecessary convoluted. Similarly complex and inconsistent is the specification of uplink reports in 38.331 (RRM measurements, UE assistance information, UE Information Request/Response, …). The procedural text for those reports if often unnecessary lengthy while lacking important information. The individual components of those reporting procedures (UE capability, report availability, report creation and message formats) are difficult to locate which is probably one reason why different procedures are specified in many ways.

NR’s uplink reporting procedures (measurements, indications, UAI, information request/response, …) don’t follow consistent principles, which makes it increasingly difficult to maintain consistent specifications and unlikely that value of those reports justifies the development cost.

In our attempt to identify NR functionality that drives product implementation and testing complexity we discovered especially two classes of uplink reports: CSI reports and UL HARQ feedback for PDSCH. Both are traditionally dealt with by RAN1 but they are tightly connected to HARQ procedures defined in MAC and similar to other reports that RAN2 maintains (e.g. BSR, PHR).

Unlike RAN2’s MAC- and RRC reports, the UL HARQ-FB and CSI reports are carried on a tailor-made physical channel (PUCCH) defined by RAN1. To save signalling overhead, the HARQ feedback reports comprise only of a set of ACK/NACK bits without explicit process IDs. UE and network derive the HARQ process IDs from the previous scheduled HARQ processes, from DAI fields in the DL grants and from the PUCCH’s transmissions time. While this was feasible in early LTE days (FDD only) the PUCCH signalling puzzle became increasingly complex in later LTE releases (carrier aggregation, TDD) and in NR (more CA, mixed numerologies, code-block groups, …). Similarly, the channel state information evolved from a simple “CQI+rank” in LTE to a diverse set of CSI report types of various sizes and processing different complexity. The latter results in different processing delays which restrict when the network may schedule those reports and regular data. This complexity of HARQ-FB and CSI reports became one of the biggest pain points for network and UE implementation and it hinders the swift introduction of new carrier aggregation variants or advanced CSI types. In [2] we provide more details about those problems and suggestions how to overcome those. In short, we observe that aligning those HARQ- and CSI reports with the handling of BSR, PHR and others would greatly simplify the system while improving end to end performance and resource utilization.

The uplink control signalling of HARQ feedback and CSI reports is one of main complexity pain points in NR. Simplifications and practical performance enhancements require a holistic view on the Uu protocol stack and end-to-end protocols.

When defining uplink and downlink control messages for 6G, RAN working groups should carefully analyse the reports’ properties and requirements and choose an appropriate control channel taking the entire protocol stack and the impact on e2e protocols into account. Whenever possible, an existing channel that offers the required functionality should be re-used. Only if none is available, a new transport solution for the specific message should be introduced.

When defining uplink and downlink control messages for 6G, RAN working groups should carefully analyse the messages’ requirements (reliability, latency, error cases, …) and choose an existing control channel fulfilling those requirements. Tailor-made channels and error recovery procedures should be avoided.

## 2.2      Security

Access stratum (AS) security in 5G is handled within the PDCP layer, i.e. PDCP SDUs are encrypted and integrity protected, meaning that both DRBs (e.g. IP packets) and SRBs (e.g. RRC messages) undergo security. Despite the use of hardware accelerators for cryptographic operations these steps consume a non-negligible amount of time. Hence, it is beneficial to perform these steps in a pre-processing manner, i.e. upon arrival of data instead of upon transmission. Thereby, the operation is done outside the time-critical scheduling and transmission loop.

For 6G, open questions regarding AS security would be usage of new cryptographic algorithms in 6G, such as AEAD (Authenticated Encryption with Associated Data), as well as need for security for messages which are not protected in NR such as uplink and downlink control messages which NR maps into MAC CEs or  for broadcast system information (SI). Security requirements may have significant impact on the overall RAN protocol design, as well as on the beforementioned pre-processing capability, which is why the LS from RAN plenary to SA3 [RP-252665] suggests to align on security requirements between RAN2 and SA3 at a defined checkpoint of the 6G study item (proposed is June 2026).

For 5G, need for security for lower layer signalling (e.g. MAC CEs) had been discussed in the context of e.g. LTM. The challenge with introducing security for MAC CEs would be that the cryptographic operation would be needed to be applied in the time-critical scheduling/transmission loop. One could accommodate for this by extending scheduling timeline, i.e. the time from when the gNB takes the scheduling decision until the time when the transmission happens on the radio interface and the time at which the UE obtains an UL grant until it performs its UL transmission. Even if only a subset of the control messages require protection and even if only a fraction of the slots contain such messages, the Uu timeline would need to be extended for all transmissions. Hence, the requirements and solutions should be evaluated with great care. Furthermore, considering 6G’s target to avoid redundant functionality, security functionality shouldn’t be replicated on MAC in addition to PDCP. Instead, and in the light of the unified UL reporting principle discussed in the previous section, signalling elements which require security in 6G and which NR carried in MAC CEs could in 6G be sent via a type of SRB i.e. through PDCP. The ciphering process would take the same amount of time but affect only those control elements that require ciphering.

Integrity protection of system information (SI) (e.g. signature in MIB/SIB) is a potential further security requirement to consider for 6G. However, before introducing SI integrity protection it's important to understand the potential attacks that are possible with unprotected SI. For example, an attack that results in compromise of user data is more important to protect against and may justify a more complex solution than an attack that results in temporary denial of service. Security requirements also depend on the type of network, for example users in private network may have stricter security requirements than users in a public network. Another aspect to consider is the recent rise of post-quantum cryptography (PQC) which e.g. requires significantly longer signatures than current classic cryptography.

A related area is confidentiality protection (encryption) of system information. Encryption is used today in 5G for positioning SIBs (posSIB) to ensure that only authorized users have access to the contents. The encryption is carried out by the LMF and the encrypted SIBs are sent to the UEs transparently via the gNB. For 6G, a generalized encryption mechanism could be considered that can be used also for other types of SIBs.

Study placement of access stratum security for data and signalling while avoiding having security in multiple layers. Also consider security for broadcast system information.

## 2.3      Data collection and transfer over the air interface

The concept of data transfer in 6G includes the transfer of various types of data (e.g. AI/ML, sensing, etc.) from the UE to the network [1]. In this section we focus on data transfer over the air interface for network-side data collection for AI/ML, sensing, and observability.

A number of AI/ML use cases are being introduced in NR Rel-19 and Rel-20, such as beam management and mobility. In the AI/ML use cases, there can be AI models both at the network side and at the UE side. There are associated data collection procedures for both of these cases - a network-side data collection procedure for training of network-side models and a UE-side data collection procedure for training of UE-side models.

For AI/ML network-side data collection, the model training may be handled at the RAN (xNB), where the xNB is the data consumer of the collected data from the UE to the network. Since the xNB is the data consumer, a requirement for 6G is that this kind of data needs to terminate at the xNB rather than at another network node. Furthermore, in NR, logged MDT data is reported via SRB2 while the data collected for network-side AI/ML models is reported via SRB6, which has a configurable priority different from the priority of SRB2 (they both have lower priority than SRB1). It can be assumed that the radio bearer used for network-side data collection in 6G will have a configurable priority.

In case of network-side data collection, the xNB can be the consumer of the collected data, e.g. if the AI/ML model is handled at the xNB side.

A difference between sensing and AI/ML is that for sensing the data must be consumed immediately by the sensing client whereas for AI/ML the data can be stored for training purpose. However, some synergies are still applicable in terms of Uu interface design.

In our understanding of the Sensing architecture a core network Sensing Function (Sensing Core Function) sends a request to xNB for sensing. xNB identifies UEs which can assist in sensing and configures the sensing measurements towards such UEs. UE provides the measurement report to the base station. The advantage with xNB-terminated measurement reports is that the obtained measurement results, if needed, can be consumed by xNB for improving communication performance and can also be dispatched to core network by further processing it, if required, or can be forwarded transparently. The design of the interface between RAN and Sensing Core Function is up to RAN3.

For both AI/ML data collection and sensing, there is a need for data (measurement report) to be terminated in xNB. A common solution in the Uu interface should be envisioned.

Another key aspect to consider is how the support for observability fits into the data collection framework. In 6G, the need for observability, (i.e., the tools to monitor, analyse and ensure the network performance), is stronger than in any previous generation of mobile networks. The fundamental enabler for supporting such observability functionalities over-the-air interface are data collection procedures, which are described in the following.

In 5G networks, MDT procedures have been standardised as a tool to support network observability. Immediate MDT has been primarily utilised for coverage analysis (using M1 measurement) as well as QoS monitoring and verification (using user plane measurements such as M4-M7 measurements). Notably, the collection of M1 measurement in MDT leverages the RRM measurement framework which was originally designed to support RRM functions and mobility management.

The data collection framework in 6G should support deployment and operation in multiple ways. Like in earlier generations, it is important that deployment-related data (propagation characteristics, neighbouring cells and mobility information, etc.) is collected and to be used for understanding the shortcomings of the deployment, etc. Such information allows for easier deployment of the new generation, especially at early phases of deployments. At the same time, 6G will cover new services and business models beyond eMBB. It is crucial that CSPs can collect information in a way that service planning and delivery can be automated. For example, by understanding where a service can be delivered and the required functionality (e.g., features to be used, etc.), or by assessing during the service delivery phase whether the corresponding conditions are fulfilled (e.g., user plane KPIs, location or coverage of the UE, etc.). Thus, it is not only important to collect the relevant information, but also to be able to correlate the different elements.

In 6G networks, assuming that data collection for the AI/ML model training will be supported from Day 1, the data collection for observability purposes could leverage the framework for data collection for AI/ML model training, in particular over the air interface (resembling the way that M1 measurements in MDT were piggybacked on the RRM measurement framework in 5G).

Reusing the same data collection framework for AI/ML model training and observability reduces the implementation cost and complexity.

A common framework in the Uu interface is designed for collecting data from the UE to the network, for network-side AI/ML models, sensing measurements and for network observability measurements, where the data is terminated in the xNB. The data collected for AI/ML network-side models is also used for observability purpose.

## 2.4      UE energy saving

### 2.4.1	Idle and Inactive mode

Low Power Wake-Up Signal (LP-WUS) has the potential for significant power savings, especially for IoT devices. In 5G the Low Power Wake-Up Receiver (LR) is unable to perform neighbour cell measurements, i.e. the power saving gains can be further improved when wake up of the MR can be delayed or prevented during Idle mode mobility.

Paging monitoring and RRM measurements are the two main contributors to the power consumption in Idle/Inactive mode. But if certain conditions are met (e.g. low mobility) the RRM measurement requirements can be relaxed.

Dependent on the progress for RRM and mobility in 6G RRM neighbour cell measurement relaxation should be evaluated for 6G. The same principle as in 5G should apply that devices in low mobility and/or not-at-cell-edge may relax RRM measurements until the device starts moving again.

### 2.4.2	Connected mode

In 4G and 5G the MAC entity’s DRX functionality determines when the UE monitors the PDCCH of the PCell and of activated SCells. In addition, RAN1 introduced means by which the network may adjust the monitoring occasions of individual PDCCH’s in time and frequency domain (search space switching, BWP adaptation, …). Compared to traditional DRX rules, which force the UE into active state upon every data arrival, the DCI-controlled schemes allow the UE to monitor the PDCCH rarely even upon arrival of some sparse data packets.

With a low-power wake-up-signal and a corresponding low-power receiver in the UE, the network may actively instruct the UE to activate its PDCCH reception. This allows the UE to reduce proactive PDCCH reading further. Due to the comparably low energy consumption of the low-power receiver specifically 5G’s “option 1-2” reduce the DL latency without increasing the UE power consumption compared to the above-mentioned C-DRX schemes.

For 6G, RAN1 and RAN2 should work jointly on a simplified set of UE energy saving functions. 5G’s duplicate and partially contradicting PDCCH-monitoring schemes should be reviewed and aligned. Herein, we consider a LP-WUS as a key building block. Traditional DRX, SCell-De-/Activation and PDCCH switching should be reviewed and possibly merged.

Solutions for UE energy saving must be addressed in combination with means for network energy saving (NES, see next section).  For example, the UEs’ PDCCH- and LP-WUS monitoring occasions should be placed in a way that is favourable also in terms of NES configuration.

Based on the discussion above we propose for UE power saving in 6G:

Study LP-WUS and RRM neighbour cell measurement relaxation for IDLE/INACTIVE and review and align the toolbox of LP-WUS, C-DRX, SCell (de-)activation and PDCCH-switching features for CONNECTED mode.

## 2.5      Network energy savings

Network energy saving (NES) features were introduced in the late phases of NR, and as a result, their benefits are limited as only a small amount of UEs may support those NES features. As realized during the discussions, the need for backwards compatibility typically prohibits the introduction of NES features into a deployed system. A new generation is hence a rare opportunity to reduce the network energy consumption at large scale. But it is crucial that both networks and UEs support those techniques upon the initial deployment of the new generation. To achieve this, NES functionality must be kept reasonably simple for UEs and networks. Unnecessary options must be avoided as they would fragment the market which makes all NES features unusable in practice.

Network energy saving techniques introduced in NR can provide limited benefits due to backwards compatibility constraint and limited UE support. 6G’s NES functionality must hence be specified and implemented by UEs and networks from day one.

The NR design is lean with a default SSB periodicity of 20 ms. While this is an improvement compared to LTE, there is still a lot of room for improvements. As described in [R1-2505625], by extending the default SSB periodicity to 160 ms, significant network energy savings are achieved. For UEs in connected, 160 ms would however not be sufficient to provide good enough measurement performance. This can be addressed by on-demand SSB transmissions.

Studies have shown significant reduction in the network energy consumption if default SSB periodicity is extended to at least 160 ms.

Extending the default periodicity of “always-on” SSBs may impact UE timing and frequency synchronization. To avoid such impact while maintaining energy savings from a longer default SSB periodicity, additional SSBs, i.e., on-demand SSB, and/or burst of SSBs can be transmitted during limited periods.

Applicability of OD-SSB to PCells is a potential venue to be explored in the first release of 6G as backwards compatibility concerns is no longer a limiting factor.

To reach full lean-carrier design potential, other common channel and signal transmissions and receptions targeting UEs in non-connected states, such as paging and random access, should also be made sparser, but more consolidated in time domain when provided, and most importantly, aligned with the sparser SSB transmissions. Some progress towards this direction was already made in 5G in Release 19, by making paging and random-access occasions adaptable. Paging and random-access adaptations standardized in Release 19 contributed to improved network energy efficiency and should be adopted as 6G baseline.

To extend the deep sleep opportunities for improving network energy savings, adaptations of common channel and signalling transmissions/receptions, such as paging and random-access occasions, should be coordinated accordingly.

Cell DTX was introduced in NR as a means to overlay gNB’s PDCCH transmissions with a pattern allowing the gNB to sleep even in times when the PDCCH config suggests the gNB should/can transmit. This was added because the PDCCH config was not flexible enough to allow the gNB to sleep. But the DTX pattern overruled the PDCCH config and hence the gNB could sleep. Cell DTX can be seen as a hack to overcome the inflexible patterns for the PDCCH config.

To summarize, in 6G, with no backward compatibility constraints, a fully lean carrier design can be realized by specifying sparsity mechanisms across all transmissions and receptions, irrespective of use case (e.g., data transfer, sensing, positioning), already from first 6G release.

Network energy efficiency should be considered as a key requirement for all features developed for 6G. Configuration options (e.g. placement of reference signals and paging occasions) that optimize energy efficiency should be the primary/mandatory choice.

## 2.6      Reliability, Robustness and Resiliency

Addressing use cases beyond best effort requires that the connectivity is always available, performance is stable and resilient to disturbances and failures. The objective is that the communication is dependable from a user perspective that service providers can provide such a guarantee.

To ensure stable performance, standards should incorporate tools that enable more fine-grained scheduling decisions and allow the scheduler to maintain clear visibility into the various queues – such as data volumes, delay budgets and other relevant factors. The performance expectations should be based on flexible, rather than overly strict, requirements for bounded latency and fixed reliability. Accordingly, UE reporting formats should be harmonized and unified to prevent the need for multiple overlapping features and report types. Furthermore, for better queue management, 6G should natively support rate control mechanisms such as L4S.

To ensure resiliency, robustness should be adopted as a core design principle for network procedures, minimizing or eliminating single points of failure. For example, in 5G carrier aggregation, the connectivity relies solely on the PCell. Baseline procedures for connection and mobility management – such as RRC re-establishment, handover, conditional handover – must be designed to effectively prevent or recover from failures with little or no service interruption time. Furthermore, in 5G, existing mechanisms for detecting connectivity issues (e.g., RLF) remain limited (e.g. in terms of responsiveness) and often lead to longer service interruption times i.e., RLF declaration, cell search and RRC re-establishment procedures.

6G standards should enable early identification of issues – whether in the PHY, in the UP or in the CP – and use these detections to trigger appropriate prevention or recovery procedures (e.g., handover, etc.). It is also crucial that the different procedures are themselves robust against failures, allowing for completion even with partial failures (e.g., completion of a conditional handover after failure of the serving base station).

Finally, we emphasize that for commercial success it is crucial that they are based on the mainstream protocols and procedures.

Addressing advanced use cases beyond best effort connectivity requires that 6G standards support tools for delivering communication with stable performance and which are resilient to issues and failures.

The 6GR basic protocol design supports delivering resilient communication with stable performance.

## 2.9      Further 6G use cases

### 2.9.1	XR

In the start of 5G standardization XR as a use case was not in focus. The XR use case comes with new traffic characteristics, e.g. very high data rates with periodic bursty behaviour and tight time requirements, which put new tough requirements on the radio network. To understand the impact on the radio network the XR traffic was studied extensively during later part of the 5G standardisation. During this standardization work many features were introduced to improve the network support of XR traffic. The XR enhancements in 5G were primarily targeting user plane, including improvements to UE reporting, scheduling and packet handling. Furthermore, the enhancements were in many cases general in nature, e.g. the enhancements were not restricted to XR use cases but could also improve network performance for other types of traffic.

XR was extensively studied during later parts of 5G standardization, and a number of user plane enhancements were identified to improve the support of XR traffic.

Since the XR use case is expected to be widely used when 6G is deployed it should be supported in the 6G baseline. Furthermore, since XR was studied extensively in earlier releases the findings should be taken into consideration when standardizing the 6G features. For 6G the most generally useful of the standardized XR features, i.e. with clear advantages and commercialization possibilities, should be identified and further studied. Examples of such features are the UE reporting enhancements, like the DSR feature introduced in Rel18, which has potential to provide timing information for any type of traffic and thus help the network scheduler in making scheduling decisions between many different UEs, and the queue handling features like the RLC enhancement for avoiding unnecessary retransmissions introduced in Rel19, which can reduce network load from multiple types of traffic.

The XR use case should be supported from day one of 6G standard and focus should be on identifying and improving the most generally useful of previously standardized XR features, such as the UE reporting and queue handling enhancements.

### 2.9.2	IoT

The 6G SI has as an objective to “target scalable and forward compatible design for diverse device types”. All 6G UEs, including the low-end devices, shall support a common mandatory baseline functionality. Unlike in previous generations, the ambition is to avoid specifying an even-lower-end device in a later release of 6G since that is known to create backwards compatibility issues and unnecessary complexity.

It is easier to scale up functionality in a later release than to scale down as experience from earlier radio generations has shown.

The key aspect at this stage is to define the minimum requirements, i.e., the mandatory baseline functionality, for low-end 6G UEs. In case there is a concern regarding combinations of capabilities that are not desired for certain use-cases, this can be prevented with defining capabilities conditional to other capabilities, as in NR, e.g. support of 1 Rx antenna can be associated to support of peak data rate. Furthermore, more advanced features can be introduced with additional capabilities, thereby enabling scalability of device functionalities.

The design of 6G physical channels, reference signals, protocols, procedures (e.g. initial access, measurements and reporting, …), capability and configuration signalling must support low-end UEs as well as high-end UEs.

A challenge to solve in 6G is ensuring the longevity and coexistence required by massive IoT applications. To name a few, key industrial sectors, such as automotive, smart utilities, and critical infrastructure monitoring, operate on cycles that demand long term solutions and support, at least for a decade. Securing these use-cases calls for a massive IoT framework integrated into 6G from initial phase.

The commercial adoption of 6G for massive IoT needs to provide a stable, forward-compatible solution from its initial design.

### 2.9.3	NTN

It is a key requirement that 6G specifications support NTN to allow providing ubiquitous coverage for basic mobile broadband, FWA, and IoT services. Seamless integration comprises: a harmonized radio interface, seamless mobility within NTN and between TN and NTN, as well as GNSS-free operation.

Similar to the introduction of NTN in NR, the design of the 6G protocol must account for the NTN-specific requirements, including (much) higher propagation delay, satellite movement, etc. However, RAN2 should aim for common solutions that address TN and NTN deployments. In other words, RAN2 should refrain from discussing NTN specific requirements and solutions separately from the main track.

One goal of the study should be to derive principles for seamless mobility within NTN and between TN and NTN from an early design phase, while trying to minimize signalling overhead as well as UE complexity and energy consumption. When studying mobility, RAN2 should focus on LEO scenarios, where satellite movement causes frequent cell changes, and the consequences thereof.

The study should start from the assumption of GNSS-free operation. 6G NTN cannot fulfil its role as an essential complement to 6G TN while depending on GNSS, which is prone to jamming and spoofing. RAN2 should hence study how to overcome the dependency on GNSS for mobility, RRM measurement framework, geofencing, etc. By overcoming this dependency, 6G NTN also overcomes the coverage limitations of GNSS (such that 6G NTN coverage is not limited by GNSS coverage, as is the case, e.g., for NR NTN coverage) as well as the energy consumption (UE battery lifetime impact) of the GNSS receiver, which is particularly important for lower-capability UEs.

6G must support NTN operation without GNSS in order to truly fulfil its role as an essential complement to 6G TN for providing ubiquitous (and reliable) coverage for basic mobile broadband, FWA, and IoT services.

Study how a single unified protocol stack for 6G can address a wide range of use cases including XR, IoT and NTN, by different configurations of the same unified protocol stack.

# 3	Conclusion

We propose:

Proposal 1	When defining uplink and downlink control messages for 6G, RAN working groups should carefully analyse the messages’ requirements (reliability, latency, error cases, …) and choose an existing control channel fulfilling those requirements. Tailor-made channels and error recovery procedures should be avoided.

Proposal 2	Study placement of access stratum security for data and signalling while avoiding having security in multiple layers. Also consider security for broadcast system information.

Proposal 3	A common framework in the Uu interface is designed for collecting data from the UE to the network, for network-side AI/ML models, sensing measurements and for network observability measurements, where the data is terminated in the xNB. The data collected for AI/ML network-side models is also used for observability purpose.

Proposal 4	Study LP-WUS and RRM neighbour cell measurement relaxation for IDLE/INACTIVE and review and align the toolbox of LP-WUS, C-DRX, SCell (de-)activation and PDCCH-switching features for CONNECTED mode.

Proposal 5	Network energy efficiency should be considered as a key requirement for all features developed for 6G. Configuration options (e.g. placement of reference signals and paging occasions) that optimize energy efficiency should be the primary/mandatory choice.

Proposal 6	The 6GR basic protocol design supports delivering resilient communication with stable performance.

Proposal 7	Study how a single unified protocol stack for 6G can address a wide range of use cases including XR, IoT and NTN, by different configurations of the same unified protocol stack.

# 4	Reference

RP-251881, New SID: Study on 6G Radio, 3GPP RAN#108, June 9-13, 2025.

R2-2507071, User plane aspects, Ericsson, Prague, Czech Republic, 2025-10-13 - 2025-10-17