3GPP TSG-RAN WG2 Meeting #131bis                                     R2-2507087

Prague, Czech Oct 13th – 17th, 2025

Source:	ZTE Corporation, Sanechips

Title:	MAC corrections for R19 IoT NTN

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

RAN2 has completed the basic functions of R19 IoT NTN in RAN2#131 meeting.

In this contribution, we will focus on the possibly required corrections for MAC specification, especially for the sub-topic of CB-Msg3-EDT.

Discussion

Issue#1: Details of Backoff scheme for CB-Msg3-EDT

For CB-Msg3-EDT, UEs that select the same Msg3 Tx window will monitor the same CB-RNTI. In some cases, if many UEs select a same Msg3 Tx window, some may transmit Msg3 successfully, while some may fail. Both successful and unsuccessful UEs will monitor the same CB-RNTI and will see the same Backoff Indicator in Msg4.

After the CB-Msg3 transmission, the MAC entity shall: - start the CB-Msg3ResponseTimer at the subframe that contains the end of the selected CB-Msg3 transmission window plus UE-eNB RTT; - monitor the PDCCH while CB-Msg3ResponseTimer is running; - if notification of a reception of a PDCCH transmission is received from lower layers; and - if the CB-Msg4 is successfully decoded: - if the CB-Msg4 contains a CB Backoff Indicator subheader: - set the CB-Msg3-EDT backoff parameter value as indicated by the BI field of the CB Backoff Indicator subheader and Table 7.2-1, except for NB-IoT where the value from Table 7.2-2 is used. - else: - set the CB-Msg3-EDT backoff parameter value to 0 ms. - if the CB-Msg4 contains one or more MAC CMRs; and - if there is a UE Contention Resolution Identity in a MAC CMR that matches the first 48 bits of the CCCH SDU transmitted in CB-Msg3(s): - consider CMR reception successful; - discard the CB-RNTI; - stop CB-Msg3ResponseTimer if running; - flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer; - if the corresponding CMR contains a Timing Advance Command field: - process the received Timing Advance Command (see clause 5.2). - start the timeAlignmentTimer associated with this TAG; - if the corresponding CMR contains a C-RNTI field: - set the C-RNTI to the value of the C-RNTI field. - finish the disassembly and demultiplexing of the MAC PDU; - consider CB-Msg3 transmission successfully completed; - if the C-RNTI field is absent in the corresponding CMR and no corresponding MAC SDU is present in CB-Msg4: - indicate the successful completion of CB-Msg3-EDT procedure to the upper layers. NOTE: The UE can receive CB-Msg4 after the CB-Msg3ResponseTimer expired if the corresponding PDCCH is received before the CB-Msg3ResponseTimer expired. - if the CB-Msg3ResponseTimer expires: - when MAC entity has finished decoding of all received CB-Msg4 and if CMR reception is not successful: - flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer; - increment CB_MSG3_TRANSMISSION_COUNTER_CE by 1; - if CB_MSG3_TRANSMISSION_COUNTER_CE = cb-Msg3-MaxAttemptNum for the corresponding enhanced coverage level + 1: - discard the CB-RNTI; - consider the CB-Msg3 transmission unsuccessfully completed; - indicate the transmission using CB-Msg3-EDT has failed to the upper layers. - else: - based on the CB-Msg3-EDT backoff parameter, select a random backoff time according to a uniform distribution between 0 and the CB-Msg3-EDT backoff Parameter value; - delay the subsequent CB-Msg3 transmission by the backoff time; - proceed to the CB-Msg3 transmission (see clause 5.1x.2).

According to the current MAC spec as above, the UE will firstly store the backoff parameter value indicated by the BI field of the CB Backoff Indicator subheader upon successfully decoding the CB-Msg4, then after the CB-Msg3ResponseTimer expires and if the CB-Msg3-EDT attempt has not yet reached the maximum number, the UE would apply the backoff parameter value and trigger the backoff operation, e.g., waiting for a random time period before re-selecting another resource and re-transmitting CB-Msg3.

In such procedure, two issues can be noticed:

- Since there is no any resources information where real conflicts occur in the Backoff Indicator, the UEs that finally successfully receive CB-Msg4 and complete contention resolution may also store and set the backoff parameter value in an earlier stage if they receives CB Backoff Indicator subheader before reception of their own CB-Msg4. If Backoff Indicator can be addressed by a RNTI corresponding to the real PUSCH resources, instead of a common resource information (start of CB-Msg3 Tx window), such issue can be reduced. Taking legacy RA-RNTI as example, RAR message generally indicates as many resource location information as possible, thereby it can enable UEs that trigger access using different resources to identify whether a RAR is targeted for itself more accurately. Since there is no UE’s transmission resources-specific RNTI, another way we can consider is to indicate resources information in the Backoff Indicator.

- For UEs that indeed occur Msg3 transmission conflict, they also have to parse all information units for other UEs in CB-Msg4 and wait for the expiration of CB-Msg3ResponseTimer before determining failure and initiating a backoff operation. However, if they can learn the resource position where real conflicts occur from the Backoff Indicator information, the UE could detect the failure earlier and initiate re-access sooner, thereby reducing unnecessary processing of other UEs' information.

Considering the above issues, we think merely including waiting time information in the Backoff Indicator isn’t sufficient.

In order to only trigger and also efficiently trigger the failed UEs to backoff a certain time period and re-transmit CB-Msg3, it's suggested to introduce resource information of the transmission location in the Backoff Indicator in Msg4. Such transmission location is the place where CB-Msg3 transmission collisions are detected.

Proposal 1: For CB-Msg3 EDT procedure, Backoff Indicator also needs to include the resource information of time occasion and frequency occasion where collisions are detected.

Conclusion

Based on the above discussion, the following proposals are given:

Proposal 1: For CB-Msg3 EDT procedure, Backoff Indicator also needs to include the resource information of time occasion and frequency occasion where collisions are detected.

Reference

- RP-250472 Revised WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc., RAN#107, March 2025