3GPP TSG-RAN WG2 Meeting #131bis	R2-2507089

Prague, Czech Oct 13th – 17th, 2025

Source:	ZTE Corporation, Sanechips

Title:	Other corrections for R19 IoT NTN

Agenda Item:	8.9.4

Document for:	Discussion and Decision

Introduction

RAN2 has completed the basic functions of R19 IoT NTN in RAN2#131 meeting.

In this contribution, we will discuss the remaining issues or possibly required corrections, except the RRC and MAC corrections, for all the sub-topics, e.g., CB-Msg3-EDT, S&F operation and PWS support.

The possibly required corrections are mainly for TS 36.304 and TS 36.306.

Discussion

  - Issue#1: Use of NAS layer S&F monitoring list for relaxation of idle mode operations

In RAN2-131 it was concluded that the S&F monitoring list configured by NAS layer is not considered for cell reselection procedure in AS layer. RAN2 also agreed to send a LS to ask CT1 whether the Satellite ID information broadcast in SIB should be sent to NAS, e.g., to facilitate NAS layer to make full use of the configured S&F monitoring list.

Furthermore, it’s no time to discuss another topic, e.g., whether the NAS layer S&F monitoring list can have impacts on the idle mode tasks, e.g., paging monitoring. RAN2 indicated this can be further discussed in this meeting.

During [post-RAN2#131-308] email discussion, there was some discussion on this issue but no consensus can be achieved.

In SA2 spec TS 23.401, some notes related to usage of S&F Wait Timer and S&F Monitoring List are cited as below:

4.13.9 Support of Store and Forward Satellite operation 4.13.9.1 General - The MME may provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting or rejecting a NAS procedure. The MME may provide S&F Monitoring List to the UE as part of detach procedure. …………………………. - When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause, with a satellite of the same PLMN that is operating in S&F Mode. NOTE 5: When the S&F Wait Timer is running, the power consumption optimization behaviours, if any, are left for UE implementation e.g. whether to listen to paging or deactivate its Access Stratum functions. NOTE 6: The S&F Wait Timer or S&F Monitoring List doesn't affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode. NOTE 7: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one. 5.3.2.1 E-UTRAN Initial Attach ………………… If the MME is operating in S&F Mode, the MME may reject the Attach Request and if the UE supports Store and Forward satellite operation, optionally provide to the UE in the Attach Reject message any of the following: a S&F Wait Timer, a S&F Monitoring List (see clause 4.13.9). ………………… If the UE supports Store and Forward Satellite operation and the MME is operating in S&F Mode, the MME may optionally provide to the UE in the Attach Accept message any of the following: a S&F Wait Timer, a S&F Monitoring List, an Estimated S&F UL Delivery Time (see clause 4.13.9). 5.3.3.1 Tracking Area Update procedure with Serving GW change …………………… If the MME is operating in S&F Mode, the MME may reject the TAU Request and if the UE supports Store and Forward satellite operation, optionally provide to the UE in the TAU Reject message any of the following: a S&F Wait Timer, a S&F Monitoring List (see clause 4.13.9). NOTE 4: The S&F Wait Timer and S&F Monitoring List can be provided in normal TAU and periodic TAU. ……………… If the UE supports Store and Forward Satellite operation and the MME is operating in S&F Mode, the MME may optionally provide to the UE in the TAU Accept message any of the following: a S&F Wait Timer, a S&F Monitoring List, an Estimated S&F UL Delivery Time (see clause 4.13.9). NOTE 12: S&F Wait Timer, S&F Monitoring List, and Estimated S&F UL Delivery Time can be provided in normal TAU and periodic TAU. 5.3.3.2 E-UTRAN Tracking Area Update without S-GW Change If the MME is operating in S&F Mode, the MME may reject the TAU Request and if the UE supports Store and Forward satellite operation, optionally provide to the UE in the TAU Reject message any of the following: a S&F Wait Timer, a S&F Monitoring List (see clause 4.13.9). NOTE 5: The S&F Wait Timer and S&F Monitoring List can be provided in normal TAU and periodic TAU. …………………………. If the UE supports Store and Forward Satellite operation and the MME is operating in S&F Mode, the MME may optionally provide to the UE in the TAU Accept message any of the following: a S&F Wait Timer, a S&F Monitoring List, an Estimated S&F UL Delivery Time (see clause 4.13.9). NOTE 14: S&F Wait Timer, S&F Monitoring List, and Estimated S&F UL Delivery Time can be provided in normal TAU and periodic TAU. 5.3.4B.2 Mobile Originated Data Transport in Control Plane CIoT EPS Optimisation with P-GW connectivity If the MME is operating in S&F Mode, the MME may reject the NAS PDU sent in step 1 and if the UE supports Store and Forward satellite operation, optionally provide to the UE any of the following: a S&F Wait Timer, a S&F Monitoring List (see clause 4.13.9). 4a. If the S11-U connection is not established, the MME sends a Modify Bearer Request message (MME address,. ……………………….. If the UE supports Store and Forward Satellite operation and the MME is operating in S&F Mode, the MME may optionally provide in the NAS PDU message any of the following: a S&F Wait Timer, a S&F Monitoring List, an Estimated S&F UL Delivery Time (see clause 4.13.9). 12a. The eNodeB sends a RRC Downlink data message including the Downlink data encapsulated in NAS PDU. ………………………………..

It can be seen that S&F Wait Timer and S&F Monitoring List are received from the network during an attach procedure, a tracking area updating procedure or a service request procedure, via NAS signaling of either failed or successful NAS procedures.

Moreover, in NAS layer spec TS 24.301, there is also some description on the usage of S&F Wait Timer, e.g., timer T3451 with S&F wait timer duration. Some parts are cited as below:

If the UE supporting S&F satellite operation receives the S&F satellite operation parameters IE in the ATTACH REJECT message: - if the S&F wait timer duration field is present in the S&F satellite operation parameters IE, the UE shall start timer T3451 with the S&F wait timer duration provided in the S&F satellite operation parameters IE. While T3451 is running, the UE shall not initiate NAS procedures over satellite E-UTRA cells, of the current PLMN, operating in S&F mode; ………………………….. If the UE supporting S&F satellite operation receives the S&F satellite operation parameters IE in the TRACKING AREA UPDATE REJECT message: a) if the S&F wait timer duration field is present in the S&F satellite operation parameters IE, the UE shall start timer T3451 with the S&F wait timer duration provided in the S&F satellite operation parameters IE. The UE shall not attempt or reattempt to perform NAS procedures over S&F satellite E-UTRAN cell, of the current PLMN, as long as timer T3451 is running; ………………………….. If the UE supporting S&F satellite operation receives the S&F satellite operation parameters IE in the SERVICE ACCEPT message: - if the S&F wait timer duration field is present in the S&F satellite operation parameters IE, the UE shall start timer T3451 with the S&F wait timer duration value provided in the S&F satellite operation parameters IE. While timer T3451 is running, the UE shall not initiate NAS procedures over S&F satellite E-UTRA cells in the TAI list assigned to the UE, unless the S&F satellite E-UTRA cell broadcasts the same satellite ID broadcasted by the S&F satellite E-UTRA cell from which the SERVICE ACCEPT message was received; …………………………..

We understand, generally, S&F Wait Timer (T3451) is mainly used to prevent UE from re-initiating NAS procedure to an nsuitable satellite. UE cannot initiate anther NAS procedure when T3451 is running, especially after S&F wait timer duration is received from a NAS signaling of failed NAS procedure (e.g., Attach Reject message). But, if last time S&F wait timer duration (and also S&F Monitoring List) are received from the NAS signaling of a successful NAS procedure, UE is still allowed to initiate another NAS procedure, e.g., on the same satellite.

Moreover, from the following description, we understand when T3451 is running, UE is also allowed to access another cell which is not a S&F satellite E-UTRAN cell (e.g., normal mode cell) or another cell belonging to a new PLMN. It’s obvious that those cells may not be in the S&F Monitoring List.

TS 24.301 4.11.5.2 Handling of timers The UE supporting S&F satellite operation shall have no more than one timer T3451 running at a time. The UE maintains timer T3451, which is started with the value of the S&F wait time duration received from the network during an attach procedure, a tracking area updating procedure or a service request procedure. The UE stops timer T3451 if the UE successfully performs an attach procedure or a tracking area updating procedure: a) via a cell which is not a S&F satellite E-UTRAN cell; b) via a cell being in a TA which is not in the list of TAs; or c) in a new PLMN.

In a summary, we think even from NAS layer perspective, the S&F Wait Timer and S&F Monitoring List would not always restrict UE from triggering another NAS procedure, especially if the S&F Wait Timer and S&F Monitoring List are received from NAS signaling of a successful NAS procedure. They also cannot give restriction on UE performing NAS procedure to another normal mode cell or a cell in a new PLMN. Then let alone using this information to impose restrictions on the behavior of the AS layer, e.g., cell measurement/reelection, SIB monitoring or Paging monitoring.

Observation 1: Even from NAS layer perspective, the S&F Wait Timer and S&F Monitoring List would not always restrict UE from triggering another NAS procedure, especially if these information are received from NAS signaling of a successful NAS procedure. Then let alone using these information to impose restrictions on the behavior of the AS layer.

Proposal 1: It’s unreasonable to use NAS layer S&F monitoring list for relaxation of idle mode operations, e.g., to let UE stop Paging monitoring in the cell that is not within the S&F monitoring list. RAN2 no longer needs to discuss this topic.

  - Issue#2: Support of PWS in TN and NW indication

In the [post-RAN2#131-309] email discussion for UE capability, for PWS support, there was a discussion on whether UE capability for supporting acceptable cell is needed. Based on the companies’ views, later the Rapp’s proposal is that, no need of a UE capability but can adding “It is optional for a PWS-capable NB-IoT UE to support the reception of PWS message on an acceptable cell” in the description of UE capability “pws-Support-r19”.

Moreover, company indicated another issue about whether PWS capable UE supports acceptable cell in NB-IoT TN. And if acceptable cell is to be supported for TN, considering there are already deployed NB-IoT networks that will never intend to support PWS, a network indication of support of PWS is needed to prevent UEs camping on cells that will not support PWS broadcasting.

Per our understanding, from spec and signaling aspect, it’s no need to restrict that support of PWS and acceptable cell is only for NTN. Whether PWS can be supported for TN can be left to NW and UE implementation, e.g., whether SIB10~12-NB messages can be broadcast from NB-IoT TN can be left to TN implementation.

However, we don’t think it’s reasonable to prevent UEs from camping on cells that will not support PWS broadcasting. Considering the emergency alerts occur sporadically, incidentally, and is the very rare case, under any scenario, a UE must not refrain from camping on a network only because it does not support PWS provision. Such cell may still can provide normal service to the UE. Based on such thought, there is no need to introduce any kind of NW indication about PWS broadcasting.

Proposal 2a: Whether PWS can be supported for TN can be left to NW and UE implementation, e.g., whether SIB10~12-NB messages can be broadcast from NB-IoT TN can be left to TN implementation.

Proposal 2b: Under any scenario, a UE must not refrain from camping on a network only because it does not support PWS provision. There is also no need to introduce any kind of NW indication about PWS broadcasting.

Conclusion

Based on the above analysis, the following proposals are given:

Observation 1: Even from NAS layer perspective, the S&F Wait Timer and S&F Monitoring List would not always restrict UE from triggering another NAS procedure, especially if these information are received from NAS signaling of a successful NAS procedure. Then let alone using these information to impose restrictions on the behavior of the AS layer.

Proposal 1: It’s unreasonable to use NAS layer S&F monitoring list for relaxation of idle mode operations, e.g., to let UE stop Paging monitoring in the cell that is not within the S&F monitoring list. RAN2 no longer needs to discuss this topic.

Proposal 2a: Whether PWS can be supported for TN can be left to NW and UE implementation, e.g., whether SIB10~12-NB messages can be broadcast from NB-IoT TN can be left to TN implementation.

Proposal 2b: Under any scenario, a UE must not refrain from camping on a network only because it does not support PWS provision. There is also no need to introduce any kind of NW indication about PWS broadcasting.

Reference

[1] RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc., RAN#105, September, 2024

[2] RP-250472, Revised WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc. March, 2025