3GPP TSG-RAN WG2 Meeting #131bis	R2-2507097

# Prague, Czechia, 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5508 | rev | - | Current version: | 17.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PC5 Relay RLC channel configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-Core |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For L2 U2N relay UE, the PC5 Relay RLC channel configurations in sl-ConfigDedicatedNR in RRCReconfiguration needs to be configured accordingly based on the need of SRAP configuration included in sl-L2RelayUE-Config. This is because the PC5 relay RLC channel configurations included in sl-ConfigDedicatedNR are destination-agnostic and do not indicate which PC5 relay RLC channel is to be established towards which remote UE. If the relay UE blindly add all configured PC5 relay RLC channels towards all its remote UEs, that would waste PC5-RRC signaling overhead and result a lot of unused RLC channels. If not, then, based on the current specification, necessary PC5 Relay RLC channel(s) will not be setup if only SRAP configuration has been changed but sl-ConfigDedicatedNR remain unchanged. Because SRAP-config change alone will not trigger PC5-RRC procedure. Hence, the dependency on SRAP configuration needs to be clarified when L2 U2N relay UE adds a new PC5 relay RLC channel. For PC5 relay RLC channel modification/release cases, no change is needed because there is no ambiguity about which RLC channel is referred to which remote UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 5.8.9.7.2, added the sentence to describe L2 U2N relay UE only initiate PC5 relay RLC channel addition towards a remote UE if the remote UE is indicated to use this RLC channel in accordance with sl-SRAP-ConfigRelay. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality Layer 2 Sidelink UE-to-NW Relay Inter-operability: If the UE implements the CR but not the network, there is no inter-operability issue. If the network implements the CR but not the UE, there is no inter-operability issue. If one UE implements the CR but not the other UE, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | L2 U2N relay UE will either unable to set up needed PC5 relay RLC channels towards its connected L2 U2N remote UEs or set up useless PC5 Relay RLC channels. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.9.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.8.9.7.2	PC5 Relay RLC channel addition/modification

Upon PC5-RRC connection establishment between the L2 U2N Relay UE and L2 U2N Remote UE, the L2 U2N Relay UE shall:

1>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

1>	apply RLC specified configuration of SL-RLC0 as specified in clause 9.1.1.4:

1>	apply RLC default configuration of SL-RLC1 as defined in clause 9.2.4 if the L2 U2N Relay UE is in RRC_IDLE/INACTIVE state;

A L2 U2N relay UE initiates a PC5 Relay RLC channel addition towards a L2 U2N Remote UE if the sl-RLC-ChannelID reveived in sl-ConfigDedicatedNR is indicated for this L2 U2N Remote UE in accordance with sl-SRAP-ConfigRelay, and a PC5 Relay RLC channel with the received sl-RLC-ChannelID was not configured before.

The UE shall:

1>	if the PC5 Relay RLC channel addition/modification was triggered due to the reception of the RRCReconfigurationSidelink message; or

1>	after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel addition/modification was triggered due to the configuration received within the sl-ConfigDedicatedNR:

2>	if the current configuration contains a PC5 Relay RLC channel with the received sl-RLC-ChannelID or sl-RLC-ChannelID-PC5:

3>	reconfigure the sidelink RLC entity in accordance with the received sl-RLC-Config or sl-RLC-ConfigPC5;

3>	reconfigure the sidelink MAC entity with a logical channel in accordance with the received sl-MAC-LogicalChannelConfig or sl-MAC-LogicalChannelConfigPC5;

2>	else (a PC5 Relay RLC channel with the received sl-RLC-ChannelID or sl-RLC-ChannelID-PC5 was not configured before):

3>	establish a sidelink RLC entity in accordance with the received sl-RLC-Config or sl-RLC-ConfigPC5;

3>	configure the sidelink MAC entity with a logical channel in accordance with the received sl-MAC-LogicalChannelConfig or sl-MAC-LogicalChannelConfigPC5.

End of change