3GPP TSG-RAN WG2 Meeting #131bis	R2-257120

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	10.4

Source:	Apple

Title:	Planning for 6G Mobility Study

WID/SID:	FS_6G_Radio

Document for:	Discussion / Decision

# Introduction

According to SID (RP-252912), the design of 6GR should be simple, efficient, with optimal user experience. This applies to Mobility as well. And Mobility is a complex topic that impacts several components of the 6G RAT. In this paper, we dissect this and attempt at proposing a skeleton on how to approach the study of Mobility towards a simple, yet efficient architecture.

# Dissection of Mobility feature

Figure 1 below is taken from NR TS38.300 where the components that are impacted by Mobility are rounded in. One can imply that Mobility permeates through several layers and impacts a large set of functionalities in both AS and NAS layers. Even though we focus on RAN aspects here, it can be seen that the impact is big nonetheless.

Figure 1 Mobility impacts to RAN/CN architecture

## Architecture impact study

RAN architecture heavily influences the mobility operation. For example, the below would shape how mobility would be designed:

- The definition of a cell/BWP, and mobility within or across cells.

- The components/nodes (including the UE) that are actors in a typical handover (for eg., CU change needed or not)

- The (change and impact of) security across the nodes, including the UE

- The need of Control and User Plane (re)establishment across the nodes and the UE. This also includes the potential new plane which might be required for new 6G services (ISAC for eg)

To avoid 5G pitfalls, RAN2 should try to study the impacts and influence the RAN architecture to ensure that Mobility operation is efficient (minimize the interruption time and impacted components for example) and also be scalable for future 6G releases. It is important to note that such effort requires co-ordination with RAN3, SA2 and SA3 (for security) and so early identification of RAN architecture design directions and preferences from RAN2 would be beneficial to feedback to the relevant WGs for their input and consideration.

Observation 1: Architectural (and deployment) impact was not fully considered from 5G day 1, resulting in mobility additions to multiple layers in later releases, creating multiple solutions adding complexity.

Observation 2: Latency reduction efforts for mobility started in later releases of NR (for eg LTM) and the architectural aspects (for eg., CU-DU split related) were not considered in initial NR releases.

Observation 3: Security requirements result in multiple layers of the protocol stack getting impacted by Mobility, also increasing latency and signaling load across nodes of the RAN ecosystem.

Observation 4: SA3 security requirements and RAN architecture design heavily impacts mobility KPIs. In the areas of RAN architecture and security assumptions, early identification should be performed on design directions that can improve mobility performance and engagement with RAN3/SA2/SA3 WGs would be needed about these, would help with better mobility design.

We see that this is the critical first step in the mobility study to ensure that RAN2’s efforts align with progress from other concerned WGs.

Proposal 1: RAN2 mobility study to include a sub-section on RAN architectural design changes that minimize the interruption to control plane and user plane (for eg., cell definition, aggregation definition, network functional placement etc) including identifying the security assumptions that facilitate efficient mobility handover. The study also aims at indicating these to RAN3/SA2/SA3 early for collaboration and alignment.

## Reduced latency and mobility robustness

Several improvements were considered in 5G NR in later releases to reduce the latency/interruption time and increase the mobility robustness.  Rel-16 DAPS, Rel-16 CHO, Rel-18 LTM, Rel-19 CLTM, RACH-less HO, RACH-less LTM etc.  However, incremental additions led to inefficient progress. Also many of the features from the 5G NR were not adopted.

Observation 5: Many of the 5G NR mobility enhancements were not adopted due to the complexity and/or practical deployment issues.

Observation 6: It would be prudent that RAN2 picks the topics for 6G DAY 1, that are commercially useful.

While we acknowledge that first release in any technology cannot have all the bells and whistles, RAN2 should use the study-item phase to evaluate the options that were in 5G with the aim of improving it for 6G as well as attempting to make these simpler and practical.

### Link (re)establishment

Radio Link establishment and link failure recovery for 5G NR mobility also follows the multiple option theme where RAN2 has specified a different method for each of the mobility types. In addition, we also have different link procedures for different UE states in 5G NR. For example, the UE link establishment method coming from RRC_INACTIVE state is different from link failure recovery which is followed by UE re-establishment. For both of these procedures, the UE context is likely present in the network and UE needs to re-connect to that context. But the security handling procedure is different for each of these cases.

Observation 7: Link (re)establishment procedures are different depending on UE mobility and UE RRC states in 5G NR, while the intent is very similar. 6G study should evaluate this for unification and simplification.

Proposal 2: Starting with 5G NR methods as the baseline, RAN2 mobility study to include a sub-section on studying various methods/procedures (eg., RACH-less handover, conditional handover, link establishment and recovery etc) to reduce the latency/interruption time and increase the mobility robustness in 6G Mobility. The evaluation should be done with practicality and adaption aspect as also the guiding principles. Collaboration with RAN1 and RAN4 as needed.

## Simplification of Mobility

### Mobility procedure study

Through the course of multiple releases of 5G NR, RAN WGs have introduced multiple procedures for handling UE mobility within RAN network, including triggering the handover via multiple layers. This has resulted in different procedures at the UE and the network for handling UE mobility, and this is something the needs to be studied to see if we can simplify to a single procedure.

Figure 2 Handover triggering procedures in 5G NR

Observation 8: 5G NR Mobility has multiple procedures in different layers of the protocol stack that trigger mobility. 6G study should evaluate if the redundancy can be removed and the mobility procedures can be simplified/unified and limited to one layer in the protocol stack.

### RRM

#### Measurement related improvements

5G Mobility has resulted in multi-layer RRM operations (for example, cell-level L3 measurements, beam level L1 measurements with or without filtering etc). The critical aspects for RRM is the need for the UE to perform a measurement (based on requirements) and provide this to the NW to aid with mobility decisions. In other cases, this measurement can also result in UE making the mobility decision in co-ordination with the network.  In the interest of simplification (which is one of the aims of the 6G study), RAN2 should study if and how the RRM aspect can be simplified towards a unified procedure for this operation.

Observation 9: 5G NR Mobility RRM has multiple options impacting multiple layers for the measurement operation. RAN2 should study the RRM procedures with aim of simplifying this and trying to limit the impacted layers.

#### Measurement framework

Similar to the presence of multiple measurement procedures at the UE in 5G NR, we also have multiple layers where the UE measurement reporting can be done (L3 RRC based, L2 MAC CE based, L1 RSRP based).

In addition, the measurement framework is quite complex with many sub-components that vary with the type of measurement. Figure 4 provides a snapshot of different sub-component related to measurement framework.

|  | RRM measurement | LTM measurement | MIMO measurement |
| --- | --- | --- | --- |
| Configuration (RS) | Measurement config -> MO Config | LTM config -> CSI-ResourceConfig | serving cell config -> CSI-MeasConfig |
| Configuration (Report) | Measurement config -> Report Config | serving cell config -> CSI-MeasConfig | serving cell config -> CSI-MeasConfig |
| Association between resource and report cfg | Meas ID to associate MO & ReportCfg | Report config is associated with resource config (in LTM config) | Report config is associated with resource config(in serving config) |
| Beam level report | beam result (after L3 filter) configurable | Beam level | Beam level |
| Cell level report | cell quality derived from beam result | N/A | N/A |
| L1 filter (Up to UE Impl.) | Yes | Yes | Yes |
| L3 filter | In RRC, applicable for cell/beam result | N/A | N/A |
| Report type | RRM Event, periodical, others | LTM Event, Periodical, SP, AP | MIMO Event, Periodical, SP, AP |
| Event triggered report | Support (Ax,Bx, Hx, AxHx) | Support (LTM#2-5) | Support (event#1,2,7) |
| Layer | RRC | MAC, L1 | L1, L1 |
| Report | RRC measurement report | MAC MR MAC CE, L1 UCI | L1 UCI |
| Report Content | Serving cell, neighbor cell (cell&beam) | Neighbor cell’s beam, serving cell’s beam | Current beam, candidate beam |
| Transmisison | Generate before transmission | Generate MR when transmitted | Generate MR when transmitted |

Figure 3 Different measurement frameworks in 5G NR

While we acknowledge that many of the sub-components are needed, the 6G study should also look at simplifying this while keeping it scalable for 	future additions.

Observation 10: 5G NR measurement framework is complex with many sub-components, and with multiple measurement types, all these sub-components have multiple options/means further complicating Mobility. 6G study should try to simplify this part.

Proposal 3: RAN2 to agree on a sub-section in 6G mobility study which evaluates the feasibility of simplifying the UE mobility and measurement procedures. The study includes (but not limited to) evaluations on simplifying configurations, measurements (for eg., cell vs beam). and measurement evaluations, reporting, triggering of handover etc (aiming towards a single procedure for each of these, where possible).

#### Measurement Gap Impact

Another big aspect of RRM is the Measurement Gap (MG) sub-feature, where NR has been adding several MG types and procedures. Figure 3 below provides some details on the MG complexity in NR. While anything related to MG needs RAN4 input, it would be beneficial if RAN2 studies on simplification and scalability of this and can co-ordinate with RAN4 towards simplifying the MG operation for 6G (assuming that we need this).

Main features of MG. Rel-15; - Per UE gap - Independent gap (FR1 gap, FR2 gap) - Gapless inter-RAT measurement (LTE->NR meas without gap). Rel-16; - Dynamic NeedForGap - interFrequencyMeas-NoGap. Rel-17; - IndependentGapConfig-maxCC (hardware sharing for FR1/FR2.) - Pre-MG (BWP specific gap on/off) - NCSG (network controlled small gap): RF retuning/baseband preparation - Concurrent gaps - FR2 UL gap - MUSIM gap - parallelMeasurementGap-r17 in NTN (2 gaps on one MeasObject). Rel-18; - No gap no interruption, No gap with interruption - Mobility WI: inter-frequency L1 measurement gap (only R15/R16 gap) - LTE measurement without gap (CRS contained in BWP, or available spare RF) - Joint gap configuration from RAN4.

Figure 3 Measurement Gap features of 5G NR

Observation 11: 5G NR Measurement Gap sub-feature has become complex over several releases and it would be better for RAN2 to evaluate methods to make this simpler and scalable.

Proposal 4: RAN2 to evaluate on the feasibility of simplification of measurement gap dynamics (configuration, handling, capability reporting etc) compared to 5G NR with early identification of aspects that simplify and make it scalable, with the plan to indicate this to RAN4 (and RAN1) for collaboration and alignment.

## AI/ML impact study

Only the later releases of 5G NR (Rel-19 and Rel-20) have considered AI/ML as a factor that influences the mobility operation. And the scope of 5G NR is limited to only some aspects (for eg., LTM is not considered). For 6G, we should avoid this and consider AI/ML as an inherent part of Mobility where AI can be involved in different aspects of the mobility operation. The 6G study should ensure that proper evaluation of the impacts of AI/ML is done in this exercise. Also, we should reuse the effort from 5G NR (LCM etc) and evaluate our study considering this as the baseline.

Observation 12: While RAN2 started late in considering the AI/ML impact to Mobility, 6G should make use of the work/study done in 5G NR as the baseline for further enhancement study.

Proposal 5: A sub-agenda item should be added for AI/ML integration into Mobility with the study focuing on enhancing this for 6G using 5G NR AI/ML as the baseline.

## Harmonization

### NTN impacts

NTN in NR was introduced a few releases after the first NR release and the mobility aspects of NTN were considered NTN-specific (due to characteristics of NTN operation) and were largely separate. 6G TR objective (shown below) requires that evaluation should be done to see if TN and NTN cane be harmonized from mobility perspective.

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

### MIMO and Mobility inter-play

One of the important learnings from 5G NR LTM feature is the impact of it’s design with the MIMO enhancements work-item where both of these were providing enhancements on the TRP operation with beams. The LTM WI differed from MIMO WI in that the beams were from the neighbor cells while for MIMO they were from the same serving cell.

However, considering the minor difference (from UE perspective), the features diverged in UE procedures quite a bit, even when the intended functionality is the same (for eg., reporting of L1 beam measurements or triggered events – MIMO used PUCCH, while LTM used MAC CE). This resulted in UEs implementing the same functionality more than once at different layers. 6G should attempt to avoid such occurrences.

Observation 13: MIMO and LTM WIs from 5G NR produced different UE procedures for the same functionality, which created redundancy and added inefficiency and additional implementation cost to the 3GPP ecosystem. 6G study should attempt to avoid such occurrences.

Proposal 6: 6G Mobility study also includes a section on the feasibility of unification of mobility procedures with collaboration with RAN1 and RAN4 as needed.

  - for TN and NTN mobility

  - for inter-TRP beam mobility and inter-cell beam mobility

## IRAT and NON-CONNECTED mode improvements

The mobility study for 6G should also include the remaining aspects, viz., mobility in states where the UE is not in CONNECTED mode, and also facilitate an efficient IRAT mobility.

RANP-109 has an agreement for the WGs to focus on IRAT mobility for migration and deployment option aspects assuming 6G standalone operation. In our view, making IRAT mobility more efficient for 6G-5G could alleviate the problems that migration is trying to address. Also the RAN 6G study objective directs the WGs to make 6G more efficient that 5G and IRAT mobility can be included as part of this.

Proposal 7: 6G mobility study to include a sub-agenda for improving non-connected mode mobility (for other RRC states than CONNECTED) and for studying the design choices that make 6G-5G IRAT mobility more efficient when compared to previous generation.

# Observations

Observation 1: Architectural (and deployment) impact was not fully considered from 5G day 1, resulting in mobility additions to multiple layers in later releases, creating multiple solutions adding complexity.

Observation 2: Latency reduction efforts for mobility started in later releases of NR (for eg LTM) and the architectural aspects (for eg., CU-DU split related) were not considered in initial NR releases.

Observation 3: Security requirements result in multiple layers of the protocol stack getting impacted by Mobility, also increasing latency and signaling load across nodes of the RAN ecosystem.

Observation 4: SA3 security requirements and RAN architecture design heavily impacts mobility KPIs. In the areas of RAN architecture and security assumptions, early identification should be performed on design directions that can improve mobility performance and engagement with RAN3/SA2/SA3 WGs would be needed about these, would help with better mobility design.

Observation 5: Many of the 5G NR mobility enhancements were not adopted due to the complexity and/or practical deployment issues.

Observation 6: It would be prudent that RAN2 picks the topics for 6G DAY 1, that are commercially useful.

Observation 7: Link (re)establishment procedures are different depending on UE mobility and UE RRC states in 5G NR, while the intent is very similar. 6G study should evaluate this for unification and simplification.

Observation 8: 5G NR Mobility has multiple procedures in different layers of the protocol stack that trigger mobility. 6G study should evaluate if the redundancy can be removed and the mobility procedures can be simplified/unified and limited to one layer in the protocol stack.

Observation 9: 5G NR Mobility RRM has multiple options impacting multiple layers for the measurement operation. RAN2 should study the RRM procedures with aim of simplifying this and trying to limit the impacted layers.

Observation 10: 5G NR measurement framework is complex with many sub-components, and with multiple measurement types, all these sub-components have multiple options/means further complicating Mobility. 6G study should try to simplify this part.

Observation 11: 5G NR Measurement Gap sub-feature has become complex over several releases and it would be better for RAN2 to evaluate methods to make this simpler and scalable.

Observation 12: While RAN2 started late in considering the AI/ML impact to Mobility, 6G should make use of the work/study done in 5G NR as the baseline for further enhancement study.

Observation 13: MIMO and LTM WIs from 5G NR produced different UE procedures for the same functionality, which created redundancy and added inefficiency and additional implementation cost to the 3GPP ecosystem. 6G study should attempt to avoid such occurrences.

# Conclusions

Proposal 1: RAN2 mobility study to include a sub-section on RAN architectural design changes that minimize the interruption to control plane and user plane (for eg., cell definition, aggregation definition, network functional placement etc) including identifying the security assumptions that facilitate efficient mobility handover. The study also aims at indicating these to RAN3/SA2/SA3 early for collaboration and alignment.

Proposal 2: Starting with 5G NR methods as the baseline, RAN2 mobility study to include a sub-section on studying various methods/procedures (eg., RACH-less handover, conditional handover, link establishment and recovery etc) to reduce the latency/interruption time and increase the mobility robustness in 6G Mobility. The evaluation should be done with practicality and adaption aspect as also the guiding principles. Collaboration with RAN1 and RAN4 as needed.

Proposal 3: RAN2 to agree on a sub-section in 6G mobility study which evaluates the feasibility of simplifying the UE mobility and measurement procedures. The study includes (but not limited to) evaluations on simplifying configurations, measurements (for eg., cell vs beam). and measurement evaluations, reporting, triggering of handover etc (aiming towards a single procedure for each of these, where possible).

Proposal 4: RAN2 to evaluate on the feasibility of simplification of measurement gap dynamics (configuration, handling, capability reporting etc) compared to 5G NR with early identification of aspects that simplify and make it scalable, with the plan to indicate this to RAN4 (and RAN1) for collaboration and alignment.

Proposal 5: A sub-agenda item should be added for AI/ML integration into Mobility with the study focuing on enhancing this for 6G using 5G NR AI/ML as the baseline.

Proposal 6: 6G Mobility study also includes a section on the feasibility of unification of mobility procedures with collaboration with RAN1 and RAN4 as needed.

  - for TN and NTN mobility

  - for inter-TRP beam mobility and inter-cell beam mobility

Proposal 7: 6G mobility study to include a sub-agenda for improving non-connected mode mobility (for other RRC states than CONNECTED) and for studying the design choices that make 6G-5G IRAT mobility more efficient when compared to previous generation.

# References

- RP-252912	Study on 6G Radio