3GPP TSG-RAN WG2#131-bis	R2-2507131

Prague, Czech Republic, Oct 13th – 17th 2025

Source: 	Fujitsu

Agenda item:	10.3.2

Title:	Initial consideration on Control plane aspects in 6G

Document for:	Discussion and decision

# Introduction

The SID of Rel-20 Study on 6G Radio was approved in RAN#108 and updated in RAN#109 [1].

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

The SID also describes the following objectives about the Radio interface protocol architecture and procedures for 6GR.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this contribution, we would like to provide initial views on 6GR Control Plane (CP) aspects in terms of possible improvements/enhancements from NR.

# Discussion

## Background: Learnt from NR Control Plane

The 6G RAN architecture as per [1], RAN2 and RAN3 will study “RAN internal functional split, interfaces, protocol stacks and procedures”. As per [2], at RP#109 the “Requirements for architecture and migration” were agreed to be captured in the TR 38.914 section 5.2. One of the requirement states “The 6G RAN architecture shall allow for control plane and user plane separation”. 5G NR supports the separation of control plane and user plane not only from protocol perspective but also from RAN architecture perspective. This means the control plane signalling (i.e. RRC and NAS messages) is mapped to SRB and the user data is mapped to DRB. Further, CU-CP and CU-UP functions are separate. CU-CP and CU-UP as network elements interact on standardized E1 interface. In practical 5G NR deployments with split CU-CP and CU-UP architecture, separate handling of the control plane and user plane functions, the network operators benefit from separate dimensioning of the network elements, pooling gains of the deployed capacity leading to improved resource utilization and minimization of disruption of service when fault occurs in the system. Further, the distributed split RAN architecture with separation of CP and UP encourages the network operator to choose cloud-based implementation of the network which allows containerization and virtualization of the network elements, more efficient resource allocation, reduced operational costs, and enhanced network flexibility. It is desirable that these benefits which the network operators harvest in 5G NR deployment due to separation of control plane and user plane shall be made available in the 6GR.

Therefore, we propose the following:

Proposal#1

- Design of the 6GR control plane and user plane shall maintain the separation not only from protocol perspective but also architecture perspective.

- RAN2 and RAN3 need to jointly discuss the separation of CP and UP from architecture perspective.

## 6G Control plane

According to [1], a “Single technology framework based on a stand-alone architecture” is envisaged for next generation 6G systems targeting the IMT-2030 requirements. This means the initial deployment of 6G system will be primarily standalone system unlike the initial 5G NR system deployment which were based on the Non-standalone (NSA) architecture relying on tight interworking between LTE RAT as anchor/master and NR as secondary RAT. The 5G NR control plane architecture is flexible and modular supporting inter node RRC messages, encapsulation of RRC messages generated by one RAT to be transported in RRC container messages of another RAT in case of tight inter-RAT interworking and embedding NAS container in RRC messages. To support new services and additional deployment architecture (apart from the standalone) in future the 6GR control plane architecture needs to be flexible, modular and forward compatible right from the beginning so that backward incompatibility can be avoided in future 6GR releases.

Further, existing LTE and 5G NR deployment (both standalone and non-standalone) in many countries leverage on the distributed split RAN architecture with the standardized interfaces between CU-CP and DU (i.e. F1 interface) and CU-CP and CU-UP (i.e. E1 interface). The 6G RAN architecture as per [1], RAN2 and RAN3 will study “RAN internal functional split, interfaces, protocol stacks and procedures”. The 5G NR control plane architecture does not restrict the deployment option decided by the mobile operator whether it is centralized (i.e. non-split) or distributed (i.e. Split) and whether it is virtualized using cloud-native principles or monolithic software using customized hardware. It is therefore desirable that 6GR control plane design is not restricted to a particular approach of deploying the 6G system.

Control plane signalling robustness is an important requirement for some use cases and services requiring ultra-high reliability communication links. In 5G NR robustness of control plane signalling is supported in both standalone architecture for carrier aggregation scenario and in dual connectivity scenario. Therefore, given that existing 5G services and new services with ultra-high reliability requirement need to be supported by 6G, the 6GR control plane architecture design shall support robustness for the control plane signalling.

Additionally, according to [1] it states “6GR and RAN design shall ensure that the 6G System can also operate without AI/ML” so this means AI/ML may not necessarily put special requirement for the 6GR control plane architecture. The generic control plane framework shall be extendable (if needed) to support signalling required to configure the UE for AI/ML aspects. The framework that may be required for data collection from UE for AI/ML purposes is independent from the control plane design.

In summary we propose the 6GR control plane architecture needs to consider the following:

Proposal#2

- 6GR control plane design shall be flexible, modular and forward compatible to support existing/new services and additional architecture deployment (apart from standalone) in future.

- 6GR control plane design shall support both centralized (i.e. Non-split) and distributed (i.e. Split) RAN architecture deployments.

- 6GR control plane design shall support robustness for the control plane signalling.

- 6GR control plane baseline design shall be generic and shall be extendable to introduce AI/ML specific requirements.

### 6GR Control Plane architecture

The control plane functionalities and design are directly impacted by the RAN architecture choice, the key performance indicators to be met by 6GR (e.g. KPIs for latency, mobility, energy efficiency etc), the separation of CP and UP, the support for virtualization, the support for cloudification of the RAN, the support for RAN sharing, network slicing, support for existing and new services etc. As per TS 38.300 the following functions are provided by the RRC layer in 5G NR.

Table 1 - Summary of Functions provided by NR RRC

| System Information Provisioning, Emergency Warning messages and Positioning Information | Paging notifications, Bearer configuration (SRB/DRB) and handling, QoS management |
| --- | --- |
| Connection control: establishment/resume, re-establishment, release, configuration/reconfiguration | Lower protocol layer configuration and reconfiguration, radio resource configuration, SCell addition, modification and release etc |
| Security key management, access stratum security configuration | Measurement configuration, Mobility and UE context management |
| PLMN selection, cell selection and reselection, cell access control and barring | UE capability handling |
| RLF handling, SON, ANR, MDT, etc | Services: VoNR, Sidelink, MBMS, etc, |
| Network sharing, network slicing etc | Other functions like allocation of UE Identifiers, handling UE assistance information, transport of NAS messages and generic error handling |

The above listed 5G NR RRC functions have played crucial role in commercialization of 5G NR and some of them extended in 5G Advanced to support new capabilities. Some of the above-mentioned RRC functions may need to be supported in the baseline 6GR and some functions may get specified in future releases. As the technical discussion during study item progresses some of the functions may get reused from 5G NR RRC, some of them get simplified by avoiding duplicate methods or avoiding unnecessary complexity, some of them get re-designed from scratch as per 6G requirements which are changed compared to 5G requirements, some of them get distributed between RRC and MAC depending on the architecture choice and enhancements needed to improve the performance compared to 5G NR. For example, if the CU/DU split RAN architecture is considered for 6GR then it may be possible that for some RRC functions the initial configuration and release is performed by CU-CP based on the L3 input while the subsequent re-configuration/modification of the functionality is performed by CU or DU depending on whether the trigger condition or feedback is available at L2 or L3. This basically means splitting the RRC function between CU-CP and DU as shown in Figure 1 below. The master or anchor RRC resides in the CU-CP and takes care of all configurations and re-configurations. The slave or auxiliary RRC resides in the DU and handles some re-configurations depending on the L2 trigger or feedback and latency requirement. Such split RRC architecture avoids frequent coordination between DU and CU-CP to provide the L2 trigger or feedback at CU-CP and subsequently the CU-CP taking action to prepare the final RRC message leading to some delay and increased processing load on the CU-CP. From the UE perspective, the split RRC function at the 6GR gNB is transparent to the UE. This means the UE is unaware the RRC message is generated by CU-CP or DU. RRC messages generated by the CU-CP, the security function for encryption and integrity protection is managed by the CU-CP. RRC messages generated by the DU, the security function for encryption and integrity protection is managed by the DU. This may come for free if the DU needs to also provide the security function for the MAC-CE. The security function for decryption and integrity check of the received RRC messages at the UE side is handled at the PDCP layer.

Figure 1 - 6GR RRC Architecture

We propose the following:

Proposal#3

- NR RRC functions are considered as baseline for 6GR RRC.

- List of baseline 6GR RRC functions to be considered during study item is FFS.

- Distribution of 6GR RRC functions between CU-CP and DU is not precluded.

### RRC state design

5G NR standards support the RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. The RRC_INACTIVE state is not only a power efficient state from the UE perspective but also reduces latency allowing quick data transfer. However, in majority of the network deployments around the globe the RRC_INACTIVE state is not supported widely. From the 5G NR experience it seems the RRC_INACTIVE support from network perspective is more beneficial for specific services catering to those use cases the operator intends to provision (e.g. RedCap, IoT etc.). Further, from network operator perspective the implementation of the RRC_INACTIVE on top of the RRC_IDLE introduces complexity to support the RAN paging in addition to the support of CN paging, configuration of the RAN-based Notification area, RRC suspend/resume procedures for connection control, the UE context management at RAN and small data transfer without state transition. This is independent of the Core network states that SA2 may study for the 6G system. RAN2 and SA2 may need coordination in the beginning of study item in the form of joint discussion and exchange of the usual liaison statements on the RRC and CN states and their relation, possible avoidance of duplicate functions like RAN paging and CN paging, UE state transparent to the CN etc.

Considering the 5G NR experience, we propose the following:

Proposal#4

- 6GR shall support the RRC_IDLE and RRC_CONNECTED states as baseline.

- During the study consult with SA2 on the CN defined UE states and its relationship with RRC states.

- Support of RRC_INACTIVE state shall be further studied during the study item to alleviate some of the pain points inherent in 5G NR.

### Connection management

As mentioned in above section regardless of the RRC states to be considered and studied in 6GR, from the UE perspective the UE at a given point of time is expected to be in a single RRC state.

Regardless of the RAN architecture, evolution of the RAN architecture, support of multiple radio spectrum sharing (MRSS), support of dual registration from the UE perspective etc., the UE supports one RRC instance i.e. RRC connection management of the UE is under the control of single RAT during the lifetime of the RRC connection.

We propose the following:

Proposal#5

- From UE perspective for connection management, the UE is expected to be in a single RRC state at a given point of time.

- From the UE perspective the UE supports one RRC instance i.e. RRC connection management of the UE is under the control of single RAT during the lifetime of the RRC connection.

### System information

LTE System Information (SI) provisioning is based on periodic broadcast of all SI. 5G NR from Day 1 introduced on-demand system information provisioning for Other System Information (i.e. SIB2, SIB3 etc) catering to the energy efficiency and signalling overhead requirement. However, the minimum system information (i.e. MIB and SIB1) were always periodically broadcasted up to Rel-18. In Rel-19, the mechanism to provide SIB1 on-demand in a NES cell was introduced based on SIB1 Request Config information received by the UE from an anchor cell. 6GR system information provisioning design shall cater to the network energy saving requirement such that system information can be provisioned in on-demand manner based on UE request in the standalone scenario with and without support of the anchor cell.

LTE and 5G NR the periodic broadcast of system information is based on the concept of non-overlapping staggered SI windows which are of same size and the concept of BCCH modification period for the change in system information. The broadcast of system information in the SI windows sometimes create scheduling restriction for user data scheduling especially during moderate to high cell traffic conditions. In the CU-DU distributed split RAN architecture deployment the minimum system information (i.e. MIB and SIB1) broadcasting is handled by the DU and the OSI broadcasting is handled by the CU. The concept of SI windows for OSI broadcasting requires coordination between the CU and DU on the F1 interface. Similar coordination on the F1 interface is required when SI change notification through paging/short message is sent to the UEs in the cell considering the BCCH modification period. Given that 6GR is expected to provision system information in on-demand manner, the concept of SI window and BCCH modification period shall be revisited considering the constraints it introduces for scheduling the user data when SI is broadcasted. Alternate concepts to existing ones used for SI broadcasting and SI change shall be explored during the study item considering the energy saving requirement and alleviating the scheduling constraints.

In addition, in 5G NR, the scheduling information and the broadcast status of other SI is indicated by SIB1. If a UE wants to acquire an on-demand SI, the UE needs to initiate on-demand SIB1 request procedure first to acquire SIB1 and check the broadcast status of the requested SI and then initiate on-demand SI procedure to acquire the requested SI. These two procedures including SIB1 request procedure and SI request procedure would cause more UE power and more radio resources. Therefore, in 6GR, it can be studied how to acquire an on-demand SI with a simplified procedure.

In summary we propose the 6GR system information provisioning needs to consider the following:

Proposal#6

- 6GR system information provisioning design shall cater to the network energy saving requirement such that system information can be provisioned in on-demand manner based on the UE request in the standalone scenario with and without support of the anchor cell.

- The concept of periodic non-overlapping staggered SI windows and the concept of BCCH modification period shall be revisited in study item. 6GR system information broadcast shall explore alternate concepts to the existing ones to be used for SI broadcasting and SI change during the study phase.

- 6GR study shall strive to simplify the on-demand system information request procedure to save the UE power, the network energy and the radio resources consumption for provisioning SI in on-demand manner.

# Conclusion

In this paper we presented some initial views on RAN2 study directions for 6GR CP aspects and have come up with the following proposals to be discussed and agreed:

Proposal#1 on CP and UP separation

- Design of the 6GR control plane and user plane shall maintain the separation not only from protocol perspective but also architecture perspective.

- RAN2 and RAN3 need to jointly discuss the separation of CP and UP from architecture perspective.

Proposal#2 for 6GR RRC design requirements and principle

- 6GR control plane design shall be flexible, modular and forward compatible to support existing/new services and additional architecture deployment (apart from standalone) in future.

- 6GR control plane design shall support both centralized (i.e. Non-split) and distributed (i.e. Split) RAN architecture deployments.

- 6GR control plane design shall support robustness for the control plane signalling.

- 6GR control plane baseline design shall be generic and shall be extendable to introduce AI/ML specific requirements.

Proposal#3 for RRC Functions

- NR RRC functions are considered as baseline for 6GR RRC.

- List of baseline 6GR RRC functions to be considered during study item is FFS.

- Distribution of 6GR RRC functions between CU-CP and DU is not precluded.

Proposal#4 for RRC States

- 6GR shall support the RRC_IDLE and RRC_CONNECTED states as baseline.

- During the study consult with SA2 on the CN defined UE states and its relationship with RRC states.

- Support of RRC_INACTIVE state shall be further studied during the study item to alleviate some of the pain points inherent in 5G NR.

Proposal#5 for RRC Connection Management

- From UE perspective for connection management, the UE is expected to be in a single RRC state at a given point of time.

- From the UE perspective the UE supports one RRC instance i.e. RRC connection management of the UE is under the control of single RAT during the lifetime of the RRC connection.

Proposal#6 for System Information Provisioning

- 6GR system information provisioning design shall cater to the network energy saving requirement such that system information can be provisioned in on-demand manner based on the UE request in the standalone scenario with and without support of the anchor cell.

- The concept of periodic non-overlapping staggered SI windows and the concept of BCCH modification period shall be revisited in study item. 6GR system information broadcast shall explore alternate concepts to the existing ones to be used for SI broadcasting and SI change during the study phase.

- 6GR study shall strive to simplify the on-demand system information request procedure to save the UE power, the network energy and the radio resources consumption for provisioning SI in on-demand manner.

# References

- RP-252912, Revised SID: Study on 6G Radio

- RP-252870, pCR for 38.914 on requirements for architecture and migration for 6G