3GPP TSG-RAN WG2 Meeting #131bis			         R2-2507141

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	10.2

Source: 	LG Electronics Inc.

Title:         	Consideration on general aspects for 6G

Document for: 	Discussion and Decision

Introduction

In this contribution, we identify key pain points observed in NR and discusses their implications for 6G. Based on these observations, we share our initial views on RAN2 general aspects to achieve the key goals for successful 6G introduction, including support for new deployment scenarios, a unified solution for all device types, and sustainability through energy saving, and efficient spectrum utilization.

Discussion

New potential deployment scenarios

For 6GR, we can take into account the following deployment scenarios which may affect common channel design/procedure, idle mobility and initial access.

A single/multi-cell multi-carrier scenario

6G is expected to operate across a wide range of spectrum, including sub-6 GHz, mmWave, and sub-THz. In RAN#109, RAN approved the requirement that 6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR.

In our view, one of the deployment scenarios related to this 6G requirement can be based on support of sparse sync rasters in frequency domain e.g., even sparser sync raster than that in NR. The main intention of 6GR design based on the sparse sync raster is to reduce the UE power consumption and cell search delay during initial cell selection. Assuming that wider channel bandwidth for specific frequency band or frequency ranges could be one way to define sparse sync rasters, 6GR can be deployed on a multi-carrier single cell where a cell-defining SSB can be periodically transmitted on the sync raster while on-demand SSB is irregularly provided on off-sync raster for better spectrum utilization on wide channel bandwidth with diverse device types.

Anther deployment scenario related to this 6G requirement is non-contiguous spectrum for which spectrum utilization efficiency can be improved e.g. by combining multiple frequency blocks that are not adjacent to each other, either within the same band or across different bands, to create a larger effective bandwidth for a single user or service. Well-known CA-based deployment i.e. multi-carrier multi-cell deployment can support aggregation of fragmented spectrum resources, while non-CA based deployment e.g. multi-carrier single cell deployment can be also studied in 6GR.

We think that the multi-carrier deployments can be studied for UEs in RRC_IDLE/INACTIVE as well as in RRC_CONNECTED. Especially, for UEs in RRC_IDLE/INACTIVE, we can study how to design common channel procedure, idle mobility and initial access in the multi-carrier deployments. In addition, we can study a scenario where a UE is directed from a primary carrier e.g. a sync raster carrier to a secondary carrier e.g. a off-sync raster carrier. Moreover, early multi-carrier setup for initial access, e.g. fast SCell setup in NR, can be also considered in multi-carrier scenarios, which is beneficial for efficient management of network energy and UE power by releasing/suspending the RRC connection immediately data transmission/reception.

Pain Points of NR: Lack of spectrum utilization and operations over one or more carriers/bands for RRC_IDLE/INACTIVE

Observation 	6GR can be designed to support a multi-carrier single cell deployment as well as multi-carrier multi-cell deployments for improved spectrum utilization and operations over one or more carriers/bands, compared to NR.

Intra-frequency cell-cluster massive MTRP scenario

To improve reliability at the cell edge, multi-TRP operation will be crucial. By allowing UEs in RRC_IDLE/INACTIVE to better interact with multiple TRPs in a cluster, we can extend cell coverage and provide a scalable cell deployed with multiple TRPs for robust initial access transmissions as well as reliable data transmissions afterwards. The TRPs can belong to a same cell or different cells depending on network implementation.

We think that 6GR can allow UEs in RRC_IDLE/INACTIVE to manage measurements and common channels for multiple TRPs, in order to reduce cell change/reselection at a boundary of a single TRP. In addition, considering that aggregating multiple TRPs can lead to reliable data transmission possibly with a higher overall data transmission rate, we can study early setup of MTRP during initial access or right after initial access which can reduce the time it takes to configure multiple TRPs, leading to faster user data transmission/reception with higher reliability and data rate for UE and a short stay of a RRC connection, which is beneficial for efficient management of network energy and UE power by releasing/suspending the RRC connection immediately data transmission/reception.

Pain Points of NR: Need for improved performance for initial access at cell edge e.g. with MTRP

Observation 	For 6GR, MTRP deployment scenario can be extended to UEs in RRC_IDLE/INACTIVE for robust initial access, compared to NR where MTRP is only applicable to UEs in RRC_CONNECTED.

Multi-RAT Spectrum Sharing (MRSS)

Multi-RAT spectrum sharing (MRSS) is expected to play a crucial role in enabling a smooth transition from 5G to 6G. Given the limited availability of new low-frequency bands for 6G and the expected requirement for wide-area coverage, efficient spectrum reuse may help address both coverage and deployment cost challenges. Reusing existing 5G sites and radio nodes for 6G may also offer advantages in terms of infrastructure efficiency.

For MRSS operation, the potential impact of duplicated signal or channel transmissions between NR and 6G—such as signalling overhead, energy consumption, and performance degradation—should also be considered, particularly in context of initial access. For example, a certain common channel can be shared by 6G cell and coexisting 5G cell. Sharing a common channel between 5G and 6G cells can be studied for initial access in 6G MRSS cells. It may be worth studying whether MRSS operation can be supported in a transparent or mode-aware manner at the UE side.

Pain Points of NR: signalling overhead, energy consumption, and performance degradation due to duplicated signal and common channel transmissions between different RATs.

Observation 	For 6GR, considering signalling overhead and energy consumption due to duplicated signal or channel transmissions between NR and 6G cells in MRSS, it is beneficial to study the MRSS scenario where a common channel is shared between 5G and 6G cells for initial access.

Proposal 1	Study 6G mechanisms for support of a multi-carrier single/multi-cell, intra-frequency cell-cluster massive MTRP and MRSS scenarios

Deployable RRC_INACTIVE state

In NR, the RRC_INACTIVE state was introduced as an intermediate mode between RRC_CONNECTED and RRC_IDLE. The objective was to reduce network management overhead for UEs with ongoing sessions while enabling greater UE power savings compared to RRC_CONNECTED. Specifically, when data transmission is temporarily suspended, the network can place the UE into RRC_INACTIVE and quickly resume the connection when new data arrives. In practice, however, commercial deployments have rarely adopted RRC_INACTIVE. While there are multiple pain points to be addressed in RRC_INACTIVE, two issues are outstanding from our point of view: a) long latency of the RRC resume procedure and b) necessity of transmitting paging messages over a wide RAN paging area when downlink data arrives. With these two pain points, operators seem to perceive limited benefits over RRC_IDLE and have little incentive to manage the additional complexity of supporting RRC_INACTIVE.

Pain Points of NR RRC_INACTIVE

- Considerable resume latency from RRC_INACTIVE to RRC_CONNECTED

- Signaling overhead due to paging across a large RAN paging area

For 6G, it is important to revisit the design of RRC_INACTIVE to ensure tangible performance gains. One potential enhancement is to enable faster RRC resume procedure, e.g. via RACH-less resume, so that network can transition UEs to RRC_INACTIVE without being constrained by resume latency. Beyond faster RRC resume, enhancements of network control over UE mobility and paging optimization could also be considered. The primary goal should be to make RRC_INACTIVE a practical and deployable feature in 6G, rather than a theoretically defined but seldom used state.

Proposal 2	Study enhanced RRC_INACTIVE to provide clear operational benefits over RRC_IDLE, with a focus on enabling low-latency RRC resume, optimizing paging and enhancing network control over UE mobility.

UE capability sigalling

Experiences from NR have shown that the size of UE Capability Information message has grown significantly and signalling of such excessive size of UE capabilities put a severe burden to both network and UE. Such large UE capabilities are resulted from a large flexibility of NR features, many per-band capabilities, and a quite large number of band combinations which UE supports for CA or DC.

To reduce the size of UE capability signalling, NR supports a capability filtering mechanism, where network can indicate RAT(s) or band(s) for which UE capability reporting is required. While the filtering mechanism helps reduce the size of reported capabilities, the reported capabilities may be still large due to a large number of band combinations. This motivates to study enhanced UE capability filtering mechanisms which supports more selective and fine-grained filtering for UE capability reporting. For example, instead of reporting all band combinations or large filtered subsets for a concerned set of bands, the UE might be asked to provide UE capabilities for only a subset of the supported band combinations to the network, or to omit band combinations that are known to be irrelevant. Other possibilities could include exclusion-based filtering or introducing ways to minimize duplication across reports.

Pain Points #1 in NR UE capability reporting: The resulting size of UE capabilities after applying capability filtering is still large.

Proposal 3	Study enhanced UE capability filtering mechanisms to reduce the size of UE capability reporting.

Another important lesson learned from 5G is that UE capabilities cannot always be assumed to remain constant. Several factors may affect applicable UE capabilities subject to changes: UE’s power preference, thermal conditions (i.e. overheating), UE capabilities split between networks for multi-SIM operations, or user-selected modes, etc. In NR, UE Assistance Information (UAI) is used to provide information on the change of UE capabilities. Typically, UE is allowed to trigger a new UAI if there is at least one information element that is changed against the previously reported, and prohibit timer is applicable for some cases. That is, network has little control on triggering of UAI transmission. More efficient UE capability update mechanisms are necessary for 6G. The mechanisms should be able to reduce signalling overhead under frequent capability updates, and allow for fine-grained network control on triggering conditions of the updates. Extending index-based capability representation used in NR based on feature sets and feature set combinations could be also considered for the enhanced capability signalling. In NR, the indices for feature sets and feature set combinations are constructed within each report and not maintained across reports. If index-based UE capability structuring in NR is extended to support reusing the reported indices across multiple UE capability reports for 6G, UE capability signalling burden may be significantly reduced particularly when UE capabilities are frequently changed.

The study on enhanced UE capability updates should consider several aspects: how UE capability signaling can be reduced across multiple reports, what mechanisms are required to avoid capability misunderstanding between UE and network, generalized UE capability signaling/update mechanisms applicable for various scenarios including dual registration or multi-SIM, granularity of network control on UE-sided triggering of UE capability updates

Pain Points #2 in NR UE capability reporting: lack of effective mechanisms to efficiently report dynamic UE capability changes

Proposal 4	Study efficient UE capability update mechanism with low signaling overhead, fine-grained network control on triggering of capability updates, and extension of index-based UE capability signalling across multiple capability reports.

Cell DTX/DRX operation

In NR, cell DRX and cell DTX were introduced with the objective of achieving network energy saving by allowing gNBs to temporarily suspend downlink or uplink activity. However, in practice, the NES gain from these features is very limited. Specifically, during the cell DTX off period, the network does not fully switch off downlink transmissions: certain signals such as SSB and HARQ retransmissions are still transmitted. Likewise, during the cell DRX off period, uplink transmissions are not completely prohibited, as UEs are allowed to initiate random access procedures if triggered. As a result, the gNB cannot entirely switch off its transmitter or receiver, preventing the network from entering a deep sleep state.

The primary reason for these exceptions is backward compatibility with legacy UEs. Since cell DRX/DTX were introduced in Release 18, these mandatory UL/DL transmissions during the off periods ensure that pre-Rel-18 UEs continue to operate correctly. However, this design choice significantly undermines the energy saving benefits for the network.

Pain Points of NR Cell DTX/DRX: Limited NES gain due to many DL/UL transmissions allowed during cell DTX/DRX off periods

For 6G, it is important to avoid such legacy-driven limitations by introducing cell DRX/DTX from 6G DAY1. A promising approach is to minimize the allowed downlink and uplink activity during cell DTX/DRX off periods. For example, the network could significantly extend the periodicity of reference signal transmissions during cell DTX off, and restrict uplink transmissions to the minimum necessary during cell DRX off (and, if feasible, conditionally prohibit all UL/DL transmissions). This would allow the network to spend more time in low-power states, maximizing NES without being constrained by legacy compatibility.

Proposal 5	Study Cell DTX/DRX for 6G, focusing on significantly reducing downlink and uplink activity during OFF periods (and potentially achieving complete suspension when feasible), in order to maximize network energy saving while minimizing UE experience degradation.

NTN

In 5G NR, to simplify the specification work, it is assumed that all the UEs have the valid UE location. Based on this assumption, the specification works for NTN support are simplified especially for the physical channels and procedure design to cover the high Doppler shift and large propagation delay in NTN scenario. Moreover, in the NTN scenario, the RSRP measurement results could be similar within a cell area. To mitigate this issue, the location-based handover and cell managements were introduced instead of the RSRP-based procedure. However, in 6GR, it is important to support the UE without the GNSS capability to extend the applicable UE types for the NTN. In this case, the location-based procedures in NR NTN would need to be revisited while the measurement-based procedure may not work properly.

For the quasi-earth fixed cells, the UE needs to perform the satellite switching to acquires the DL/UL timing for the new target satellite. Even during soft satellite switching, the UE may not receive and transmit data for the source satellite during the time that the UE tries to detect SSB for the target satellite. During the satellite switching, the UE could suffer from the service interruption. Meanwhile, in the earth-moving cells, the NTN UE experiences frequent handover due to the satellite mobility. As the satellite moves fast, the serving cell of a UE keeps changing, even though the UE doesn’t move. In 6GR NTN, VLEO can be further considered for the relatively high throughput and low latency. However, due to the lower altitude of the VLEO (e.g., 300km) and the higher satellite speed, it is expected that satellite switching will occur more frequently. Depending on the target scenario (e.g., voice call or emergency alert), the service continuity would be critical, and then the service interruption needs to be resolved or mitigated further.

In NR NTN, due to the large propagation delay and the limited HARQ buffer at the UE side, it is supported that the NW disables HARQ feedback for some HARQ processes. In this case, the data associated with the HARQ process without HARQ feedback may depend on the RLC ACK mechanism for the reliable communication. However, it can cause huge end-to-end latency. Moreover, in 6GR, currently it is unclear whether the RLC ACK mechanism will be introduced or not. In those situation, more reliable communication via NTN would need to be further considered in 6GR. The reliable transmission could save the resources for the retransmissions, and then the overall latency could be further enhanced. The enhancements for reliability and latency cause throughput increasing. These kinds of enhancements would be useful for the successful commercialize the NTN. In 6GR, it would be necessary to study how to improve reliability/throughput for the NTN UE.

Observation	According to newly considered aspects (e.g., GNSS-less, VLEO) in 6GR NTN, some of 5G NTN mechanisms might be no longer feasible or be inefficient. To aim at a harmonized 6GR design for TN and NTN, an introduction of new scenario-related functions would need to be studied together from the early stage of 6GR.

Proposal 6	Study enhanced mechanisms for 6G NTN to support additional aspects beyond those considered in 5G NTN, such as a) UEs without GNSS capability b) services with reduced interruptions (due to handovers/satellite switching) and/or improved reliability/throughput.

Conclusion

Pain points (lessons learned from NR)

Scenarios/deployments

- Lack of spectrum utilization and operations over one or more carriers/bands for RRC_IDLE/INACTIVE

- Need for improved performance for initial access at cell edge e.g. with MTRP

- Considerable resume latency from RRC_INACTIVE to RRC_CONNECTED

- Signaling overhead due to paging across a large RAN paging area

- Signalling overhead, energy consumption, and performance degradation due to duplicated signal and common channel transmissions between different RATs (MRSS)

RRC_INACTIVE state

- Considerable resume latency from RRC_INACTIVE to RRC_CONNECTED

- Signaling overhead due to paging across a large RAN paging area

UE capability

- Large UE capability even after applying capability filtering

- Lack of effective mechanisms to efficiently report dynamic UE capability changes

Cell DTX/DRX (NES)

- Limited NES gain due to many DL/UL transmissions allowed during cell DTX/DRX off periods

NTN

- GNSS-based procedures for 5G NTN is inefficient or may no longer relevant if GNSS-less NTN/VLEO is considered for 6G NTN

- Disabled HARQ feedback due to long RTT results in low reliability and low throughput.

Proposal

Proposal 1	Study 6G mechanisms for support of a multi-carrier single/multi-cell, intra-frequency cell-cluster massive MTRP and MRSS scenarios.

Proposal 2	Study enhanced RRC_INACTIVE to provide clear operational benefits over RRC_IDLE, with a focus on enabling low-latency RRC resume, optimizing paging and enhancing network control over UE mobility.

Proposal 3	Study enhanced UE capability filtering mechanisms to reduce the size of UE capability reporting.

Proposal 4	Study efficient UE capability update mechanism with low signaling overhead, fine-grained network control on triggering of capability updates, and extension of index-based UE capability signalling across multiple capability reports.

Proposal 5	Study Cell DTX/DRX for 6G, focusing on significantly reducing downlink and uplink activity during OFF periods (and potentially achieving complete suspension when feasible), in order to maximize network energy saving while minimizing UE experience degradation.

Proposal 6	Study enhanced mechanisms for 6G NTN to support additional aspects beyond those considered in 5G NTN, such as a) UEs without GNSS capability b) services with reduced interruptions (due to handovers/satellite switching) and/or improved reliability/throughput.