3GPP TSG-RAN WG2 Meeting #131bis	R2-2507155

Prague, Czech Republic, Oct. 13th –17th, 2025

Agenda item:	8.4.2

Source:	Huawei, HiSilicon

Title:	Discussion on open issues 38304-1 and 38304-3 for R19 LP-WUS

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the below open issues in TS 38.304 for R19 LP-WUS [1].

38304-1: Discuss whether “low mobility” criterion based on LR measurements needs to be introduced for LP-WUS. 38304-3: FFS the impact on legacy low-mobility criteria for MR when the UE exits serving cell measurement offloading case.

# Discussion

## (38304-1) Supporting “low mobility” criterion based on LR measurements

For the existing Rel-16 relaxed measurement rules, in addition to the criteria for not being at cell edge (based on the absolute serving cell quality), the criteria for low mobility (based on serving cell quality variation over time) was also defined. However, introducing mobility-based criteria for LP-WUS would significantly increase the complexity of the current decision rules for entering/exiting both RRM measurement relaxation and RRM measurement fully offloading, which are merely based on the serving cell quality.

Furthermore, implementing mobility criteria could potentially impact the availability of the LP-WUS function for high-mobility UEs, such as smartphones or in-vehicle devices. Rather than imposing network-defined mobility restrictions, we recommend allowing UE implementation to determine when to exit LP-WUS monitoring during high mobility state.

Based on these considerations, we propose to leave it to UE implementation to determine whether to exit LP-WUS monitoring when in high mobility state.

Proposal 1: (38304-1) Do not introduce “low mobility” criteria based on LR measurements for LP-WUS.

Proposal 2: It is up to UE implementation to determine whether to exit LP-WUS monitoring when in high mobility state.

## (38304-3) Impact of legacy “low mobility” criteria for MR with LP-WUS

When LP-WUS UE exits fully offloading, it turns on MR and can start evaluation for legacy relaxed measurement criteria if configured. As it’s the MR that measures the serving cell quality after fully offloading, there is no need to introduce any additional conditions to legacy “low mobility” criteria stating UE turns on MR when exiting fully offloading.

Proposal 3: (38304-3) No need to introduce any additional conditions to legacy “low mobility” criteria when LP-WUS UE exits fully offloading.

# Conclusion

In this contribution, we discussed the open issues 38304-1 and 38304-3 and proposed the following:

Proposal 1: (38304-1) Do not introduce “low mobility” criteria based on LR measurements for LP-WUS.

Proposal 2: It is up to UE implementation to determine whether to exit LP-WUS monitoring when in high mobility state.

Proposal 3: (38304-3) No need to introduce any additional conditions to legacy “low mobility” criteria when LP-WUS UE exits fully offloading.

# References

- 1. R2-2506862 List of open issues for LP-WUS 38.304 CR