3GPP TSG-RAN WG2 Meeting #131-bis	R2-2507157

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	10.3.1	User plane

Source:	Nokia, Nokia Shanghai Bell

Title:	6G UP design

WID/SID:	FS_6G_Radio - Release 20

Document for:	Discussion and Decision

# 1	Introduction

In 6G, the overall 5G NR user plane protocol (SDAP/PDCP/RLC/MAC) is expected to be retained, with certain simplifications and enhancements rather than a completely new design of the radio protocol layers. This makes it important to reflect on the lessons learned from 5G.

5G has set an important technical baseline for mobile networks, supporting diverse verticals such as eMBB, URLLC, and mMTC while advancing the performance in terms of latency, power consumption, and throughput to support various use cases such as NTN, eRedCap, and XR, etc. At the same time, the introduction of a vast number of features and configuration options brought significant complexity to both implementation and deployment. This experience highlighted that success is not only about achieving the peak performance, but also depends on avoiding excessive fragmentation, aligning innovation with realistic business use cases, and allowing for sustainable deployments.

With this in mind, we will discuss potential areas for simplification and enhancement in each sublayer.

# 2	Design of 6G User Plane

## 2.1	SDAP

SDAP has been introduced in 5G to ensure that the QoS flow from the CN is properly enforced in the RAN by mapping each QoS flow to a RB.

In SDAP, reflective QoS is specified to allow a UE to infer the uplink QoS rules from the received downlink data, i.e., from the SDAP header, thereby reducing signaling overhead. While reflective QoS can be useful for fast setup of the services like voices and gaming - where the uplink and downlink traffic share similar QoS requirements, it has not been widely used in practice today. This is because explicit QoS rules are more precise and providing static QoS rule is just sufficient while offering more deterministic management where, e.g., rules are removed explicitly rather than by a timer.

In 6G, real-time and low latency traffic is expected to increase, which could make reflective QoS more appealing. Still, since the main benefit is reducing signaling overhead, which makes sense particularly for short-duration traffic, use of reflective QoS remains uncertain. Given these considerations, it would be worthwhile to discuss whether reflective QoS should remain in 6G – determining if the value of initial signaling and latency reduction for short, real-time traffic is critical that it outweighs the operator’s preference for precise and deterministic control of QoS rules.

Proposal 1: For SDAP, RAN2 discusses if reflective QoS remains in 6G.

## 2.2	PDCP

The 6G PDCP is expected to be based on 5G, inhering most of the core functions of PDCP such as header compression, ciphering, integrity protection, reordering, and in-order/out-of-order delivery. The fundamental security framework -PDCP anchors UP security through encryption and integrity protection - is also expected to be maintained in 6G while some enhancements can be considered to address specific vulnerabilities identified from 5G NR. The security aspect will be addressed in our another TDoc on common CP/UP aspect.

One key lesson from 5G is that the coexistence of non-standalone (NSA) and standalone (SA) has created excessive deployment options, leading to market fragmentation and delaying adoption of SA. To avoid repeating this, 6G should aim at ensuring a lean and implementation-friendly system with a standalone-only architecture from Day-1, as emphasized in [RP-251990] and [R2-2507079]. In this sense, complex initial feature like the split bearer for DC may not be an essential Day-1 feature.

Proposal 2: The PDCP in 6G should aim at designing a simplified Standalone-only architecture from Day-1 that de-prioritizes complex initial feature, e.g., split bearer operation for DC.

In 5G NR, each IP packet requires separate security processing and carries its own headers across PDCP, RLC, and MAC, resulting in inefficient data handling and processing burden that scales with increasing data rate. As new services requiring higher data rate emerge, this linear scaling of processing load risks becoming a major bottleneck for the design of an energy-efficient, low-latency, and scalable 6G UP.

PDCP concatenation can address this issue by bundling multiple packets into a single PDCP PDU, so that only one round of processing is needed instead of many. The starting point for discussion on PDCP concatenation can be to identify the scenarios or conditions where it shows clear gains, such as for high-rate traffic where latency requirements are relaxed, for the traffic with short inter-arrival time, or in scenarios where the concatenation can be implemented without complex buffer handling or adverse effects on pre-processing.

Proposal 3: The PDCP in 6G should aim at reducing processing load and header overhead. PDCP concatenation can be one option for this goal.

## 2.3	RLC

In Rel-19 XR, following new mechanisms were introduced for RLC AM operation:

-	Avoiding unnecessary retransmissions by stopping transmission when PDCP indicates discard, and allowing the receiver to discard missing PDUs after a configured duration with positive acknowledgement.

-	Ensuring timely retransmissions by triggering retransmission or polling when the remaining lifetime of an RLC SDU falls below configured thresholds.

We believe the benefits of these mechanisms are not limited to XR, and therefore they should be adopted as baseline features in 6G.

Meanwhile, one of the key functions of RLC is error correction through ARQ, which ensures reliability by retransmitting the lost or corrupted PDUs. However, this may appear redundant, given that HARQ also performs retransmission of the lost or corrupted PDUs in lower layer. If HARQ reliability is sufficiently high, RLC retransmissions may in fact become unnecessary. In addition, the Rel-19 XR RLC mechanisms still rely on timers or thresholds to trigger retransmission, which may leave room for undesirable delay. Since HARQ feedback can provide immediate information about transmission success/failure, leveraging HARQ feedback to earlier RLC retransmissions could be considered as an additional enhancement in 6G, especially for latency-critical traffic from e.g., AI services.

At the same time, it should be noted that relying on HARQ feedback at the RLC layer may have some implication on MAC because MAC would need to track the packets belonging to each TB. In addition, HARQ retransmission and feedback reliability depend on 6G physical layer performance. Therefore, it is difficult for RAN2 to make assumptions on this – whether HARQ retransmission and feedback can ensure sufficiently high reliability, and it would be important to receive early input from RAN1.

Proposal 4: The RLC in 6G should be built on Rel-19 RLC, incorporating Rel-19 XR mechanisms. RAN2 further study if 6G RLC supports avoidance of redundant retransmission under reliable HARQ or faster retransmission using HARQ feedback, where such considerations require coordination with RAN1.

## 2.4	MAC

### 2.4.1	Random Access

In 5G, RA partitioning has been done for several features in different releases, e.g. Coverage Enhancement (CE), RedCap, SDT, Slicing, etc, on top of beams and preamble group A/B, which results in fragmentation of the RA resources as well as increased specification and implementation complexity. For 6G, we should aim to minimize the RA resource fragmentation to only do it for the features that RAN1 concluded as absolutely needed (e.g. CE) that requires distinguishing at random access response step, but not for the ones that is enough to be indicated in MSG3 or later (e.g. SDT, Slicing).

Besides, RA procedure design initially relied only on the 4-step RA procedure, and the 2-step RA procedure was introduced in a later release to reduce the latency of the sequential steps of RAP-RAR-Msg3-Msg4. While 2-step RA procedure enables faster access with possibility to fallback to 4-step, it also introduces complexity due to, for instance, RA configuration, RA type selection, RA resource selection, and multiple response formats.  Additionally, in the context of Rel-19 NTN, CB-MSG3 was introduced without preamble transmission with the assumption of TA compensation at the UE side.

In our view, for 6G, depending on RAN1 decides which types of the request are to be supported, the goal in RAN2 should be to design a unified RA procedure that integrates all supported types, which would allow handling of all cases consistently, thereby simplifying the design.

Proposal 5: For random access, we aim at minimizing the need of RA resource partitioning and having a unified Random Access procedure to support any types of requests that RAN1 decides to support.

### 2.4.2	BSR/DSR and LCP

For scheduling information reporting, BSR has been supported since LTE Rel-8 and NR Rel-15 and DSR with additional delay information was introduced in Rel-18 to enable the UE to report delay-critical status information for certain LCGs which was then enhanced further in Rel-19 to report finer granularity of different remaining time thresholds.

Interaction between DSR and BSR procedure have been proposed in 3GPP since Rel-18, though not specified considering the backward compatibility and specification complexity. Separate BSR and DSR triggering and cancellation conditions and independent reports using different MAC CEs could result in information reported in a DSR and BSR be either partially or totally overlapping depending on the thresholds configuration and the buffer status. Since we are designing everything from scratch for 6G, it makes sense to simplify the procedure with unified buffer status and delay status report that can provide the gNB with configuration flexibility while reducing the signaling overhead and simplifying the specification and implementation of the corresponding features.

How to handle the AI/ML data, sensing data in the scheduling information report and LCP can be decided later after the discussion on those becomes clearer. However, the aim should target a unified procedure for buffer status and delay status reporting covering also the report of these new types of data for 6G. Different periodicity for triggering, report granularity etc. can be considered if seen needed.

Proposal 6: For scheduling information report, we aim at unified procedure for buffer status and delay status reporting, covering also the report of new types of data for 6G.

### 2.4.3	MAC Control Elements

In uplink, MAC CEs carry various types of control information such as buffer status, power headroom, etc., and with each release, new MAC CEs have been introduced to support new features, for instance, Beam, NR-unlicensed, sidelink, etc. This has led to a shortage of available LCID values, and in Rel-16, eLCID was introduced, which added extra one byte overhead in the MAC subheader. This also raised complicated discussion about which MAC CEs should continue using the LCID and which should rely on eLCID. Given that the MAC CEs are typically linked to the feature configured or supported for the UE, many of the LCID or eLCID values reserved for the MAC CEs appear unnecessarily reserved for the UE not supporting or not configured with a certain feature. For instance, LCIDs reserved for the MAC CEs related to sidelink or NR-U are just wasted for the UEs not supporting or not configured with sidelink or NR-U operation.

In our view, instead of reserving a fixed LCID or eLCID for each MAC CE in the specification, the network should be able to configure an (e)LCID for the MAC CEs flexibly considering the features configured or supported for the UE, while still reserving some LCIDs for essential type of MAC CEs such as BSR/DSR, and PHR, etc. This way would allow more relevant features – that matters to the UE – to benefit from the low overhead LCID.

Proposal 7: For MAC CEs, we aim at reducing inefficiency and overhead caused by the static reservation of LCID/eLCID values, given the increasing number of UE-specific features in 6G.

# 3	Conclusion

The followings are proposed in this contribution:

SDAP:

Proposal 1: For SDAP, RAN2 discusses if reflective QoS remains in 6G.

PDCP:

Proposal 2: The PDCP in 6G should aim at designing a simplified Standalone-only architecture from Day-1 that de-prioritizes complex initial feature, e.g., split bearer operation for DC.

Proposal 3: The PDCP in 6G should aim at reducing processing load and header overhead. PDCP concatenation can be one option for this goal.

RLC:

Proposal 4: The RLC in 6G should be built on Rel-19 RLC, incorporating Rel-19 XR mechanisms. RAN2 further study if 6G RLC supports avoidance of redundant retransmission under reliable HARQ or faster retransmission using HARQ feedback, where such considerations require coordination with RAN1.

MAC:

Proposal 5: For random access, we aim at minimizing the need of RA resource partitioning and having a unified Random Access procedure to support any types of requests that RAN1 decides to support.

Proposal 6: For scheduling information report, we aim at unified procedure for buffer status and delay status reporting, covering also the report of new types of data for 6G.

Proposal 7: For MAC CEs, we aim at reducing inefficiency and overhead caused by the static reservation of LCID/eLCID values, given the increasing number of UE-specific features in 6G.