3GPP TSG-RAN WG2 Meeting #131bis	R2-2507171

Prague, Czech Republic, October 13th – 17th, 2025

Source:	vivo

Title:	Discussion on release independent of 6Rx

Agenda Item:	8.20.1

Document for:	Discussion and Decision

- Introduction

There is RAN4 LS in R2-2506735 to RAN2 to ask on release independent of 6Rx.

1. Overall Description: RAN4 is discussing release independence for 6Rx UE requirements with max 6 DL MIMO layers and max 4 DL MIMO layers. Since release independence aspects are related to the max 6 DL MIMO layers feature with new UE capability signalling, RAN4 respectfully requests RAN2 to provide feedback on whether UE capability signalling to allow indication of the support of maximum 6 DL MIMO layers for PDSCH as in LS R4-2505225 will be specified in Rel-19. If yes, can this signalling be implemented early in Rel-18? 2. Actions: To RAN2 ACTION: RAN4 respectfully requests RAN2 to provide feedback on whether UE capability signalling to allow indication of the support of maximum 6 DL MIMO layers for PDSCH as in LS R4-2505225 will be specified in Rel-19. If Yes, can this signalling be implemented early in Rel-18?

- Discussion

Based on RAN4 LS in R4-2505225 RAN2 have agreed two R19 CRs for 6 Rx in [1] and [2]. Indication of the support of maximum 6 DL MIMO layers for PDSCH for FWA as in LS R4-2505225 has been specified in Rel-19.

RAN4 also ask if this signalling can be implemented early in Rel-18?  If one feature can be implemented in early release it should be captured in TS 38.331 Annex C as below.

As highlight in yellow, the basic requirement of early implementable features is no interoperability issue.  Based on the agreed the CR, we know that the introduction of this 6 Rx feature has no interoperability issue. However, as we known, the main release independent feature is spectrum-related features.  the rationale here for release independence of spectrum-related features is that spectrum is a very valuable and expensive resource that needs to be managed as efficiently as possible. For non-spectrum-related features we should discuss carefully case by case in addition to no interoperability issue.  Whether 6Rx can be early implemented can be discussed in RAN4 based on valuable and important aspects.

Proposal: From RAN2 point of view, this signalling of 6Rx implemented early in Rel-18 does not give interoperability issue based the agreed CRs in R2-2506466 and R2-2506467, however whether 6Rx supporting shall be early implemented can be discussed in RAN4 based on valuable and important aspects.

- Conclusion

In this contribution, we discussed the LS from RAN4, and give our proposal as below.

Proposal: From RAN2 point of view, this signalling of 6Rx implemented early in Rel-18 does not give interoperability issue based the agreed CRs in R2-2506466 and R2-2506467, however whether 6Rx supporting shall be early implemented can be discussed in RAN4 based on valuable and important aspects.

- Reference

- R2-2506466 Introduction of UE capability on 6 DL MIMO layers

- R2-2506467 Introduction of UE capability on 6 DL MIMO layers.