3GPP TSG-RAN WG2 Meeting #131bis	R2-2507177
Prague, Czech Republic, October 13 – 17th, 2025

Agenda Item:	8.5.3

Source:	InterDigital

Title:	[MAC Issue 1] RA-RNTI for PRACH adaptation

Document for:	Discussion, Decision

# Introduction

The Rel-19 WID on Network Energy Savings for NR has the following objective on adaptation of common signalling and channels, including adaptation of PRACH in time domain.

In RAN2#130, the following agreements were made for RACH domain adaptation

No need to extend the RA-RNTI space for additional PRACH occasions, i.e. no RAN2 impact for RA-RNTI calculation. RAN2 assumes that f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs. RAN2 understand there is no MAC

And accordingly, an LS was sent to RAN1 [2]. A reply LS was received from R1 mentioning that there is no proper place in RAN1 specifications to capture the impact on f_id, and the index nRA in RAN1 specification is an index for physical signal generation and cannot be directly used for determination of f_id.

This contribution discusses this remaining open issue and how to address it in specifications.

# Discussion

## Adaptation of PRACH in time domain

The RA-RNTI collision issue was first discussed in RAN2#130, including the following options:

- introducing a separate offset related to the RACH configuration index when calculating the RA-RNTI.

- No need to extend the RA-RNTI space for additional PRACH occasions, assuming that f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs.

- introduce a configurable offset for the symbol index (s_id) in the RNTI equation.

The second option was agreed on the pretext that it will rely on nRA parameter in RAN1 specs is modified to be interpreted as the RO index in addition to the number of legacy FDMed ROs. However, RAN1’s reply LS indicates that this option is not feasible to be captured properly. Therefore, it can be captured in the MAC spec. A text proposal is provided in Annex A.

Proposal 1:	Adopt the TP in Annex A for TS 38.321, to specify that if the UE selects an additional RO (configured by addlRACH-Config-Adapt) and the RO overlaps in the frequency domain with legacy ROs, the UE interprets f_id as the index of the PRACH occasion in the frequency domain plus the number of legacy FDMed ROs.

# Conclusion

This contribution discusses this remaining open issue on RA-RNTI for PRACH adaptation in MAC. The following proposal is provided:

Proposal 1:	Adopt the TP in Annex A for TS 38.321, to specify that if the UE selects an additional RO (configured by addlRACH-Config-Adapt) and the RO overlaps in the frequency domain with legacy ROs, the UE interprets f_id as the index of the PRACH occasion in the frequency domain plus the number of legacy FDMed ROs.

# References

RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson

R2-2504724, “LS on RA-RNTI for PRACH adaptation”, RAN2

R1-2506587, “Reply LS on RA-RNTI for PRACH adaptation”, RAN1

# Appendix A: RA-RNTI for PRACH adaptation in MAC

### 5.1.3	Random Access Preamble transmission

The MAC entity shall, for each Random Access Preamble:

1>	if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and

1>	if the notification of suspending power ramping counter has not been received from lower layers; and

1>	if LBT failure indication was not received from lower layers for the last Random Access Preamble transmission; and

1>	if SSB or CSI-RS selected is not changed from the selection in the last Random Access Preamble transmission; and

1>	if the Random Access procedure is not initiated by the PDCCH order for an LTM candidate cell:

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell as preamble re-transmission; and

1>	if the PDCCH order indicates the same LTM candidate cell and the same SSB as the last Random Access Preamble transmission:

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	select the value of DELTA_PREAMBLE according to clause 7.3;

1>	if the selected PRACH occasion is of the second PRACH occasions (as defined in TS 38.213 [6]) and sbfd-RACH-SingleConfig-preambleReceivedTargetPower is configured for the Random Access Procedure:

2>	set PREAMBLE_RECEIVED_TARGET_POWER to sbfd-RACH-SingleConfig-preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE.

1>	else if the selected PRACH occasion is of the second PRACH occasions (as defined in TS 38.213 [6]) and sbfd-RACH-DualConfig is configured for the Random Access Procedure:

2>	set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower (included in the sbfd-RACH-DualConfig) + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE.

1>	else:

2>	set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE.

1>	except for contention-free Random Access Preamble for beam failure recovery request and contention-free Random Access Preamble triggered by a PDCCH order for an LTM candidate cell, compute the RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted;

1>	instruct the physical layer to transmit the Random Access Preamble using the selected PRACH occasion, corresponding RA-RNTI (if available), PREAMBLE_INDEX, and PREAMBLE_RECEIVED_TARGET_POWER.

1>	if the Random Access Procedure is triggered by a PDCCH order for an LTM candidate cell:

2>	consider this Random Access procedure completed.

1>	if LBT failure indication is received from lower layers for this Random Access Preamble transmission:

2>	if lbt-FailureRecoveryConfig is configured:

3>	perform the Random Access Resource selection procedure (see clause 5.1.2).

2>	else:

3>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

3>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

4>	if the Random Access Preamble is transmitted on the SpCell:

5>	indicate a Random Access problem to upper layers;

5>	if this Random Access procedure was triggered for SI request:

6>	consider the Random Access procedure unsuccessfully completed.

4>	else if the Random Access Preamble is transmitted on an SCell:

5>	consider the Random Access procedure unsuccessfully completed.

3>	if the Random Access procedure is not completed:

4>	perform the Random Access Resource selection procedure (see clause 5.1.2).

The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier). If the PRACH occasion is configured by addlRACH-Config-Adapt, f_id is the index of the PRACH occasion in the frequency domain plus the number of other PRACH occasions not configured by addlRACH-Config-Adapt that overlap with it in the frequency domain.