3GPP TSG RAN WG2 Meeting #131bis                                                       R2-2507184

# Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:	10.2

Source:	Ofinno

Title:	Scenarios and architectural principles for 6G RAN2 design

Document for:	Discussion and decision

# Introduction

This document discusses the scenarios and architectural principles for RAN2 work considering the agreed 6G SID [1] and the endorsed TDoc [2] summarizing the overall principle for 6G:

The contributing companies propose to endorse an overall principle in 3GPP to create lean and streamlined standards for 6G, e.g., by dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, avoiding excessive configurations, etc. Any exception to the above shall be well justified.

# Discussion

## Architectural Principles and Lessons Learned from 5G

The design of the 6G RAN architecture will be guided by lessons learned from 5G to avoid repeating past challenges. Key principles include:

- Simplicity and Leanness: Avoid excessive flexibility, configurability, and feature options that led to complexity in 5G. This includes streamlining protocols and reducing duplicated functionalities across layers.

- Scalable and Unified Design for Diverse Devices: 6G must natively support a wide range of device types from Day 1, from low-power devices (e.g., LPWA/IoT) to high-performance devices (e.g., eMBB and XR), within a single, scalable RAT. This avoids the fragmented, add-on approach of 5G that created inefficiencies.

- Native Support for Key Features: Critical functionalities like energy saving, NTN integration, AI/ML, and security must be designed into the system from the start, not as later additions. This is crucial for ensuring widespread adoption and avoiding backward compatibility constraints that hindered 5G enhancements.

- Cloud-Native and Open Interfaces: The architecture should be cloud-friendly and support open interfaces, continuing the disaggregation trend from 5G.

RAN2 design of 6G standards should be scalable, lean, and streamlined. It must natively support diverse device types and key features from Day 1, while reducing unnecessary optionality and configurability to avoid the complexities of 5G.

## Deployment Scenarios and Migration Strategy

The agreed 6G SID [1] captures the guidance on the scenarios to start focusing RAN work (as highlighted in yellow on the relevant SI objectives copied below).

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1, RAN2, RAN3, RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel BW (at least 200MHz) support for 6G deployments at least > 2 GHz, ~ 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

…

(7) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4].  RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s).

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Figure below provides a high-level overview of 6G standalone and coexistence/interworking with 5G considering also a 6G/5G MRSS (Multi-Radio Spectrum Sharing) scenario.

The initial 6G study work will focus on a single, standalone (SA) architecture. This decision is a direct lesson from 5G, where numerous non-standalone (NSA) options led to unforeseen complexity, delayed SA deployments, and sometimes a suboptimal user experience. The 6G core may interface with an evolved 5G Core (e5GC), reusing its Service-Based Architecture and key network functions where possible to ensure a smooth migration path.

For seamless migration and spectrum utilization, key enablers identified are:

- 5G-6G Multi-RAT Spectrum Sharing (MRSS): This is considered an essential component from Day 1 to enable the deployment of 6G in existing 5G bands, facilitating a gradual refarming of spectrum. The goal is to make MRSS significantly more efficient than the 4G-5G Dynamic Spectrum Sharing (DSS).

- Intra-6G Carrier Aggregation (CA): CA will be crucial for aggregating new and existing spectrum bands to achieve high performance and competitive user experiences from the start.

- Inter-RAT Mobility: Robust mobility between 5G NR and 6GR is critical for service continuity.

While the study will initially prioritize the SA scenario, RAN Plenary will guide any future work on additional migration options, such as dual-stack or dual-connectivity, after March 2026.

RAN2 study of 6G design should prioritize the standalone deployment scenario. Interworking will be based on mobility with 5G NR, supported by MRSS and intra-6G CA. Further guidance from RAN Plenary is awaited for other migration options.

## Key Areas of Study for RAN2

For RAN2 study work of 6G, key areas are highlighted in the SID [1] as shown above and below references.

(2) Physical Layer structure for 6GR,

  - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2]

  - Initial access [RAN1, RAN2, RAN4]

    - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4]

    - Studies on initial access proc., random access proc., system information and paging [RAN2, RAN1, RAN4]

  - 6GR spectrum utilization and aggregation.  [RAN1, RAN2, RAN4]

  - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4]

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design

- Control Plane architecture (including RRC states) and protocol design

- Access stratum security aspects, in alignment with requirements from SA3

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc)

(4) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

The following points summarizes key areas of study that RAN2 could initially focus the 6G design discussion considering 6G SID [1] and the architectural principles outlined:

- 6G CP/UP protocol simplification

This study may investigate a modular protocol design to efficiently support a diverse range of devices, from low-cost/IoT to high-end UEs, using an anchor baseline protocol with optional additional modules. The goal is to create a lean and implementation-friendly protocol stack by eliminating duplicated functions across layers, such as merging RLC functions into MAC/PDCP. This approach aims to reduce signalling overhead and latency by simplifying RRC configurations and reusing stored configurations across state transitions. The simplified, modular structure will enable native support for parallel processing and hardware-friendly implementation, ultimately improving energy efficiency.

- Study 6G CP/UP protocol simplifications to reduce latency and signalling overhead.

- 6G end-to-end energy saving

This study may focus on creating a joint, harmonized framework for network (NES) and device (UEPS) energy savings that is natively designed into 6G from day one. Key techniques include coordinating network and UE sleep/wake cycles through enhanced DRX/DTX and minimizing "always-on" signals by making transmissions like SSB and SIBs on-demand, triggered by a low-power Wake-Up Signal (WUS). The framework will also explore adaptive transmissions in the time, frequency, and spatial domains based on traffic load to maximize "Less ON, More OFF" periods. This holistic approach is critical for sustainability and for improving the user experience on battery-constrained devices.

- Study 6G end-to-end energy saving techniques to reduce power consumption in UE and RAN.

- 6G Diverse Access Control

This study aims to design a single, scalable Radio Access Technology (RAT) that natively supports all device types from day one, including LPWA/RedCap, eMBB, and NTN, thereby avoiding the fragmented, add-on approach of 5G. The framework will focus on maximizing commonality between Terrestrial (TN) and Non-Terrestrial Networks (NTN) to simplify implementation and enable seamless mobility. It will feature a unified initial access procedure, including cell search and RACH, which is efficient for a wide range of device capabilities. This approach will reduce complexity and market fragmentation while providing ubiquitous connectivity.

- Study how to provide a common/unified 6G access framework.

- 6G QoS/QoE framework

This study will develop a new QoS framework to meet the stringent demands of emerging services like immersive XR and multi-modal applications. A possible approach is to introduce an adaptive resource type, allowing the RAN to autonomously adjust QoS within soft boundaries (e.g., target and max values) without heavy signalling for faster adaptation and improved Quality of Experience (QoE). This enhanced framework may support finer granularity, potentially down to the SDF-level, to enable more precise and efficient radio resource scheduling for individual application data flows.

- Study a 6G QoS/QoE framework to provide more adaptive and fine-granular quality of service/experience than in 5G.

- 6G Data framework.

This study will investigate a holistic, data-centric framework to efficiently manage the vast amounts of non-user-plane data generated by AI/ML, sensing (ISAC), and network operations. This "Data Plane" or "Data Bus" will provide a unified mechanism for the collection, transfer, processing, and exposure of this system data, avoiding the inefficient and siloed approaches of 5G. The framework will ensure operators have visibility and control over data collection while enabling new monetization opportunities like Data-as-a-Service (DaaS).

- Study a 6G data framework for efficient data-centric operations.

- 6G Robust Security

This study will establish a "security by design" framework that extends protection to lower protocol layers or procedures, which were left vulnerable in 5G. Key goals include developing lightweight confidentiality and integrity protection for MAC CEs and digital signatures for broadcast system information to defend against threats like false base stations. The 6G architecture must also be quantum-resistant from day one, incorporating for example support for post-quantum cryptography and 256-bit keys to address future computational threats.

- Study 6G light and robust security for all protocol layers (e.g., L2/L1) and procedures (e.g., broadcast).

# Conclusion

The observations captured are the following:

Observation 1.	RAN2 design of 6G standards should be scalable, lean, and streamlined. It must natively support diverse device types and key features from Day 1, while reducing unnecessary optionality and configurability to avoid the complexities of 5G.

Observation 2.	RAN2 study of 6G design should prioritize the standalone deployment scenario. Interworking will be based on mobility with 5G NR, supported by MRSS and intra-6G CA. Further guidance from RAN Plenary is awaited for other migration options.

The proposals captured are the following:

Proposal 1.	Study 6G CP/UP protocol simplifications to reduce latency and signalling overhead.

Proposal 2.	Study 6G end-to-end energy saving techniques to reduce power consumption in UE and RAN.

Proposal 3.	Study how to provide a common/unified 6G access framework.

Proposal 4.	Study a 6G QoS/QoE framework to provide more adaptive and fine-granular quality of service/experience than in 5G.

Proposal 5.	Study a 6G data framework for efficient data-centric operations.

Proposal 6.	Study 6G light and robust security for all protocol layers (e.g., L2/L1) and procedures (e.g., broadcast).

# Reference

- 3GPP RP-252912, “Study on 6G Radio, SID,” September 2025.

- 3GPP RP-250766, “Lean and Streamlined 6G Standards,” March 2025.