3GPP TSG RAN WG2 Meeting #131bis                                                       R2- 2507189

# Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:	10.4

Source:	Ofinno

Title:	Key considerations for mobility in 6G

Document for:	Discussion and decision

# Introduction

This document discusses the key considerations for mobility is 6G system based on the agreed 6G SID [1].

Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

# Discussion

The foundational approach for 6G mobility function is driven by lessons learned from 5G, particularly the need to move beyond multi modal mobility.

Mobility mechanisms have been ever-present in cellular technologies and have evolved across generations. However, the baseline handover mechanism presented significant challenges for 5G's advanced use cases. The main KPIs against which a mobility mechanism is evaluated is based on its interruption time and robustness during the mobility. Interruption time during mobility refers to the time duration for which the UE does not receive a packet or transmit a packet during mobility. It can be understood as the shortest time duration during which a UE is not able to exchange user plane packets with any base station during a handover. Robustness refers to the ability of the mobility mechanism to maintain a stable and reliable connection between UE and network, i.e., to avoid failures during the mobility e.g., handover failures (HOFs) and radio link failures (RLFs).

## CONNECTED mobility

Release 15 supported the baseline handover that is a network-controlled break-before-make handover where the source gNB decides and commands the UE to perform a handover; the network coordinates context transfer over Xn (or NG) and the UE executes the RRC handover procedure (HO Command → random access in target → HO Complete). The network makes the handover decision based on L3 measurements from UE.

Release 16 introduced 2 new mechanisms – DAPS for reducing interruption time, and CHO for improving mobility robustness. In DAPS, the UE maintains active stacks on both source and target during HO attempt so user-plane/data transmission can continue on the source while the target access is being established. However, this resulted in a complex specification. In CHO, the network pre-configures one or more handover commands/ target cell configuration for handover in advance before the actual handover. Based on satisfying an execution condition configured by the network, the UE executes CHO to the target cell that satisfied the execution condition.

While CHO improves the robustness, it suffers the same interruption time as a legacy handover. Similarly, while DAPS improves the interruption time, it suffers from the same robustness issue as a legacy handover. This resulted in developing a new mechanism, LTM, which allows for a reasonable improvement in interruption time while providing a high mobility robustness. It borrows the principle of pre-configuration of one or more handover commands/ target cell configuration for cell switch in advance before the actual cell switch. However, in LTM, the cell switch to the target cell is network-controlled and indicated using a MAC CE. LTM underwent a phased evolution in release 18 and release 19. In a first phase, the fundamentals of LTM were developed and the scope was limited to intra-CU mobility. In the next phase, LTM was expanded to support inter-CU mobility as well, and finally culminating in a conditional LTM where the UE triggers the cell switch based on satisfying an LTM execution condition received from the network. Figure 1 below illustrates the evolution of mobility in 5G from release15 through release 19.

Figure 1: mobility evolution in 5G

Table below illustrates many key properties are similar across multiple mobility mechanisms. For example, Although LTM supports many advance techniques like early UL synchronization to a candidate cell, L1 measurement reporting etc., there is very little difference in key performance aspects and concept of CHO and LTM i.e., they share a lot of similarities. Each mobility mechanism has its own signaling, configurations, decision logic, and reporting. UEs and networks must support multiple parallel mobility mechanisms —leading to implementation complexity, high memory requirements, increased signaling overhead/ control plane load and suboptimal resource utilization.

| Mechanism | Interruption Time | Robustness | Complexity | UE Requirements |
| --- | --- | --- | --- | --- |
| Legacy HO | High | Low | Low | Low |
| DAPS | Very Low (0 ms) | Moderate | High | Dual Stack |
| CHO | High | High | Moderate | Conditional Logic |
| LTM | Very Low | Moderate to high | Moderate | L1/L2 Aware |
| CLTM | Very Low | High | Moderate | L1/L2 Aware, conditional logic |

Table 1: key properties of mobility mechanisms

5G utilizes different, complex mobility solutions (DAPS, LTM, CHO) across various scenarios, radio access technologies (RATs), and deployment types (e.g., mmWave vs. Sub-6 GHz), leading to a highly complex operational framework. Furthermore, 5G also struggles to seamlessly manage mobility across TN and NTN coverage. However, a certain mobility mechanism may serve a purpose better than other mobility mechanisms. For example, CHO may be useful in lower frequencies for improved robustness whereas LTM may be useful in cases where the handover decision and execution need to be quickly completed. In 5G, both the configurations are prepared, signalled and stored separately.

Single mobility mechanism may not be optimal to manage seamless mobility in all scenarios. Existing mechanisms for handover preparation, signaling and configuration storage, may result in increased memory requirements, signaling overhead and suboptimal resource utilization.

3GPP studied the application of AI/ML for mobility mechanisms. In the near future, mobility management will likely become predictive, using AI/ML to anticipate UE movement and pre-configure network resources seamlessly, minimizing signaling even further. The evolution from the common baseline handover (for all use cases and devices) to a suite of specialized mechanisms like DAPS, CHO, and LTM is 5G's answer to diverse and stringent mobility demands. With increased device types, traffic profiled and mobility patterns, a fusion of multiple mobility mechanisms may become crucial in providing higher reliability in some scenarios e.g., a CHO with early UL synchronization. A unified mobility mechanism in 6G will not be a single protocol but an intelligent, adaptive and coherent framework that can seamlessly orchestrate these techniques to provide the best mobility experience and performance.

- RAN2 to study and adopt an adaptive framework to support a unified mobility mechanism that allows for fusion of need-based different techniques.

The evolution of mobility mechanisms has seen the introduction of many features that optimize of facility one or the other aspects involved in mobility. Figure 1 also highlights the main features introduced across different mobility mechanisms. Many of these features are exclusive to that mobility mechanism. With kind of use cases that may be integrated at the beginning of 6G, it may be beneficial to include some of these features into the unified mobility framework. LTM (Release 18 and 19 cumulatively) appears to be the most feature advance and can be taken as a baseline to develop the unified mobility framework.

- The key features of conditional LTM is baseline for developing unified mobility framework.

## Unified measurement framework

5G NR uses several kinds of measurement signals: CSI-RS, SSB – each serves different purposes (channel estimation, beam management etc.).

Measurements for Radio Resource Management (RRM) and Channel State Information (CSI) are configured and reported separately, using lengthy and complex RRC signaling with overlapping events, thresholds, and filtering. CSI and RRM measurement reports occur frequently, especially in massive MIMO and beam management scenarios. CSI is reported via L1 signals while RRM measurements are reported via L2 or L3 signaling for mobility management. In 5G, the each of the measurement and reporting mechanisms are configured separately leading to a large size of configuration and a staggered/ stacked configuration structure.

UEs have to monitor and report several reference signals in parallel, resulting in higher control-plane overhead and increased UE power consumption. This creates high configuration complexity, increased risk of misconfiguration, and vendor-specific inconsistencies in coverage and performance evaluation. Since these reporting is not coordinated, the network may make decisions based on incomplete information because CSI and RRM measurements are not jointly optimizes. The evolution of mobility mechanisms and introduction of L1 measurement report via L2 signaling i.e., MAC CE only added to the complexity of the already complex configuration and reporting of these measurements.

With the increase in the number of antenna elements the overhead for measurement increases even more. The current CSI framework in 5G, with different reporting types (periodic, semi-persistent, aperiodic) is not designed to efficiently handle extremely large antenna arrays and the associated beam management. On the other hand, RRM measurements require the UE to measure multiple cells and beams, which leads to significant power consumption and processing overhead. The problem will aggravate in 6G with dense networks and integrated NTN.

CSI and RRM measurement reporting are configured separately for L3, L2 and L2 reporting and the reporting itself may occur frequently, especially in massive MIMO and beam management scenarios. UEs have to monitor and report several reference signals in parallel, resulting in higher control-plane overhead and increased UE power consumption.

In addition to existing use cases, 6G may integrate Integrated Sensing and Communication (ISAC). In 6G, a unified measurement framework would provide a holistic view of the radio environment by integrating existing and possibly new types of measurements (e.g., ISAC based measurements) into a single, coherent framework as shown in figure 2. AI based measurement predictions that are studies in release 19 of 5G can also be considered as a reference to enhance the application of AI for 6G measurement framework. This would help configure context-aware measurement configuration to the UE. This would enable simultaneous optimization of link adaptation, beam management, and mobility management. This would also reduce the signaling overhead by avoiding duplicate configurations, measurements and reporting.

Figure 2: example of 5G distributed/ stacked measurement framework against 6G unified measurement framework

- RAN2 to introduce a unified measurement framework and study how to minimize/ optimize the measurement configuration, measurement evaluation and measurement reporting.

## IDLE/ INACTIVE mobility

Cell reselection in 5G NR (New Radio) enables a UE in RRC Idle or Inactive state to autonomously select and camp on the most suitable cell, prioritizing service continuity and efficient resource usage. The reselection process relies on the related information broadcast in SIBs, or received from dedicated signaling via RRC release. The reselection process is primarily based on radio signal strength and basic quality metrics (e.g., RSRP, RSRQ), not on end-to-end user experience or full network performance. This can sometimes result in selection of cells that are suboptimal from a throughput or latency perspective, especially in dense deployments or multi-layer networks. The decision-making is unaware of cell load, service type, or actual user requirements—it does not factor visibility into current traffic, congestion, or load-balancing needs. Thus, even where a cell is available with better real-time performance, it might not be selected unless its signal metrics are superior.

5G NR introduces advanced concepts like beam-level measurements and adjustments for mobility in RRC_CONNECTED, but cell reselection typically triggers on cell-level status rather than optimal beam selection, which could miss finer-grained mobility optimization. After reselection, the UE has to re-read basic system information (SIB1), which can delay the resumption of higher-level services. The reselection parameters are broadcast and is the same for all UEs. There is no device type based or context-aware dynamic updates to the reselection procedure.

6G should target a shift from simple, autonomous UE procedure to an intelligent, network-cooperative process for cell reselection in IDLE and INACTIVE states. This may enable optimal load distribution and resource utilization, ensuring that the UE selects the best cell for its operation, and not the cell with the best downlink coverage/ signal strength. AI/ML based prediction may come in handy as well.

- RAN2 to study context-aware and AI/ML based intelligent cell reselection procedure to select cell that is best for the device/ service type.

## Inter-RAT mobility

Inter-RAT (Inter-Radio Access Technology) Mobility in 5G (NR) refers to the procedures that allow a User Equipment (UE) to seamlessly move between the 5G New Radio (NR) network and a legacy network, primarily 4G LTE (E-UTRA). The inter-RAT mobility procedures in 5G are largely based on the Handover (HO) and Cell Reselection mechanisms inherited and adapted from LTE. 5G also supports a service-driven inter-RAT mobility to LTE for support of VoLTE (i.e., VoNR to VoLTE). These scenarios are illustrated in figure 3.

Figure 3: Inter-RAT mobility from 5G to 4G

Although designed for seamless mobility between 4G and 5G, there are several technical challenges and limitations in the RRC-based inter-RAT handover:

- a Break-Before-Make process, involving complex cross-RAT signaling, security procedures, full RRC reconfiguration on the target side, QoS remapping to the target RAT (e.g., QoS flow to EPS bearer conversion), lack of L1/L2 controlled mechanisms etc. to name a few. This leads to high interruption times and are not suitable for services with low latency requirements. RRC-based inter-RAT HO requires the N26 interface to be deployed between the 5GC AMF and the EPC MME.

- Lack of N26 interface support forces the use of less efficient procedures like RRC Connection Release/Redirection, which significantly increases the interruption time and potential for service drops.

- Inter-RAT reestablishment is not supported and when a UE detects failure at coverage edge, the connection is dropped, and a new connection is established on the target RAT.

Inter-RAT cell reselection based on priority and using periodic detection measurement and evaluation can be used as a baseline in 6G. 5G supports inter-RAT mobility in RRC_IDLE and RRC_CONNECTED state. Inter-RAT mobility in RRC_INACTIVE is not support and an RNA area is limited to cells of the same RAT. We think it is beneficial to support inter-RAT mobility in RRC_INACTIVE state. To truly blend inter-RAT mobility as a seamless procedure between 5Gand 6G, it would be beneficial to overcome or minimize these limitations.

- Inter-RAT mobility between 5G and 6G is supported in IDLE, INACTIVE and CONNECTED state. RAN2 to study how to:

- reduce the interruption during inter-RAT handover,

- support re-establishment and RAN notification are across RATs.

# Conclusion

The contribution discusses aspects related to mobility in 6G. The following observations and proposals are made:

Observations:

- Single mobility mechanism may not be optimal to manage seamless mobility in all scenarios. Existing mechanisms for handover preparation, signaling and configuration storage, may result in increased memory requirements, signaling overhead and suboptimal resource utilization.

- CSI and RRM measurement reporting are configured separately for L3, L2 and L2 reporting and the reporting itself may occur frequently, especially in massive MIMO and beam management scenarios. UEs have to monitor and report several reference signals in parallel, resulting in higher control-plane overhead and increased UE power consumption.

- Proposals:

- RAN2 to study and adopt an adaptive framework to support a unified mobility mechanism that allows for fusion of need-based different techniques.

- The key features of conditional LTM is baseline for developing unified mobility framework.

- RAN2 to introduce a unified measurement framework and study how to minimize/ optimize the measurement configuration, measurement evaluation and measurement reporting.

- RAN2 to study context-aware and AI/ML based intelligent cell reselection procedure to select cell that is best for the device/ service type.

- Inter-RAT mobility between 5G and 6G is supported in IDLE, INACTIVE and CONNECTED state. RAN2 to study how to:

- reduce the interruption during inter-RAT handover,

- support re-establishment and RAN notification are across RATs.

# Reference

- RP-252912, Study on 6G Radio, SID, September 2025.