3GPP TSG-RAN WG2 Meeting #131bis	R2-2507207

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item:		8.2.2

Source:		Futurewei

Title:		Discussions on remaining issues for Rel-19 AIoT

Document for:		Discussion and Decision

# Introduction

During RAN meeting#109 meeting,

In RAN2 meeting#131, how to handle the delay NAS response was discussed and the following agreements were made, wherein the issue that needs to be further checked is highlighted below [1]:

Agreements on no data available due to delay in NAS

- The reader determines no data available case by SDU length 0.   As more data indication is mandatory, the device sets this bit to "0".

- The reader, in response to 0 SDU in the device’s MAC response, may send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device.

- The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command.

- RAN2 would like to check if there is a case where NAS doesn’t provide a response at all.   If this case exists, RAN2 will discuss this issue.

In this paper, we first list additional cases other than delayed NAS response where no NAS response is to be sent. Then, we analyze all possible scenarios of failure in receiving the NAS response immediately. We discuss the need for differentiating all these scenarios to enable the reader and/or the AIOTF to react to the failure with proper mitigation. We compare the solution of using the timely MAC response with 0 SDU and MDI=1 with other solutions that have been suggested for addressing the delayed NAS response problem, in the presence of other possible reasons for failure. We conclude that the solution of using the timely MAC response with 0 SDU and MDI=1 remains the best solution, because it works most effectively in enabling the reader/AIOTF to differentiate all possible scenarios of failure so as to timely apply the correct remedy, and the solution presents a simple and unified design with the segmentation solution. Hence, we propose that RAN2 adopt it. Finally, we propose TP, in the Annex, for revising 38.391 accordingly.

# Discussions

## On additional cases other than delayed NAS response where no NAS response is to be sent

### Cases due to NAS protocol error handling

Following cases of no NAS response due to protocol error handling are described in the latest A-IoT NAS protocol specification (TS 24.369) [2]:

# 6 Handling of unknown, unforeseen, and erroneous protocol data

…

### 6.2.1	Message too short

When a message is received that is too short to contain a complete message type information element, that message shall be ignored, c.f. 3GPP TS 24.007 [3].

…

## 6.3	Unknown or unforeseen message type

If the AIoT device or the network receives a AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall ignore the AIoT NAS message.

…

## 6.4	Non-semantical mandatory information element errors

When on receipt of a message,

a)	an "imperative message part" error; or

b)	a "missing mandatory IE" error;

is diagnosed or when a message containing a syntactically incorrect mandatory IE is received, the AIoT device shall ignore the AIoT NAS message and the network shall:

…

## 6.7	Messages with semantically incorrect contents

When a message with semantically incorrect contents is received, the AIoT device shall perform the foreseen reactions of the procedural part of clause 5. If, however no such reactions are specified, the UE shall ignore the message.

### Cases due to integrity verification failure on command request at the NAS layer

In addition, when a MAC SDU is received in R2D Upper Layer Data Transfer message correctly (i.e., CRC checked) but the integrity verification has failed at the A-IoT NAS layer, it is expected that the device will discard the NAS message (and treating it as if not received), based on RAN2’s own experience with defining NR UE’s behaviors when integrity verification fails on a PDCP PDU at the receiving PDCP entity.

It has been suggested, during the previous meeting, that in case of integrity failure, the device should send a MAC response containing a NAS response indicating the integrity failure. However, when integrity verification fails, very often it is due to the security keys used by the device and by the network have become out of synch. In this situation, the NAS response sent by the device will likely fail in the integrity verification on the AIOTF side and be discarded.

Therefore, it is still reasonable for RAN2 to assume no command response from the device when the command request has failed in the integrity verification at the A-IoT NAS layer of the device.

Observation 1: Additional cases of no NAS response (other than due to NAS delay) do exist, due to NAS protocol error handling and due to failure of integrity verification on the command request at the NAS layer.

Unlike the delayed NAS response case where the expected NAS response is still pending, all the above cases of no NAS response are conclusive.

Observation 2: Unlike the delayed NAS response case where the expected NAS response is still pending, all the above cases of no NAS response are conclusive.

## Analysis of all possible scenarios of failure in receiving NAS response timely

The reader’s failure in receiving the command response timely from the device may be due to one of the following reasons, with detailed analysis of the symptoms and remedies (and with the author’s apologies for the lengthy discussion and the blog style in this section):

- Energy-related reason:

  - Symptoms: no MAC response is received.

  - Remedy: If the communication failure is due to an energy-related failure (i.e., the device ran out of energy), the reader can wait for sufficient time for the device to be recharged and sends the command request again. Note that the reader can fix this type of failure all by itself unless there is a stringent delay requirement.

- Coverage failure:

  - Symptoms: no MAC response is received, despite the reader’s effort in retransmitting the same command request

  - Remedy: the reader can initially try to fix the issue as a case of energy-related failure. However, after the reader had waited for sufficient time for the device to be recharged and sent the command request again, repeatedly if necessary, if the reader still receives no MAC response, this may not be an energy-related failure but instead could be a coverage-related failure (e.g., the device has moved out of the coverage area of the reader).

    - In this case, the reader should send, to the AIOTF, the command failure message with a cause indicating that no lower layer response is received from the device at all.

    - Then, in response to the command failure message and the cause, the AIOTF may select other reader(s), possible near-by reader(s), to send the command request again.

- NAS protocol error:

  - Symptoms: 1) a MAC response with 0 SDU and MDI=0 is immediately received; or 2) no MAC response is received, as suggested in [3] and [4].

    - 2) will not be differentiable from the cases of energy-related failure or coverage failure, preventing the reader/AIOTF to timely apply the correct remedy.

    - Therefore, 2) should not be pursued. The device should send a MAC response with 0 SDU and MDI=0 in the case of NAS protocol error. Note that the device sending a MAC response is important for confirming that the communication still works and hence filtering out the unlikely reasons for failure to enable the reader/AIOTF to timely apply the correct remedy.

  - Remedy: if 1) is adopted, when the reader receives the timely MAC response with 0 SDU and MDI=0, the reader should send a command failure message to the AIOTF with a cause indicating that the MAC response is received without NAS response, suggesting that the failure is not at the AS layer.

    - In response to the command failure message from the reader, the AIOTF can check if there has been a NAS message coding error in the original command request. If yes, re-code the command request and send it to the same reader again. If not, the AIOTF may consider that the security keys may have been out of synch between the device and the AIOTF during the failed round, and initiate another “inventory-and-command” procedure with the same reader with the same command request (in plaintext), hoping that the keys will be derived correctly this time.

- Integrity failure:

  - Symptoms: 1) a MAC response with 0 SDU and MDI=0 is timely received; 2) no MAC response is received, as suggested in [3] and [4], which has been deemed as inadequate in the previous analysis for the NAS protocol error case and hence should not be pursued.

    - Therefore, the best behaviour of the device in case of integrity failure is still 1), i.e., sending a MAC response with 0 SDU and MDI=0.

  - Remedy: already described in the remedy for the NAS protocol error.

- Time delay (not enough time for the device to finish the execution of the command and generate a NAS response in time for the MAC response, however, the NAS response is coming, only later,  i.e., the case of delayed NAS response):

  - Symptoms: the reader received a MAC response with 0 SDU as a response to the command request. The current debate is whether MDI should be set to 0 or 1.

    - If MDI is set to 0, as in the current baseline text, first, the reader would not be able to differentiate this case from the cases of NAS protocol error case or integrity failure case, preventing the reader to timely apply the correct remedy. For example, the reader may treat the time delay case the same way as the NAS protocol error case and the integrity failure case by sending the command failure message to the AIOTF, while the problem could have been solved by the reader itself, had it received enough information to differentiate the case of time delay from the cases of NAS protocol error and integrity failure.

    - Instead, if MDI is set to 1 in this case, combined with 0 SDU in the timely MAC response, the reader would be able to know that a) communication with the device is good and the command request has been received (due to the MAC response); b) the device currently has no NAS response to send (due to 0 SDU); c) but the device will have the NAS response to send later (due to MDI=1). The combination of a) to c) ensures that the reader can differentiate the time delay case from all the other failure cases described above, therefore, the reader can timely apply the correct remedy that has been adopted in the current baseline text for the delayed NAS response case, as below.

  - Remedy in the current baseline text: the reader later sends another R2D Upper Layer Data Transfer message with the Received Data Size field indicating 0 without the command request.

Observation 3. Considering how to handle all above scenarios effectively and consistently, the device sending the timely MAC response with 0 SDU and MDI=1 remains as the best solution for the case of “no data available due to delay in NAS”.

## On the compatibility between the segmentation status and the delay status

A case of segmentation and a case of delayed NAS response can never happen at the same time, i.e., if an SDU is currently segmented, it cannot be currently delayed, and if an SDU is currently delayed, it cannot be currently segmented. Therefore, when combined with the presence/absence of the SDU in the MAC response, the MDI value can be used for differentiating the case of segmentation and the case of delayed NAS response. There is a unified way  for determining when the R2D Upper Layer Data Transfer message should include the Data SDU field (to carry the command request) and when it should include the Received Data Size field (to indicate the offset) without changing the design that these two fields are mutually exclusive. If the Received Data Size field is included, its value ranges for the case of segmentation and for the case of delayed NAS response are also mutually exclusive. In our opinion, there is no need to introduce a separate bit for indicating the delay status, as suggested in [5].

Proposal 1. MDI is set to 1 (instead of 0, as previously agreed) for the case of “no data available due to delay in NAS”.

# Conclusions

Observation 1: Additional cases of no NAS response (other than due to NAS delay) do exist, due to NAS protocol error handling and due to failure of integrity verification on the command request at the NAS layer.

Observation 2: Unlike the delayed NAS response case where the expected NAS response is still pending, all the above cases of no NAS response are conclusive.

Observation 3. Considering how to handle all above scenarios effectively, the device sending a timely MAC response with 0 SDU and MDI=1 remains as the best solution for the case of “no data available due to delay in NAS”.

Based on the above observations, we propose the following:

Proposal 1. MDI is set to 1 (instead of 0, as previously agreed) for the case of “no data available due to delay in NAS”.

# References

R2_131_Chairnotes_25-08-29_17-00_final.

TS 24.369 V1.0.0.

R2-2506915, Discussion on A-IoT remaining issues, Spreadtrum, UNISOC, RAN2#131bis.

R2-2507101, Discussion on open issue A-IoT, Apple, RAN2#131bis.

R2-2507210, Discussion on open issues for AIoT, LGE, RAN2#131bis.