3GPP TSG-RAN WG2 Meeting #131bis	R2-2507217

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	10.4

Source:	Samsung, Verizon

Title:	Study on 6G Mobility Framework

Document for:	Discussion and Decision

Introduction

In RAN#108 plenary, a study item on “6G RAN” was approved [1]. Based on the SID, the study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements. This contribution provides a summary of Samsung’s views on the general mobility principles, types (e.g. L3, CHO, LTM, RLM/RLF, cell reselection), and measurements including Inter-RAT and intra-RAT mobility.

Discussion

General 6G Mobility Principles

The transition from 4G LTE (Long Term Evolution) to 5G NR (New Radio) has revolutionized mobile network capabilities, significantly enhanced data throughput, latency, and reliability. Mobility technologies—encompassing seamless handover (HO) mechanisms, cell reselection, and tracking area management—play a pivotal role in maintaining uninterrupted connectivity for users, particularly in dynamic environments.

LTE established foundational mobility protocols, including intra- and inter-frequency handovers and idle-mode cell reselection, optimized for macro-cell deployments. In contrast, 5G NR introduced ultra-reliable low-latency handovers, and AI/ML-based predictive algorithms to anticipate UE’s measurement results on top of the L3-based handover principles introduced in LTE. These innovations minimize signalling overhead, reduce latency, and ensure service continuity across networks, enabling next-generation applications. Since at least the same level of mobility performance as NR is expected, for 6G, the similar mobility KPI should be considered. In this aspects, among many objectives of mobility enhancements, latency reduction, robustness, simple design to implement and resource efficiency would be the main considerations on 6G mobility.

Figure 1. 5G NR mobility schemes

NR mobility enhancements were mainly targeted to reduce the latency and improve the robustness during the HO. In addition, the basic L3-based regular HO should be the basic mobility principle in 6G as well because this cell-level NR HO shows the stable and good performance in general scenario (e.g. normal speed and service cases).

In Figure 1, various mobility schemes introduced in NR are depicted. These NR mobility schemes have improved in many years, so that the mobility performances of wireless communication are enhanced a lot in terms of latency reduction and robustness. As we know, there is no specific KPI requirement in mobility performance specific to the new 6G service requirement, therefore we think inheriting NR design metric is also the way 6G mobility aims to go. So, the main design principle of 6G mobility framework would be latency reduction, enhancing robustness and the increasing resource efficiency similar as NR. For this purpose, the current NR mobility schemes could be the baseline for 6G, while these NR schemes should be optimized for further enhancement. The additional important design metric is to reduce signalling overhead and UE operation complexity for the mobility procedure. In this aspect, the unified design of the 6G mobility framework is required to consider the real implementation impact.

Observation 1: 5G NR introduced ultra-reliable low-latency handovers, and AI/ML-based predictive methods to anticipate UE’s measurement results on top of the L3-based handover principles introduced in LTE.

Proposal 1: 6G mobility framework aims to achieve latency reduction, enhance robustness and resource efficiency considering the practical implementation, and the L3-based regular HO is the baseline for the 6G mobility.

Unified L1/L3 Measurement Framework

According to NR measurement model in Figure 2, NR measurement model uses the beam measuring and the cell quality is determined by the detected beam measurements. We call this as Layer 3 RRM measurement model. That is, UE measures each detected beams of the serving cell and neighbor cells and these beam measurement results are averaged to derive the cell quality. For doing this, the UE is configured to consider a subset of the detected beams. Filtering takes place at two different levels: at the physical layer to derive beam quality and then at RRC level to derive cell quality from multiple beams. Cell quality from the beam measurements is derived in the same way for the serving cell(s) and for the non-serving cell(s). The main role of RRM measurement is to report the cell quality, while reporting the L3 filtered beam quality is an optional feature. Therefore, the measurement reports may contain the measurement results of the X best beams (L3 filtered) if the UE is configured to do so by the gNB. However, these X best beams are not exactly same as the beams which are used for calculating the cell quality based on the measurement model.

Figure 2. NR Measurement Model

In addition, NR LTM introduced the different measurement model for Layer 1 measurements with the Layer 3 ones, we call this as Layer 1 RRM measurement model. In this measurement model, the cell quality is not used for target cell selection and only the beam level measurements are used for mobility decision. This Layer 1 RRM measurement model follows the CSI measurement/report configurations so the measurement resources are configured explicitly to UE for this RRM purpose. From the NW perspective, this measurement resource information sharing should be done in the preparation phase in order that the source cell provides these measurement resource in L1 measurement/report configurations for LTM preparation to UE. Plus, this reference signal configuration might be duplicated for CSI measurement resource configuration and L1 measurement resource configuration in serving cell and candidate cells. For example, L1 RSes in CSI-MeasConfig (CSI) and LTM-CSI-ResourceConfig (RRM) could configure the same RSes for candidate cells but the usage is different. So, this redundant measurement configuration could be optimized to reduce the RRC signaling overhead.

Since these L1 based mobility schemes was not considered in the beginning of NR, NR measurement models for RRM and CSI are designed independently, and also these models can work in parallel. Beam measurements would be used for both RRM and CSI purpose if 6G will introduce the L1-based mobility scheme, so the unifying any duplicated signaling and optimizing for beam and cell measurement configuration and reporting, in L1/L3 measurement framework is required in 6G.

Observation 2: Since L1-based mobility scheme (LTM) was not considered in the beginning of NR, NR measurement models for RRM and CSI are designed independently, and also these models can work in parallel.

Proposal 2: Unified framework for L1/L3 measurement configuration, which is used for L3-based RRM, L1-based RRM and CSI acquisition purpose, is studied.

Connected Mode Mobility Support

For HO latency reduction, dual stack protocol stack (DAPS) to achieve 0 ms HO interruption, L1/L2 triggered mobility (LTM) and RACH-less HO for both L3 HO and LTM were introduced. The features introduced in LTM (e.g. early DL/UL synchronization, pre-processing of RRC decoding, optimized beam management, etc.) reduces HO interruption time significantly. Here, the mobility interruption time for LTM is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell. The latency of the RACH-based and RACH-less LTM procedure is characterized by the terms illustrated in Figure G-2 and Figure G-3 in [2]. This LTM feature does not require big hardware upgrade so LTM is now implemented in many UE and NW vendors, so we believe the LTM will be used in real NR deployment soon. However, DAPS has not been implemented due to its complexity from UE side even though DAPS could achieve 0 ms HO interruption theoretically. 6G mobility framework should be simplified to avoid complex design with various mobility schemes. We think this HO latency reduction scheme what NR implemented (i.e. LTM) could be studied as the basic 6G mobility framework.

Plus, the early UL TA acquisition was introduced in LTM to derive the TA value of the target cell in advance (i.e. RACH-less LTM). The advantage of this feature is to reduce the interruption caused by the RACH procedure during the cell switch. To support this feature, the network side needs to reserve some dedicated RACH resource. In the current NR specification, the dedicated RACH resource for early TA acquisition can be shared among the UEs from the same DU. Thus, for each candidate cell, it has to reserve the separate RACH resource sets, each of which are prepared for UEs belonging to one DU, such per-DU resource reservation will consume more RACH resource. We believe this early TA acquisition could be extended to many additional cases in 6G. On the other hand, such extension may result in more RACH resource reservation for different purposes. Thus, we may consider the unified early TA acquisition scheme for resource efficient design.

The robustness of mobility aims at reducing the potential failures during UE mobility (i.e. missing the HO command from NW due to the bad channel condition). To achieve this purpose, conditional mobility mechanisms, e.g., conditional handover (CHO), conditional LTM (CLTM), conditional PSCell addition/change (CPAC), were introduced in NR. We also think this CHO and CLTM could be considered as the basic 6G mobility framework but CPAC study in 6G could be dependent on DC support in 6G.

Observation 3: For HO latency reduction and enhance robustness, LTM and CHO related features have been implemented in real NR deployments, but the DAPS has not been implemented due to its complexity.

NR mobility enhancements have been carried out over several releases, resulting in a very complex design with multiple mobility schemes to cover different scenarios. While these enhancements provide flexibility, they have also introduced redundancy in configuration, measurements, and reporting. In many cases, the network sends duplicate configuration parameters, and the UE reports duplicate measurements (e.g., for L3, LTM, etc.) to support different procedures. This leads to inefficient signaling and increased complexity in mobility management. In addition, NR LTM use the MAC CE (L2 signal) as a cell switch command but it is not secured (e.g. lacks integrity protection). This cell switch command makes UE to go another cell, so it creates the performance loss if this cell switch command has been tempered with. In 6G, the unified simple design for mobility by considering the following aspects are required.

- Avoid duplicated cell preparation and configuration: One cell may be prepared for multiple times for different purpose (e.g. (C)LTM, CHO).

- Avoid unnecessary measurements and duplicate measurement reporting: Both L1 and L3 measurements for the same cell can be used for mobility triggering.

- Secured L3 or L2 cell switch command with prepared target cell configuration.

- Ensure the efficient resource allocation, e.g., early TA resource.

Figure 3. 6G mobility framework

Proposal 3: RAN2 study the unified design for 6G mobility supporting following aspects:

- HO execution initiated by UE for robustness

- HO execution initiated by network via secured HO command for security enhancement

- Early TA acquisition for latency reduction

- Pre-configured candidate configuration for signaling overhead reduction and resource efficiency

Finally, AI/ML-based mobility solutions have been investigated in NR and some features can facilitate the mobility performance improvement. One such feature is RRM measurement prediction, which enables the UE to anticipate future measurement results for serving/neighboring cells. This allows the UE to initiate early measurement reporting with the predicted values, effectively predicting measurement events. Based on this early report, NW can proactively prepare or trigger HO procedure earlier to avoid too late handover, compared to the legacy HO that relies solely on historical measurement results. From this aspect, the benefits of the AI/ML-based mobility features (e.g., HOF rate decrease) have been evaluated and validated under L3 HO framework in Release 19, as outlined in the AI/ML MOB SI.

To enhance the robustness of mobility, we believe that AI-based measurement/event prediction can be further extended to (C)LTM. For instance, the UE could report the predicted L1 measurements for neighbor cells using a new CSI report format. This would enable NW to select target or candidate cells that are likely to maintain stable channel quality even after (C)LTM. Additionally, for CHO/CLTM execution conditions, which currently rely solely on historical measurement results, incorporating predicted future measurement results could further improve robustness and reliability.

In Release 19 AI/ML MOB SI, RLF prediction was initially included in SID as one of the use cases for study but was eventually deprioritized due to time constraints. However, the value of RLF prediction remains significant, as this failure event directly and negatively impacts user experience. If an RLF is predicted, the UE could report it to NW to request an early HO or initiate a fast recovery procedure, thereby minimizing its adverse effects. Building on this motivation and approach, beam failure prediction can also be explored for 6G. Given that beam failure events are likely to occur more frequently and earlier than RLF, beam failure event prediction can represent an even more useful and important use case. For the feasibility of RLF and beam failure prediction, we believe these can be effectively implemented by leveraging the existing features such as RRM measurement prediction (from Rel-20 AI/ML MOB) and beam prediction (from Rel-19 AI/ML PHY), both of which have already been evaluated. Thus, RAN2 can study RLF and beam failure prediction for 6G.

Observation 4: AI/ML-based mobility solutions have been investigated in NR basic mobility scheme (e.g. L3 HO) and some features (e.g. RRM measurement prediction, RLF prediction) can facilitate the mobility performance improvement.

Proposal 4: RRM measurement/event prediction is considered as Day-1 feature for 6G mobility. RAN2 study the application of AI-based measurement/event prediction for CHO, (C)LTM, RLF, and beam failure.

RLM/RLF and Fast Recovery

As a radio channel monitoring and the radio link failure handling procedure, NR defined the Radio Link Monitoring (RLM) and Radio Link Failure (RLF) procedures. For RLM, UE in the active BWP monitors the configured reference signals and check this measured quality with the configured thresholds. This RLM is used for both beam failure detection (BFD) and RLF cases.

For example, the UE declares RLF when one of the following criteria are met in NR:

-	Expiry of a radio problem timer started after indication of radio problems from the physical layer; or

-	Expiry of a timer started upon triggering a measurement report for a measurement identity for which the timer has been configured while another radio problem timer is running; or

-	Random access procedure failure; or

-	RLC failure; or

-	Detection of consistent uplink LBT failures.

After RLF declared the RRC re-establishment timer T311 starts, the UE selects a suitable cell and then initiates RRC re-establishment. Otherwise, UE goes to RRC IDLE state. We think this NR RLF procedure is also needed in 6G as a baseline. For the enhancement of radio link recovery procedure, NR introduced the fast recovery procedure in CHO and LTM, the detail procedure is captured below.

-	in case of CHO, for RLF in the source cell:

-	selects a suitable cell and if the selected cell is a CHO candidate and if network configured the UE to try CHO after RLF then the UE attempts CHO execution once, otherwise re-establishment is performed;

-	enters RRC_IDLE if a suitable cell was not found within a certain time after RLF was declared.

-	in case of MCG LTM, for RLF in the source cell:

-	selects a suitable cell and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM after RLF then the UE attempts RACH-based LTM execution once, otherwise re-establishment is performed;

-	enters RRC_IDLE if a suitable cell was not found within a certain time after RLF was declared.

We think the initial 6G mobility framework considers the pre-configurations of the candidate target cells as introduced in NR CHO and LTM as we proposed above. Then, this fast recovery procedure could be also applied for the unified radio link recovery scenario (i.e. RLF, HO failure).

Observation 5: RLM procedure in NR is commonly defined for both RLF and BFR, and the fast recovery procedures introduced in CHO and LTM enhanced the performance of the radio failure cases in terms of recovery time.

Proposal 5: RLM/RLF procedure in NR is used for the baseline and the unified fast recovery mechanism is applied for RLF and HO failure cases.

Cell Reselection

In NR, the IDLE/INACTIVE mode mobility is defined based on following cell reselection criteria which involves measurements of the serving and neighbour cells.

- Cell reselection is always based on cell defining sync signals

- Intra-frequency reselection is based on ranking of cells

- Inter-frequency/inter-RAT reselection is based on absolute priorities

- Cell reselection can be speed dependent

- Cell/Service specific prioritization

We think this NR cell reselection criteria is stable in performance so it could be reused in 6G cell reselection framework.

Proposal 6: NR cell reselection framework is the baseline of 6G cell reselection framework.

Conclusion

From lessons learned from NR, the following observations are justified:

Observation 1: 5G NR introduced ultra-reliable low-latency handovers, and AI/ML-based predictive algorithms to anticipate UE’s measurement results on top of the L3-based handover principles introduced in LTE.

Observation 2: Since L1-based mobility scheme (LTM) was not considered in the beginning of NR, NR measurement models for RRM and CSI are designed independently, and also these models can work in parallel.

Observation 3: For HO latency reduction and enhance robustness, LTM and CHO related features have been implemented in real NR deployments, but the DAPS has not been implemented due to its complexity.

Observation 4: AI/ML-based mobility solutions have been investigated in NR basic mobility scheme (e.g. L3 HO) and some features (e.g. RRM measurement prediction, RLF prediction) can facilitate the mobility performance improvement.

Observation 5: RLM procedure in NR is commonly defined for both RLF and BFR, and the fast recovery procedures introduced in CHO and LTM enhanced the performance of the radio failure cases in terms of recovery time.

RAN2 is requested to discuss and agree the following proposals:

Proposal 1: 6G mobility framework aims to achieve latency reduction, enhance robustness and resource efficiency considering the practical implementation, and the L3-based regular HO is the baseline for the 6G mobility.

Proposal 2: Unified framework for L1/L3 measurement configuration, which is used for L3-based RRM, L1-based RRM and CSI acquisition purpose, is studied.

Proposal 3: RAN2 study the unified design for 6G mobility supporting following aspects:

- HO execution initiated by UE for robustness

- HO execution initiated by network via secured HO command for security enhancement

- Early TA acquisition for latency reduction

- Pre-configured candidate configuration for signaling overhead reduction and resource efficiency

Proposal 4: RRM measurement/event prediction is considered as Day-1 feature for 6G mobility. RAN2 study the application of AI-based measurement/event prediction for CHO, (C)LTM, RLF, and beam failure.

Proposal 5: RLM/RLF procedure in NR is used for the baseline and the unified fast recovery mechanism is applied for RLF and HO failure cases.

Proposal 6: NR cell reselection framework is the baseline of 6G cell reselection framework.

References

[1]	RP-251881		New SID: Study on 6G Radio	NTT DOCOMO.

[2]	TS 38.300		NR and NG-RAN Overall Description; Stage 2		Rel-18.