3GPP TSG-RAN WG2 Meeting #131bis	R2-2507259

Prague, Czech Republic, Oct. 25th – 29rd, 2025

Agenda Item	: 8.13.2

Source	: LG Electronics Inc.

Title	: Discussion on Control Plane Procedure for multi-hop U2N relay

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink multi-hop relay was approved for Rel-19 and the WID is shown in RP-242349 including the following objectives.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this document, we mainly investigate the notification message delivery condition considering the other working group spec works (e.g., CT1/SA2).

2.	Discussion

2.1 Notification message delivery condition

RAN2 sends LS to SA2 (R2-2504800) for inform the RAN2 agreement about the notification message. The followings are captured from the LS.

Agreements: The notification message is used for the L2 child UE in case of upstream relay (re)selection triggered by AS layer failure cases (Uu/PC5 link failure/release, using legacy cause value) where the upper layer does not trigger link release towards the child. For reselection triggered by upper layers we rely on the upper-layer notification/release mechanism already defined. Remote UE handling of the notification is up to upper layer implementation. Further handling of any inter-layer issues on this topic can be done in maintenance phase.. Regarding to the notification message mentioned in the above agreements, it is defined in TS 38.331 as NotificationMessageSidelink since Rel-17, in details, the notification message is used to inform L2 or L3 U2N Remote UE(s) when the L2 or L3 U2N Relay UE performs handover, detects Uu RLF, or its Uu RRC connection establishment/resume fails. Upon reception of the notification message, it is up to U2N Remote UE implementation whether to release or keep the unicast PC5 link. If U2N Remote UE decides to release the unicast PC5 link, it triggers the PC5 release procedure and may perform cell or relay reselection.

According to our understanding based on the RAN2 agreement, the notification message can be used for L3 or L2 U2N intermediate Relay UE or Remote UE. If the upper layer keeps maintaining the child link after the AS layer failure cases in the upstream, the AS layer can send the NotificationMessageSidelilnk message to the child UE. When the child UE receives the NotificationMessageSidelilnk message from its parent UE, it’s up to the child UE’s implementation whether to maintain or release the link with the parent UE. In other words, the UE receiving the NotificationMessageSidelilnk message has an opportunity to decide whether to maintain or release the link.

Meanwhile, according to the CT1 spec (TS 24.554), they are describing the case of relay reselection triggering conditions and how to handle the relay reselection procedure. The followings are partly captured from TS 24.554.

#### 8b.2.3.2 Multi-hop UE-to-network relay reselection procedure initiation The 5G ProSe multi-hop remote UE or the 5G ProSe intermediate UE-to-network relay UE shall trigger the multi-hop UE-to-network relay reselection procedure if one of the following conditions is met: a) the UE has received a lower layers indication that the serving 5G ProSe multi-hop UE-to-network relay UE or the serving 5G ProSe intermediate UE-to-network relay UE no longer fulfills the lower layers criteria as specified in 3GPP TS 38.331 [13]; <Omitted> To conduct multi-hop UE-to-network relay reselection process, the UE shall first initiate one of the following procedures or both depending on UE's configuration parameters for 5G ProSe multi-hop UE-to-network relay as specified in clause 5.2.8: a) monitoring procedure for UE-to-network relay discovery as specified in clause 8b.2.1.2.3; or b) discoverer procedure for UE-to-network relay discovery as specified in clause 8b.2.1.3.2. After the execution of the above discovery procedure(s), the 5G ProSe multi-hop remote UE performs the multi-hop UE-to-network relay selection procedure as specified in clause 8b.2.5. For a 5G ProSe intermediate UE-to-network relay UE based on model B discovery, if the 5G ProSe intermediate UE-to-network relay UE detects link failure or link quality degradation, the the 5G ProSe intermediate UE-to-network relay UE may send a relay reselection indication toward the 5G ProSe multi-hop remote UE by using the 5G ProSe direct link modification procedure as specified in clause 8b.2.2.3. If the 5G ProSe multi-hop remote UE receives the relay reselection indication, it may initiate the multi-hop UE-to-network relay reselection procedure.

According to the CT1 spec, the relay UE shall trigger relay reselection when the lower layers do not fulfil the lower layer criteria as specified in 3GPP TS 38.331. The lower-layer criteria to initiate relay reselection triggering at the intermediate Relay UE may be Uu/PC5 link failure/release with the parent UE. When the relay reselection is triggered, the UE initiates the discovery procedure.

If relay reselection is triggered at the intermediate Relay UE, the intermediate Relay UE receives the announcement message from the candidate parent UEs and makes the PC5 connection with one of the candidate parent UEs before forwarding the announcement message to the child UE in the discovery model A. If the child UE receives the announcement message, the child UE can update the changed information via the announcement message, such as hop-count or root information changes, while maintaining the PC5 link with the intermediate Relay UE. According to the CT1 spec, in the discovery model B, the intermediate Relay UE may send a relay reselection indication toward the child UE when relay reselection is triggered. If the child UE receives the relay reselection indication from its parent UE, the child UE initiates relay reselection, i.e., transmitting a solicitation message. If the child UE maintains the PC5 link with the parent UE, the solicitation message can be delivered via the current maintaining PC5 link.

For these procedures, there is no clear description of whether the link between the intermediate Relay UE and child UE is maintained or released when relay reselection is triggered. The intermediate Relay UE may perform the relay reselection procedure while maintaining the child links. The upper layer can perform the link modification, such as hop-count or path change, via the discovery message or relay reselection indication while maintaining the child links. In the multi-hop relay, it takes a longer time to set the multi-hop connection than to set a single-hop connection, including the discovery procedure. So, partly recovering a link while maintaining the downstream links can help save signalling load and reduce connection effort.

Comparing the the NotificationMessageSidelilnk usage to the CT1 spec, the NotificationMessageSidelilnk gives a possibility to the child UE whether to maintain the link with the parent UE or release. The unnecessary propagation of the NotificationMessageSidelilnk message can make unnecessary link release and relay reselection triggering at each intermediate Relay UE or Remote UE.

Observation 1: According to the CT1 spec (i.e., TS 24.554), the upper layer can handle the link modification, such as hop-count or path information changes via discovery message or relay reselection indication, while maintaining the downstream links.

Observation 2: The NotificationMessageSidelilnk message applied to L2 or L3 U2N Relay, and it gives the opportunity whether to maintain or release the upstream link to the intermediate Relay UE or Remote UE.

- When the intermediate Relay UE receives path switching commend (i.e., indirect-to-(in)direct path switching) within the same cell.

In the RAN2#131 messing, RAN2 made the following agreements:

- Notification by an intermediate relay UE at least in idle/inactive, when caused by an upstream reselection/RLF/link release, occurs upon the intermediate relay UE’s handling after receiving the notification or release message (e.g., relay reselection or cell selection), but not triggered by the reception of the upstream notification itself.  RRC_CONNECTED case to be considered as part of service continuity.

- In case of notification message triggered by an error (e.g., RLF), for intermediate relay UE in idle/inactive, the intermediate relay UE can send cell/relay reselection cause value to the remote UE in RRC_IDLE/INACTIVE if it successfully performs a recovery action. If recovery action is not successful or if it opts to release the upstream PC5 link, it sends relayUE-PC5-RLF cause value if PC5 RLF is detected by itself or the same cause value received in the notification message from the parent node.

- The intermediate relay UE in RRC_IDLE/INACTIVE may omit sending the notification message if the relay reselection or cell selection (from multihop to direct path for the intermediate relay) does not cause the change of the serving cell.  This does not preclude the implementation considering other parameters such as hop count if they are made visible to AS layer by implementation.  Capture this in a NOTE in the stage 3 spec.

According to the above agreements, IDLE/INACTIVE intermediate Relay UE may not send the notification message to the child UE if the cell ID is not changed after relay reselection or cell selection. However, the intermediate Relay UE can send a notification message to the child UE by its implementation if the hop count is visible at the AS layer and the value has changed. The reason that the intermediate UE may not send the notification message to the child UE was to prevent the child relay/Remote UEs being triggered relay reselection unnecessarily.

The same principle can be applied to the connected intermediate Relay UE. Unlike the legacy U2N Relay UE in the single-hop, the RRC_CONNECTED intermediate Relay UE in the multi-hop can change its parent UE, i.e., by receiving path switching message, within the same cell. In the legacy single-hop operation, only the Remote UE can receive the path switching command within the same cell, e.g., the Remote UE can change the path from direct to indirect, and the target Relay UE can be in the same cell as the cell of the previous direct path. However, in this case, the Remote UE in a single-hop doesn’t need to send a notification message to the downstream since there are no child UEs. But in the multi-hop operation, if the intermediate Relay UE receives the path switching command within the same cell, i.e., the case that the target parent Relay UE is in the same cell as the cell of the source Relay UE, the we need to discuss whether it is necessary for an intermediate Relay UE in RRC_CONNECTED may send the notification message to the child UEs.

We think that if the RRC_CONNECTED intermediate Relay UE receives the path switching command within the same cell, the intermediate Relay UE may not send the notification message to the child UEs as the same principle that has applied to the IDLE/INACTIVE intermediate Relay UE. This is possible under the assumption that the gNB has the context of all the child UEs. The gNB can send proper configurations to the child UEs before/after sending a path switching command to the intermediate Relay UE. If the gNB cand sends proper configurations to the child UEs before/after sending a path switching command, the notification message doesn't need to be delivered. Because gNB already has the context of all the downstream relay UEs of the intermediate Relay UE, the gNB can configure a proper configuration to the intermediate Relay UE to keep the downstream links. It’s different from the group HO in terms of just changing the path.  Even if the hop-count is changed, it can be handled by the upper layer via discovery message or relay reselection indication.

More specifically, if the intermediate Relay UE receives path switching command from the gNB and the target parent UE is in the same cell as the previous parent UE, the intermediate Relay UE doesn’t send the NotificationMessageSidelilnk message immediately to the child UE. Or, if the intermediate Relay UE receives path switching command from the gNB and the target path is a direct path to the same cell, the intermediate Relay UE doesn’t send the NotificationMessageSidelilnk message immediately to the child UE. In those cases, the intermediate Relay UE doesn’t send the NotificationMessageSidelilnk message to the child UE when the connection establishment to the target path is successfully performed. The change of the upper layer information, such as path information or hop-count change, can be handle at the upper layer. The AS layer sends the NotificationMessageSidelilnk message only when the cause of the NotificationMessageSidelilnk should give the opportunity to the child UE whether to maintain or release the connection with the parent UE.

Proposal 1: If the RRC_CONNECTED intermediate Relay UE receives path switching command and the target parent Relay UE is in the same cell of the previous parent UE, the intermediate Relay UE doesn’t need to send NotificationMessageSidelilnk message to the child UE regardless of the hop-count or path information changes.

Proposal 2: If the RRC_CONNECTED intermediate Relay UE receives path switching commend and the target path is the direct path of the same cell, the intermediate Relay UE doesn’t need to send NotificationMessageSidelilnk message to the child UE.

3.	Conclusion

Observation 1: According to the CT1 spec (i.e., TS 24.554), the upper layer can handle the link modification, such as hop-count or path information changes via discovery message or relay reselection indication, while maintaining the downstream links.

Observation 2: The NotificationMessageSidelilnk message applied to L2 or L3 U2N Relay, and it gives the opportunity whether to maintain or release the upstream link to the intermediate Relay UE or Remote UE.

Proposal 1: If the RRC_CONNECTED intermediate Relay UE receives path switching command and the target parent Relay UE is in the same cell of the previous parent UE, the intermediate Relay UE doesn’t need to send NotificationMessageSidelilnk message to the child UE regardless of the hop-count or path information changes.

Proposal 2: If the RRC_CONNECTED intermediate Relay UE receives path switching command and the target path is the direct path of the same cell, the intermediate Relay UE doesn’t need to send NotificationMessageSidelilnk message to the child UE.

4.	Reference

- RP-241609, Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNet.