3GPP TSG-RAN WG2 Meeting #131bis														R2-2507261

Prague, Czech Republic, Oct. 13th-17th

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 5160 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rapporteur correction on IoT NTN Ph3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR is to capture the corrections from the agreed RILs based on the RIL status in R2-2507060. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR captures the corrections from the following agreed RILs based on the RIL status in R2-2507060: V212, V213, N011, N013, V216, W801. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There would be errors in the RRC specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.3a, 5.3.3.3b, 5.3.3.3e, 6.3.1, 6.3.2, 6.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

5.3.3.3a	Actions related to transmission of RRCConnectionResumeRequest message

If the UE is resuming the RRC connection from a suspended RRC connection, the UE shall set the contents of RRCConnectionResumeRequest message as follows:

1>	if the UE is a NB-IoT UE; or

1>	if the UE is initiating UP-EDT for mobile originating calls in accordance with conditions in 5.3.3.1b; or

1>	if the UE is initiating UP transmission using PUR in accordance with conditions in 5.3.3.1c; or

1>	if field useFullResumeID is signalled in SystemInformationBlockType2:

2>	if the UE connected to 5GC is a BL UE or UE in CE:

3>	set the fullI-RNTI to the stored fullI-RNTI;

2>	else:

3>	set the resumeID to the stored resumeIdentity;

1>	else:

2>	if the UE connected to 5GC is a BL UE or UE in CE:

3>	set the shortI-RNTI to the stored shortI-RNTI;

2> else:

3>	set the truncatedResumeID to include bits in bit position 9 to 20 and 29 to 40 from the left in the stored resumeIdentity.

1>	if the UE is resuming the RRC connection after release with redirect with mpsPriorityIndication:

2>	set the resumeCause to highPriorityAccess;

1>	else if the UE supports mo-VoiceCall establishment cause and UE is resuming the RRC connection for mobile originating MMTEL voice and SystemInformationBlockType2 includes voiceServiceCauseIndication and the establishment cause received from upper layers is not set to highPriorityAccess:

2>	set the resumeCause to mo-VoiceCall;

1>	else if the UE supports mo-VoiceCall establishment cause for mobile originating MMTEL video and UE is resuming the RRC connection for mobile originating MMTEL video and SystemInformationBlockType2 includes videoServiceCauseIndication and the establishment cause received from upper layers is not set to highPriorityAccess:

2>	set the resumeCause to mo-VoiceCall;

1>	else if the UE is initiating UP-EDT for mobile terminating calls in accordance with conditions in 5.3.3.1b:

2>	set the resumeCause to mt-EDT;

1>	else:

2>	set the resumeCause in accordance with the information received from upper layers;

1>	set the shortResumeMAC-I to the 16 least significant bits of the MAC-I calculated:

2>	over the ASN.1 encoded as per clause 8 (i.e., a multiple of 8 bits) VarShortResumeMAC-Input (or VarShortResumeMAC-Input-NB in NB-IoT);

2>	with the KRRCint key and the previously configured integrity protection algorithm; and

2>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	if the UE is a NB-IoT UE:

2>	if the UE supports DL channel quality reporting in MSG3 and cqi-Reporting is present in SystemInformationBlockType2-NB:

3>	set the cqi-NPDCCH to include the latest results of the downlink channel quality measurements of the carrier where the random access response is received or set the cqi-NPDCCH to include the latest results of the downlink channel quality measurements of the anchor carrier in case CB-Msg3 is transmitted on the anchor carrier, as specified in TS 36.133 [16];

NOTE 0:	The downlink channel quality measurements use measurement period T1 or T2, as defined in TS 36.133 [16].

2>	if the UE is connected to EPC, set earlyContentionResolution to TRUE;

1>	restore the RRC configuration and security context from the stored UE AS context, except for the following:

-	MCG SCell(s) configuration, if stored,

-	nr-SecondaryCellGroupConfig, if stored;

1>	if the UE is initiating UP-EDT for mobile originating calls in accordance with conditions in 5.3.3.1b:

2>	if the UE is a NB-IoT UE connected to EPC:

3>	if the UE has ANR measurements information available in VarANR-MeasReport-NB and if the RPLMN is included in plmn-IdentityList stored in VarANR-MeasReport-NB:

4>	set anr-InfoAvailable to TRUE;

1>	if the UE is resuming an RRC connection after early security reactivation in accordance with conditions in 5.3.3.18:

2>	if the UE is initiating UP-EDT in accordance with conditions in 5.3.3.1b; or

2>	if the UE is initiating UP transmission using PUR in accordance with conditions in 5.3.3.1c:

3>	restore the PDCP state and re-establish PDCP entities for all SRBs and all DRBs;

3>	if drb-ContinueROHC has been provided in immediately preceding RRC connection release message, and the UE is requesting to resume RRC connection in the same cell:

4>	indicate to lower layers that stored UE AS context is used and that drb-ContinueROHC is configured;

4>	continue the header compression protocol context for the DRBs configured with the header compression protocol;

3>	else:

4>	indicate to lower layers that stored UE AS context is used;

4>	reset the header compression protocol context for the DRBs configured with the header compression protocol;

3>	resume all SRBs and all DRBs;

2>	else:

3>	if the UE is a NB-IoT UE or the UE is connected to EPC, restore the PDCP state and re-establish the PDCP entity for SRB1;

3>	if the UE is connected to 5GC:

4>	apply the default configuration for SRB1 as specified in 9.2.1.1;

4>	except for NB-IoT, apply the default NR PDCP configuration as specified in TS 38.331 [82], clause 9.2.1 for SRB1;

3>	resume SRB1;

2>	derive the KeNB key based on the KASME key to which the current KeNB is associated, using the stored value of nextHopChainingCount received in the RRCConnectionRelease message in the preceding connection, as specified in TS 33.401 [32] for EPC and TS 33.501 [86] for 5GC;

2>	derive the KRRCint key associated with the previously configured integrity algorithm, as specified in TS 33.401 [32] for EPC and TS 33.501 [86] for 5GC;

2>	derive the KRRCenc key and the KUPenc key associated with the previously configured ciphering algorithm, as specified in TS 33.401 [32] for EPC and TS 33.501 [86] for 5GC;

2>	configure lower layers to resume integrity protection using the previously configured algorithm and the KRRCint key derived in this clause to all subsequent messages received and sent by the UE;

2>	configure lower layers to resume ciphering and to apply the ciphering algorithm and the KRRCenc key derived in this clause to all subsequent messages received and sent by the UE;

2>	configure lower layers to resume ciphering and to apply the ciphering algorithm and the KUPenc key derived in this clause immediately to the user data sent and received by the UE;

2>	if the UE is initiating UP-EDT for mobile originated calls in accordance with conditions in 5.3.3.1b:

3>	configure the lower layers to use EDT;

2>	else if the UE is initiating UP transmission using PUR in accordance with conditions in 5.3.3.1c:

3>	configure, except pur-TimeAlignmentTimer, the lower layers to use transmission using PUR;

3>	deliver the UL grant for transmission using PUR to the MAC entity;

1>	else:

2>	if SRB1 was configured with NR PDCP:

3>	for SRB1, release the NR PDCP entity and establish an E-UTRA PDCP entity with the current (MCG) security configuration;

NOTE 1:	The UE applies the LTE ciphering and integrity protection algorithms that are equivalent to the previously configured NR security algorithms.

2>	else:

3>	for SRB1, restore the PDCP state and re-establish the PDCP entity;

If the UE is resuming the RRC connection from RRC_INACTIVE, the UE shall set the contents of RRCConnectionResumeRequest message as follows:

2>	if field useFullResumeID is signalled in SystemInformationBlockType2:

3>	set the fullI-RNTI to the stored fullI-RNTI value provided in suspend;

2>	else:

3>	set the shortI-RNTI to the stored shortI-RNTI value provided in suspend;

2>	restore the RRC configuration, RoHC state, the stored QoS flow to DRB mapping rules and the KeNB and KRRCint keys from the UE Inactive AS context except for the following:

-	MCG physical layer,

-	MCG MAC configuration,

-	NR pdcp-Config,

-	MCG SCell configurations, if stored,

-	nr-SecondaryCellGroupConfig, if stored;

2>	set the shortResumeMAC-I to the 16 least significant bits of the MAC-I calculated:

3>	over the ASN.1 encoded as per clause 8 (i.e., a multiple of 8 bits) VarShortINACTIVE-MAC-Input;

3>	with the KRRCint key in the UE Inactive AS Context and the previously configured integrity protection algorithm; and

3>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

2>	derive the KeNB key based on the current KeNB or the NH, using the stored nextHopChainingCount value, as specified in TS 33.501 [86];

2>	derive the KRRCenc key, the KRRCint and the KUPenc key, as specified in TS 33.401 [32];

2>	apply the default configuration for SRB1 as specified in 9.2.1.1;

2>	apply the default NR PDCP configuration as specified in TS 38.331 [82], clause 9.2.1 for SRB1;

2>	configure lower layers to resume integrity protection for all SRBs except SRB0 using the configured algorithm and the KRRCint key derived in this clause immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE;

2>	configure lower layers to resume ciphering for all radio bearers except SRB0 and to apply the configured ciphering algorithm, the KRRCenc key and the KUPenc key derived in this clause, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE;

Following procedures are applied for both suspended RRC connection and RRC_INACTIVE:

2>	resume SRB1;

NOTE 2:	Until successful connection resumption, the default physical layer configuration and the default MAC Main configuration are applied for the transmission of SRB0 and SRB1, and SRB1 is used only for the transfer of RRCConnectionResume message, and RRCConnectionRelease message if security has been re-activated.

The UE shall submit the RRCConnectionResumeRequest message to lower layers for transmission.

The UE shall continue cell re-selection related measurements as well as cell re-selection evaluation.

If the UE is resuming the RRC connection from RRC_INACTIVE and if lower layers indicate an integrity check failure while T300 is running, the UE shall perform actions specified in 5.3.3.16.

5.3.3.3b	Actions related to transmission of RRCEarlyDataRequest message

The UE shall set the contents of RRCEarlyDataRequest message as follows:

1>	if upper layers provide an S-TMSI:

2>	set the s-TMSI to the value received from upper layers;

1>	else if upper layers provide a 5G-S-TMSI:

2>	set the ng-5G-S-TMSI to the value received from upper layers;

1>	set the establishmentCause in accordance with the information received from upper layers;

1>	if the UE is a NB-IoT UE:

2>	if the UE supports DL channel quality reporting and cqi-Reporting is present in SystemInformationBlockType2-NB:

3>	set the cqi-NPDCCH to include the latest results of the downlink channel quality measurements of the carrier where the random access response is received or set the cqi-NPDCCH to include the latest results of the downlink channel quality measurements of the anchor carrier in case CB-Msg3 is transmitted on the anchor carrier, as specified in TS 36.133 [16];

NOTE:	The downlink channel quality measurements may use measurement period T1 or T2, as defined in TS 36.133 [16]. In case period T2 is used the RRC-MAC interactions are left to UE implementation.

1>	set the dedicatedInfoNAS to include the information received from upper layers;

The UE shall:

1>	if the UE is initiating CP-EDT in accordance with conditions in 5.3.3.1b:

2>	configure the lower layers to use EDT;

1>	else if the UE is initiating CP transmission using PUR in accordance with conditions in 5.3.3.1c:

2>	configure, except pur-TimeAlignmentTimer, the lower layers to use transmission using PUR;

2>	deliver the UL grant for transmission using PUR to the MAC entity;

1>	submit the RRCEarlyDataRequest message to the lower layers for transmission.

#### 5.3.3.3e	UE actions upon receiving CB-Msg3-EDT indications from lower layers

For CP transmission using CB-Msg3-EDT, upon indication from lower layers that CB-Msg3-EDT is successfully completed, the UE shall perform the actions as specified in 5.3.3.4b as if an empty RRCEarlyDataComplete message was received.

Upon reception of CB-Msg3-EDT failure indication from lower layers, the procedure ends.

NOTE:	When receiving the CB-Msg3-EDT failure indication from lower layers, the RRC procedure is re-initiated. Which procedure (e.g. EDT using random access procedure, random access procedure, CB-Msg3-EDT) is initiated is up to UE implementation.

Next Change

6.3.1	System information blocks

–	SystemInformationBlockType33

The IE SystemInformationBlockType33 contains satellite assistance information for neighbour cells. When the SystemInformationBlockType33 is signalled in a TN cell, it may contain satellite assistance information for BL UEs, UEs in enhanced coverage, and/or NB-IoT NTN capable UEs.

SystemInformationBlockType33 information element

-- ASN1START

SystemInformationBlockType33-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18	NeighSatelliteInfoList-r18			OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING							OPTIONAL,

...,

[[

neighSatelliteInfoListNR-r19	NeighSatelliteInfoListNR-r19			OPTIONAL,	-- Need OR

neighSatelliteInfoList-v1900	NeighSatelliteInfoList-v1900			OPTIONAL,	-- Need OR

neighSatelliteInfoListNB-r19	NeighSatelliteInfoListNB-r19			OPTIONAL	-- Need OR

]]

}

NeighSatelliteInfoListNR-r19 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfoNR-r19

NeighSatelliteInfoList-v1900 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-v1900

NeighSatelliteInfoListNB-r19 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18

NeighSatelliteInfoList-r18 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18

NeighSatelliteInfo-r18 ::=	SEQUENCE {

satelliteId-r18				SatelliteId-r18,

ephemerisInfo-r18				CHOICE {

stateVectors-r18				EphemerisStateVectors-r17,

orbitalParameters-r18			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r18		SEQUENCE {

nta-Common-r18					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r18			INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r18	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

epochTime-r18					SEQUENCE {

startSFN-r18					INTEGER (0..1023),

startSubFrame-r18				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Mac-r18						INTEGER (1..512)				OPTIONAL,	-- Need OP

t-ServiceStartNeigh-r18		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

NeighSatelliteInfoNR-r19::=	SEQUENCE {

satelliteId-r19				SatelliteId-r18,

ephemerisInfo-r19				CHOICE {

stateVectors-r19				EphemerisStateVectors-r17,

orbitalParameters-r19			EphemerisOrbitalParameters-r17

}																OPTIONAL,	-- Need OP

nta-CommonParametersNR-r19		SEQUENCE {

nta-CommonNR-r19				INTEGER (0.. 66485757)	OPTIONAL,	-- Need OP

nta-CommonDriftNR-r19			INTEGER (-257303..257303)	OPTIONAL,	-- Need OP

nta-CommonDriftVariationNR-r19	INTEGER (0..28949)			OPTIONAL	-- Need OP

},

epochTime-r19					SEQUENCE {

startSFN-r19					INTEGER (0..1023),

startSubFrame-r19				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Mac-r19						INTEGER (1..512)				OPTIONAL,	-- Need OP

ntn-PolarizationDL-r19			ENUMERATED {rhcp,lhcp,linear}	OPTIONAL	-- Need OR

}

NeighSatelliteInfo-v1900 ::=	SEQUENCE {

sf-OperationModeNeigh-r19		ENUMERATED {barred, notBarred}	OPTIONAL,	-- Need OR

t-ModeSwitchingNeigh-r19		TimeOffsetUTC-r17					OPTIONAL	-- Need OR

}

-- ASN1STOP

Next Change

6.3.2	Radio resource control information elements

–	CB-Msg3-ConfigSIB

The IE CB-Msg3-ConfigSIB is used to specify the CB-Msg3-EDT configuration.

CB-Msg3-ConfigSIB information element

-- ASN1START

CB-Msg3-ConfigSIB-r19 ::=			SEQUENCE {

cb-Msg3-MinRSRP-Threshold-r19		RSRP-Range							OPTIONAL,	--Need OR

cb-Msg3-RSRP-CE-Level-r19			RSRP-Range							OPTIONAL,	--Need OR

cb-Msg3-ConfigList-r19				CB-Msg3-ConfigList-r19,

powerRampingParameters-r19			PowerRampingParameters-r19

}

CB-Msg3-ConfigList-r19 ::=		SEQUENCE (SIZE (1.. maxCE-Level-CB-Msg3-r19)) OF CB-Msg3-Config-r19

CB-Msg3-Config-r19 ::=				SEQUENCE {

cb-Msg3-TBS-r19							ENUMERATED {b144, b328, b408, b504, b600, b712,

b808, b936},

cb-Msg3-NumOfReplicas-r19				INTEGER(1..4),

cb-Msg3-TimeResource-r19		SEQUENCE {

pusch-Periodicity-r19					ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128,

sf256},

pusch-StartSFN-r19					INTEGER (0..1023),

pusch-StartSubframe-r19				INTEGER (0..9)

},

cb-Msg3-MPDCCH-Config-r19				CB-Msg3-MPDCCH-Config-r19,

cb-Msg3-PUCCH-Config-r19				CB-Msg3-PUCCH-Config-r19,

cb-Msg3-PUSCH-Config-r19				CB-Msg3-PUSCH-Config-r19,

cb-Msg3-TxWindow-r19			SEQUENCE {

windowSize-r19						ENUMERATED {n1, n2, n4, n8, n16, n32, n64,

n128},

windowPeriodicity-r19				ENUMERATED {n1, n2, n4, n8, n16, n32, n64,

n128}				OPTIONAL	--Need OP

}																		OPTIONAL,	--Need OP

cb-Msg3-ResponseWindow-r19				ENUMERATED {sf240, sf480, sf960,

sf1920, sf3840, sf5760, sf7680, sf10240},

cb-Msg3-MaxAttemptNum-r19				ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n10}

OPTIONAL,	--Need OP

...

}

CB-MSG3-MPDCCH-Config-r19 ::=		SEQUENCE {

mpdcch-Narrowband-r19			SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13),

mpdcch-PRB-PairsConfig-r19		SEQUENCE{

numberPRB-Pairs-r19				ENUMERATED {n2, n4, n6, spare1},

resourceBlockAssignment-r19		BIT STRING (SIZE(4))

},

mpdcch-NumRepetition-r19		ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256},

mpdcch-StartSF-CSS-r19			ENUMERATED {v1, v1dot5, v2, v2dot5, v4, v5, v8, v10},

mpdcch-Offset-CSS-r19			ENUMERATED {zero, oneEighth, oneQuarter,

threeEighth, oneHalf, fiveEighth,

threeQuarter, sevenEighth}

}

CB-Msg3-PUCCH-Config-r19 ::=		SEQUENCE {

n1PUCCH-AN-r19						INTEGER (0..2047)			OPTIONAL,	-- Need OR

pucch-NumRepetitionCE-Format1-r19	ENUMERATED {n1, n2, n4, n8} OPTIONAL	-- Need OR

}

CB-Msg3-PUSCH-Config-r19 ::=		SEQUENCE {

numRUs-r19							BIT STRING (SIZE(2)),

prb-AllocationInfoSet-r19			SEQUENCE (SIZE(1..48)) OF BIT STRING (SIZE(10)),

mcs-r19								BIT STRING (SIZE(4)),

numRepetitions-r19					BIT STRING (SIZE(3)),

p0-UE-PUSCH-r19						INTEGER (-8..7),

alpha-r19							Alpha-r12

}

PowerRampingParameters-r19 ::=			SEQUENCE {

powerRampingStep-r19					ENUMERATED {dB0, dB2, dB4, dB6},

cb-Msg3-InitialReceivedTargetPower-r19	ENUMERATED {

dBm-120, dBm-118, dBm-116, dBm-114, dBm-112,

dBm-110, dBm-108, dBm-106, dBm-104, dBm-102,

dBm-100, dBm-98, dBm-96, dBm-94,

dBm-92, dBm-90}

}

-- ASN1STOP

CB-Msg3-ConfigSIB field descriptions

alpha Parameter: αc(3). See TS 36.213 [23], clause 5.1.1.1.

cb-Msg3-ConfigList CB-Msg3-EDT configuration for each CE level applicable to a UE performing CB-Msg3-EDT. The first entry in the list is the CB-Msg3-EDT configuration for CE level 0, and the second entry in the list is the CB-Msg3-EDT configuration for CE level 1.

cb-Msg3-InitialReceivedTargetPower Initial power for CB-Msg3 transmission as specified in TS 36.321 [6]. Value in dBm. Value dBm-120 corresponds to -120 dBm, dBm-118 corresponds to -118 dBm and so on.

cb-Msg3-MaxAttemptNum The maximum number of attempts of CB-Msg3-EDT within this CE level. If the field is absent, the UE shall assume no re-attempt.

cb-Msg3-MinRSRP-Threshold Indicates the minimum RSRP threshold for initiating CB-Msg3-EDT.

cb-Msg3-NumOfReplicas Indicates the number of replicas that UE should send for CB-Msg3-EDT.

cb-Msg3-PUSCH-Config Indicates PUSCH resource for CB-Msg3-EDT. numRUs indicates DCI field for PUSCH number of resource units, see TS 36.213 [23] clause 8.1.6. prb-AllocationInfoSet contains a list of information for PRB allocation which indicates DCI field for PUSCH resource block assignment, see TS 36.212 [22], clause 5.3.3.1.10. mcs indicates DCI field for PUSCH modulation and coding scheme, see TS 36.213 [23] clause 8.6. numRepetitions indicates DCI field for PUSCH repetition number, see TS 36.213 [23] clause 8.0. numRUs set to '00' indicates use of full-PRB resource allocation, otherwise sub-PRB resource allocation as defined in TS 36.213 [23], clause 8.1.6.

cb-Msg3-ResponseWindow MPDCCH search space window duration. See TS 36.321 [6] and TS 36.213 [23]. Value in subframes. Value sf240 corresponds to 240 subframes, value sf480 corresponds to 480 subframes and so on.

cb-Msg3-RSRP-CE-Level RSRP threshold for UEs to select the configuration for CB-Msg3-EDT.

cb-Msg3-TBS Indicates the TB size threshold for initiating CB-Msg3-EDT within this CE level. Value b114 corresponds to 114 bits, b328 corresponds to 328 bits and so on.

cb-Msg3-TxWindow CB-Msg3 transmission window configuration. The start time of the CB-Msg3 transmission window is aligned with the PUSCH start time indicated by pusch-StartSFN-r19 and pusch-StartSubframe-r19. When cb-Msg3-NumOfReplicas-r19 equals 1, this field is absent. When windowPeriodicity is absent, the window periodicity uses the same value as windowSize. Value n1 corresponds to 10ms, n2 corresponds to 20ms and so on.

mpdcch-Narrowband Indicates a set of narrowbands on which the UE monitors for MPDCCH, see TS 36.213 [23], clause 9.1.5. Field values (1..maxAvailNarrowBands-r13) correspond to narrowband indices (0..maxAvailNarrowBands-r13-1) as specified in TS 36.211 [21].

mpdcch-NumRepetition Maximum number of repetitions levels for CSS for MPDCCH, see TS 36.213 [23].

mpdcch-Offset-CSS Fractional period offset of starting subframe for an MPDCCH common search space, see TS 36.213 [23].

mpdcch-PRB-PairsConfig Indicates the configuration of physical resource-block pairs used for MPDCCH. See TS 36.213 [23]. mpdcch-PRB-Pairs indicates the number of PRB pairs. Value n2 corresponds to 2 PRB pairs; n4 corresponds to 4 PRB pairs and so on. resourceBlockAssignment indicates the index to a specific combination of PRB pair for MPDCCH set. See TS 36.213 [23], clause 9.1.4.4.

mpdcch-StartSF-CSS Starting subframe configuration for an MPDCCH common search space, see TS 36.213 [23]. Value v1 corresponds to 1, value v1dot5 corresponds to 1.5, and so on.

n1PUCCH-AN Indicates PUCCH ACK resource offset, see TS 36.213 [23], clause 10.1.

p0-UE-PUSCH Parameter: P0_UE_PUSCH,c (3). See TS 36.213 [23], clause 5.1.1.1, unit dB.

powerRampingStep Power ramping factor in TS 36.321 [6]. Value in dB. Value dB0 corresponds to 0 dB, dB2 corresponds to 2 dB and so on.

pucch-NumRepetitionCE-Format1 Number of PUCCH repetitions for PUCCH format 1/1a, see TS 36.211 [21] and TS 36.213 [23]. Value n1 corresponds to 1 repetition, value n2 corresponds to 2 repetitions, and so on.

Next Change

6.7.3	NB-IoT information elements

6.7.3.2	NB-IoT Radio resource control information elements

–	CB-Msg3-ConfigSIB-NB

The IE CB-Msg3-ConfigSIB-NB is used to specify the CB-Msg3-EDT configuration.

CB-Msg3-ConfigSIB-NB information element

-- ASN1START

CB-Msg3-ConfigSIB-NB-r19 ::=		SEQUENCE {

cb-Msg3-MinRSRP-Threshold-NB-r19		NRSRP-Range-NB-r14				OPTIONAL,	--Need OR

cb-Msg3-RSRP-CE-Levels-NB-r19			CB-Msg3-RSRP-CE-Levels-NB-r19	OPTIONAL,	--Need OR

cb-Msg3-ConfigList-NB-r19				CB-Msg3-ConfigList-NB-r19,

powerRampingParameters-NB-r19			PowerRampingParameters-NB-r19

}

CB-Msg3-ConfigList-NB-r19 ::=		SEQUENCE (SIZE (1.. maxCE-Level-CB-Msg3-NB-r19)) OF

CB-Msg3-Config-NB-r19

CB-Msg3-Config-NB-r19 ::=			SEQUENCE {

cb-Msg3-TBS-NB-r19						ENUMERATED {b144, b328, b408, b504, b584, b680, b808,

b936},

cb-Msg3-NumOfReplicas-NB-r19				INTEGER (1..4),

cb-Msg3-TimeResource-NB-r19		SEQUENCE {

npusch-Periodicity-r19					ENUMERATED {ms40, ms80, ms160, ms240,

ms320, ms640, ms1280, ms2560},

npusch-StartSFN-r19						INTEGER (0..1023),

npusch-StartSubframe-r19				INTEGER (0..9)

},

cb-Msg3-PhysicalConfig-r19		SEQUENCE {

npusch-NumRUsIndex-r19					INTEGER (0..7),

npusch-NumRepetitionsIndex-r19			INTEGER (0..7),

npusch-SubCarrierSetList-r19	SEQUENCE (SIZE(1..48)) OF NPUSCH-SubCarrierSet-r19,

npusch-MCS-r19						CHOICE {

singleTone							INTEGER (0..10),

multiTone							INTEGER (0..13)

},

ack-NumRepetitions-NB-r19			ACK-NACK-NumRepetitions-NB-r13,

p0-UE-NPUSCH-r19						INTEGER (-8..7),

alpha-NB-r19							ENUMERATED {al0, al04, al05, al06,

al07, al08, al09, al1},

npdcch-CarrierIndex-r19					INTEGER (1..maxNonAnchorCarriers-NB-r14)

OPTIONAL,	-- Need OP

npdcch-NumRepetitions-r19				ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1},

npdcch-StartSF-CSS-r19					ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64},

npdcch-Offset-CSS-r19						ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

},

cb-Msg3-TxWindow-NB-r19			SEQUENCE {

windowSize-NB-r19						ENUMERATED {n8, n16, n32, n64, n128, n256,

n512, n1024},

windowPeriodicity-NB-r19				ENUMERATED {n8, n16, n32, n64, n128, n256,

n512， n1024}	OPTIONAL	--Need OP

}																		OPTIONAL,	--Need OP

cb-Msg3-ResponseWindow-NB-r19					ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32,

pp64},

cb-Msg3-MaxAttemptNum-NB-r19				ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n10} 																			OPTIONAL,	--Need OP

...

}

CB-Msg3-RSRP-CE-Levels-NB-r19 ::=	SEQUENCE (SIZE(1..2)) OF RSRP-Range

NPUSCH-SubCarrierSet-r19 ::=		CHOICE {

khz15								INTEGER (0..18),

khz3dot75							INTEGER (0..47)

}

PowerRampingParameters-NB-r19 ::=			SEQUENCE {

PowerRampingStep-NB-r19						ENUMERATED {dB0, dB2, dB4, dB6},

cb-Msg3-InitialReceivedTargetPower-NB-r19	ENUMERATED {

dBm-130, dBm-128, dBm-126, dBm-124, dBm-122,

dBm-120, dBm-118, dBm-116, dBm-114, dBm-112,

dBm-110, dBm-108, dBm-106, dBm-104, dBm-102,

dBm-100, dBm-98, dBm-96, dBm-94, dBm-92, dBm-90,

dBm-88, dBm-86, dBm-84,dBm-82, dBm-80}

}

-- ASN1STOP

End of Change