3GPP TSG-RAN WG2 Meeting #131bis	R2-2507287

Prague, Czechia, 13th – 19th Oct, 2025

Agenda item:	8.9.4

Source:	Samsung

Title:	Idle mode and capability-related corrections for IoT NTN

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#107 the latest WID for IoT NTN Phase 3 was agreed [1], where Capacity enhancements for uplink:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In this contribution, we provide some corrections on idle mode and capabilities.

# Capabilities-related correction

## Support of acceptable cell in TN/NTN

One open issue is on whether acceptable cell shall be considered support only in NTN or both in TN and NTN. In principle and according to the WID, it may be support if there is no further work required.

However, situation in TN and NTN is different and the acceptable cell camping feature is tricky. This is because current terrestrial networks are very unlikely to be able to support PWS as they would be tailored to IoT services. The tricky part is that a UE may camp on a cell as an acceptable cell without the knowledge of cell that is being camped on, or for the cells that are part of the UEs PLMN. Thus in order to support this, there would need to be a way for a network to allow for UEs to camp or not to camp as an acceptable on an NB-IoT cell. One way to allow it is for the network to indicate whether it supports PWS broadcasting or not, and if it does the UE is can camp on the cell as an acceptable cell.

Proposal 1: Acceptable cell camping on a TN cell can be allowed if there is a way of preventing UEs to camp on the cell as an acceptable cell.

Proposal 2: Introduce a network indication of support of PWS broadcasting. If indicated, a UE may camp on the cell as an acceptable cell.

# Idle mode corrections

## On S&F list use in idle mode

In the last meeting the following was agreed:

The S&F mode indication (i.e., sf-OperationMode) and the S&F mode transition time (i.e., t-ModeSwitching) of the neighbor satellite are signaled in SIB33 per neighbor satellite.

A Rel-19 UE may deprioritize the neighbor cells operating in the S&F mode (FFS on the detailed specification impact, i.e. whether a note is sufficient or some normative text. To be considered in the post meeting CR review)

And based upon this, the following note was captured:

NOTE: 	If the highest ranked cell or best cell according to absolute priority reselection rules is a cell operating in store and forward mode the UE may deprioritise or not consider this cell as candidate cell for cell reselection and may continue considering other cells not operating in store and forward mode for cell reselection.

The main issue with making the text in the note to normative text seems to be that “deprioritise” may need to be defined. However, we think that this can be resolved by clarifying that what deprioritise means is up to UE implementation.

Proposal 3: Make the NOTE “If the highest ranked cell or... “ normative text and a note that “deprioritisation” is up to UE implementation.

Proposal 4: Agree text proposal on 36.304 in Annex A.

## On paging optimization

In the last meeting, the following was discussed:

R2-2505437	Further consideration on Store and Forward	Huawei, HiSilicon, China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

Paging in S&F satellite operation

Proposal 1:An indication is introduced in the System information to indicate that the UE could skip monitoring paging and initiating RRC connection for MT data reception.

Continue the discussion offline 305

This solution is fairly simple and can thus likely be implemented quite easily. However, there are a lot of potential issues. First of all, turning “ON” the feature is quite simple, as it just an indication for UE to stop monitor paging. However, turning “OFF” the feature is likely very difficult, as the feature actually stops the UE monitoring for system information update etc. This introduces a lot of risk, even for a non-malicious network operator. If there is any need for any type of system information update for whatever reason, then there is no way to indicate this. PSM was introduced whereby a UE is indicated over NAS to sleep for a long time, but this feature is dedicatedly signalled. Thus the feature likely has to be more complicated in order to function, for instance introducing a timer for how long to not monitor paging, and there may need to be dedicated signalling to support this. We think that instead, idle mode tasks can be relaxed based on the S&F monitoring list.

Proposal 5: Do not consider paging enhancements whereby a UE stops monitoring paging based on an indication in System Information.

# Conclusion

In this contribution we discussed other corrections for IoT NTN:

Proposal 1: Acceptable cell camping on a TN cell can be allowed if there is a way of preventing UEs to camp on the cell as an acceptable cell.

Proposal 2: Introduce network indication of support of PWS broadcasting. If indicated, a UE may camp on the cell as an acceptable cell.

Proposal 3: Make the NOTE “If the highest ranked cell or... “ normative text and a note that “deprioritisation” is up to UE implementation.

Proposal 4: Agree text proposal on 36.304 in Annex A.

Proposal 5: Do not consider paging enhancements whereby a UE stops monitoring paging based on an indication in System Information.

# Reference

RP-250472, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#107, Incheon, ROK, March 2025.

R2-2502773, Report of [Post129][307][R19 IoT NTN] CB-msg3/CB-msg4 (Mediatek), Mediatek, RAN2#129bis, Wuhan, China, April 2025.

# Annex A

-------------------------------------- Based on R2-2506556 --------------------------------------

#### 5.2.4.4	Cells with cell reservations, access restrictions or unsuitable for normal camping

For the highest ranked cell (including serving cell) according to cell reselection criteria specified in clause 5.2.4.6, for the best cell according to absolute priority reselection criteria specified in clause 5.2.4.5, the UE shall check if the access is restricted according to the rules in clause 5.3.1.

If that cell and other cells have to be excluded from the candidate list, as stated in clause 5.3.1, the UE shall not consider these as candidates for cell reselection. This limitation shall be removed when the highest ranked cell changes.

If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable for a CN type due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for the CN type for a maximum of 300s. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

If the highest ranked cell or best cell according to absolute priority reselection rules is an inter-RAT cell which is not suitable due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency as candidates for reselection for a maximum of 300s. In case of UTRA further requirements are defined in the TS 25.304 [8]. In case of NR further requirements are defined in the TS 38.304 [38]. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

If the highest ranked cell or best cell according to absolute priority reselection rules is a CSG cell which is not suitable due to not being a CSG member cell, the UE shall not consider this cell as candidate for cell reselection but shall continue considering other cells on the same frequency for cell reselection.

If the highest ranked cell or best cell according to absolute priority reselection rules is a cell operating in store and forward mode the UE may deprioritise or not consider this cell as candidate cell for cell reselection and may continue considering other cells not operating in store and forward mode for cell reselection.

NOTE: 	How to deprioritise a cell in store and forward mode is up to UE implementation.

-------------------------------------- Based on R2-2506556 --------------------------------------