3GPP TSG-RAN WG2 Meeting #131bis					R2-2507297

Prague, Czech Republic, Oct. 13th-17th

Source:	Continental Automotive

Title:                       Discussion on Topology 2 for AIoT

Agenda Item:	9.2.2

Document for:	Discussion and Decision

# Introduction

According to the approved Work Item Description for Release 20 Ambient IoT Phase 2 [1], Objective 2 specifies support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions:

- According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769 (No specification support for CW control)

- According to D2T2-C for active Device(s) (Start after March 2026)

- Traffic types DO-DTT and DT, and DO-A (only for active Device(s))

- Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command)

- FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum

The following objectives are defined for RAN2 [1]:

- The objective includes specification of necessary Uu procedures to enable the A-IoT operations.

- RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios.

- A-IoT radio resource of the UE reader is only valid within the serving cell.

- Resource allocation to the UE Reader shall be via Uu RRC signaling.

This contribution discusses enhancements to RRC procedures for Radio Link Failure (RLF) recovery and handover for intermediate UE Readers operating in D2T2 configurations, addressing the above requirements and constraints.

# Discussion

2.1 Radio Link Failure Handling

In D2T2 deployments, intermediate UE Readers play a key role in enabling communication between the gNB and A-IoT devices. These Readers must maintain RRC CONNECTED state to support continuous Ambient IoT operations. However, when RLF occurs due to mobility, interference, or coverage limitations, the gNB immediately releases the allocated resources and context, resulting in service interruption and the need for full reconfiguration.

To address these challenges, we propose that the UE Reader includes a Reader-ID and a dedicated cause value (e.g., ambientIoTRLF) during RRC re-establishment. The gNB may optionally retain the A-IoT context for a limited duration using a configurable timer. If the UE reconnects within the same cell during this period, the gNB can restore the previous configuration, allowing the UE Reader to resume operations without full reconfiguration.

Proposal 1:RAN2 should specify enhanced RRC re-establishment procedures for UE Readers, including A-IoT-specific context retention and restoration mechanisms to support seamless recovery after RLF.

2.2 Handover Procedure

Handover of intermediate UE Readers currently involves RRC reconfiguration, which may interrupt ongoing A-IoT operations such as passive device scanning or data collection. This can negatively impact continuous-use applications like indoor inventory and command.

To reduce latency and improve continuity, we propose predictive, RACH-less cell switching based on mobility profiles and pre-coordinated context sharing between source and target gNBs. The UE Reader and gNB may exchange mobility profiles to anticipate movement. The source gNB forwards A-IoT context (e.g., Reader ID, session info, resource config) to the target gNB, which pre-allocates resources. The UE synchronizes with the target cell and resumes operations without full RRC reconfiguration.

Proposal 2: RAN2 should consider enhancements to handover procedures that support predictive mobility, context transfer, and fast resource activation for intermediate UE Readers, enabling seamless transitions and minimizing service disruption.

2.3 A-IoT UE Reader Capability Framework

Different UE Readers have different capabilities. Some UE Readers support only Device 1, while others support Device 2b or both. Some UE Readers can generate carrier wave, while others cannot. The number of devices that a UE Reader can handle also varies between implementations.

Different cells have different deployments. Some cells have only Device 1 deployed, some have only Device 2b, and some have both. In some cells, the network provides carrier wave. In other cells, the UE must generate carrier wave.

To support diverse Ambient IoT deployments, it is essential to define standardized mechanisms for UE Readers to report their capabilities (e.g., supported device types, waveforms, CW generation, traffic types) and for the network to inform UE Readers about cell-specific configurations. This enables efficient resource allocation, avoids operational mismatches, and ensures interoperability across vendors.

Proposal 3: RAN2 should consider specifying capability exchange procedures between UE Readers and the network to support flexible Ambient IoT deployments.

# Conclusions

This contribution presents proposals to enhance RRC re-establishment and handover procedures for intermediate UE Readers in Ambient IoT deployments. These enhancements aim to improve service continuity, reduce signaling overhead, and support reliable operation in mobile scenarios, in alignment with the objectives of the Release 20 WID. Additionally, we request RAN2 to discuss the following proposals:

Proposal 1: RAN2 should specify enhanced RRC re-establishment procedures for UE Readers, including A-IoT-specific context retention and restoration mechanisms to support seamless recovery after RLF.

Proposal 2: RAN2 should consider enhancements to handover procedures that support predictive mobility, context transfer, and fast resource activation for intermediate UE Readers, enabling seamless transitions and minimizing service disruption.

Proposal 3: RAN2 should consider specifying capability exchange procedures between UE Readers and the network to support flexible Ambient IoT deployments.

# References

[1] RP-252894: Work Item Description for Ambient IoT Phase 2