3GPP TSG-RAN WG2 Meeting #131bis	R2-2507335

Prague, Czech Republic, Oct. 13th-17th

Agenda item:		10.3.3

Source:	Rakuten

Title:	Data Plane for AI-ML data collection

Document for:		Discussion and Decision

# Introduction

The SID of Study on 6G Radio in RAN #108 includes the following objectives [1]

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

The highlighted text mentions about the data collection for AI-ML use-cases and sensing data from the UEs/devices and network support to transfer this data. In this contribution, we present our analysis and views regarding a new data plane for transfer of various new types of data including AI-ML data and sensing related data for 6GR.

# Discussion

## Key issues associated with AI-ML, Sensing data

In our understanding, some of the key issues associated with data collection and data transfer over the air-interface for AI/ML, sensing, etc are listed below.

- The information transfer involves huge data volume, as it encompasses multiple use-cases associated with thousands of UEs/devices.

- Need of flexible data routing (e.g. UE-RAN, UE-CN), as the data could be consumed at RAN and/or CN depending on the use-case and on-demand processing (e.g. compression)

- Data reuse is a key requirement to avoid duplicated data collection from UE/NF, as UEs keep moving from one NG-RAN node to another.

## Existing framework for AI-ML data transfer

The Control Plane (SRBs) has clear bottleneck for carrying AI/ML data with limited segmentation, as such data may be too frequent/large. The UE is only required to support 45kB RRC buffer size, according to TS 38.306, while the dataset/parameters are in the order of hundreds of MBs. The User Plane (DRB) is unsuitable for carrying AI/ML data, due to the difference in characteristics of data handled in UP and the need to process/consume the data at nodes within RAN (eg: at DU, CU-CP and CU-UP). Moreover, the AI/ML data may have to be terminated at RAN or CN depending on the use-case.

The priority of AL/ML data can be treated differently from Control Plane data and User Plane data, i.e., if Control Plane data is missed, the network and UEs cannot exchange User Plane data effectively or even the UEs being disconnected from the network; some User Plane data becomes useless if the end-to-end delay is long, while the time to arrive at the destination for majority of AL/ML data can be with less stringent delay requirements. The priority of CP, UP and AI-ML data are inherently different and is very important information for scheduler, which is responsible to schedule transmissions with limited radio resources (bandwidth, power and number of antennas).

Therefore, a new mechanism is required to transfer operational data including AI/ML data collection, sensing data, etc between UE and the network.

## Data Plane DRBs

A new Data plane Data Radio Bearer (DP-DRB) (different from the control plane SRBs and user plane DRBs) could be a way-forward to address the above issues.

- A DP-DRB can be configured between UE  RAN and/or UE  CN. i.e a DP-DRB can be terminated either at RAN or at CN, based on the use-case and requirement.

- A DP-DRB can be configured for AI-ML data collection and training, transfer of sensing data to/from sensing equipment.

Figure 1: Exemplary diagram illustrating Data Plane DRB

Study the introduction of a new Data Plane DRB for the purpose of AI-ML data collection over the air interface and transfer of sensing data to/from sensing equipment.

# Conclusion and Proposals

Based on the discussion above, we conclude this contribution with the following observations and proposals:

Proposal 1		Study the introduction of a new Data Plane DRB for the purpose of AI-ML data collection over the air interface and transfer of sensing data to/from sensing equipment.

# References

- RP-251881. (n.d.). New SID: Study on 6G Radio. NTT Docomo.