3GPP TSG-RAN WG2 Meeting #131bis			         R2-2507341

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	10.3.2

Source: 	LG Electronics Inc.

Title:         	Consideration on control plane aspects for 6G

Document for: 	Discussion and Decision

Introduction

In this contribution, we examine key pain points in NR control plane procedures and, based on these observations, share our initial views on 6G control plane design, aiming to enable flexible on-demand mechanisms, energy-efficient operations, and enhanced support for diverse UE types.

Discussion

On-demand system information & SSB transmission

The on-demand transmission mechanism for system information was introduced in NR to reduced broadcast overhead. However, only UE’s request procedure was specified to achieve it, while many aspects still followed the legacy SI transmission mechanism. As a result, the on-demand approach has not been able to fully provide its potential benefits.

Pain Point of NR OD #1: Lack of integration between OD-SIB1 operation and OD-SI operation

In NR, on-demand SIB1 has been specified after on-demand SI. Thus, on-demand SIB1 operation and on-demand SI operation are not so well integrated. For instance, UE can know the time window when OD-SIB1 is transmitted while UE cannot for OD-SI. Besides, there is the case when SIB1 is not broadcast while SI is broadcast. However, necessity of this case is not clear. The case seems eventually there and became something to be handled in the specification due to separate standard work in different releases.

Considering on-demand SSB is also on the table for 6G discussion, we can discuss a single framework to efficiently support on-demand SSB/SIB1/SI. The transmissions, procedures and RACH resources for on-demand SSB/SIB1/SI can be handled based on a unified approach.

Pain Point of NR OD #2: Delayed SI reception & NES constraints due to the semi-static scheduling

In NR, the transmission of a requested SI message is constrained by pre-configured SI window. After a UE requests a specific SI message, the network can only schedule its transmission within the next available SI window associated with the SI message. This fixed scheduling approach forces the UE to wait for the start of the next SI window, resulting in an avoidable latency. This limitation constrains the usage of the system information, which is critical for future latency-sensitive features.

The fixed SI windows also impose a significant constraint on network energy saving. They force the network to remain active during the pre-defined SI windows, preventing the full use of energy-saving mechanisms like Cell Discontinuous Transmission. A more flexible 6G framework should support dynamically scheduled SI transmissions. This would not only reduce UE latency by decoupling SI delivery from fixed SI window but also enable the network to more effectively implement energy saving modes, aligning with 6G efficiency goals.

Pain Point of NR OD #3: Signalling inefficiency due to lack of delta configuration support

The current NR on-demand SI mechanism forces the network to transmit the entire SI message, even when only a small portion has changed. This leads to redundant signalling overhead, as large amounts of unchanged information are repeatedly delivered. This inefficiency is particularly problematic in scenarios requiring frequent updates to specific parameters, such as paging/measurement related configuration for network energy saving. A more efficient 6G approach should move beyond this "all-or-nothing" signalling and allow for the transmission of only the modified information. This would significantly reduce downlink overhead, conserve radio resources, and shorten the time UEs need to decode and apply updates.

Pain Point of NR OD#4: Lack of network energy efficiency for SSB/PBCH transmissions

In NR, UE assumes 20 msec as default synchronization signal (SS) periodicity during initial access procedure. The NR study on NES in TR 38.864 showed that substantial NES gains (up to 84 %) can be achieved by adopting longer SS periodicity. Therefore, it would be beneficial in terms of NES if default SS periodicity can be increased to the value greater than 20 msec for 6G.

However, even if NW transmits SS with a longer periodicity (e.g., 160 msec), UE may require frequent SS transmission in some cases. For instance, when UE attempts to access a cell, or finds a new beam during a beam recovery procedure, it can be more efficient for UE to receive SS with a shorter periodicity (e.g., 20 msec). Depending on whether NW is aware of such UE’s situation, NW-initiated (similar to Rel-19 OD-SSB) or UE-initiated on-demand SS (or SS periodicity adaptation) can be considered.

Alternatively, SS can be separately transmitted from PBCH. For instance, SS-burst (which consists of different SS beams) is periodically transmitted while PBCH-burst (which consists only of PBCH with different beams) can be transmitted in response to UE’s request (i.e., on-demand manner). For another instance, SS-burst is transmitted with a shorter periodicity (e.g., 20 msec) while PBCH-burst is transmitted with a longer periodicity (e.g., 160 msec). It is noted that NES gains can be obtained by reducing PBCH transmissions.

Pain Point of NR OD#5: Resource overhead due to cell-specific always-on sync signals and common channels for idle/inactive UEs

Meanwhile, as described in [1], we think that 6GR needs to be designed to support not only conventional cellular scenarios but also new potential 6G scenarios such as e.g. multi-carrier single/multi-cell, intra-frequency cell-cluster massive MTRP and MRSS scenarios. Considering new potential 6G scenarios, we can study how to efficiently support common channels applicable to new scenarios.

For example, 6GR can be deployed on a multi-carrier single cell where normal SIB1 can be periodically transmitted on a primary carrier while on-demand SIB1 may be provided on a secondary carrier for better spectrum utilization on wide channel bandwidth with diverse device types. Particularly, considering that SIB1 size is limited based on coverage target, on-demand SIB1 may not need carry all information elements for diverse device types. For instance, RedCap specific common channel configuration can be sent on a secondary carrier (e.g. RedCap specific initial BWP) only based on request by RedCap UEs while non-RedCap specific common channel configuration does not need to be sent to RedCap UEs.

In addition, multi-cell deployment over multi-carriers in NR demands cell-specific always-on overhead signals on each carrier which inefficiently consumes periodic resources and network energy. This pain point can be alleviated by on-demand SSB/SIB1/SI. For example, a cell-defining SSB can be periodically transmitted on the sparse sync raster as a primary carrier while on-demand SSB can be provided on off-sync raster as a secondary carrier for better spectrum utilization on wide channel bandwidth with diverse device types and NES.

Proposal 1	Study an adaptation for SS/PBCH and a unified approach for on-demand SSB/SIB1/SI.

Proposal 2	Study enhanced on-demand SI, focusing on improving signalling efficiency (particularly for SI update) and reducing SI reception latency, e.g. by enabling optimized SI delivery and flexible scheduling that also support more flexible network energy saving.

NTN SI acquisition

In NTN, it takes long time to accomplish the initial access due to long propagation delay. Additionally, the satellites move fast along their orbits, the time that UE stays in the particular beam coverage of a satellite is short. To successfully complete a connection within the satellite’s visibility, the UE has to complete quickly the initial connection setup procedure. However, NR NTN didn’t consider fast initial access specially. In 6GR NTN, as the new service scenarios, e.g., voice call or emergency alert, etc., are emerged, fast initial access should be addressed. In NR NTN, the UE can start the initial access procedure after receiving SS/PBCH, SIB1, and SIB19. Specifically, to obtain NTN configuration including the ephemeris information and common TA information for PRACH transmission, the UE needs to wait to receive SIB19 while the SIB19 can be received after obtaining the SI scheduling information from SIB1. Moreover, since the SI scheduling information is provided by the RRC message, it will take some time (e.g., few or few tens of msec) for the UE to acquire where the UE tries to detect the SIB19. It implies that the UE may not receive the NTN configuration right after the SIB1 reception. Considering that the SS/PBCH and common channel periodicities could be increased due to the limited active beam ratio at the satellite, this kind of steps would cause additional latency for the reception of the NTN configurations. Before receiving the NTN configuration, the UE cannot pre-compensate the TO/FO due to the satellite moving, so it cannot start the PRACH transmission. Also, the UE needs to pre-compensate the timing drift and the Doppler drift based on the received SS/PBCH. Moreover, since the satellites (especially for LEO) move fast along their orbits, it is important to ensure that the UE completes the initial connection setup procedure within the satellite’s visibility. Otherwise, the UE may need to restart the DL synchronization by detecting SS/PBCH from the target satellite since the information related to the satellite switching is currently included in SIB19. Considering that the VLEO scenario could be further taken into account in 6GR, it would be necessary to carefully treat these cases.

In 5G NR, depending on the system information change notifications and valueTag in SIB1, the UE can determine whether to consider the stored SIB as valid for the cell or try to receive the updated SIB for the cell. However, due to the different validity timeline of the NTN configurations, such mechanism cannot be used for the NTN configurations. Instead, for the NTN configurations, another timer (e.g., ntn-UlSyncValidityDuration) is used for the UE to reacquire the new NTN configurations. According to the NR NTN, UE should attempt to re-acquire SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime. Since the SIB19 is not distinguishable from other SIBs (due to the same SI-RNTI), the UE would try to receive SIBs other than SIB19 even though the UE does not need to do based on the SI change notification. In short, the existing 5G NR SI reacquisition may not work properly or be inefficient in NTN scenario. In 6GR, it would be necessary to carefully investigate the SI (re)acquisition by taking the NTN characteristics into account.

Pain Points of NR NTN SI acquisition

- The system information required for initial access to NR NTN is defined separately from that for TN and is received within an independent time window, which results in additional latency and UE energy consumption.

Proposal 3	Study mechanism for (re)acquisition of NTN-related SI to reduce initial access latency and/or improve UE energy saving under NTN operation.

Energy-saving friendly Paging mechanism

In NR, various enhancements, e.g. OD-SI, SIB1 and SSB, have been introduced to reduce always-on signaling overhead. By contrast, paging has mainly enhanced to improve UE power saving, e.g. Paging Early Indication and Low Power Wake-Up Signal. Though a minor enhancement was specified for network energy saving in release-19, its effectiveness remains very limited due do the need to maintain support for legacy UEs.

Pain Point of NR Paging #1: Limited flexibility due to static PO configuration

All configuration required for paging reception is provided through SIB1, including even the parameters introduced for PO adaptation in Release 19. Since SIB1 carries information that must be updated simultaneously by all UEs, its update is only allowed at predefined instances, i.e., the start of the next modification period. Consequently, SIB1 is not well suited for (re-)configuring parameters such as paging related, which requires more flexible and timely optimization. This restricts the network’s ability to optimize the number and distribution of paging occasions in response to real-time paging load. A more flexible mechanism would allow adaptive configuration of paging occasions, thereby minimizing the network’s active time for paging transmission.

Pain Point of NR Paging #2: Two different paging configurations for legacy NR UEs and Rel-19 NES UEs

Moreover, Rel-19 NES introduced paging adaptation in time domain. In NR, since legacy NR UEs and Rel-19 NES UEs coexist, NW may need to individually configure two different paging configurations where one is for legacy NR UEs and the other is for Rel-19 NES UEs, which is clearly inefficient from NW operational point of view. Therefore, more flexible paging configuration for paging frame (PF) and for paging occasion (PO) than Rel-15 needs to be considered from 6G Day-1. For instance, PFs/POs can be allocated in a compressed manner in time domain, instead of evenly distributed PFs in time domain.

Pain Point of NR Paging #3: NW energy consumption and resource inefficiency for paging transmissions with all beams over multiple cells of a large area and coarse-grained UE location knowledge from network side

Paging DCI/PDSCHs normally happen over multiple cells of a large area, but UE will respond to the paging only for a single cell. In addition, paging DCI/PDSCHs are always broadcast with all beams over multiple cells of a large area. From network energy saving perspective and resource efficiency perspective, such legacy operation is not so a proper way to transmit paging.

Considering this pain point, paging transmissions can be based on request by UE actually present at a cell for network energy saving. In addition, paging can be transmitted only with specific beams corresponding to PRACH on the request to further reduce NW energy consumption for paging transmission and improve resource efficiency.

In NR, mechanisms such as PEI and LP-WUS have been introduced for the purpose of UE power saving. These allow the network to notify UEs whether paging exists for a given paging group, thereby avoiding unnecessary wake-ups and improving UE power saving. Despite these enhancements, each cell has no knowledge of which paging-group UEs are camped on it. Consequently, paging messages for all paging groups must be transmitted blindly from all cells, regardless of the actual presence of UEs. This results in significant inefficiency and unnecessary signalling overhead.

To address this limitation, 6G should consider new mechanisms that enable the network to more efficiently identify the presence and location of UEs. Such mechanisms would allow the network to minimize redundant paging transmissions while preserving UE power-saving benefits. However, achieving this goal may require UEs to report their presence or location more frequently. While this could enable more optimized paging transmission, it inevitably increases UE power consumption. Network energy saving should not come at the cost of excessive UE power consumption, and the impact on UEs should be treated as a critical consideration when specifying any paging-related enhancements.

Proposal 4	Study energy-saving friendly paging mechanisms for 6G, focusing on improving flexibility in paging occasion configuration/adaptation and enabling finer-grained UE presence awareness for on-demand paging to reduce unnecessary paging transmissions.

Decoupling paging reception cell and access execution cell

In NR, paging reception and random access execution are tightly coupled within a single serving cell. A UE in RRC_IDLE receives a paging message from its serving cell and is then expected to initiate random access in that same cell. While simple, this design creates inefficiencies in multi-frequency deployments. When multiple cells across different frequencies are available for camping, the network must broadcast paging across all of them to guarantee UE reachability. Conversely, if paging transmission were restricted to a single frequency, UEs would converge on that frequency for paging, leading to access congestion and unbalanced resource utilization.

Pain Points of coupling of paging reception cell and access execution cell:

- Redundant paging transmission across all candidate frequencies to guarantee UE reachability.

- Restricting paging to a limited set of frequencies leads to access congestion on those frequencies.

To overcome this limitation, 6G could decouple the paging reception cell from the access execution cell. In this design, a UE would receive paging on one frequency but be able to perform random access on a different cell operating on another frequency. This allows the network to reduce paging transmissions, limiting them to M out of N frequencies, thereby saving radio resources and network energy. At the same time, access attempts can remain distributed across multiple frequencies, preventing overload on a single carrier and improving overall system stability.

A key challenge in this approach is avoiding access delays. If a UE searches for an access cell on inter-frequency only after receiving paging or generating MO data, the access procedure would be delayed, compared to the legacy design. To prevent this, the UE should pre-identify candidate access cells and pre-acquire the required system information in advance. With this preparation, the UE can immediately initiate access on the selected cell, even if it differs from the paging reception cell. This enhanced UE behaviour improves paging efficiency, reduces signalling overhead, and supports balanced access distribution across frequencies. It also enables more flexible scheduling of system information.

Proposal 5	Study de-coupling of paging reception and access execution across different frequencies to allow flexible paging transmission across multiple frequencies while supporting distributed and low-latency access across multiple frequencies.

RRC connection establishment

In RAN#109, RAN approved the requirement that 6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR. We think that it is beneficial to improve initial access mechanisms in multi-carrier deployment and MRSS deployment.

Pain Point of NR IA #1: Lack of load distribution with NR initial access mechanisms in multi-carrier deployment and MRSS deployment

In NR, in multi-carrier deployment where inter-frequency cells are overlaid, UE selects one frequency based on reselection priority given by the network and then performs initial access to a cell on that frequency. With this operation, the network cannot move this UE to another frequency based on varying load situation for better distribution over multi-carriers unless the UE completes RRC connection establishment. This situation can also happen in multi-RAT deployment. We think that this pain point can be alleviated based on enhancement to initial access assuming that NR cell and 6GR cell in MRSS or multi-carriers can be well coordinated.

Pain Point of NR IA #2: Lack of early setup for spectrum aggregation and MTRP for fast/reliable data transmission/reception leading to releasing/suspending the immediate RRC connection afterwards.

Early SCell setup has been specified to accelerate use of a SCell for faster user data transmission/reception leading to a short stay of a RRC connection, which seems beneficial for efficient management of network energy and UE power by releasing/suspending the RRC connection immediately after data transmission/reception. This is obviously based on CA. If multi-carrier single cell deployment is considered in 6G, this mechanism needs to be extended for non-CA based spectrum utilization.

Meanwhile, multi-TRP operation has been introduced in NR to enhance reliability and robustness and increase throughput and capacity. However, MTRP can be set up only after complete of RRC connection establishment. This delay prevents the network from using advanced MIMO techniques for initial data transmission. Thus, early MTRP setup can be studied for fast reliable data transmission/reception leading to releasing/suspending the RRC connection immediately after data transmission/reception.

In addition, by allowing UEs in RRC_IDLE/INACTIVE to better interact with multiple TRPs in a cluster, we can extend cell coverage and provide a scalable cell deployed with multiple TRPs for robust initial access transmissions as well as reliable data transmissions afterwards. The TRPs can belong to a same cell or different cells depending on network implementation.

We think that 6GR can allow UEs in RRC_IDLE/INACTIVE to manage measurements and common channels for multiple TRPs, in order to reduce cell change/reselection at a boundary of a single TRP. In addition, considering that aggregating multiple TRPs can lead to reliable data transmission possibly with a higher overall data transmission rate, we can study early setup of MTRP during initial access or right after initial access which can reduce the time it takes to configure multiple TRPs, leading to faster user data transmission/reception with higher reliability and data rate for UE and a short stay of a RRC connection, which is beneficial for efficient management of network energy and UE power by releasing/suspending the RRC connection immediately data transmission/reception.

Moreover, early multi-carrier setup for initial access, e.g. fast SCell setup in NR, can be also considered in multi-carrier scenarios. Both early MTRP setup and multi-carrier setup can provide faster user data transmission/reception and will lead to a short stay of a RRC connection, which is beneficial for efficient management of network energy and UE power by releasing/suspending the RRC connection immediately data transmission/reception.

Furthermore, one of new deployments considered for 6G is MRSS. Considering that one of MRSS options is to share SSB/PBCH between 5G and 6G cell, we can study how to select 5G cell and 6G cell on the same frequency for initial access in this MRSS deployment.

Proposal 6 	Study cell access and initial access for better distribution across different carriers and RATs and fast/reliable data transmissions, with extension to new potential scenarios such a single/multi cell multi-carrier scenario, intra-frequency cell-cluster massive MIMO scenario, MRSS.

Fine-grained Access Control

The Unified Access Control (UAC) in NR does not fully reflect the actual usage patterns of modern smartphones. For example, the same application may run in the foreground at one moment and in the background at another, but NR UAC handles both cases identically. In addition, UAC does not take into account detailed traffic attributes, such as data volume, which could be used to prioritize small, time critical transmissions when the network is congested.

Pain Points of NR UAC

- NR UAC does not account for realistic usage patterns in modern smartphones (e.g., cannot distinguish between foreground and background operation of the same application).

- Lack of consideration for detailed traffic attributes (e.g., data volume) limits the efficiency of scheduling under high network load.

For 6G, access control should be enhanced to better match real smartphone usage and detailed traffic attributes. By adapting access control policies to the actual traffic attributes, the network can improve fairness, reduce access congestion, and allocate resources more efficiently to meet UE needs, eventually enhancing both system performance and user experiences.

Proposal 7	Study fine-grained Access Control capable of differentiating foreground and background applications, and reflecting detailed traffic attributes (data volume, etc.).

Conclusion

Paint points of NR

On-demand SI

- Lack of integration between OD-SIB1 operation and OD-SI operation

- Delayed SI reception & NES constraints due to the semi-static scheduling

- Signalling inefficiency due to lack of delta configuration support

- Lack of network energy efficiency for SSB/PBCH transmissions

- Resource overhead due to cell-specific always-on sync signals and common channels for idle/inactive UEs

NTN SI acquisition

- The system information required for initial access to NR NTN is defined separately from that for TN and is received within an independent time window, which results in additional latency and UE energy consumption.

Paging

- Limited flexibility due to static PO configuration

- Two different paging configurations for legacy NR UEs and Rel-19 NES UEs

- NW energy consumption and resource inefficiency for paging transmissions with all beams over multiple cells of a large area and coarse-grained UE location knowledge from network side

Coupling of paging reception cell and access execution cell in NR

- Redundant paging transmission across all candidate frequencies to guarantee UE reachability.

- Restricting paging to a limited set of frequencies leads to access congestion on those frequencies.

Initial access

- Lack of load distribution with NR initial access mechanisms in multi-carrier deployment and MRSS deployment

- Lack of early setup for spectrum aggregation and MTRP for fast/reliable data transmission/reception leading to releasing/suspending the immediate RRC connection afterwards.

Access control

- NR UAC does not account for realistic usage patterns in modern smartphones (e.g., cannot distinguish between foreground and background operation of the same application).

- Lack of consideration for detailed traffic attributes (e.g., data volume) limits the efficiency of scheduling under high network load.

Proposal

Proposal 1	Study an adaptation for SS/PBCH and a unified approach for on-demand SSB/SIB1/SI.

Proposal 2	Study enhanced on-demand SI, focusing on improving signalling efficiency (particularly for SI update) and reducing SI reception latency, e.g. by enabling optimized SI delivery and flexible scheduling that also support more flexible network energy saving.

Proposal 3	Study mechanism for (re)acquisition of NTN-related SI to reduce initial access latency and/or improve UE energy saving under NTN operation.

Proposal 4	Study energy-saving friendly paging mechanisms, focusing on improving flexibility in paging occasion configuration/adaptation and enabling finer-grained UE presence awareness for on-demand paging to reduce unnecessary paging transmissions.

Proposal 5	Study de-coupling of paging reception and access execution across different frequencies to allow flexible paging transmission across multiple frequencies while supporting distributed and low-latency access across multiple frequencies.

Proposal 6 	Study cell access and initial access for better distribution across different carriers and RATs and fast/reliable data transmissions, with extension to new potential scenarios such a single/multi cell multi-carrier scenario, intra-frequency cell-cluster massive MIMO scenario, MRSS.

Proposal 7	Study fine-grained Access Control capable of differentiating foreground and background applications, and reflecting detailed traffic attributes (data volume, etc.).

Reference

R2-2507141 Consideration on general aspects for 6G		LG Electronics Inc