3GPP TSG RAN WG2 #131-bis	R2-2507361

Prague, Czechia, 13th – 17th October 2025

Agenda item	: 10.2

Source	: Jio Platforms Limited

Title	: Single Frequency HetNet Deployment Scenario for 6GR

Document for	: Discussion

# Introduction

The new SID “Study on 6G Radio” [1] has been agreed in RAN#108 and revised [2] in RAN#109 meeting and objective 1 of the study item is as shown below:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

In this document, we introduce a single frequency HetNet scenario for 6GR.

# Discussion

3GPP had introduced HetNet framework during LTE time frame and further it was extended to HetNet dual connectivity as well as carrier aggregation in 3GPP release 12. However, 5G NR does not specify about HetNet till release 19 study items although there is clear need of HetNet deployment in NR.

Heterogeneous Network (HetNet) consist of various types of cells including Macro, Micro, Pico and indoor small cells such as residential and enterprise cells deployed together and operating in the same spectrum. As the frequency increases in 6G, the penetration loss increases that pose a potential challenge in providing indoor customer experience with the Homogeneous network of Macro cells. More than 80% data consumption comes from the Indoors and therefore, it is important to have a better 6G Indoor coverage with the help of HetNet. A typical HetNet deployment scenario with single frequency is illustrated in Figure 1 below.

Figure 1: Concept of HetNet

Figure 1. The figure shows a heterogeneous cellular network topology with single frequency incorporating different forms of small cell deployments under the umbrella coverage of Macro cell network.

Heterogeneous Networks (HetNets) deliver three key benefits: (a) Localised indoor coverage and (b) Increase indoor customer experience and c) Increase the capacity of access network. By integrating small cells with existing networks, HetNet focus high-speed, high-capacity bandwidth on specific hotspot areas. This targeted approach is ideal for dense urban environments and business districts where reliable service is necessary to support a growing array of data-heavy applications. In short, HetNets can achieve a multi-fold benefit of increase network capacity and customer experience when compared to traditional homogeneous network architectures of Macro cells alone. As 6GR is being studied and there is an opportunity to introduce single frequency HetNet framework.

As 6GR is being studied and there is an opportunity to introduce single frequency HetNet framework.

Table 1: Recommended attributes for the new scenario of Intra Frequency heterogeneous network

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 7 GHz Around 30 GHz |
| Aggregated system bandwidth | Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: 1 GHz (DL+UL) |
| Layout | Single layer ( Single Frequency ): - Macro or Small cells same frequency layer |
| Other attributes, e.g., attributes related to BS and UE | Same as the value for basic deployment scenarios |

For 6GR deployment scenario, a new deployment scenario of single frequency heterogeneous network should be introduced, to improve the capacity, coverage and cell-edge performance.

In such case, the recommended attributes assumptions listed in Table 1 should be supported.

While HetNet provides better capacity and coverage but one of the major challenges is Inter cell Interference due to reference signal overlap. In HetNet deployment there will be an inter cell interference at small cell due to Reference Signal (RS) transmitted by the 6G Macro cells.

In HetNet, Macro cells and small cells use the same time-frequency spectrum resources. Both type of cells generally transmits reference signals like Channel state information RS, Synchronization signal (SSB), Tracking reference signal and position reference signal in overlap region of time frequency resources. This can lead to reference signal interference in HetNet deployment where macro and small cells are from different vendors and both are having their own implementation. Such kind of interference degrades the SINR, CQI and throughput that reduces the customer experience.

In current 5G NR deployment where Massive MIMO of 32T32R high power radio and low power small cells of 4T4R are deployed in overlap coverage. Figure 2 shows a typical HetNet deployment diagram below.

Figure 2: HetNet deployment scenario

Two types of UEs – one is connected to Macro, and other is connected to small cell at the same 	RSRP level of -85 dBm as shown in the Figure 2. Due to significant power difference between Macro and small cell, although UE receives the similar signal level in downlink, the UE attached to small cell gets interference in uplink by the UE attached to Macro cell (by 22 dB as shown in the Figure 2). Therefore, all UEs are at either the cell edge or mid-cell of Macro will create an Uplink interference to the UE attached to the small cells.

Small cell UEs suffer heavy uplink interference due to Macro cells UEs due to significant UE Tx power of both types of UEs. This interference does not occur in homogenous network deployment where there is no path loss difference ideally.

Figure 3 below shows one more example of downlink interference for UE attached to small cell because of reference signal of Macro cells. Figure 3 shows the reference signals of Macro and small cells.

| Slot | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Macro Cell |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Small Cell |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | TRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CSI RS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 32 port CSI RS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 3: Interference by small cell UE because of Reference signal by Macro cell

As it is shown in Figure 3, whenever Macro cell transmits reference signals, small cell UEs will get interfered in the downlink.

Since DL Reference signal are periodic in nature generally and in dense environment of HetNet deployment overlapping RS degrade UE channel estimation and throughput. In NR, the situation is improved by making RS more configurable and on-demand, but interference is still possible in SSB and CSI-RS without proper coordination. The reasons for reference signal interference are as follows:

- Co-channel Deployment: Macros and small cells using the same carrier frequency.

- Dense Small-Cell Layers: Higher density increases the likelihood of overlapping RS resources.

- Inadequate PCI/SSB Planning: Conflicting reference signal patterns.

- High Macro Transmit Power: Strong CRS/SSB signals penetrate small cell coverage areas.

In LTE, to avoid this type of HetNet interference, below features were introduced:

- Enhanced Inter cell interference (eICIC): This feature is introduced in release 10 to mitigate the HetNet interference on data as well as control channels. It uses the ABS frame information (almost blank subframe) where macro cells do not transmit anything, and the same information is conveyed to small cells via X2 interface. Based on this small cell user can be scheduled in these ABS subframes.

- Co-ordinated Multi Point (CoMP): It provides the feasibility to transmit or receive data across multiple base stations in coordinated way which enhances cell edge user performance. CoMP supports coordinated scheduling and joint processing for multiple cells.

Possible reference signal interference mitigation solutions should be studied in 6GR.

RAN2 should send LS to RAN3 to discuss the possibility of including the interference avoidance procedures in 6GR single frequency HetNet scenario.

# Conclusion

Observation 1: As 6GR is being studied and there is an opportunity to introduce single frequency HetNet framework.

Proposal 1: For 6GR deployment scenario, a new deployment scenario of single frequency heterogeneous network should be introduced, to improve the capacity, coverage and cell-edge performance.

Observation 2: Small cell UEs suffer heavy uplink interference due to Macro cells UEs due to significant UE Tx power of both types of UEs. This interference does not occur in homogenous network deployment where there is no path loss difference ideally.

Proposal 2: Possible reference signal interference mitigation solutions should be studied in 6GR.

Proposal 3: RAN2 should send LS to RAN3 to discuss the possibility of including the interference avoidance procedures in 6GR single frequency HetNet scenario.

# References

3GPP RAN#108, RP-251881, New SID: Study on 6G Radio

3GPP RAN#109, RP-252912, Revised SID: Study on 6G Radio