3GPP TSG-RAN WG2 Meeting #131-bis	R2-2507389

Prague, CZ, 13 - 17 October 2025

Agenda item	: 10.3.1

Source	: Tejas Networks

Title	: Discussion on user plane functions

Document for	: Discussion & Decision

# Introduction

In this contribution, we will point out a few issues related to user plane functionalities for both TN and NTN deployments.

# 2 User Plane for Terrestrial & Non-Terrestrial Network

## 2.1 MAC

It will be sooner than later that 6G NTN will be expected to carry application data riding on the TCP protocol. Support for TCP protocol becomes a challenge with GEO satellite deployments which will have delays of the order 500-600ms which can cause performance issues for the TCP protocol performance which are well known. Though, solutions exist which handle these performance issue at the application layer what 6G should stive is to design solution(s) whose scope is within the RAN.

Consider the performance of a TCP application deployed over Non-Terrestrial Network provisioned by Geo Synchronous Orbit (GEO) Satellite. Consider HARQ-based transmission/retransmission of MAC PDUs and assume that a MAC PDU is lost on the air interface. Considering the long RTT and assuming at most one retransmission is needed, it would be well above 1000ms by the time source TCP application received the acknowledgement. Given the way TCP protocol is designed, source TCP application brings the transmission rate down to zero upon Retransmission Time-Out (RTO) and enters additive increase phase. Further, it takes time for the source TCP application to evolve post-packet-loss transmission rate to the value where packet loss is detected. As a consequence, TCP rate will never grow beyond a rate of few bytes/seconds if the losses persist.

Thus, 6GR study item should focus on addressing this issue early on.

Proposal 1: 6GR study item should study techniques that make TCP performance over NTN links is close to performance achievable on terrestrial links.

## 2.2 RLC

4G specification allowed both segmentation and concatenation of RLC SDUs to match resource assignment made by the scheduler. However, concatenation is possible only after the number of PRBs available for transmission becomes known leading to delay in composing the RLC PDU with concatenation. The concatenation of RLC SDUs is discontinued in 5G to reduce the delay incurred in the formation of MAC PDU and this was mainly motivated by the ultra-low-latency requirement of the URLLC service. However, we must note that RLC SDU concatenation still happens but at the MAC layer through the corresponding MAC SDUs leading to increased control overhead.

This tradeoff between increased control overhead and the delay reduction was considered important in the case of URLCC service. However, this tradeoff may not be needed for other services like eMBB and can be left skewed in favor of reducing the control overhead.

The above observation motivates us to ask more pertinent question as to whether user plane functionalities should be tailored to suit a particular service instead of adopting one-size-fits-all idea.

Proposal 2: User plane functionalities should be tailored to a particular service as different services are expected to provide different performance.

# Conclusion

Proposal 1: 6GR study item should study techniques that make TCP performance over NTN links is close to performance achievable on terrestrial links.

Proposal 2: User plane functionalities should be tailored to a particular service as different services are expected to provide different performance.