3GPP TSG-RAN WG2 Meeting #131bis		                                    R2-2507393

Prague, Czech Republic, 13th – 17th Oct. 2025

Source:	TCL

Title:	Discussion on general aspects for 6GR

Agenda item:	10.2

Document for:	Discussion and decision

- Introduction

This contribution discusses several fundamental aspects of 6GR, including overall design principles, support for new service, high-level architecture, and the user equipment (UE) capability framework. Based on the emerging requirements and use cases being identified by 3GPP SA1 in TR 22.870 [1] and the objectives outlined in the newest Study on 6G Radio [3], the aim is to present a coherent overview of these topics and provide technical rationale along with initial proposals for 6GR design.

- Discussion

  - Design principles for 6GR

As 6G requirements crystallize, two key design principles are identified for 6GR architecture and protocol development: Modular design and On-demand functionality. These principles will help ensure the 6GR is flexible, scalable, and forward-compatible to accommodate a broad range of use cases and device types while avoiding unnecessary complexity.

Modular Architecture and Protocol Design

The 6GR is expected to support a diverse set of services, types of devices and features (from traditional mobile broadband to new capabilities like integrated sensing and AI). A monolithic design would become unwieldy and inflexible in the face of such diversity. There is a need to structure the 6GR architecture and protocols into well-defined, self-contained modules. Each functional component (or protocol sublayer) should be as decoupled as possible, enabling independent development, evolution, and deployment. This modular approach would allow new functionalities to be introduced or updated with minimal impact on existing ones. On other hand, a modular 6GR enables service decoupling and reusability, dovetails with initiatives to define network functions as services and adopts service-based interfaces and programmability more easily. In summary, a modular architecture and protocol design is proposed as a guiding principle for 6GR.

Principles 1：Modular-based architecture and protocol designing for 6GR

On-Demand Functionality Design

Alongside modularization, 6GR should natively support on-demand activation of functionalities. Given the wide range of device capabilities and service requirements envisioned in 6G, not all network functions need to be active for every user or at all times. The current 5G system often requires UEs to signal extensive capabilities and the network to support a multitude of features regardless of whether they are utilized in a particular session. In 6G, it is desirable that RAN features (especially advanced or resource-intensive ones) can be enabled, configured, or instantiated dynamically when required, and remain dormant or omitted when not needed. This on-demand design principle applies both to control-plane procedures and user-plane functions.

An on-demand functionality framework yields a more efficient and flexible RAN. Network resources (air interface, processing, signaling overhead) are only expended for capabilities in use to improve overall efficiency. On-demand functionality framework also allows the RAN to adapt rapidly to context changes. By designing protocols and interfaces that can negotiate and trigger functionalities on-the-fly (for example, on-demand channel state reporting, on-demand interference mitigation functions, etc.), the 6GR can achieve a high degree of customization and agility. Consequently, on-demand functionality is proposed as a core design principle for 6GR. Combined with modular architecture, this will help keep the baseline RAN as lightweight as possible for simple use cases, while allowing more complex operations to be supported when required.

Principles 2: On-Demand Designing for advanced functionalities of 6GR

  - New services

Among the new service paradigms under consideration for 6G, Artificial Intelligence (AI)-enabled services and Integrated Sensing and Communication (ISAC) services stand out as transformative additions. In fact, the 3GPP RAN plenary has already recognized AI and sensing as new service categories to be included in the 6G requirements study (TR 38.914 [2]) alongside other usage scenarios. These services impose unique demands on the RAN in terms of data, connectivity and latency.

    - AI

6G networks are expected to natively support AI-driven applications and network intelligence. This includes use cases like distributed machine learning, AI-assisted network optimization, and user-oriented AI services (e.g. AI agent communication, immersive AI-driven experiences). TR 22.870, for example, outlines numerous AI-related 6G use cases such as AI for connected vehicles, personalized AI for healthcare, collaborative AI agents, and more. A common thread in these scenarios is that the RAN will play an active role in collecting, processing, and delivering data for AI.

To support AI services, a 6G RAN node will need capabilities beyond traditional baseband processing. It must be able to ingest and distribute large data sets in real time. Notably, data will need to flow between RAN nodes and UEs, between different RAN nodes, and between RAN and core-network nodes in support of AI functionalities. For instance, an AI-driven application might require gathering high-resolution sensor data from many UEs (upwards link to RAN), aggregating or exchanging data between neighboring RAN nodes (for example, to get a holistic view in a smart city scenario), and forwarding relevant data to a core or edge cloud service for AI model processing (RAN to CN exchange). The RAN may also receive AI inferencing tasks or machine learning model updates from the core side that need to be distributed to UEs or other base stations. All of these imply the RAN should handle intensive data exchange across all interfaces in a reliable and timely manner, likely more so than in previous generations.

Observation 1: The support of AI functionality in 6GR will require RAN nodes to exchange significant amounts of data across the Uu interface (RAN–UE), between RAN nodes (inter-base station), and between RAN and core network nodes.

    - Sensing

Integrated Sensing and Communication (ISAC) is another pillar of 6G’s service landscape. In simple terms, 6G networks are anticipated to sense the environment – using radio signals to detect, localize, and characterize objects or happenings – in addition to providing communications. RAN nodes (base stations) can act as radar-like sensors by transmitting signals and analyzing echoes, and UEs might also participate in sensing by collecting or relaying measurements. The 3GPP RAN plenary has affirmed sensing as a key use case in the 6G study. To enable these, 6G RAN nodes will embed sensing functions alongside communication functions.

Realizing sensing services imposes new demands on the RAN infrastructure and protocols. A 6G base station would need to transmit specialized waveforms or signals for sensing and process the reflections or feedback. It may also coordinate with other RAN nodes to perform multi-static sensing. Indeed, various sensing deployment modes are being considered – including monostatic (single node transceiving) and bistatic configurations involving multiple Transmit/Receive Points (TRPs) and UEs. This means RAN-to-RAN communication could be leveraged to exchange sensing information or cross-synchronize sensing signals. Additionally, results of sensing (raw or processed data about the environment) might be sent from the RAN to the core network or application servers for further analysis or to offer sensing-as-a-service to third parties. Like AI, the sensing function will generate new data flows in the network: uplink (UE reports or echoes), downlink (probe signals, control), and laterally between nodes (joint processing of sensing data).

Observation 2: The support of sensing functionality in 6GR will similarly require extensive data exchange between RAN nodes and UEs, among RAN nodes themselves, and between RAN and core network entities.

  - 6G RAN Architecture

A number of guiding architecture requirements were agreed at RAN Plenary #109. In particular, it has been agreed that “the 6G RAN architecture shall allow for control plane and user plane separation.” As in the figure 1, 5G RAN (NR) architecture introduced a functional separation in the gNB: a Central Unit (CU) and Distributed Unit (DU), and the CU itself is further subdivided into a Control-Plane part (CU-CP) and User-Plane part (CU-UP) for the gNB. Effectively, 5G RAN distributes RAN functions across three logical nodes (CU-CP, CU-UP, DU) to allow some degree of centralization and separation of concerns. However the case of three logical entities increases system complexity and the case of two logical entities (CU and DU) only partially achieves CP/UP separation.

For cleaner CP/UP separation, a more streamlined architecture with two primary RAN nodes could be considered: a CP node and a UP node as illustrated in figure 2. In one possible realization, a 6G base station could be functionally split such that all control-plane functions reside in a dedicated CP node, while all user-plane functions reside in a dedicated UP node. The CP node would handle RAN control protocols (RRC signaling, connection management, mobility control, radio resource allocation decisions, etc.), and the UP node would handle the data transport (PDCP-user, user plane RLC/MAC, scheduling execution, etc.). This approach would effectively implement a full control/user plane separation within the RAN architecture.

Figure 1: 5G RAN architecture for function separation     Figure 2: 6G RAN architecture for CP/UP separation

Beyond the traditional user plane (which primarily carries end-user communications), 6G will introduce new categories of data as discussed (e.g. sensing data, AI model data, etc.). The RAN will be not just forwarding end-user IP payloads, but also handle direct data exchanges between RAN and UE, between RAN nodes and between RAN and other network entities as required by services. The 6G UP node could incorporate an expanded “data plane” functionality to support these emerging data flows which will effectively add a flexible data routing layer within the RAN architecture to accommodate service-specific data dissemination (distinct from standard user payload forwarding).

Proposal 1: In the 6G RAN architecture, adopt a clear control plane (CP) and user plane (UP) separation by defining two main RAN logical nodes – a CP node responsible for all control-plane functions, and a UP node responsible for all user-plane functions. Furthermore, the UP node should include a generalized data-plane capability to efficiently support emerging 6G data services.

  - UE capability framework

The current (5G) framework treats capabilities in a relatively flat manner – a UE provides exhaustive information on every supported option (radio bands, maximum layers, supported channel bandwidths per band, supported features like URLLC, positioning, etc.). As UEs vary from very simple (e.g. low-cost IoT modules) to very advanced (high-end smartphones or 6G terminals), not all capability parameters are relevant to every device. In 5G, however, even a low-tier device must signal many fields (some just set to “not supported”), and a high-tier device must enumerate all combinations it supports. This one-size-fits-all signaling is inefficient and difficult to extend. It also complicates network-side interpretation, as networks must parse through capabilities to determine the “type” of device (IoT, enhanced mobile broadband, etc.) and adapt their configuration accordingly. For 6G, it is worth exploring a more scalable and structured capability framework that can accommodate diverse device types without burdening the system with excessive signaling.

To overcome the Shortcomings of the 5G approach, a hierarchical or multi-level capability representation could be considered for UE capability framework in 6G. For example, at the top level, a UE could declare its broad device type (or category), which encapsulates key characteristics of its radio and service capabilities. This device type would effectively classify the UE among a limited set of categories (for example, categories might distinguish an IoT-centric device, a standard enhanced Mobile Broadband (eMBB) device, an ultra-low latency device, an XR/immersive device, etc., as needed for 6G). After indicating its device type, a UE would then only need to report the specific capabilities relevant within that category. This constitutes the second level of the proposed multi-level framework. These specific capabilities could largely follow the existing 5G NR capability parameters, but pruned and focused for the device type. The hierarchical or multi-level capability representation would result in a leaner, more modular capability signaling and also inherently supports on-demand capability exchange: the network could query additional details only if needed.

The RAN plenary has begun examining the notion of device types and associated parameter sets for 6G. By assigning each UE to a well-defined type with known baseline capabilities, the network can handle each class optimally (e.g., scheduling strategies for IoT vs eMBB devices), and the capability signaling overhead is significantly reduced for each UE. This layered approach can improve scalability as 6G connects everything from simple sensors to complex AI-powered devices.

Proposal 2: Adopt a multi-level UE capability framework for 6G and the device type could be the top level.

- Conclusion

In this contribution, we discussed the following general aspects for 6GR and provided our proposals as follows:

- Design principles for 6GR

Principles 1：Modular-based architecture and protocol designing for 6GR

Principles 2: On-Demand Designing for advanced functionalities of 6GR

- New services

Observation 1: The support of AI functionality in 6GR will require RAN nodes to exchange significant amounts of data across the Uu interface (RAN–UE), between RAN nodes (inter-base station), and between RAN and core network nodes.

Observation 2: The support of sensing functionality in 6GR will similarly require extensive data exchange between RAN nodes and UEs, among RAN nodes themselves, and between RAN and core network entities.

- 6G RAN Architecture

Proposal 1: In the 6G RAN architecture, adopt a clear control plane (CP) and user plane (UP) separation by defining two main RAN logical nodes – a CP node responsible for all control-plane functions, and a UP node responsible for all user-plane functions. Furthermore, the UP node should include a generalized data-plane capability to efficiently support emerging 6G data services.

- UE capability framework

Proposal 2: Adopt a multi-level UE capability framework for 6G and the device type could be the top level.

- References

- TR 22.870: "Study on 6G Use Cases and Service Requirements".

- TR 38.914: "Study on 6G Scenarios and Requirements".

- RP-252912: " Revised SID: Study on 6G Radio“NTT DOCOMO, CMCC, AT&T, Vodafone Beijing, China, September 15-18, 2025.