3GPP TSG-RAN WG2 Meeting #131bis	R2-2507397

13-17 October, Prague, Czech Republic

Source: 	Vodafone

Title: 		Early alignment on the air interface security

Agenda Item: 10.3.3

Introduction

During RAN Plenary #109, Liaison Statement RP-252891 was sent to the SA Plenary requesting early alignment on the air interface security requirements across SA and RAN working groups by June 2026.

“RAN invites SA to consider adding a June 2026 check point (in SA3’s SID on 6G security S3-252973) for the alignment between SA3 and RAN2 on any security requirements that impact the AS protocol design.”

SA #109 handled the RAN LS and the SA3 Study Item on 6G security was updated to include the related milestone and agreed in SP-251233.

Discussion

Early Considerations for Control and User Plane Protocols in RAN2 – Including AI/ML Data Transfer and Security Requirements:

RAN2 is initiating discussions on the architecture and functionalities of the control and user plane protocol stacks for 6GR, including support for AI/ML data transfer. A key objective of this work is to define the roles and responsibilities of each protocol layer in handling various types of traffic and control information.

Security considerations are critical and must be addressed from the outset. To ensure that protocol design aligns with security needs, it is essential that the first set of air interface security requirements be made available as early as possible. This will allow RAN2 to incorporate them into the design process from the beginning, rather than retrofitting security measures later.

During the development of 5G, the system was architected such that the PDCP layer provided ciphering and integrity protection for both user data and RRC signaling, ensuring secure transmission of information above the MAC layer. Subsequently several control functions traditionally handled by the RRC layer were moved to the MAC layer, e.g. LTM related or SCell activation/deactivation. This shift was driven by the need for performance optimization, including more precise execution time, reduced interruption times, and improved energy efficiency for the UE.

Looking ahead to 6G, it is reasonable to expect that this trend will continue. This evolution raises important questions about the appropriate level of security protection required at each layer, particularly below RRC.

Although RAN2 is still in the early stages of protocol stack development, and neither the architecture nor the specific functions of the protocols have been finalized, it is imperative to begin defining security requirements now. Early dialogue with SA3 and timely input from SA3 will enable RAN2 to make informed decisions about protocol responsibilities, data handling, and protection mechanisms, especially considering existing or new services like Mobility, Carrier Aggregation, AI/ML data exchange or Sensing, which may introduce new security and privacy challenges.

Proposal:

It is proposed that this RAN 2 meeting sends an LS to SA3 with the following action:

RAN2 kindly ask SA3 to provide security requirements for RRC Layer and below (e.g. MAC) by June 2026, and RAN2 invites SA3 to engage in early dialogue to facilitate this.