3GPP TSG-RAN WG2 Meeting #131bis	R2-2507423

Prague, Czech Republic, 13 – 17 October, 2025

Agenda item:	8.10.1

Source:	Huawei, HiSilicon

Title:	WI Comments file for TS 36.331 CR for R19 SONMDT

Document for:	Discussion and Decision

1 Introduction

This is a WI Comments file for TS 36.331 CR for R19 SONMDT, based on the email discussion of RIL issue collection for TS 36.331 CR.

2 SONMDT for LTE Comments file

SONMDT for LTE Comments file

Template:

Xnnn

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Xnnn |  |  |  |  |  |  | vnnn | ToDo |

[Description]:

[Proposed Change]:

[Comments]:

Instructions:

- Copy the template RIL comments fields above (including the Heading Xnnn)

- Paste the RIL comments fields at its position while respecting the order of the RILs in the Review file (i.e. keep the order of the spec).

- Fill in the fields, see R19 ASN.1 Guideline.

- Companies may comment whether they agree or disagree.

- Can copy spec text and use Word “Track changes”, etc.

- Do not delete text added by other companies.

C064

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C064 | SONMDT | 1 | reconfiguration with sync failure |  | Tangxun |  | V002 | PropAgree |

[Description]: “reconfiguration with sync failure” should be used instead of “handover failure” to cover LTM case, and this is also to align with the corresponding description in NR RRC spec.

[Proposed Change]: update the procedural text as below:

4>	if the selected PCell is a suitable cell as defined in TS 36.304 [4]:

5>	if the UE supports RLF-Report for MCG LTM and if ltm-RecoveryCellId in VarRLF-Report of TS 38.331 [82] is set:

6>	set timeUntilReconnection in VarRLF-Report of TS 38.331 [82] to the time that elapsed since the radio link failure or reconfiguration with sync failure experienced in the failedPCellID stored in VarRLF-Report of TS 38.331 [82];

5>	else:

6>	set timeUntilReconnection in VarRLF-Report of TS 38.331 [82] to the time that elapsed since the last radio link failure or reconfiguration with sync failure;

[Comments]:

[Huawei] ok for this change.

[Rapporteur] The change is agreeable.

N001

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N001 | SONMDT | 1 | What is capability SCG failure information for EN-DC MRO? |  | Jarkko Koskela |  | V003 | PropAgree |

[Description]: in 5.6.13a.2	on uses “if the UE supports SCG failure information for EN-DC MRO”. As I’m not expert on this feature I was not able to map this to any capability. Which one this refers to?

[Proposed Change]: Remove ambiquity regarding to which capability this refers to.;

[Comments]:

[Huawei] This UE capability is from the agreed 36.306 CR. I suggest to add "as specified in TS 36.306 [5]." after the above-mentioned text for clarification.

R2-2505209	Introduction of SONMDT UE Capabilities	CATT	CR	Rel-19	36.306	18.5.0	1915	-	B	NR_ENDC_SON_MDT_Ph4-Core

Agreed

[Rapporteur] Suggest to use Huawei's suggestion to fix this RIL issue.

H345

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H345 | SONMDT | 0 | Missing TS reference number for TS 38.331 |  | Jun Chen |  | V004 | Duplicate |

[Description]: in 5.6.13a.3, TS reference number for TS 38.331 is missing.

3>	set perRA-InfoListNR to indicate the performed random access procedure related information as specified in 5.7.10.5 of TS 38.331

[Proposed Change]: Suggest to add TS reference number for TS 38.331:

3>	set perRA-InfoListNR to indicate the performed random access procedure related information as specified in 5.7.10.5 of TS 38.331 [82]

[Comments]:

[Rapporteur] This editorial change is agreeable.

Lenovo commented that CR: H345 (adding missing spec reference [82]) is editorial and can be already fixed during CR implementation review. So the status is changed to Duplicate, and this RIL will not be captured in the rapporteur CR.

H346

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N001 | SONMDT | 1 | perRA-InfoListNR |  | Jun Chen |  | V004 | PropAgree |

[Description]: in section 6.2.2, the following wording "RA information for NR RACH" is not accurate, and instead the it should be about NR RACH report information (which has been used in TS 38.331).

perRA-InfoListNR

This field is used to indicate per RA information for NR RACH.

[Proposed Change]: Suggest to change the wording into the following:

perRA-InfoListNR

This field is used to indicate per RA information for NR RACH per NR RACH report information.

[Comments]:

[Rapporteur] See no comments from other companies, so consider it to be agreeable.

B001

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B001 | SONMDT | 1 | Missing “OPTIONAL” for field perRA-InfoListNR-r19 |  | Lenovo (Hyung-Nam) |  | V007 | ToDo |

[Description]: Any extension that is introduced in FailureReportSCG-NR-r15 after the extension marker should be optionally present. Therefore, "OPTIONAL" is missing for field perRA-InfoListNR-r19.

[Proposed Change]: Add missing “OPTIONAL” for field perRA-InfoListNR-r19 as shown below.

[[

previousPSCellId-r19			SEQUENCE {

physCellId-r19					PhysCellIdNR-r15,

carrierFreq-r19					ARFCN-ValueNR-r15

}																	OPTIONAL,

failedPSCellId-r19				SEQUENCE {

physCellId-r19					PhysCellIdNR-r15,

carrierFreq-r19					ARFCN-ValueNR-r15

}																	OPTIONAL,

timeSCG-Failure-r19				INTEGER (0..1023)					OPTIONAL,

perRA-InfoListNR-r19			SEQUENCE {

perRA-InfoList-r16				OCTET STRING					OPTIONAL,

perRA-InfoList-v1660			OCTET STRING					OPTIONAL,

perRA-InfoList-v1800			OCTET STRING					OPTIONAL

}																	OPTIONAL

]]

[Comments]:

[Rapporteur] This change makes sense, so it is agreeable.