3GPP TSG-RAN WG2 Meeting #131bis                                    	R2-2507441

Prague, Czech Republic, 13th - 17th October 2025

Agenda item:	8.9.2

Source:	Qualcomm Incorporated

Title:	Remaining issues on CB-EDT

Document for:	Discussion and Decision

- Introduction

In RAN2#131 meeting, the work item was considered complete. However, there are some issues left open. In this document, we address those issues.

- Discussion

CB EDT Msg3 transmission window

Open issue: Whether to configure the PUSCH resource within the Tx window configuration or as a separate configuration, and what is the unit for window configuration;

Defining larger periodicity of PUSCH occasions and then grouping consecutive PUSCH occasions into a CB-Msg3-EDT transmission window will lead to long latency for the UE to complete the CB-Msg3-EDT procedure.

For example in figure 1, if the network can allocate only 4 PUSCH occasions per second, having periodic PUSCH occasions will result in 4 PUSCH occasions starting at 0ms, 250ms, 500ms and 750ms. Assuming N=2 occasions within a window, the first two occasions belong to window 1 while the next two belong to window 2. Let us also assume K=2 is configured.

Figure 1 An example of CB Msg3 (DSA) window with larger PUSCH periodicity

A UE transmitting in Window 1 will transmit in TO#1, wait until start of TO#2, transmit in TO#2 and start the response window after the end of TO#2 plus RTT. So the earliest time the UE can receive success/failure response in this case is 250ms + PUSCH duration + RTT.

To reduce this latency the PUSCH peridiocity can be reduced. However this leads to increase amount of resources allocated for PUSCH occasions.

Instead, if the PUSCH occasions within a window are packed closer together (optimally back-to-back) the latency can be decreased without increased resource utilization.

Figure 2 An example of CB Msg3 (DSA) window with shorter PUSCH periodicity

The earliest time the UE can receive a success/failure response in this example in figure 2 would be 80ms + PUSCH duration + RTT.

We think network should have option to configure the CB-Msg3-EDT transmission window efficiently optimizing the resource, latency and probability of success. For example, there can be gap between two transmission windows and there should be no PUSCH resources configured in the gap. Network should also have option to configure the PUSCH occasions back-to-back to reduce latency. For example, when UE is in CE level 0, the minimum gap can be zero or as small as 10ms. In our view, the cleaner configuration is configuring the PUSCH resource within the Tx window configuration.

CB-Msg3-configuringng9 ::=				SEQUENCE {

cb-Msg3-TBS-r19							ENUMERATED {b144, b328, b408, b504, b600, b712,

b808, b936},

cb-Msg3-NumOfReplicas-r19				INTEGER(1..4),

cb-Msg3-MPDCCH-Config-r19				CB-Msg3-MPDCCH-Config-r19,

cb-Msg3-PUCCH-Config-r19				CB-Msg3-PUCCH-Config-r19,

cb-Msg3-PUSCH-Config-r19				CB-Msg3-PUSCH-Config-r19,

cb-Msg3-TxWindow-r19			SEQUENCE {

windowSize-r19						ENUMERATED {n1, n2, n4, n8, n16, n32, n64,

n128},

windowPeriodicity-r19				ENUMERATED {n1, n2, n4, n8, n16, n32, n64,

n128}				OPTIONAL	--Need OP

pusch-Periodicity-r19					ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128,

sf256},

startSFN-r19					INTEGER (0..1023),

startSubframe-r19				INTEGER (0..9)

}																		OPTIONAL,	--Need OP

cb-Msg3-ResponseWindow-r19				ENUMERATED {sf240, sf480, sf960,

sf1920, sf3840, sf5760, sf7680, sf10240},

cb-Msg3-MaxAttemptNum-r19				ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n10}

OPTIONAL,	--Need OP

...

}

As shown above, the CB-Msg3 window can include window length, start time, periodicity and PUSCH periodicity. Then special case of number of PUSCH occasions of 1 can be configured as:

windowSize-r19 = n1

windowPeriodicity-r19 = absent

configure the PUSCH resource within the Tx window configuration

Unit of Transmission window and periodicity

In our view, the transmission window size can be configured in number of PUSCH periodicities. Value n1 should correspond to duration of (1 * pusch-Periodicity-r19) and n2 to (2 * pusch-Periodicity-r19). Since maximum number of replicas is 4, we suggest using 2 bits of information though it is possible to configure window size larger than duration of 4 replicas (for example N = 5 and K = 4).

windowSize-r19						ENUMERATED {n1, n2, n3, n4},

However, for the case of window periodicity, we need to align it with the way RNTI formula is derived. As we see from the formula that the window periodicity should be in multiple 10ms.

However, first we need to decide what is window periodicity whether it is the gap between two windows or interval between starts of the windows. From the RNTI calculation perspective, we think the window periodicity is the interval between starts of the windows, which means the window periodicity should be larger or equal to the window length. If we configure the window length in terms of PUSCH periodicity, then PUSCH periodicity also needs to be in multiple 10ms. For, NB-IoT, it is already the case. However, for eMTC, PUSCH periodicities of 2, 4, 8 are also possible.

Therefore, we suggest defining the window periodicities in absolute values in such as a way that it is in multiple of 10ms as well as greater or equal to window length.

#### 5.1x.3 CB-Msg4 reception Once all the CB-Msg3s of a CB-Msg3 transmission window are transmitted, the MAC entity shall monitor the PDCCH of the SpCell, while CB-Msg3ResponseTimer is running, for CB-Msg4(s) identified by the CB-RNTI defined below. For BL UEs and UEs in CE mode A, the CB-RNTI associated with the CB-Msg3 transmission window in which the CB-Msg3(s) are transmitted, is computed as: CB-RNTI = floor (SFN_id/WP) modulo (32) + 32*CE_level + 2401 Where: - SFN_id is the first SFN of the selected CB-Msg3 transmission window. - WP is the configured CB-Msg3 transmission window periodicity in unit of 10ms provided by windowPeriodicity. - CE_level is the selected enhanced coverage level (0 <= CE_level < 2). For NB-IoT UEs, the CB-RNTI associated with the CB-Msg3 transmission window in which the CB-Msg3(s) are transmitted, is computed as: CB-RNTI = floor (SFN_id/WP) modulo (32) + 32*CE_level + 96*carrier_id + 4097 Where: - SFN_id is the first SFN of the selected CB-Msg3 transmission window. - WP is the configured CB-Msg3 transmission window periodicity in unit of 10ms provided by windowPeriodicity. - CE_level is the selected enhanced coverage level (0 <= CE_level < 3). - carrier_id is the index of the UL carrier associated with the selected UL grants (0 <= carrier_id < 16). The carrier_id of the anchor carrier is 0.

For windowSize, use 2 bits of information to signal n1, n2, n3 and n4 where value n1 should correspond to duration of (1 * pusch-Periodicity-r19) and n2 to (2 * pusch-Periodicity-r19) and so on.

Select the range of values for windowPeriodicity-r19 which are in multiple of 10ms and larger than or equal to windowSize.

TA command

When TA command is received in Msg4 and also C-RNTI (not going to IDLE), that means the UE is supposed to monitor the PDCCH for further downlink message. It is possible UE receives RRC release message to go to IDLE mode but only after transmitting HARQ feedback in UL. Similarly, the UE may receive RRC setup message to transition to the connected mode. In this case also, the UE would need to continue UL transmission for RRC setup complete message.

However, currently the timeAlignmentTimer is not started upon receiving TA command in Msg4 for CB-EDT procedure. This means it is not clear whether the UE needs to maintain TA accumulation or use absolute TA for the next UL transmission.

In our view, the TA command received in this case is the absolute TA command and the UE should start timeAlignmentTimer and maintain TA accumulation for subsequent UL transmission.

start timeAlignmentTimer when TA command is received in a successful CMR for a serving cell belonging to a TAG.

HARQ feedback resource field in Msg4

RAN2 received the LS [1] from RAN1 and provided following response for the 2 bits field of HARQ feedback resource.

RAN1 reply to Q1:

- RAN1 sees no issue with the RAN2 agreement that HARQ feedback resource information is included in the CB-Msg4

- RAN1 also notes that using 2 bits for “HARQ ACK resource offset” in eMTC allows multiplexing only up to 4 users in the same response.

Figure 3 CB-EDT Msg4 CMR format for eMTC and NB-IoT

If we look at the HARQ feedback resource for eMTC and NB-IoT, it is clear that HARQ feedback resource for 16 users can be provided with 4 bits for NB-IoT. However, this is significantly low and limited to just 4 users for eMTC which has higher bandwidth than NB-IoT.

Use 4 bits of HARQ ACK resource field in CMR for eMTC in line with NB-IoT design.

Applicable of new Msg4 format (and group RNTI) to 4 step RACH

One of the objectives of UL capacity enhancements of Rel-19 WID was to resolve the bottle neck issue of 4 step RACH/EDT that results from individually addressing many UEs for contention resolution in Msg4. However, the Msg4 enhancement was only considered for new CB-EDT feature. The new Msg4 format can be supported on the top of existing 4 steps EDT procedure as shown in figure 4.

Figure 4 An example of using new Msg4 CMR format in Rel-15 EDT

We think this is doable as the UE can report the support of Msg4 format in Msg3. In case of 4 step EDT, this indication can be added in RRCEarlyDataRequest and in case of 4 step Random Access procedure, it can be added in RRCConnectionRequest message (e.g., using a spare bit or a new LCID code point).

Then UE starts contention resolution timer as existing procedure. The other change is that the UE would have to monitor both TC-RNTI for legacy Msg4 receiving procedure and a new group RNTI for new Msg4 receiving procedure. In this case, the group RNTI can be configured as cell specific group RNTI in SIB, it can be per CE level.

Support new Msg4 multicast response format for legacy 4 step RACH or EDT procedure.

- Conclusion

Following proposals are made:

Proposal 1	configure the PUSCH resource within the Tx window configuration

Proposal 2	For windowSize, use 2 bits of information to signal n1, n2, n3 and n4 where value n1 should correspond to duration of (1 * pusch-Periodicity-r19) and n2 to (2 * pusch-Periodicity-r19) and so on.

Proposal 3	Select the range of values for windowPeriodicity-r19 which are in multiple of 10ms and larger than or equal to windowSize.

Proposal 4	start timeAlignmentTimer when TA command is received in a successful CMR for a serving cell belonging to a TAG.

Proposal 5	Use 4 bits of HARQ ACK resource field in CMR for eMTC in line with NB-IoT design.

Proposal 6	Support new Msg4 multicast response format for legacy 4 step RACH or EDT procedure.

- References

[1] R2-2506717, “Reply LS on on CB Msg3 EDT for IoT NTN Ph3”, Prague, Czech Republic, 13th - 17th October 2025.