3GPP TSG-RAN WG2 Meeting #131bis                                    	R2-2507442

Prague, Czech Republic, 13th - 17th October 2025

Agenda item:	8.17

Source:	Qualcomm Incorporated

Title:	Remaining issues for NB-IoT NTN TDD mode

Document for:	Discussion and Decision

- Introduction

In RAN2#131 meeting, following agreements were made.

Agreements:

1.	IoT NTN TDD mode capability is conditionally mandatory (as in the draft CR) if the UE supports the relevant TDD band

2.	No need to introduce a new list of neighbour cells operating in TDD mode in Rel-19

3.	RAN2 assume that for IoT NTN TDD mode, serving cell and neighbor cells are not always synchronized.

4.	There is no benefit in introducing a new idle mode DRX cycle length for IoT NTN TDD mode.

5.	There is no specification impact for RAN2 (36.304) related to paging scheduling.

6.	There is no specification change required in the existing SIB1 scheduling mechanism.

7.	RAN2 assumes the network will take into account the invalid subframes when setting the explicit EpochTime

8.	RAN2 assumes that OCC defined for IoT NTN Rel-19 can be supported for IoT NTN TDD mode but ask RAN1 for confirmation

9.	We aim at introducing the needed changes (if any) to support CB-Msg3 and CB-Msg4 for IoT NTN TDD mode during the ASN.1 review, including at least

Adding configurations that are aligned with the 90 ms periodicity for npusch-Periodicity and windowPeriodicity-NB

(FFS other needed enhancements based on the progress of the CB-Msg3-EDT discussion this week)

10.	IoT NTN PWS Rel-19 can be supported in IoT NTN TDD (no spec impact)

11.	TDD radio frame offset between serving cell and neighbor cell(s) can be provided via SIB33 to the UE for neighbor cell measurement.

In this document, we provide details of remaining issues that are not resolved over email discussion.

- Discussion

Collision due to postponed transmission

In NB-IoT TDD mode, it is agreed that when UL SPS occasion falls on an invalid frame, it will be postponed to the next valid frame. When SPS occasion is postponed to the next valid frame, multiple UE’s UL transmission may collide. This would lead to interference and failure of the transmission. We think the simple solution is to interpret current values of the SPS periodicities as new values that are multiple of 90ms.

For example, the value sf128 of semiPersistSchedIntervalUL-r15 can be interpreted as value sf180 in NB-IoT TDD mode.

For SPS interval, the values of semiPersistSchedIntervalUL-r15 are interpreted as new values that are multiple of 90ms.

Measurements

First, for neighbor cell measurements, UE may need to know neighbor’s timing information such that it knows when the neighbor’s invalid DL SFN duration is. RAN2 agreed following in RAN2#131 meeting.

- TDD radio frame offset between serving cell and neighbor cell(s) can be provided via SIB33 to the UE for neighbor cell measurement.

A NB-IoT NTN cell operating in TDD mode may need to be deprioritized as there may be NB-IoT cells operating in normal FDD mode. As NB-IoT does not support frequency priority, the UE may select TDD mode cell over FDD cell.

As we see there is a chance that UE capable of both normal mode and TDD mode is camping on normal TN cell and the neighbor cell list includes both normal TN neighbor cell frequency and NB-IoT TDD mode cell frequency, then it may be possible that UE finds the NB-IoT TDD as better ranked neighbor cell. We think the cleaner solution would be to have a new list of NB-IoT neighbor cells operating in TDD mode.

However, RAN2 made following decision in RAN2#131 meeting.

- No need to introduce a new list of neighbour cells operating in TDD mode in Rel-19

Therefore, we suggest either to revisit the last agreement or clarify that it is up to UE whether to perform TDD mode neighbor cell measurements when performing normal mode neighbor cell measurements.

As NB-IoT does not support frequency priority, the UE may select TDD mode cell over FDD cell.

Clarify that it is up to UE whether to perform TDD mode neighbor cell measurements when performing normal mode neighbor cell measurements.

- Conclusion

Following proposals are made:

Proposal 1	For SPS interval, the values of semiPersistSchedIntervalUL-r15 are interpreted as new values that are multiple of 90ms.

Observation 1.	As NB-IoT does not support frequency priority, the UE may select TDD mode cell over FDD cell.

Proposal 2	Clarify that it is up to UE whether to perform TDD mode neighbor cell measurements when performing normal mode neighbor cell measurements.