3GPP TSG-RAN WG2 Meeting #131bis	R2-2507470

# Prague, Czech Republic, 13th – 17th October 2025

Agenda Item:	8.7.2

Source:	Ericsson

Title:	N091, S038

Document for:	Discussion, Decision

# 1	Introduction

The following are the list of open issues as contributed by companies and the opinion of the rapporteur..

| RIL ID | Class | Title | Status |
| --- | --- | --- | --- |
| N091 | 1 | UAI for measurement gap cancellation preference | ToDo |
| S038 | 1 | Checking T346o running status | PropRej |

In this contribution, we discuss the open issues as listed above.

# 2	Discussion

## 2.1 	S038

If UE has preference for gap occasion cancellation ratio and has not sent gapOccasionCancelRatio since UE was configured to do so, the UE triggers UAI transmission without considering if timer T346o is running.

Our understanding is that this causes no issue if either UE considers all prior OtherConfigs including gapOccasionCancelRatioReportConfig or if it can be guaranteed that the timer T346o is not running upon reception of the OtherConfigs including gapOccasionCancelRatioReportConfig . However, the timer T346o is not stated to be stopped if running upon reception of OtherConfigs including gapOccasionCancelRatioReportConfig . We see the following solutions:

- Solution-1: RAN2 clarifies that “since it was configured to do so” includes all prior OtherConfigs including gapOccasionCancelRatioReportConfig and not only the last configuration, or

- Solution-2: T346o timer is stopped, if running, upon reception of each OtherConfigs including gapOccasionCancelRatioReportConfig  or “timer T346o is not running” added as condition also for the first time after configuration.

We believe solution-1 is the correct interpretation as the gapOccasionCancelRatioReportConfig is Need M and should maintain this configuration i.e., the current spec is correct and no specification impact.

(S038) Clarify that the UE behaviour related to T346o timer includes all prior OtherConfigs including gapOccasionCancelRatioReportConfig and not only the last configuration. No spec impact is foreseen.

## 2.2	N091

To send the UAI with UE preference of gap occasion cancellation ratio is not a very time critical process, thus there is no need for this further optimization of the UAI triggering. If network has made a new measurement gap configuration the transmission of the preference for the gap occasion can wait until the UAI prohibit timer runs out.

(N091) No optimization of the UAI triggering is considered for the case of newly configured measurement gaps.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

No table of contents entries found.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	(S038) Clarify that the UE behaviour related to T346o timer includes all prior OtherConfigs including gapOccasionCancelRatioReportConfig and not only the last configuration. No spec impact is foreseen.

Proposal 2	(N091) No optimization of the UAI triggering is considered for the case of newly configured measurement gaps.