3GPP TSG-RAN WG2 #131bis	R2-2507473

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	7.0.2.16

Source: 	Ericsson

Title:  	Clarification on DSR triggering

Document for:	Discussion and Decision

# Introduction

In RAN2#131 a correction on DSR triggering for Rel18 was discussed triggered by [1]. In this CR it was argued that remaining RLC SDU segments belonging to a PDCP SDU cannot trigger a DSR, since part of the PDPC SDU has already been transmitted in a MAC PDU. The proposed change was to add the word “completely transmitted” to make it clear that a remaining RLC segment can also trigger the DSR. However, during the session there were different views on how the current specification should be interpreted and it was decided that RAN2 should first align the views on the current specification.

After long offline discussions on what the interpretation of current specification should be, two camps emerged:

- companies that believe a PDCP SDU has only been transmitted when all the data belonging to the SDU has been transmitted,

- companies that believe that only part of a PDCP SDU need to be transmitted to consider the SDU as transmitted.

No agreement could be made in RAN2#131 and the decision of what the interpretation of current specification should be was postponed to future meeting.

In this contribution we provide our view of what the correct interpretation of the current specification should be.

# Discussion

## SDU Definition in Specification

There are many examples describing the use of SDUs in the 3GPP standards. Looking at these examples should clarify how usage of protocol SDUs should be interpreted. Starting with the L2 data flow picture from 38.300 chapter 6.6:

The figure shows the mapping of SDUs between protocol layers. This can also be understood by checking the definitions in the RLC specification (38.322, chapter 3.1) where the delay critical RLC SDU is described as:

Delay-critical RLC SDU: RLC SDU corresponding to a PDCP PDU indicated as delay-critical by PDCP (see TS 38.323 [4]).

And the RLC segment as:

RLC SDU segment: A segment of an RLC SDU.

From the mappings and definitions, it is obvious that an RLC SDU maps to a complete PDCP PDU while the RLC SDU segments consist of only parts of the PDCP SDU.

For the purpose of MAC buffer status reporting, the UE shall consider the following as RLC data volume:

- RLC SDUs and RLC SDU segments that have not yet been included in an RLC data PDU;

- RLC data PDUs that are pending for initial transmission;

- RLC data PDUs that are pending for retransmission (RLC AM).

For the purpose of MAC single entry delay status reporting, the UE shall consider the following as delay-critical RLC data volume:

- delay-critical RLC SDUs and delay-critical RLC SDU segments that have not yet been included in an RLC data PDU;

- RLC data PDUs pending for initial transmission, and containing a delay-critical RLC SDU or a delay-critical RLC SDU segment;

- RLC data PDUs that are pending for retransmission (RLC AM).

Then looking into the usage of the RLC SDUs and RLC SDU segments there are many examples, but one closely related to the DSR is the delay critical data volume calculation which is described in chapter 5.5 Data Volume Calculation in the RLC specification:

It can be observed that a distinction is being made between SDUs and SDU segments. Any time part of the data can be considered then segments are referenced. Thus, mention of the SDU is referencing the complete SDU, i.e. the complete PDCP PDU, while the mention of SDU segments is referencing parts of the SDU, i.e. parts of the PDCP PDU.

There are many examples in specification that a SDUs refers to the complete PDU while the SDU segments refer to part of the PDU data.

For a PDCP SDU received from upper layers, the transmitting PDCP entity shall:

-	associate the COUNT value corresponding to TX_NEXT to this PDCP SDU;

NOTE 1:	Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4;

-	perform uplink data compression of the PDCP SDU as specified in clause 5.14.4;

-	perform integrity protection, and ciphering using the TX_NEXT as specified in the clause 5.9 and 5.8, respectively;

-	set the PDCP SN of the PDCP Data PDU to TX_NEXT modulo 2[pdcp-SN-SizeUL];

-	increment TX_NEXT by one;

-	submit the resulting PDCP Data PDU to lower layer as specified below.

Looking at the PDCP specification it is described how the PDCP PDU is created from the PDCP SDU in chapter 5.2.1 Transmit operation:

In short, the PDCP PDU is created from the PDCP SDU by addition of headers.

An RLC SDU is a PDCP PDU and a PDCP PDU is a PDCP SDU including PDCP headers.

## SDU Transmission in Specification

A second important aspect concerns the definition of when an RLC SDU is considered transmitted or received. Some companies argue that an RLC SDU should be deemed transmitted or received as soon as part of it, i.e., when some segments, have been transmitted or received.

This interpretation is incorrect. Multiple examples in the specifications clearly indicate that an RLC SDU is considered transmitted only when all of its segments have been successfully transmitted and received. In other words, an RLC SDU is regarded as delivered only when the entire unit, equivalent to the PDCP PDU, has been received — not just parts of it. The specifications do not explicitly use the term 'complete,' since it is inherently understood that an RLC SDU is only complete when it fully matches the PDCP PDU, meaning all segments have been received.

For reference, see the following specification examples:

Example 1:

5.2.3.1.1 AM transmit operations

When receiving a positive acknowledgement for an RLC SDU with SN = x, the transmitting side of an AM RLC entity shall:

- send an indication to the upper layers of successful delivery of the RLC SDU;

- set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which a positive acknowledgment has not been received yet.

The transmitter side updates state variables and indicates to higher layers that an RLC SDU has been successfully delivered when a positive acknowledgement has been received. If this update were triggered when only an RLC SDU segment was received, it would create inconsistencies in transmitter behavior. Moreover, the receiver side sends positive acknowledgements only when the entire RLC SDU has been received, not when just a segment is received. As above, the specifications do not use the term “complete” as it is sufficiently clear what an RLC SDU is.

Example 2:

5.3.2     Retransmission

The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following:

This section demonstrates that negative acknowledgements may refer to the RLC SDU or to specific segments. This directly contradicts the claim that an RLC SDU can be considered transmitted or received upon successful delivery or reception of a single segment.

Example 3:

5.4        SDU discard procedures

When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

In discard procedures, the PDCP instructs RLC to discard the entire RLC SDU. There is no reference to discarding only part of an SDU. In fact, this would be impossible if any of its segments were already in the process of transmission.

These examples leave no room for misinterpretation: historically and technically, 3GPP has always defined an RLC SDU as a complete unit distinct from its segments. Equating an RLC SDU with a segment or considering it transmitted or received when only a segment has been delivered, is not consistent with the specifications and is technically incorrect.

RAN2 to confirm that the transmission or reception of an RLC SDU segment is not equivalent to a transmission or reception of the said RLC SDU or the PDCP SDU.

RAN2 agrees that the interpretation in Proposal 1 remains in Release 18 and any remaining RLC SDU segments of a PDCP SDU can trigger a DSR.

# Conclusion

In the previous sections we made the following observations:

Observation 1	There are many examples in specification that a SDUs refers to the complete PDU while the SDU segments refer to part of the PDU data.

Observation 2	An RLC SDU is a PDCP PDU and a PDCP PDU is a PDCP SDU including PDCP headers.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to confirm that the transmission or reception of an RLC SDU segment is not equivalent to a transmission or reception of the said RLC SDU or the PDCP SDU.

Proposal 2	RAN2 agrees that the interpretation in Proposal 1 remains in Release 18 and any remaining RLC SDU segments of a PDCP SDU can trigger a DSR.

# References

R2-2505408, Correction on DSR triggering