3GPP RAN WG2 Meeting #131bis	R2-2507508

Prague, Czech Republic, Oct. 13th – 17th, 2025

Agenda Item:	8.11.3

Source:	InterDigital, Inc.

Title:	SBFD with CA for stage-2 spec

Document for:	Discussion

# 1. Discussion

SBFD with CA

In the RAN1#120 meeting [1], RAN1 agreed that SBFD operation with one TDD carrier in multicarrier.

Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

In the last RAN2#131 meeting [2], RAN2 agreed SBFD with NR-DC

RAN2 evaluated the specification impact to support simultaneous configuration of SBFD and DC, and concluded the following: From RAN2 point of view there may be limited specification impact (i.e., stage-2 impact but no need to introduce stage-3 spec impact). Send LS to RAN4 and RAN1 (cc RAN3) to inform this conclusion.

During the last meeting, RAN2 has an agreement that configuration of SBFD and DC has impact on stage-2 specification. Similar to DC scenario, it seems certain that SBFD operation with CA has also impact on stage-2 specification and should be captured for clear description of SBFD operation.

However, according to the current stage-2 specification (i.e., 38.300) [3], SBFD with CA operation is not supported (i.e., not captured) which is not aligned with RAN1 agreement and understanding.

Proposal 1: Support SBFD operation with CA scenario for stage-2 specification.

Proposal 2: Adopt the changes to stage-2 38.300 specification provided in the appendix.

# 2. Conclusion

Proposal 1: Support SBFD operation with CA scenario for stage-2 specification.

Proposal 2: Adopt the changes to stage-2 38.300 specification provided in the appendix.

# 3. References

[1] Chairman’s notes, 3GPP TSG RAN WG1 #120.

[2] Chairman’s notes, 3GPP TSG RAN WG4 #131.

[3] [Post131][219][SBFD] CR for TS 38.300 (CATT)

[4] R2-2506820, Open issues in TS 38.300 on Rel-19 Evolution of NR duplex operation (SBFD)

# 4 Appendix – Proposed changes to 38.300

Start of changes

# SBFD

## X.1 General

Sub-Band Full Duplex (SBFD) operation is supported for a TDD carrier, enabling simultaneous downlink transmission and uplink reception at the gNB on their non-overlapping respective sub-bands. From UE perspective, full duplex is not supported. The configurations of cell-specific SBFD time and frequency resources are provided through SIB1 or dedicated signalling.

A UE can be semi-statically configured with SBFD sub-bands in downlink symbols and flexible symbols provided by tdd-UL-DL-ConfigurationCommon and this UE is referred to as an SBFD aware UE. The maximum number of UL sub-bands for SBFD operation in an SBFD symbol within a TDD carrier is one. The maximum number of DL sub-bands for SBFD operation in an SBFD symbol within a TDD carrier is two.

A UE can be configured with SBFD symbols within a TDD carrier in CA scenario.

In an SBFD symbol, except for cross-link interference measurements, a UE transmits or receives only in RBs that are both in the active UL BWP and in the UL sub-band, or both in the active DL BWP and the DL sub-band(s), respectively.

A UE can be configured to transmit or receive only in non-SBFD symbols, only in SBFD symbols, or across both SBFD symbols and non-SBFD symbols for multiple transmission or reception occasions.

End of change