3GPP TSG-RAN WG2 Meeting #131bis	R2-2507509

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.5.3

Source:	Nokia, Nokia Shanghai Bell

Title:	User Plane issues for NES

WID/SID:	Netw_Energy_NR_enh

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss the following open issues 1 and 2 below listed by MAC rapporteur related to user plane maintenance aspects on the objectives of the R19 NES WI (Issue 3 to be addressed by the rapporteur):

MAC Issue 1: RA-RNTI for PRACH adaptation (described in LS R1-2506587) MAC issue 2: whether "initialDownlinkBWP-RedCap" can be configured before SIB1 request procedure (e.g. acquired by UE earlier) and can be used for PDCCH monitoring. If so, can the network know that for a redcap UE performing an OD-SIB1 request. Further, the rapporteur proposed capturing a resolution for the following open issue part of the miscellaneous corrections rapporteur CR. Companies need not address this in their contribution. MAC Issue 3: Monitoring adap-SSBPeriodicityIndication-RNTI during C-DRX.

# 2	MAC Issue 1: RA-RNTI for PRACH adaptation

In the RAN2 LS R2-2504724, we indicated the RAN2 agreed assumption of f_id take into account both the legacy ROs and the additional ROs:

RAN2 assumes that f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs.

RAN1 reply LS (R1-2506587) pointed out that there is no proper place in PHY to capture the RA-RNTI for PRACH adaptation and requested us to capture it in MAC. In addition, it also indicated we should capture their agreement of additional PRACH resource is invalidated if it overlaps with legacy PRACH resource.

RAN1 thanks RAN2 for providing the LS on RA-RNTI for PRACH adaptation. From RAN1 perspective, there is no proper place in RAN1 specifications to capture the impact on f_id since f_id is not defined in RAN1 specifications. The index nRA in RAN1 specification is an index for physical signal generation and cannot be directly used for determination of f_id. RAN1 kindly requests RAN2 to take the above into account, as well as the below RAN1 agreements, with regards to the impact on f_id of valid RO in TS38.321: Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping - Note: the overlapped RO for legacy resource is not impacted - FFS: Clarification on configuration of legacy ROs Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported

Based on the reply LS, we should capture in MAC both aspects of f_id considering both legacy RO and additional RO, and also the additional RO is invalidated if it overlaps with legacy RO, i.e. the f_id should only calculate the valid ones.

Proposal 1: (MAC-1) Capture in MAC that “F_id for a valid non-overlapping additional PRACH occasion is the index of the valid additional PRACH occasion in the frequency domain plus the number of FDMed PRACH occasions apart from the additional PRACH occasions”.

TP for MAC:

The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as: RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier). F_id for a valid non-overlapping additional PRACH occasion is the index of the valid additional PRACH occasion in the frequency domain plus the number of FDMed PRACH occasions apart from the additional PRACH occasions.

# 3	MAC Issue 2: OD-SIB1 on redcap specific initial BWP

During the CR review, it was pointed out that the UE does not have initialDownlinkBWP-RedCap yet before it acquires SIB1, thus the following change in MAC should be removed (Sharp001):

1> if initialDownlinkBWP-RedCap is configured: 2> if the Random Access procedure was initiated for SI request or SIB1 request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier: 3> monitor the PDCCH on the BWP configured by initialDownlinkBWP. 2> else: 3> monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap. 1> else: 2> monitor the PDCCH on the BWP configured by initialDownlinkBWP.

While ERI005 pointed out the UE could also request SIB1 in the OD-SIB1 cell while camping on it. It was further questioned by Oppo whether redcap UE monitors initialDownlinkBWP and initialDownlinkBWP-RedCap depending on whether it has requested SIB1. We understood that should not be the case. Current text says if the RACH is for SI request for SIB1 request, then the UE monitors PDCCH on initialDownlinkBWP, which means SIB1 request on initialDownlinkBWP-RedCap is not supported as for legacy SI request. Thus, we see no issue with the change.

Proposal 2: (MAC-2) the change in MAC related to initialDownlinkBWP-RedCap can be kept.

# 4 Conclusion

MAC open issues are discussed in this contribution with the following proposals proposed:

Proposal 1: (MAC-1) Capture in MAC that “F_id for a valid non-overlapping additional PRACH occasion is the index of the valid additional PRACH occasion in the frequency domain plus the number of FDMed PRACH occasions apart from the additional PRACH occasions”.

Proposal 2: (MAC-2) the change in MAC related to initialDownlinkBWP-RedCap can be kept.