3GPP TSG-RAN WG2 Meeting #131bis	R2-2507516

# Prague, Czech Republic, 13 – 17 October 2025

Agenda item:		8.7.3

Source:	Xiaomi

Title:	Discussion on open issues of XR RLC AM enhancements

Document for:		Discussion and Decision

# Introduction

After RAN2#131 meeting, email discussion “[POST131][507][XR] Final 38.322 CR (vivo)” was held to further collect RLC open issues [1]. In this contribution, we discuss remaining XR RLC AM issues.

# Discussion

## [RLC-V01] Interraction between RLC and PDCP

The open issue is as follows:

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| vivo | When t-RxDiscard expires, the RLC entity stops waiting to receive the obsolete RLC SDUs, and the corresponding RLC SDUs will be positively acknowledged in the STATUS report. However, the PDCP t-Reordering may be still running and waiting for receiving the outdated SDUs from the RLC layer, this can lead to unnecessary delays in packet delivery. Therefore, the receiving RLC entity should indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: RLC-V01 |

The main issue of receiving RLC entity indicating the SN of obsolete RLC SDUs to the receiving PDCP entity is that there is no 1:1 relationship between PDCP SN and RLC SN due to:

- There are PDCP control PDUs, which do not have PDCP SNs, but have RLC SNs.

- For split bearer, a PDCP PDU can be transmitted via one of the associated RLC entities, therefore there is even less correspondence between PDCP SN and RLC SN.

Therefore even if receiving RLC entity indicates the SN of obsolete RLC SDUs to the receiving PDCP entity, the receiving PDCP entity does not know the corresponding PDCP SNs of those obsolete RLC SDUs.

On the other hand, with consistent setting of RLC timer t-RxDiscard and PDCP timer t-Reordering, it is expected that after expiry of t-RxDiscard, the timer t-Reordering will expire soon. In addition, SN gap reporting introduced in Rel-18 can also move the PDCP receiving window. Therefore there is almost no gain (in terms of reduction of latency of delivery of PDCP SDUs to upper layers) when introducing such internal interaction. It is therefore proposed to not consider such optimization.

Proposal 1: [RLC-V01] No enhancement is introduced for interaction between RLC and PDCP receiving entities when t-RxDiscard expires.

## [RLC-N02] Condition to stop timer t-PollRetransmit

The open issue is as follows:

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| Nokia | [N002] Below NOTE in clause 5.3.3.3 NOTE: When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit. First, we should clarify that the NOTE is for the case where stopReTxDiscadedSDU is configured because stopping/resetting based on discard indication is limited to configuration of stopReTxDiscardedSDU. Second, it seems RAN2 has not carefully discussed ‘NACK’ part in the NOTE. To us, it seems not reasonable to stop polling even when there is an unACKed RLC SDU still under transmission, i.e., waiting for ACK/NACK after retransmission or before discard. Our suggestion is: NOTE: If stopReTxDiscardedSDU is configured, Wwhen all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit. [Rapp]: This issue was discussed in RAN2#130 based on Apple’s proposal. The agreement includes the ACK/NACK/Discard cases. I understand the motivation in Apple’s contribution is that in the case where a NACK has just been received, there is no need for polling since the status is already known, and the retransmission can be performed directly. While the current Note in the specification also states “may stop and reset,” so I think this should not be a major issue. But considering it has not been extensively discussed during RAN2#130, let’s discuss it. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: RLC-N02 |

It is correct that the note is only applicable when stopReTxDiscardedSDU is configured. The main discussion point is whether to stop t-PollRetransmit when SN is negatively acknowledged. Timer t-PollRetransmit is defined as the timer “used by the transmitting side of an AM RLC entity in order to retransmit a poll”. After receiving negative acknowledgement, there is no need to poll again. TS 38.322 clause 5.3.3.3 already specifies that t-PollRetransmit is stopped when a negative acknowledge is received, as below:

Upon reception of a STATUS report from the receiving RLC AM entity the transmitting side of an AM RLC entity shall:

-	if the STATUS report comprises a positive or negative acknowledgement for the RLC SDU with sequence number equal to POLL_SN:

-	if t-PollRetransmit is running:

-	stop and reset t-PollRetransmit.

It is therefore proposed that in the related note, the “negatively” part is kept.

Proposal 2: [RLC-N02] For the note of clause 5.3.3.3, the condition “If stopReTxDiscardedSDU is configured” should be added. For the stop condition of t-PollRetransmit, no change is needed regarding “negatively acknowledged”.

# Conclusion

In this contribution, we discuss remaining XR RLC AM issues, and propose the following:

Proposal 1: [RLC-V01] No enhancement is introduced for interaction between RLC and PDCP receiving entities when t-RxDiscard expires.

Proposal 2: [RLC-N02] For the note of clause 5.3.3.3, the condition “If stopReTxDiscardedSDU is configured” should be added. For the stop condition of t-PollRetransmit, no change is needed regarding “negatively acknowledged”.

# References

[1] R2-2507017, vivo, "List of RLC open issues for R19 XR"