3GPP TSG-RAN WG2 Meeting #131bis	R2-2507517

# Prague, Czech Republic, 13 – 17 October 2025

Agenda item:		8.11.2

Source:	Xiaomi

Title:	Discussion on SBFD MAC open issues

Document for:		Discussion and Decision

# Introduction

After RAN2#131 meeting, email discussion “[Post131][221][SBFD] CR for TS 38.321 (Samsung)” was held to further collect MAC open issues [1]. In this contribution, we discuss SBFD MAC open issues.

# Discussion

## [MAC-2] preambleReceivedTargetPower for Msg3 Tx Power

The open issue is as follows:

MAC-2: How to clarify "the preambleReceivedTargetPower used in Msg3 transmission power derivation is that configured for legacy RO, in the case that Msg3 is transmitted in non-SBFD symbols and SBFD RACH config option 2 is configured."

This open issue is related to the RACH configuration associated with preambleReceivedTargetPower for Msg3. In RAN1#120bis meeting, it is agreed that preambleReceivedTargetPower configured for legacy-RO is used if Msg3 PUSCH is transmitted in non-SBFD symbols. Related part in MAC CR is:

5> else (i.e., the received UL grant indicates that the corresponding PUSCH transmission is in non-SBFD symbols as specified in clause 11.1 of TS 38.213 [6]):

6>	indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE).

During email discussion “[Post131][221][SBFD] CR for TS 38.321 (Samsung)” [1], all companies agreed that clarification is needed. The main difference is how to capture the associated RACH configuration, with the following two options:

- Option A: Negative statement ("preambleReceivedTargetPower not included in sbfd-RACH-DualConfig"), considering that it is difficult to explicitly indicate a commonly applicable parent IE for different contexts inclusively

- Option B: Positive statement ("preambleReceivedTargetPower included in yy"), for clearer and simpler UE implementation.

In our understanding, positive statement is preferred for clear guidance to UE implementation. Instead of specifying explicitly the IE names (which might be complicated as several IEs are related), it is sufficient to indicate that preambleReceivedTargetPower is associated with first PRACH occasions in this case.

Proposal 1: [MAC-2] If the received UL grant indicates that the corresponding PUSCH transmission is in non-SBFD symbols, preambleReceivedTargetPower associated with first PRACH occasions is indicated for Msg3 Tx power.

Corresponding change is shown below:

5> else (i.e., the received UL grant indicates that the corresponding PUSCH transmission is in non-SBFD symbols as specified in clause 11.1 of TS 38.213 [6]):

6>	indicate the preambleReceivedTargetPower (associated with first PRACH occasions) and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE).

## [MAC-3] UE transmit power continuity during RO type switching

The open issue is as follows:

MAC-3: Whether, and (if support) how, to support the compensation for the difference in preamble received target power between SBFD RO and legacy RO (in addition to the compensation for the power ramping difference that is already supported), for ensuring preamble transmit power continuity, during RO type switching.

The main motivation from proponent is that preambleReceivedTargetPower for SBFD RO and legacy RO can be different and should be compensated during RO type switching. It is indeed possible to configure different preambleReceivedTargetPower for SBFD RO and legacy RO. However it is expected that the difference is not significant as the configuration is determined by the interference level and length of the preamble (related to preamble format). With proper network configuration, even without compensating the difference in preamble received target power between SBFD RO and legacy RO, the preamble transmit power continuity can be still ensured.

Proposal 2: [MAC-3] During RO type switching, there is no need to compensate the difference in preamble received target power between SBFD RO and legacy RO. No specification change is needed.

## RO type indication for Enhanced LTM cell switch command MAC CE

In rapporteur CR of email discussion “[Post131][221][SBFD] CR for TS 38.321 (Samsung)” [2], the introduced RO field for Enhanced LTM cell switch command MAC CE is put in the same octet as UL TCI state ID, as below:

Figure 6.1.3.75a-1: Enhanced LTM Cell Switch Command MAC CE

However, UL TCI state ID field is optionally present, as highlighted from the description below:

-	UL TCI state ID: This field indicates and activates the uplink TCI state for the LTM target cell (i.e. the SpCell of the target configuration indicated by the Target Configuration ID field). The UL TCI state is identified by TCI-UL-StateId in ltm-UL-TCI-StateToAddModList as specified in TS 38.331 [5]. The octet containing this field (i.e. this field and the two reserved bits in the same octet) is included if the value of unifiedTCI-StateType in the ltm-TCI-Info of the configuration indicated by Target Configuration ID field is separate. The length of the field is 6 bits;

Therefore if the value of unifiedTCI-StateType in the ltm-TCI-Info of the configuration indicated by Target Configuration ID field is joint, the octet containing field UL TCI state ID is not present, and it is not possible to indicate RO type. It is proposed to use R bit in other octets, for example R bit in the octet containing field NCC value or field TCI state ID, which is always present.

Proposal 3: For RO type indication for Enhanced LTM cell switch command MAC CE, the R bit in the octet containing field NCC or field TCI state ID is used to indicate RO type.

An example of using R bit in the octet containing field NCC to indicate RO type is shown below.

# Conclusion

In this contribution, we discuss SBFD MAC open issues, and propose the following:

Proposal 1: [MAC-2] If the received UL grant indicates that the corresponding PUSCH transmission is in non-SBFD symbols, preambleReceivedTargetPower associated with first PRACH occasions is indicated for Msg3 Tx power.

Proposal 2: [MAC-3] During RO type switching, there is no need to compensate the difference in preamble received target power between SBFD RO and legacy RO. No specification change is needed.

Proposal 3: For RO type indication for Enhanced LTM cell switch command MAC CE, the R bit in the octet containing field NCC or field TCI state ID is used to indicate RO type.

# References

[1] R2-2507158, Samsung, "Summary of Rel-19 SBFD MAC open issue discussions for maintenance"

[2] R2-2507080, Samsung, "Correction on MAC spec for R19 SBFD"