3GPP TSG RAN WG2 Meeting #131bis	R2-2507557

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda Item:	8.2.2

Source:	Xiaomi

Title:	Remaining open issues for Rel-19 A-IoT

Document for:	Discussion and Decision

Introduction

Based on the guidance from the rapporteur, the following issues still need further discussion based on companies’ contributions, as per the open issue list in [1]:

-	Subgroup: Paging message content

  - Issue 1-7: Security parameter

  - Issue 1-3: Paging ID length field (related to incoming LS in S2-2507793)

-	Subgroup: D2R message content for data transmission

  - Issue 3-7: more data indication handling

-	Subgroup: Others

  - Issue 4-6: max NAS message size

In this contribution, we continue the discussion on these remaining open issues and provide corresponding proposals.

Discussion

Paging message content

- 2.1.1  	Issue 1-7: Security parameter

The situation on this open issue is cited as follows: [1]

| Issue 1-7: Security parameter | How to include the security parameters in Paging message. - Based on SA3 LS in S3-252392, it’s understood that security is a mandatory feature to be supported by the Rel-19 devices. But companies also raised question during online discussion whether the security parameters can be optional, for instance the feature can be activated/controlled by network. And companies also mentioned that the signaling overhead of the current paging message design is an important aspect from RAN2 point of view. Then how to capture the security parameter has been postponed and expected more discussion in Oct meeting. In this case, this issue is classified as “to be discussed by contribution”, and the rapporteur would like to suggest companies also to consider more from RAN2 perspective when designing signaling in their contributions, in terms of e.g., signaling compatibility, flexibility, signaling overhead, and scalability. - SA3 provided more information about the security design in LS S3-252933 in Aug meeting, which should be take into account. - Relevant agreements: - RAN2 thinks it is feasible from a signalling perspective to add the 128 bits. However, from RAN2 perspective the less overhead the better, so SA3 should avoid adding additional parameters if possible. - Indicate to SA3 that RAN2 tries to minimize number of bits required. Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG. - RAN2 will wait for SA3 conclusions in October on whether the “128bit random number in the paging request message” is always required or not. - Reply to SA3 (The LS is approved in R2-2506465) - RAN2 thinks it is feasible from a signalling perspective to add the 128 bits. However, from RAN2 perspective the less overhead the better, so SA3 should avoid adding additional parameters if possible. - Indicate to SA3 that RAN2 tries to minimize number of bits required. Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG. - Indicate space pressure from all the WG - Status in running CR: not captured yet | To be discussed by company contributions |
| --- | --- | --- |

Concerning the issue on whether the random number is optional or mandatory in the A-IoT paging message,, our preference is to make it optional from the UE vendor point of view, as the security operations now specified in [2] is rather complicated and leads to big burden for device implementation complexity. Among the rationales shown in the co-sourced paper in [3], there are the concerns on always opening the security functions especially due to the following two reasons from our side:

- Unnecessary security related operations performed by the device every time an A-IoT service is triggered.

As shown in [3], performing authentication for the already inventoried device seems not really necessary, if the device(s) were inventoried with authentication already performed before long.

- Unnecessary security related operation in the secured area/deployment without security risk.

Since the typical A-IoT deployment is to provide the area specific A-IoT services (e.g. inventory and read/write command to the devices within a given area), it should be regarded as the typical scenario that the deployment of the A-IoT is within areas that the security risk is kept away from fake device attack, especially in the indoor deployment which is only supported in Rel-19 D1T1.

As a matter of fact, in such deployment scenarios as in bullet 2 above, it is even possible to deploy the device not implementing the complicated security related operations, and this enables A-IoT device to lower its cost and is beneficial for the A-IoT commercialization.

Observation 1: Always including the random number field in A-IoT paging message will lead to unnecessary burden on device complexity, especially in some typical cases/deployments where enabling security operations to the device(s) is unnecessary.

We thus propose that RAN2 makes an attempt to see whether it is possible to change the random number field in A-IoT paging message into an optional field, and informs SA3 of this RAN2 preference for confirmation. Also, in case the random number in the A-IoT paging message is determined as optional, the A-IoT NAS layer then needs to determine whether to generate a response random number (i.e. RANDAIOT_d, in [2]) and includes it in the Inventory response (included in the Data SDU in R2D upper layer data transfer message), depending on whether the RANDAIOT_n in A-IoT paging message is present or not. Since the generation of RANDAIOT_d is the A-IoT NAS behaviour, CT1 also needs to be informed of this RAN2 preference to check whether any Spec impact is needed.

Proposal 1: (Issue 1-7) RAN2 agrees as a RAN2 preference that the random number field in A-IoT paging message is optionally present.

Proposal 1a: (Issue 1-7) If Proposal 1 is agreed, send LS to SA3 and CT1, informing them of the RAN2 preference and asking them to work on necessary specification changes (if any).

- 2.1.2 	 Issue 1-3: Paging ID length (w.r.t. SA2 LSin S2-2507793)

Current situation of this open issue is as follows: [1]

| Issue 1-3: Paging ID length field | The field to indicate the paging ID length, e.g., how many bits, taking into account of CT4 and SA2 inputs. - Relevant agreements: - 8-bit length field (in unit of bit) is assumed to indicate the paging ID length, based on current SA2/CT4 conclusion. - However, in Aug meeting, SA2 sent a new LS to RAN2 in S2-2507793 on AIoT Devicef Permanent ID Length, indicating SA2 identified requirements for longer device ID, and the rapporteur understand this will impact the length of paging ID as well, e.g., 600 bits are needed instead of 256 bits. Therefore, in Oct meeting, RAN2 needs to discuss how to reply the SA2 LS, and how to handle the impact to paging ID length field. - Status in running CR: the field name is captured in 6.2.1.1 with the detailed format. | To be discussed by company contributions |
| --- | --- | --- |

In LS S2-2507793, SA2 is asking the feasibility to enlarge the size of filtering information which corresponds to the paging ID to up to 600 bits, as follows:

To support a maximum paging ID field of around 600 bits, the Paging length field should be of 10 bits. Considering also the PIPI field needed, at least a room of approximately 611 bits needs to be left to accommodate the maximum 600-bit paging ID.

Based on the current A-IoT Paging message structure design, below are what we observe:

- The maximum Paging message size is 1000 bits, which corresponds to the value “125 bytes” of the R2D TBS field.

- The maximum size of the fields other than the paging related fields (i.e. Paging ID, Paging ID length and PIPI) is around 169 bits (assuming a 128-bit random number field being included with an optionality indicator).

Then the maximum room left for the paging ID related information is around 830 bits, which is larger than the size asked by SA2 to support the 600-bit filtering information.

Based on the above analyses, what is observed is that, in case no other information from other WG is to be further included into the paging message (e.g. no other security information than the random number), it is feasible to support the filtering information of around 600 bits as asked by SA2.

Therefore, RAN2 can reply to SA2 with this conclusion. In addition, due to the scarcity of the bits in A-IoT paging information, RAN2 also requests SA2 to minimize the filter information size whenever possible, similar to the request RAN2 made to SA3 for the security parameters in [4].

Proposal 2: (Issue 1-3) Reply to SA2 that the room available in A-IoT paging message to include the paging ID related information is around 830 bits. RAN2 thinks it is feasible to support the around 600 bit filtering information as asked by SA2 (if no other information into paging is introduced by other WGs), but would like to request SA2 to minimize the filtering information size whenever possible.

D2R message content for data transmission

- 2.2.1  	Issue 3-7: more data indication handling

As listed by Rapporteur, the issue is described as follows: [1]

| Issue 3-7: more data indication handling | How to set “more data indication” value in case of no NAS response available (i.e., zero SDU) - Relevant agreements: - The reader determines no data available case by SDU length 0. As more data indication is mandatory, the device sets this bit to "0". - The reader, in response to 0 SDU in the device’s MAC response, may send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device. - The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command. - RAN2 would like to check if there is a case where NAS doesn’t provide a response at all. If this case exists, RAN2 will discuss this issue. - Regarding the above check point, it can be up to companies’ contribution. - Status in running CR: captured. | To be discussed by company contributions |
| --- | --- | --- |

If one checks the CT1 specification for A-IoT [5], it is specified that the device shall ignore the NAS message, if the message type is unknown or unforeseen (see below citation from [5]):

- 6.3 Unknown or unforeseen message type If the AIoT device or the network receives a AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall ignore the AIoT NAS message. NOTE: A message type not defined for the AIoT NAS protocol in the given direction is regarded by the receiver as a message type not defined for the AIoT NAS protocol, see 3GPP TS 24.007 [3].

This means, there is anyway the case that the A-IoT NAS layer at the device will not provide any response after receives an A-IoT NAS message from the R2D upper layer data transfer.

Observation 2: According to CT1 spec, there are some cases where the device shall ignore the AIOT NAS message received, e.g., reception of an A-IoT NAS message with unknown message type.

Besides this case of Unknown/unforeseen message type, we further find another case, i.e. integrity check failure, for which whether there is NAS response is not clear. Specifically, according to the SA3 specification (see the citation below in [3]), if integrity check is successful, device will generate a command response.

5.3.2 Security procedure on information protection during command procedure Figure 5.3.2-1: Security procedure on the information protection during command procedure 1. The command procedure is initiated as specified in step 1-6 of clause 6.2.3 of TS 23.369 [2]. 2. The procedure as described in clause 5.2.2 shall be performed. The device and AIOTF acquire the KAIOTF key to be used for command protection. The derivation of KAIOTF key is specified in Annex A.3. 3. The AIOTF shall construct a AIOT NAS Command Request and protect the message based on the KCommand_enc, KCommand_int, the confidentiality and integrity algorithms for the AIoT device. The AIOTF shall send the protected Command Request containing an indication on whether cyphering is activated to NG-RAN. NOTE 1: The whole AIOT NAS Command Request message is integrity protected. If confidentiality algorithm is not null-scheme, the AIOT NAS Command Request message is partly ciphered with the exception that the selected protection algorithms are in clear text. 4. The NG-RAN shall send a R2D message containing the protected AIOT NAS Command Request as specified in as specified in TS 38.300 [3] and TS 38.391 [6]. 5. The device shall derive the KCommand_enc, KCommand_int and verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. The AIoT device shall construct a NAS Command Response and protect the message based on the KCommand_enc and KCommand_int key using the same algorithms. 6. The AIoT device shall send a D2R message containing the protected AIOT NAS Command Response to the NG-RAN as specified in as specified in TS 38.300 [3] and TS 38.391 [6]. 7. The NG-RAN shall forward the AIOT NAS Command Response containing the protected AIOT NAS Command Response to the AIOTF. 8-9. The AIOTF shall verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. Then, the AIOTF shall continue the procedure as specified in clause 6.2.3 of TS 23.369 [2]. NOTE 2: It is assumed that there is only one round of command procedure per device following an inventory procedure. Since the KAIOTF key is fresh, there is no need for additional freshness parameters for replay protection. NOTE 3: It is assumed that no new algorithms will ever be introduced for information protection during command procedure.

However, if the integrity check fails, whether/how to generate the command response needs to be further addressed. Actually, until now there is no clear guidance from SA3/CT1 on how A-IoT device should handle this case. Referring to legacy Uu, the handling of the integrity failure for AMF is to respond with a service reject message (see below citation from [6]). Thus, following the same logic, the A-IoT device can also reply with a service reject message if integrity check fails, which can be considered as a reject response.

4.4.4.3 Integrity checking of NAS signalling messages in the AMF […] If a SERVICE REQUEST or CONTROL PLANE SERVICE REQUEST message fails the integrity check and the UE has only non-emergency PDU sessions established, the AMF shall send the SERVICE REJECT message with 5GMM cause #9 "UE identity cannot be derived by the network" and keep the 5GMM-context and 5G NAS security context unchanged. For the case when the UE has an emergency PDU session and integrity check fails, the AMF may skip the authentication procedure even if no 5G NAS security context is available and proceed directly to the execution of the security mode control procedure as specified in subclause 5.4.2. Additionally, the AMF shall include the Additional 5G security information IE with the RINMR bit set to "Retransmission of the initial NAS message requested" in the SECURITY MODE COMMAND message as specified in subclause 5.4.2.2. After successful completion of the service request procedure, the network shall perform a local release of all non-emergency PDU sessions. The emergency PDU session shall not be released.

Observation 3: In NR Uu, in the case of integrity check failure of a service request, a service reject message is responded.

By contrast, one may also consider whether an A-IoT NAS message received with integrity check failure should be regarded as an unknown/unforeseen message type, thus to be necessarily handled by the device’s A-IoT NAS layer following the procedure in “6.3	Unknown or unforeseen message type” in [5] as cited above.

Based on the above analysis, there may be two alternatives to handle the integrity check failure case, i.e.,

- Alt.1: respond with a reject message (i.e. following NR Uu NAS design for Service Request)

- Alt.2: ignore the command and no response at all (i.e. following the AIoT NAS design on unknown message type handling)

Considering that the handling of NAS integrity check failure is out of RAN2 scope but it is relevant to the RAN2 open issue on how to cope with the “no NAS command response” case, we suggest checking with SA3 and CT1 on which way to go with, in order to further get clear whether the integrity check failure also belongs to the “no NAS response” case.

To sum up, it is proposed that RAN2 first confirms there is the case where no NAS response is available (at least for Unknown or unforeseen message type handling in A-IoT NAS), and further checks with CT1/SA3 on how to handle the integrity check failure case of a command message.

Proposal 3: (Issue 3-7) RAN2 confirms the understanding that command response is not always provided by A-IoT NAS at the device.

Proposal 3a: (Issue 3-7) RAN2 further checks with SA3/CT1 on how to handle the integrity check failure of an A-IoT NAS command message by the device:

- Alt.1: the device responds with a reject message.

- Alt.2: the device ignores the message and does not respond at all.

With the “no NAS response” case existing anyway, whether/how to indicate this information to the reader should be addressed by RAN2, since this may impact the reader behaviour on D2R scheduling. In our understanding, from resource efficiency perspective, even there is no NAS response at all, the device should still provide feedback to the reader and inform the reader about this situation. Otherwise, the reader may consider the device fails to receive the command or not ready for the response, and keep retransmitting the command or scheduling D2R resource for the response.

Observation 4: From resource efficiency perspective, even if there is no response from A-IoT NAS at all, the device should still respond to the reader and inform the reader of this situation.

Regarding how to response to the reader in this situation, 0 SDU should be included in the D2R Upper Layer Data transfer message. However, to differentiate this “no NAS response” case with the case of delayed NAS response concluded last meeting, additional information should be signalled to the reader as well. Considering that the main motivation to define MDI is for segmentation, to separate from the functionality of segmentation, the most straightforward solution is to define a new bit to indicate whether there is a response or not. In addition, if SA3/CT1 concludes that the device also ignores the NAS message in the case of integrity check failure, the device can also indicate the cause value when there is no command response at all, e.g., integrity failure, unknown message, etc. The reader can forward the cause value to CN for further differentiated handling.

Proposal 4: (Issue 3-7) RAN2 discusses the introduction of a new bit(s) to indicate whether there is a command response and/or indicate a cause value, when the D2R response includes 0 data SDU.

Others

- 2.3.1  	Issue 4-6: max NAS message size

As listed by Rapporteur, the issue is described as follows: [1]

| (New) Issue 4-6: CT1 LS on max NAS message size | In C1-255679 LS on the maximum supported AIoT NAS container length, CT1 asked about the maximum supported AIoT NAS container length in both R2D and D2R directions. - Background and relevant agreements: - In R1-2409250/R2-2409405 Reply LS to RAN2 on data block sizes for Ambient IoT, RAN1 indicated that from RAN1 perspective, a maximum TB size of around 1000 bits in PHY for R2D and D2R directions can be supported. Based on this, RAN2 design the SDU length and offset length as 7 bits, which is a sufficiently large value that can indicate the maximum integer number of bytes after subtracting the MAC overhead from 1000 bits. - The rapporteur understands that RAN2 can reply CT1 with the above situation, and there is no additional impact to the current MAC signaling format, | To be discussed by company contributions |
| --- | --- | --- |

We have similar understanding as the Rapporteur. From the AS perspective, a maximum TB size of around 1000 bits for R2D and D2R directions can be supported, which is the basis of the SDU length/offset length design. Then:

- For “R2D Upper layer Data Transfer” message, subtracting the MAC overhead, the maximum supported Data SDU size is 952 bits for R2D direction;

- For “D2R Upper layer Data Transfer” message, although the SDU length is specified in as 7 bits, the TB size indicated by RAN1 needs to be followed. Hence, subtracting the MAC overhead, the maximum supported Data SDU size is 991 bits for D2R direction.

It is proposed to provide above information as the reply to CT1.

Proposal 5: (Issue 4-6) RAN2 replies to CT1 that the maximum supported TB sizes in R2D and D2R directions are 952 bits and 991 bits, respectively.

Conclusion

Based on the above discussion, the observations and proposals in this paper are listed as follows:

Issue 1-7: Security parameter

Observation 1: Always including the random number field in A-IoT paging message will lead to unnecessary burden on device complexity, especially in some typical cases/deployments where enabling security operations to the device(s) is unnecessary.

Proposal 1: (Issue 1-7) RAN2 agrees as a RAN2 preference that the random number field in A-IoT paging message is optionally present.

Proposal 1a: (Issue 1-7) If Proposal 1 is agreed, send LS to SA3 and CT1, informing them of the RAN2 preference and asking them to work on necessary specification changes (if any).

Issue 1-3: Paging ID length (w.r.t. SA2 LS S2-2507793)

Proposal 2: (Issue 1-3) Reply to SA2 that the room available in A-IoT paging message to include the paging ID related information is around 830 bits. RAN2 thinks it is feasible to support the around 600 bit filtering information as asked by SA2 (if no other information into paging is introduced by other WGs), but would like to request SA2 to minimize the filtering information size whenever possible.

Issue 3-7: more data indication handling

Observation 2: According to CT1 spec, there are some cases where the device shall ignore the AIOT NAS message received, e.g., reception of an A-IoT NAS message with unknown message type.

Observation 3: In NR Uu, in the case of integrity check failure of a service request, a service reject message is responded.

Observation 4: From resource efficiency perspective, even if there is no response from A-IoT NAS at all, the device should still respond to the reader and inform the reader of this situation.

Proposal 3: (Issue 3-7) RAN2 confirms the understanding that command response is not always provided by A-IoT NAS at the device.

Proposal 3a: (Issue 3-7) RAN2 further checks with SA3/CT1 on how to handle the integrity check failure of an A-IoT NAS command message by the device:

- Alt.1: the device responds with a reject message.

- Alt.2: the device ignores the message and does not respond at all.

Proposal 4: (Issue 3-7) RAN2 discusses the introduction of a new bit(s) to indicate whether there is a command response and/or indicate a cause value, when the D2R response includes 0 data SDU.

Issue 4-6: max NAS message size

Proposal 5: (Issue 4-6) RAN2 replies to CT1 that the maximum supported TB sizes in R2D and D2R directions are 952 bits and 991 bits, respectively.

Reference

[1] R2-2507030 Summary of A-IoT MAC open issues (outcome of [POST131][021][AIoT] MAC spec)

[2] 3GPP TS 33.369 "NR; Ambient IoT Medium Access Control Protocol specification"

[3] R2-2506902 Issue 1-7 Security parameter in A-IoT paging

[4] R2-2506464 Reply LS on security parameter in paging request message.

[5] 3GPP TS 24.369 "Ambient IoT Non-Access-Stratum (AIoT NAS) protocol for 5G System (5GS)"

[6] 3GPP TS 24.501 "Non-Access-Stratum (NAS) protocol for 5G System (5GS) "