3GPP TSG-RAN-WG2 Meeting #131bis	R2-2507573

Prague, Czech Republic, Oct. 13th-17th 2025

Agenda item	:		8.6.3

Source	: 	Sharp

Title	: 	User Plane issues for CLTM and event triggered L1 MR

Document for	:	Discussion and Decision

Introduction

In this paper, we discuss User Plane open issues for Rel-19 Mobility suggested to be discussed [1].

Discussion

Inter-CU LTM related issues

MAC-V01

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| vivo | RACH based LTM with MIMO 2TA How to address the mismatch issue that the TCI state ID indicated in the LTM Cell Switch Command MAC CE and TA value in the RAR within the RACH-based LTM procedure are associated with different TAG? [Huawei] This is a Rel-18 issue, it was already discussed and change proposals were not pursued. We should not re-open this now. [Rapp] That is true. The conclusion in Rel-18 is leave this to NW implementation, and this is one possible way for Rel-19. But it is better to explicitly discuss it. | Issue Type: Not essential Not important How to address it: based on companies contribution Issue Number: MAC-V01 |

As Huawei and Rapp mentioned above, this issue was discussed in Rel-18 [2]. Then, we exam whether the Rel-18 solution can be applied to Rel-19.

During Rel-18, the potential issue that the TCI state ID indicated in the LTM Cell Switch Command (CSC) MAC CE and the Timing Advance (TA) value provided in the Random Access Response (RAR) are associated with different TAG was identified for RACH-based LTM procedures. Basically, gNB knows the TCI state ID indicated in the LTM CSC MAC CE. Thus, the target gNB can provide the Timing Advance (TA) value provided in the RAR associated with same TAG with the TCI state ID. Furthermore, if the mismatch occurs, gNB can recognize the mismatch based on the information, i.e., the TCI state ID and the TA value in the RAR. Then, RAN2 agreed that the gNB which recognized the mismatch can send a new PDCCH order to trigger new RACH procedure to resolve the mismatch issue. Thus, in Rel-18, RAN2 agreed to leave this matter to network implementation rather than introducing additional signaling or complexity in the specification.

Observation 1. (MAC-V01) In Rel-18, target gNB which recognized the mismatch based on the TCI state ID indicated in the LTM CSC MAC CE and TA value provided by RAR can resolve the mismatch issue by sending a new PDCCH order.

In MAC-V01, Inter-gNB LTM introduces additional considerations since the CSC is sent by the source gNB, while the RACH and TA adjustment occur at the target gNB. However, the fundamental principle remains unchanged: TCI state ID indicated in the LTM CSC MAC CE is sent to the target gNB via XnAP signalling, i.e., CELL SWITCH NOTIFICATION message [3]. Then, the target gNB can know the TCI state information. It means that the Rel-18 solution can be applied to Rel-19 inter-CU LTM.

Observation 2. (MAC-V01) In Rel-19, the target gNB knows TAG associated with the TCI state information indicated in the LTM CSC MAC CE via XnAP signalling, i.e., CELL SWITCH NOTIFICATION message, and TAG associated with the TA value provided by RAR.

Proposal 1. (MAC-V01) Rel-18 agreement and solution can be applied to Rel-19 inter-CU LTM, i.e., no spec impact is needed.

Event triggered L1 measurement related issues

MAC-O02

| OPPO | In 5.4.4, there are several note captured to clarify the SR/PUCCH resource handling for the cases that PUCCH resources are overlapped among the triggered SRs NOTE 1: Except for the cases specified in NOTE 3 below, the selection of which valid PUCCH resource for SR to signal SR on when the MAC entity has more than one overlapping valid PUCCH resource for the SR transmission occasion is left to UE implementation. NOTE 3: When the MAC entity has pending SR for SCell beam failure recovery and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery of a BFD-RS set) overlapping with PUCCH resource for SCell beam failure recovery for the SR transmission occasion, the MAC entity considers only the PUCCH resource for SCell beam failure recovery as valid. When the MAC entity has pending SR for beam failure recovery of a BFD-RS set of Serving Cell and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery) overlapping with PUCCH resource for beam failure recovery of that BFD-RS set for the SR transmission occasion, the MAC entity considers only the PUCCH resource for beam failure recovery of that BFD-RS set as valid. NOTE 6: When the MAC entity has PUCCH resource for pending SR for SCell beam failure recovery overlapping with PUCCH resource for pending SR for beam failure recovery of a BFD-RS set for the SR transmission occasion, it is up to UE implementation to select PUCCH resource for SCell beam failure recovery or PUCCH resource for beam failure recovery of a BFD-RS set. For pending SR for event triggered L1 MR MAC CE, similer text is needed for charlfication, especially for the case that PUCCH resource for pending SR for event triggered L1 MR MAC CE has overlapping with PUCCH resource for beam failure recovery. [Rapp]: As we have no conclusion on how to handle the case when pending SR for event triggered L1 MR MAC CE overlapping with other PUCCH resource, this issue is wroth to be discussed. | Issue Type: Not essential How to address it: Based on companies’ contribution. Issue Number: MAC-O02 |
| --- | --- | --- |

As mentioned by Rapp above, RAN2 need to discuss how to handle the case when pending SR for event triggered L1 MR MAC CE overlapping with other PUCCH resource. In RAN2#129 meeting, RAN2 made agreement that BFR and event-triggered L1 MR has same priority. And these two MAC CE may compete with each other if the pending SR for event-triggered L1 MR is overlapping with PUCCH resource for BFR.

In 5.4.4, it is noted that when the MAC entity has pending SR for BFR and one or more PUCCH resources overlapping with PUCCH resource for the SR, the MAC entity considers only the PUCCH resource for BFR SR as valid. However, the prioritization between pending SR for event-triggered MR and other PUCCH resources is not mentioned. Priority handling procedure between SRs for BFR and event-triggered MR is not also clear.

Observation 3. (MAC-O02) In the logical channel resource allocation, MAC CEs for Beam Failure Recovery (BFR) and event-triggered L1 MR have same priority.

Observation 4. (MAC-O02) The prioritization procedure for the case that the SR for event-triggered MR overlaps with SR for BFR is not captured in the spec.

Proposal 2. (MAC-O02) It’s up to UE implementation how to handle prioritization when SRs for event-triggered L1 MR and BFR overlap.

Proposal 3. (MAC-O02) In order to cope with the case that SR for event-triggered L1 MR MAC CE overlaps to some PUCCH resources, similer text is needed for charlfication.

MAC-V03

| vivo | Similar as L3 measurement reporting, if an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed from or modified in the current UE configuration, the MAC entity shall remove the measurement reporting entry for this ltm-CSI-ReportConfigId from the MR_LIST, stop the periodical reporting timer, and reset the associated information (e.g. timeToTrigger) for this ltm-CSI-ReportConfigId. [Huawei] Agree but this should be initiated from TS 38.331, as well as the execution of 5.x.3, see below. [Rapp] It is a possible way. Let’s discuss it. | Issue Type: Not essential but important How to address it: based on companies contribution Issue Number: MAC-V03 |
| --- | --- | --- |

In L3 measurement reporting procedure, when measObject or/and reportConfig is modified or removed, associated entries are removed and associated timers are reset. For L1 measurement reporting, same mechanism can be used i.e. when an ltm-CSI-ReportConfig (or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig) is modified or removed, MR entry in the MR_LIST and related information for this ltm-CSI-ReportConfigId should be reset. Similar to the RRC procedure (reset MR entry and related information), MAC procedure (reset MR entries and related information and stop related timers) should be triggered by RRC layer.

Proposal 4. (MAC-V03) If an ltm-CSI-ReportConfig (or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig) is modified or removed, RRC layer should trigger MAC layer to reset MR entry in the MR_LIST and related information for this ltm-CSI-ReportConfigId.

Conditional LTM related Issues

MAC-V04

There are some issues related to TAT handling for intra-CU CLTM.

| vivo | How to handle the CLTM TAT of target cell when receiving an LTM Cell switch Command MAC CE with valid TA? Which is related the previous MAC-14 open issue. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: MAC-V04 |
| --- | --- | --- |

For Rel-19 CLTM, the UE can be configured with CLTM TAT(s) associated with each CLTM candidate cell(s) and the UE can maintain TA value(s) associated with each CLTM candidate cell(s) to perform RACH-less CLTM. To deliver TA values, the source gNB can send CLTM TAC MAC CE after the UE perform random access procedure to the CLTM candidate cell triggered by PDCCH order. For Rel-18 LTM, the UE can receive a LTM Cell Switch Command (CSC) to trigger LTM, this LTM CSC can include TA value and UE can use this TA value to perform RACH-less LTM.

Rel-18 LTM and Rel-19 CLTM can be configured simultaneously, and this raises the issue of whether a valid TA included in the LTM cell switch command can be reflected to the configured CLTM TAT of the target cell.

In our view, reflecting TA value in LTM cell switch command to CLTM TAT is not so profitable. Even if this reflecting mechanism is not supported, the NW can send/update TA value by using CLTM TAC MAC CE.

Observation 5. (MAC-V04)There is already a procedure i.e. CLTM TAC MAC CE for maintaining TAs during CLTM.

Therefore, it is not necessary to use valid TA contained in LTM CSC when the UE executes the CLTM procedure. In addition, this mechanism complicates the procedure and not simple.

Proposal 5. (MAC-V04) When the UE receives LTM cell switch command including a TA value and this command indicate one LTM candidate cell, the UE does NOT maintain this TA value as a TA value for a candidate cell and does NOT start associated TAT for CLTM.

MAC-V05/MAC-K01/MAC-N01

| vivo | When current serving cell is an LTM candidate cell, whether UE maintain and update its CLTM TAT based on legacy Timing Advance Command MAC CE? [Huawei] The UE could be allowed to use the valid TA value it has for the serving cell, this could be a note. [Rapp]: This has not been concluded. Let’s discuss it. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: MAC-V05 |
| --- | --- | --- |
| ASUSTeK | If the source cell is an LTM candidate cell, upon Cell switch, does the UE store the valid TA and update its CLTM TAT of the source cell for possible RACH-less CLTM back to the source cell (e.g., if the CLTM TAT for the Source Cell is not running at the time upon Cell switch)? [Rapp]: This issue is same as above issue raised by vivo, i.e. MAC-V05. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: MAC-K01 |
| Nokia | Although the TAs of other candidates are maintained, the TA of the previous serving cell, which may also be a candidate, is discarded as it is maintained by the servingCell TAT and not the candidate TAT. For example, the corresponding candidate TAT may not have been started or it may have not been restarted upon reception of a serving cell TAC MAC CE. To address the issue: FFS whether to store the previous serving cell TA for the corresponding candidate and remaining TAT time in the CLTM TAT of the corresponding candidate upon cell switch or, as discussed above by vivo, maintain and update the cooresponding CLTM TAT based on reception of legacy TAC MAC CE for the serving cell. The former approach allows maintaining only one timer for the cell, the legacy TAT when it is serving and the candidate TAT when not. [Rapp]: This issue is same as above issue raised by vivo and ASUSTek, i.e. MAC-V05, and MAC-K01 | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: MAC-N01 |

The UE can be configured with max 8 candidate cells for CLTM and these candidate cells can include a current serving cell. In such a situation, there is an issue whether the UE stores the valid TA value for a CLTM candidate cell and updates its CLTM TAT of the source cell for possible RACH-less CLTM back to the source cell when the UE receives TAC MAC CE. As mentioned in Observation 3 part, the gNB can send CLTM TAC MAC CE to update a TA value and such additional CLTM TAT handling mechanism may complicate the procedure.

Proposal 6. (MAC-V05, MAC-K01, MAC-N01) When the UE receives a TAC MAC CE for a serving cell, UE does NOT store a valid TA value for a CLTM candidate cell and update its CLTM TAT of the source cell for possible RACH-less CLTM back to the source cell.

Conclusion

In this paper, we made the following observations and proposals:

Observation 1. (MAC-V01) In Rel-18, target gNB which recognized the mismatch based on the TCI state ID indicated in the LTM CSC MAC CE and TA value provided by RAR can resolve the mismatch issue by sending a new PDCCH order.

Observation 2. (MAC-V01) In Rel-19, the target gNB knows TAG associated with the TCI state information indicated in the LTM CSC MAC CE via XnAP signalling, i.e., CELL SWITCH NOTIFICATION message, and TAG associated with the TA value provided by RAR.

Observation 3. (MAC-O02) In the logical channel resource allocation, MAC CEs for Beam Failure Recovery (BFR) and event-triggered L1 MR have same priority.

Observation 4. (MAC-O02) The prioritization procedure for the case that the SR for event-triggered MR overlaps with SR for BFR is not captured in the spec.

Observation 5. (MAC-V04)There is already a procedure i.e. CLTM TAC MAC CE for maintaining TAs during CLTM.

Proposal 1. (MAC-V01) Rel-18 agreement and solution can be applied to Rel-19 inter-CU LTM, i.e., no spec impact is needed.

Proposal 2. (MAC-O02) It’s up to UE implementation how to handle prioritization when SRs for event-triggered L1 MR and BFR overlap.

Proposal 3. (MAC-O02) In order to cope with the case that SR for event-triggered L1 MR MAC CE overlaps to some PUCCH resources, similer text is needed for charlfication.

Proposal 4. (MAC-V03) If an ltm-CSI-ReportConfig (or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig) is modified or removed, RRC layer should trigger MAC layer to reset MR entry in the MR_LIST and related information for this ltm-CSI-ReportConfigId.

Proposal 5. (MAC-V04) When the UE receives LTM cell switch command including a TA value and this command indicate one LTM candidate cell, the UE does NOT maintain this TA value as a TA value for a candidate cell and does NOT start associated TAT for CLTM.

Proposal 6. (MAC-V05, MAC-K01, MAC-N01) When the UE receives a TAC MAC CE for a serving cell, UE does NOT store a valid TA value for a CLTM candidate cell and update its CLTM TAT of the source cell for possible RACH-less CLTM back to the source cell.

Reference

[1] R2-250xxxx_List of MAC open issue for R19 mobility_v14_Rapp2.docx

[2] R2-2407571, Report from session on V2X/SL, R18/19 MOB, and R19 NES

[3] TS38.423, v19.0.0, NG-RAN; Xn application protocol (XnAP)