3GPP TSG-RAN WG2 Meeting #131bis	R2-2507577

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	8.11.3

Source:	vivo

Title:	Discussion on RRC Remaining issues for SBFD

Document for:	Discussion and Decision

1	Introduction

In the post RAN2#131 email discussion on RRC CR for SBFD, some RIL issues are raised in [1], and we will discuss these issues in this contribution.

2	Discussion

2.1	C101/104-CBRA for BFR

At RAN2#129, we have reached the following agreements on SBFD RACH aspect.

On RO type signaling for CFRA 1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig. 2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated.

And this agreement has been reflected in the RRC CR, i.e., the field description of ra-OccasionType under BeamFailureRecoveryConfig and RACH-ConfigDedicated as follows [2].

ra-OccasionType Indicates the second PRACH occasions of CFRA to be used by a SBFD aware UE (see TS 38.213 [13], clause 8). If absent, indicates the first PRACH occasions to be used.

During the post RAN2#131 email discussion, one company mentioned that CBRA also needs to be considered for the cases mentioned above, besides CFRA. The reason is that when the CFRA resources for the BFR and HO are unavailable, the fallback to CBRA will be performed.

However, RAN2 didn’t discuss this scenario before. In our view, this scenario exists and makes sense. Given the following agreement achieved at RAN2#130 [3], i.e., the same RO type indicated in CFRA resource can be used for the CFRA resource when fallback from CFRA to CBRA is performed, the RO type indicated by ra-OccasionType under BeamFailureRecoveryConfig and RACH-ConfigDedicated can also be used for the fallback to CBRA.

Other agreements SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA.

During the email discussion, two alternatives are proposed to cover this fallback case:

- Alt 1, remove ‘of CFRA’ in the field description of ra-OccasionType

- Alt 2, RRC spec does not change, the RAN2 agreement ‘SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA’ should be captured/reflected somewhere in MAC spec.

- Alt 3, change the RRC field description to ‘Indicates the second PRACH occasions of CFRA or of the fallback CBRA to be used by a SBFD aware UE

In our view, all of the three alternatives are feasible to cover the missing scenario. Given that the initial RO type selection procedure in MAC is now separately described for the CFRA cases and CBRA cases, and it is clear in MAC spec. However, from RRC spec perspective, to make it more clearly, Alt 3 (i.e. the application scenario is explicitly described) can be adopted, since Alt 1 is just a high level description and Alt 2 does not cover the fallback CBRA case.

Proposal 1: RAN2 to confirm that the RO type indicated in BeamFailureRecoveryConfig and RACH-ConfigDedicated can be used to indicate the second PRACH occasions of the fallback CBRA to be used by a SBFD aware UE.

2.2	C701-AdditionalRACH-Config for SBFD

In the current RACH partitioning framework, when the 2-step RA resources and the 4-step RA resources are configured for the same feature combination, those two IEs are located in the same additionalRACH-Config IE, to simplify the UE implementation on identifying the RA parameters associated with the same feature combination.

During the RIL review, one company mentioned that there is similar case due to the introduction of SBFD RO, and we should discuss the case that when the non-SBFD resources and the SBFD resources are configured for the same feature combination, since the SBFD RO defined in sbfd-RACH-DualConfig can also be associated with the feature combination and located in additionalRACH-Config IE.

In our view, firstly, the case mentioned above exists and needs to discussed for optimization, to simplify the UE implementation complexity, we’d like to adopt the existing method, i.e., when different RACH resources are configured for the same feature combination, those IEs are located in the same additionalRACH-Config IE.

Secondly, given that SBFD RO is not supported for 2-step RA, the case only needs to be considered is that when the 4-step RA resources and SBFD RA resources are configured for the same feature combination.

As suggested by LGE during the post email discussion, we also propose to add the following red text into the field description of additionalRACH-ConfigList.

additionalRACH-ConfigList List of feature or feature combination-specific RACH configurations, i.e. the RACH configurations configured in addition to the one configured by rach-ConfigCommon and by msgA-ConfigCommon. The network associates all possible preambles of an additional RACH configuration to one or more feature(s) or feature combination(s). The network does not configure this list to have more than 16 entries. If both rach-ConfigCommon and msgA-ConfigCommon are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config. If both rach-ConfigCommon and sbfd-RACH-DualConfig are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config.

Proposal 2: When the SBFD RA resources and the 4-step RA resources are configured for the same feature combination, those two IEs are located in the same additionalRACH-Config IE.

3	Conclusion

Based on the discussion, we have the following proposals,

Proposal 1: RAN2 to confirm that the RO type indicated in BeamFailureRecoveryConfig and RACH-ConfigDedicated can be used to indicate the second PRACH occasions of the fallback CBRA to be used by a SBFD aware UE.

Proposal 2: When the SBFD RA resources and the 4-step RA resources are configured for the same feature combination, those two IEs are located in the same additionalRACH-Config IE.

4	Reference

[1] R2-2507001, WI SBFD ASN.1 Comments file, Huawei, HiSilicon

[2] R2-2506495, Introduction of Rel-19 Evolution of NR duplex operation (SBFD), Huawei, HiSilicon

[3] R2-2505002, Report of 3GPP TSG RAN WG2 meeting #130, St Julian's, Malta