3GPP TSG-RAN WG2 Meeting #131bis	R2-2507578

# Prague, Czech Republic, Oct 13th - 17th, 2025

Source: KDDI Corporation

Title:　 Considerations on Control Plane for 6GR

Agenda Item:	10.3.2

Document for:	Discussion

- Introduction

The new SID for “study on 6G Radio” was approved in RAN#108 meeting and was revised in RAN#109 meeting, Its objective is to develop 6G Radio (6GR) to meet the requirements of the next generation of RAN, including those related to the control plane. In this contribution, we would like to share our initial considerations on several aspects of the control plane design for 6GR, including RRC state modelling and system information acquisition, etc.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

- Discussion

  - General principles for 6GR RRC

The size of RRC messages in NR is generally about 1.5 times larger than equivalent messages in previous generations, even though some NR-specific information is included. Reducing the size of RRC messages can contribute to lower setup and handover delays, and also reduce the radio resources required for transmission.

Another concern is that the ASN.1 structure of RRC has become increasingly complex and difficult to understand. We would like to explore whether there is a way to design an RRC ASN.1 structure that is easier to use and interpret, both for implementation and specification purposes.

Proposal 1: RAN2 to aim for a simplified RRC message with reduced size.

  - RRC for 6GR

For RRC services and functions,

At minimum, the following basic functionalities can be considered

- Broadcast of system information,

- Paging initiated by 5GC or NG-RAN

- Establishment, maintenance and release of an RRC connection between UE and NG-RAN, including add/mod/rel of carrier aggregation

- Security function including key management

-Establishment, configuration, maintenance and release of signaling Radio Bearers(SRBs) and data radio beers(DRBs)

- Qos management, as Qos configuration is delivered to the UE via RRC

- UE measurement reporting and control of the reporting, due to the L3HO will still be required as a fallback mechanism, the RRC based UE measurement-related procedures will be needed.

-Detection and recovery from radio link failure due to the supportive of L3HO, as well as the unpredictable radio condition of the spectrum

-NAS message transfer to/from NAS from/to UE due to the fact that RRC serves as the signalling interface between the UE and the NW, enabling the delivery of messages such as, registration, authentication, etc.

Regarding to other functions, such as add/mod/release of Dual connectivity, these can be wait until further progress is made in the dual connectivity/dual stack-related discussion. Mobility-related functions will be discussed in section 10.4, there it can be revisited once a clearer picture of the 6G mobility mechanism.

Proposal 2 : The following functionalities can be considered as a start point for the RRC services and functions:

Broadcast of system information;Pagin;support add/mod/rel of carrier aggregation;support add/mod/rel of SRBs and DRBs;Qos management;UE measurement-related procedures;radio link failure;NAS message delivery

For RRC protocol states

In current NR specification, three RRC states are defined: RRC idle/inactive/connected. While RRC inactive was introduced to reduce signaling overhead and improve UE power efficiency, it has not been widely adopted in commercial networks. For 6GR, it is worth to re-evaluating the necessity of maintaining RRC_INACTIVE state. If a function does not contribute to the commercial network, it is suggested to consider whether to eliminate it to simplify the overall system design.

For RRC_IDLE: at least, PLMN selection, broadcast of system information, cell re-selection and paging, can be considered for initial support

For RRC_CONNECTED: at least, connection between the 6G core and 6G RAN, the cell information which the UE belongs to, Transfer of unicast data to/from the UE, network-controlled mobility including measurements, can be considered for initial support

Proposal 3: RAN2 re-evaluates the necessity on whether maintaining RRC_INACTIVE, considering the future contribution to commercial network.

Proposal 4: RAN2 consider the following basic protocol states for RRC_IDLE and RRC_CONNECTED for initial support.

RRC_IDLE: PLMN selection, Broadcast of system information, cell re-selection and paging

RRC_CONNECTED: Connection between 6G core and 6G RAN, the cell information which the UE belongs to, transfer of unicast data to/from the UE, and network-controlled mobility including measurements

System information for 6GR

System information will still be the information to provide basic information for initial access and for acquiring other system information. The detailed synchronization signal and system information design needs to be discussed and decided by RAN1, we would like to point out that in the current network, due to some reason, the RRM is primarily based on the SSB and the SSB periodicity is currently aligned across cells. Some CSI-like signal or CSI-like RRM might be designed for the future 6G networks, we would like companies to fully consider the flexibility design of SSB or SIB.

Proposal 5: The synchronization signal design, especially their periodicity, aims to have both flexibility and considering the 3GPP specification discussion cycle and the future implementation.

In 5G, the SIB1 structure evolved to include multiple feature specific cellBarred flags, such as,

cellBarred2RxXR

cellBarredATG

cellBarred-eRedCap1Rx

cellBarred-eRedcap2Rx

cellBarredFixedVSAT

cellBarredMobileVSAT

cellBarredNES

cellBarredNTN

cellBarredRedcap1Rx

cellBarredRedcap2Rx

all the above added incrementally as new features emerged. While the approach has served its purpose, it introduces scalability and maintainability as the number of features grows. For 6G, a unified and extensible structure that consider accommodate current and future access types and device categories would be helpful to reduce signaling overhead, simply decoding, and enable much more smooth evolution of the SIB1. Establishing such a design from the first release of 6G will ensure long-term flexibility and robustness of the system information signaling.

Proposal 6: RAN2 aims for a unified and extensible structure for the cellBarred-information within SIB1, if it is needed in 6G.

- 3. Conclusion

Proposal 1: RAN2 to aim for a simplified RRC message with reduced size.

Proposal 2 : The following functionalities can be considered as a start point for the RRC services and functions:

Broadcast of system information;Pagin;support add/mod/rel of carrier aggregation;support add/mod/rel of SRBs and DRBs;Qos management;UE measurement-related procedures;radio link failure;NAS message delivery

Proposal 3: RAN2 re-evaluates the necessity on whether maintaining RRC_INACTIVE, considering the future contribution to commercial network.

Proposal 4: RAN2 consider the following basic protocol states for RRC_IDLE and RRC_CONNECTED for initial support.

RRC_IDLE: PLMN selection, Broadcast of system information, cell re-selection and paging

RRC_CONNECTED: Connection between 6G core and 6G RAN, the cell information which the UE belongs to, transfer of unicast data to/from the UE, and network-controlled mobility including measurements.

Proposal 5: The synchronization signal design, especially their periodicity, aims to have both flexibility and considering the 3GPP specification discussion cycle and the future implementation.

Proposal 6: RAN2 aims for a unified and extensible structure for the cellBarred-information within SIB1, if it is needed in 6G.