3GPP TSG RAN WG2 #131	bis                                                                              R2-2507585

Prague , CZ, Oct 13th – 17th, 2025

Source:	CEWiT

Title:	Discussion on Topology 2 for Ambient IoT

Agenda Item:	9.2.2

Document for:	Discussion and Decision

- Introduction

In Rel-19, Device 1 specification for Deployment Scenario 1 with Topology 1 was completed. For Rel-20, RAN2 will begin work on Topology 2, where the UE functions as an intermediate node. In RAN#109 [1], RAN updated the guidance for RAN2’s work on Topology 2.

Objectives for RAN 2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling.

In this contribution, we discuss our views on aspects of D2T2 for A-IoT.

- Discussion

For Topology 2, SA2 has defined work task to support A-IoT services, focusing on RRC-based UE Reader connectivity. To align with SA2’s decision, the RAN plenary has updated the WID, specifying support for Deployment Scenario 2 with an intermediate UE as Reader. The work is limited to an RRC-based architecture, coordinated with SA2, while excluding UE reader coordination, or simultaneous Uu and A-IoT operation in the same band. Accordingly, RAN2’s role in Rel-20 is to focus only on RRC-based solutions and define the necessary RRC procedures for enabling A-IoT in Topology 2.

Proposal 1: RAN2’s work on Topology 2 should be centered on RRC-based architecture.

Topology 2 requires additional enhancements on the Uu interface to support A-IoT, particularly in terms of gNB-controlled radio resource allocation for UE readers. It has been agreed that resources are dedicated to a single UE reader, provided via dedicated signaling from the serving gNB, and that such resources are not valid across cells in this release. Since the Rel-19 normative work on Topology 2 was not completed, it should be reaffirmed in Rel-20 that cross-cell resource validity is not supported. A key issue still open is the granularity of resource allocation. Several options have been identified, ranging from per UE reader/service request to per transmission, per procedure, per transmission pair, per paging round, or per access round. For frequent A-IoT services (e.g., inventory use cases with many messages and large numbers of devices), allocating resources per transmission would create excessive signaling overhead on the Uu interface, as the UE Reader would need to request resources for each message. In such cases, allocating resources per UE reader is more efficient, as it reduces signaling and provides flexibility to manage frequent transmissions. Conversely, for light traffic scenarios (e.g., a few devices or command/response exchanges), per transmission or per pair allocation may be more resource-efficient.

Proposal 2: RAN2 to adopt per UE reader resource allocation as the baseline, with gNB assigning a resource set to each reader.

Proposal 3: RAN2 to evaluate additional granularities (e.g. per R2D/D2R transmission or per pair) based on the different use cases (e.g. command operations).

Two alternatives for A-IoT radio resource allocation requests were studied in Release 19 during study phase: (1)  BS allocates a set of A-IoT resources to the UE Reader in response to a request from the UE Reader, and/or  (2) BS allocates a set of A-IoT resources to the UE Reader based on a service request from the CN. The study concluded that in all cases the BS controls the resources used by the A-IoT interface between the device(s) and the UE reader, and assistance information (e.g., resource type, size etc.) may be provided to the BS to facilitate proper allocation. Both alternatives were found to be feasible and complementary: CN-based requests (Alt-2) are well suited for initial resource configuration upon service arrival, while UE reader-based requests (Alt-1) can efficiently support on-demand reconfiguration, such as adjusting or releasing resources based on dynamic conditions like collision status, D2R failures, or service completion. Therefore, it is proposed that RAN2 confirms support for both alternatives in Rel-20, with Alt-2 enabling initial setup and Alt-1 enabling flexible reconfiguration, ensuring coverage of a wide range of use cases.

Observation 1: Two alternatives for the radio resources were discussed in Rel-19:

- Alt 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or

- Alt 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN.

Proposal 4: RAN2 should support both alternatives for radio resource allocation, with the following applicability:

- Alt-1: Enables flexible on-demand reconfiguration, such as adjusting or releasing resources in response to dynamic conditions (e.g., collision events, D2R failures, or service completion).

- Alt-2: Provides an effective mechanism for initial resource configuration at the time of service establishment.

In Rel-19 study phase, resource allocation for Topology 2 was discussed under the assumption that the UE Reader operates in RRC_CONNECTED and receives its configuration via dedicated RRC signalling.

RAN2#128 agreements:

Reader UE performing AIOT operation in RRC_CONNECTED (down-selection or support of both options will be decided in the WI phase):

Option 1: UE receives resources in dedicated signalling.  The resources remain valid until the network releases them explicitly (understanding is this would be supported by RRC reconfiguration)

Option 2: UE receives resources in dedicated signalling along with a time period in which they can be used.  The resources remain valid for that time period.

TR 38.769 [2] recorded both options as feasible, with final down-selection deferred to the WI phase. In both cases, the gNB can still release resources explicitly, but how the gNB determines release remains open. It was observed that assistance information from the AIoTF and/or the UE Reader (e.g., resource release request, implicit or explicit indication of procedure completion, or service termination) could help the gNB decide when to release resources.

From a practical perspective, Option 1 is simpler and sufficient in many cases, as the network already knows when the UE reader is expected to complete a service (e.g., following an inventory or command request). In this approach, the BS configures the frequency domain resources (e.g., via RRC reconfiguration), and the UE Reader uses the allocated block until the service completes, at which point the network releases the resources. This avoids service interruption and unnecessary complexity. On the other hand, Option 2 introduces higher specification impact, as it requires defining timer configuration, start/stop conditions, and potentially additional assistance information, which may be challenging since service duration depends on device scheduling, A-IoT radio conditions, and reader implementation details. While more complex, Option 2 could still be useful in specific cases to optimize signalling overhead and resource utilization.

Proposal 5: Support both validity options, with Option 1 (explicit network release) as baseline and Option 2 (time-based validity) for further study in specific use cases.

UE readers differ in their capabilities like some support only Device 1, others support Device 2b, and some support both. Similarly, cells have different deployments some host only Device 1, some only Device 2b, and others both; in some cases the network provides the carrier wave, while in others the UE must generate it. To enable efficient and interoperable Ambient IoT deployments, it is important to standardize mechanisms for UE Readers to report their capabilities (e.g., supported device types, waveforms, carrier wave generation, traffic types) and for the network to communicate cell-specific configurations. This ensures proper resource allocation, prevents operational mismatches, and supports multi-vendor interoperability. Accordingly, RAN2 should consider specifying capability exchange procedures between UE Readers and the network to support flexible deployments.

Proposal 6: RAN2 should establish standardized mechanisms for UE Readers to report their capabilities and for the network to communicate cell-specific configurations.

- Conclusion

In this contribution, we provide our views we discuss our views on aspects of D2T2 for A-IoT.

The proposals and observations are summarized as follows.

Proposal 1: RAN2’s work on Topology 2 should be centered on RRC-based architecture.

Proposal 2: RAN2 to adopt per UE reader resource allocation as the baseline, with gNB assigning a resource set to each reader.

Proposal 3: RAN2 to evaluate additional granularities (e.g. per R2D/D2R transmission or per pair) based on the different use cases (e.g. command operations).

Observation 1: Two alternatives for the radio resources were discussed in Rel-19:

- Alt 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or

- Alt 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN.

Proposal 4: RAN2 should support both alternatives for radio resource allocation, with the following applicability:

- Alt-1: Enables flexible on-demand reconfiguration, such as adjusting or releasing resources in response to dynamic conditions (e.g., collision events, D2R failures, or service completion).

- Alt-2: Provides an effective mechanism for initial resource configuration at the time of service establishment.

Proposal 5: Support both validity options, with Option 1 (explicit network release) as baseline and Option 2 (time-based validity) for further study in specific use cases.

Proposal 6: RAN2 should establish standardized mechanisms for UE Readers to report their capabilities and for the network to communicate cell-specific configurations.

# References

[1]   RP-252894, Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2.

[2]   TR 38.769 V2.0.1 (2024-12), Study on solutions for ambient IoT (Internet of Things).